### DSL-100D

#### ADSL Modem PCI

#### Quick Installation Guide

First Edition (February, 2000)

6DSL100QML01

Printed In Taiwan

RECYCLABLE

### Introduction

This Quick Installation Guide gives step-by-step instructions for setting up the D-Link DSL-100D ADSL Modem PCI. For more detailed information about the DSL-100D, its components, making network connections and technical specifications, please refer to the User’s Guide.

#### Unpacking

Open the shipping cartons of your Internet server and carefully unpacks the contents.  The carton should contain the following items:

- One ADSL Modem PCI device.

- One DSL-100D software driver CD-ROM.

- User’s Guide.

- This Quick Installation Guide.

Inspect the device and all accompanying items.  If any item is damaged or missing, report the problem immediately to your dealer.

## Requirements

- D-Link recommends that your PC system meet the following minimum requirements to be able to use the DSL-100D ADSL PCI Modem Card:

- An IBM PC AT or compatible computer with a Pentium® 100Mhz or faster processor

- At least 6 megabytes of free hard disk space

- 32 megabytes or more of main memory (RAM)

- A 24x or faster CD-ROM drive

- Microsoft Windows 95 (OSR1 or OSR2), 98 (1st or 2nd edition), or NT 4.0 (Service Pack 3)

NOTE: You may need the Windows (95, 98 or NT) operating system CD during installation. If you do not have this CD, contact your PC vendor for details on where the Windows files are stored on your machine.

#### Driver support

| - Operating Systems | - RFC 1483 | - RFC 1577 | - RFC 2364 (PPPoA) |
| --- | --- | --- | --- |
| - Win95 – OSR1 & OSR2 | - Yes | - No | - Yes |
| - Win98 – 1st edition | - Yes | - No | - Yes |
| - Win98 – 2nd edition | - Yes | - Yes | - Yes |
| - NT 4.0 – Service Pack 3 | - Yes | - No | - Yes |

### Connecting the Card to the ADSL Line

- Connect the network cable to the RJ-11 port on the ADSL PCI modem card.

- Connect the other end of the telephone cable to your ADSL wall jack.

- Replace the PC cover and reconnect all cables.

## Installation and Initial Configuration

#### Installing the Hardware

- Shut down the system and turn it off.

- Unplug the system from the power source.

- Remove the cover of the system chassis.

- Touch the system chassis or connect the copper end of the anti-static wrist strap to a metal component of the chassis to provide grounding.

- Insert the modem into an empty PCI slot after removing the slot cover, as shown in Figure 1. (The “slot cover” is the metal strip that covers the opening in the back of the computer chassis into which the host modem’s external connector fits.)

Figure 1 Inserting the modem into the PCI Slot

- Press the modem firmly into the slot to ensure that it is properly seated. Secure the modem’s end bracket to the chassis with the bracket screw.

- Replace the chassis cover and apply power to the machine.

#### Installing the Software

The following sections provide the information necessary to complete the installation and setup of the ADSL PCI modem card. You will need to know the TCP/IP properties, including the PVC values (VPI and VCI), IP address, the gateway, and the domain server name, to complete the configuration of the ADSL PCI modem card.  Additionally, the installation CD-ROM or diskettes of your operating system are required for these sections.

#### Windows 98 (Example)

This section describes how to install the network driver when your PC is running Windows 98. There are two major revisions of Windows 98:

- Windows 98 original (henceforth referred to as the 1st edition)

- NOTE:	Before you install your ADSL PCI modem card, determine whether your operating system is either Win98, 1st edition, or Win98, 2nd edition. This information is necessary to ensure the ADSL network performs properly as Win98, 2nd edition supports advanced features which the 1st edition can not. If unsure of the operating system edition, please contact your PC dealer for this information.

Follow these steps:

- Confirm that you have already installed the ADSL PCI modem in your PC and then turn on the PC.

- Windows 98 should automatically detect the ADSL PCI modem card and will search for a driver for the card in the PC. The first Add New Hardware Wizard window will appear.

- Click “Next.”

- Select “Search for the best driver for your device. (Recommended).” on the second Add New Hardware Wizard window and then click “Next.”

- Insert the DSL-100D ADSL PCI Modem Card CD-ROM into your CD-ROM drive and select “Specify a location:” on the third Add New Hardware Wizard window. Enter the correct path name (e.g. “E:\RFC1483\Win98Com” for Windows 98 1st edition) and click “Next.”

- Windows should find the *.INF (w95dslh.inf for Windows 98 1st edition) file in the directory specified in the window above under “Location of the Driver:” (the file name may be too long too fit in the window). Click “Next” on the fourth Add New Hardware Wizard window to continue.

- Windows will start copying the drivers. The Windows 98 CD should be in your CD-ROM drive or easily accessible.

- The Service Options window will appear. Click “Next” for the recommended installation (and skip to step 11) or “Advanced” to change connection address settings.

- Example for RFC 1483, the default PVC values for Ethernet over ATM are: VPI=0 and VCI=35. If necessary, you can change these values on the window above. The range for VPI is 0 to 255 and the range for VCI is 0 to 65,535. Please consult your ADSL service provider for more detailed information. Click “OK” when you are finished modifying these settings or to accept the default values.

- NOTE:	Failure to set up the correct VPI and VCI values may cause the ADSL network to fail.

- Click “Next” on the Service Option window. You may see an Insert Disk window asking you to insert the disk labeled “Windows 98 1st Edition CD-ROM” into your drive if you have not already done this. Comply with this instruction and then click “Next.” Note that if you are storing your source code on your hard drive (that is, not using a CD-ROM), please enter the location of Windows 98 (e.g., “E:\RFC1483\Win98Com”) in the Copying Files window and then click “OK.”

- Windows has finished installing all the DSL-100D ADSL PCI Modem Card drivers as well as automatically installing an icon. Click “Finish” on the fifth Add New Hardware Wizard window to complete the installation.

- Click “Yes” on the System Settings Change window when prompted “Do you want to restart your computer now?”

- Please go to the section entitled Verifying the Driver Installation for Windows 98 and follow the instructions.

Additional help is available online at http://www.dlink.com, or through our offices listed at the back of the User’s Guide.