DES-3200-28P ver. C1

24 Port PoE 10/100Mbps + Power Over Ethernet +

2 10/100/1000BASE-T +

2 Combo 10/100/1000 BASE-T / 100/1000 SFP

DES-3200-52 ver. C1

48 Port 10/100Mbps + 2 100/1000 SFP +

2 Combo 10/100/1000 BASE-T / 100/1000 SFP

DES-3200-52P ver. C1

48 Port PoE 10/100Mbps Power Over Ethernet +

2 10/100/1000BASE-T +

2 Combo 10/100/1000 BASE-T / 100/1000 SFP

Layer 2 Management Switch

Version 1.0

Date: 2011/09/16

Revision History:

| Version | Date | Name | Content Revised |
| --- | --- | --- | --- |
| 1.00 | 2011/09/16 | Jessica Huang | First Edition |

Product Description

DES-3200-28P/52/52P aims to replace DES-3028P/52/52P to enhance product competitiveness. Compared to DES-3028P/52/52P, DES-3200-28P/52/52P have advanced Hardware and Software features such as 16K Mac address table, 6KV surge/lightning protection in Ethernet ports, 802.3at(PoE+) for DES-3200-28P/52P, IPv6 support.

# Product Specifications:

## Hardware Specifications

| General |  |
| --- | --- |
| Dimension | 19-inch, 1U Rack-mount size - DES-3200-28P: 441mm X 308mm X 44mm - DES-3200-52: 441mm X 308mm X 44mm - DES-3200-52P: 441mm X 380mm X 44mm |
| Weight | DES-3200-28P: 4.288 kg DES-3200-52: 4.030 kg DES-3200-52P: 6.043 kg |
| Operation Temperature | -5-50°C |
| Storage Temperature | -40-70°C |
| Operation Humidity | 10%-90% RH |
| Storage Humidity | 5% ~ 90% RH |
| Operation Altitude | Up to 2000 M |
| Heat Dissipation | DES-3200-28P: 855.696 BTUs per hour DES-3200-52: 113.90 BTUs per hour DES-3200-52P: 1608.156 BTUs per hour |
| Acoustic | DES-3200-28P: 37dB at low speed, 49.5dB at High speed DES-3200-52: 0dB at fan off, 32.8 dB at fan on DES-3200-52P: 39.2dB at low speed, 51.2dB at High speed |
| MTBF | DES-3200-28P: 216780 Hour DES-3200-52: 189396 Hour DES-3200-52P: 189,396 Hour |
| Default Baud Rate | 115,200 |
| Power Consumption | DES-3200-28P: Maximum power consumption: 250.78 Watts Standby power consumption: 24.4 Watts DES-3200-52: Maximum power consumption: 33.38 Watts Standby power consumption: 19.3 Watts DES-3200-52P: Maximum power consumption:471.6WWatts Standby power consumption: 31 Watts |
| Performance |  |
| Switching Capacity | DES-3200-28P: 12.8Gps DES-3200-52: 17.6Gps DES-3200-52P: 17.6Gps |
| 64-byte Max. Forwarding Rate | DES-3200-28P: 9.5 million packets per second DES-3200-52: 13.1 million packets per second DES-3200-52P: 13.1 million packets per second |
| Forwarding Mode | Store and Forward |
| Key Component |  |
| CPU | 400MHZ |
| SDRAM for CPU | 128M DDR2 |
| Flash Memory | 32MB |
| Packet Buffer | 1.5 MB |
| FAN | DES-3200-28P: smart fan 1. Fans are capable to automatically adjust speed based on system temperature. 2. Alert alarm if fan running at abnormal speed. 3. Default : High speed (7000~7500RPM) 4. When internal temperature detected by the sensor goes over38 ℃(external temperature may be around 33 ℃), the fan speed will automatically change to high speed (7000RPM~7500RPM)). 5. When internal temperature detected by the sensor goes below 34 ℃(external temperature may be around 30 ℃), the fan speed will automatically change to low speed (5000RPM). DES-3200-52: fan - Fans are capable to automatically turn on/off based on system temperature. 2. Alert alarm if fan running at abnormal speed. (Only support when FAN is on. If FAN is off, the system can't check FAN state) 3. Default : FAN on 4. When internal temperature detected by the sensor goes over 54℃(external temperature may be around 33℃), the fan speed will automatically turn on. 5. When internal temperature detected by the sensor goes below 38℃(external temperature may be around 30℃), the fan speed will automatically turn off. DES-3200-52P: smart fan 1. Fans are capable to automatically adjust speed based on system temperature. 2. Alert alarm if fan running at abnormal speed. 3. Default : High speed (7000~7500RPM) 4. When internal temperature detected by the sensor goes over 43℃(external temperature may be around 33℃), the fan speed will automatically change to high speed. (7000RPM~7500RPM).5. When internal temperature detected by the sensor goes below 37℃(external temperature may be around 30℃), the fan speed will automatically change to low speed (4000~5000RPM) |
| Power Features |  |
| Internal power supply | DES-3200-28P : 253W AC Input: 100 – 240 VAC, 50-60 Hz DES-3200-52: 40W AC Input: 100 – 240 VAC, 50-60 Hz DES-3200-52P : 589W AC Input: 100 – 240 VAC, 50-60 Hz |
| Green |  |
| RoHS | Compliant with RoHS 6 |
| Port Function |  |
| Console port | - RJ-45 interface for out-of-band CLI configuration |
| 10/100BASE-TX ports | 24 x 10/100BaseT PoE ports for DES-3200-28P 48 x 10/100BaseT ports for DES-3200-52 48 x 10/100BaseT PoE ports for DES-3200-52P Compliant to following standards: - IEEE 802.3 compliance - IEEE 802.3u compliance Support Half/Full-Duplex operations - IEEE 802.3x Flow Control support for Full-Duplex mode - Back Pressure for Half-Duplex mode - Head-of-line blocking prevention Support manual/auto MDI/MDIX configuration |
| 100/1000 Fiber Ports | Port 51 ~ 52 in DES-3200-52 Comply with following standards: - IEEE 802.3u compliance (Support 100M transceivers) - IEEE 802.3z compliance Supported SFP Transceivers: 1. DEM-210 (100Base-FX, Multi-mode, 2km) 2. DEM-211 (100Base-FX, Single-mode, 15km) 3. DEM-310GT (1000Base-LX, Single-mode, 10km) 4. DEM-311GT (1000ase-SX, Mutli-mode, 500m) 5. DEM-312GT2 (1000Base-SX, Multi-mode, 2km) 6. DEM-314GT (1000BASE-LHX, Single-mode, 50km) 7. DEM-315GT (1000BASE-ZX, Single-mode, 80km) 8. DGS-712 (1000Base-T 100m) Only supported 1000Mbps Mode (not flow control) Supported WDM transceiver 1. DEM-220T (100BASE-FX, Single-mode, 20km, TX-1550/RX-1310nm) 2. DEM-220R (100BASE-FX, Single-mode, 20km, TX-1310/RX-1550nm) 3. DEM-330T (1000BASE-LX, Single-mode, 10km, TX-1550/RX-1310nm) 4. DEM-330R (1000BASE-LX, Single-mode, 10km, TX-1310/RX-1550nm) 5. DEM-331T (1000BASE-LX, Single-mode, 40km, TX-1550/RX-1310nm) 6. DEM-331R (1000BASE-LX, Single-mode, 40km, TX-1310/RX-1550nm) |
| Combo ports | Support 10/100/1000BASE-T / 100/1000 SFP ports 1000Base-T ports compliant to following standards: - IEEE 802.3 compliance - IEEE 802.3u compliance - IEEE 802.3ab compliance Support Half/Full-Duplex operations - IEEE 802.3x Flow Control support for Full-Duplex mode - Back Pressure for Half-Duplex mode - Head-of-line blocking prevention Support manual/auto MDI/MDIX configuration SFP Transceivers Supported: - DEM-310GT (1000BASE-LX) - DEM-311GT (1000BASE-SX) - DEM-314GT (1000BASE-LHX) - DEM-315GT (1000BASE-ZX) - DEM-312GT2 (1000BASE-SX) - DEM-210 (100BASE-LX) - - DEM-211 (100BASE-LX) WDM transceiver Supported: - DEM-330T (TX-1550/RX-1310nm), up to 10km, Single-Mode - DEM-330R (TX-1310/RX-1550 nm), up to 10km, Single-Mode - DEM-331T (TX-1550/RX-1310 nm), up to 40km, Single-Mode - DEM-331R (TX-1310/RX-1550 nm), up to 40km, Single-Mode - DEM-220T (TX-1550/RX-1310 nm), up to 20km, Single-Mode - DEM-220R (TX-1310/RX-1550 nm), up to 20km, Single-Mode Compliant to following standards: - IEEE 802.3z compliance - IEEE 802.3u compliance |
| PoE Capability |  |
| PoE Capable Ports | DES-3200-28P: Port 1~24 DES-3200-52P: Port 1~48 |
| Maximum Power Budget | DES-3200-28P: - Port 1 ~ 4: 30W (Support PoE+) - Port 5-24: 15.4W (Support PoE) - Max 188 W PoE power budget per device DES-3200-52P: - Port 1~8:30W (Support PoE+) - Port 13-48: 15.4W (Support PoE) - Max 370W PoE power budget per device |
| PoE Specification | - Compliant with IEEE 802.3af/802.3at PoE standard. - Automatically discover the connection of PD device and immediately sends power to it - Auto disable port if the port current is over 350mA or short happens. - For 802.3af capable devices, provide the power following the classification below Class; Usage; Minimum power levels of output of PSE. 0; Default; 15.4W. 1; Optional; 4.0W. 2; Optional; 7.0W. 3; Optional; 15.4W. 4; Reserved; Treat as Class 0. - For 802.3at capable devices, provide power allocation of 0.1 watt granularity using LLDP method. Class; Usage; Minimum power levels of output of PSE. 0; Default; 15.4W. 1; Optional; 4.0W. 2; Optional; 7.0W. 3; Optional; 15.4W. 4; Optional; 15.4/30W. Note: class 4 is Ptype=Icable x V Port_PSE min Type1 = 15.4W Type2 = 30W - Follow the standard PSE pin-out standard of Alternative A which is sending out power over number 1,2,3,6 pins of 8 wires of Ethernet cable Pin; Alternative. 1; Negative Vport. 2; Negative Vport. 3; Positive Vport. 4. 5. 6; Positive Vport. 7. 8. |

## Software Features

| Features | Detailed Description | Standard | Release |
| --- | --- | --- | --- |
| Stackability |  |  |  |
| Virtual Stacking / Clustering | 1. Support D-Link Single IP Management v1.61 2. Use a single ip to manage the virtual stack with up to 32 D-Link xStack devices 3. Support IPv4 address |  | R4.00 |
|  | 4. Support D-Link Single IP Management v1.73 |  | TBD |
|  | 5. Support IPv6 address |  | TBD |
| L2 Features |  |  |  |
| MAC Address | 1. Support 16K MAC address table 2. Support 512 static MAC addresses 3. Support black hole MAC address 4. Support max 256 static L2 multicast addresses which share 2K addresses with both IGMP snooping groups and MLD snooping groups |  | R4.00 |
| Flow Control | 1. 802.3x when full duplex 2. Back Pressure when half duplex 3. Head-of-line blocking prevention |  | R4.00 |
| Jumbo Frame | Up to 12K Bytes |  | R4.00 |
| Spanning Tree | 1. Support 802.1D STP 2004 edition 2. Support 802.1w RSTP 3. Support 802.1s MSTP 4. Support 8 MSTP Instances 5. Support per port / VLAN / device BPDU filtering 6. Support Root Restriction(defined in 802.1Q-2005) |  | R4.00 |
|  | 7. Support 64 MSTP Instances |  | TBD |
| Ethernet Ring Protection Switching (ERPS) | - Support Single Ring ERPS | ITU G.8032 | R4.00 |
|  | - Multiple Ring ERPS, | TBD | TBD |
| Loopback Detection(LBD) | 1. D-Link Loopback Detection 4.03 (latest) 2. STP independent 3. Per port/VLAN shutdown when there is a loop detected. |  | R4.00 |
| Link Aggregation | 1. Comply with 802.1AX and 802.3ad 2. M ax group per device: DES-3200-28P : support max 14 groups per device DES-3200-52 : support max 26 groups per device DES-3200-52P : support max 26 groups per device 3. Support 8 ports per group 4. High availability LACP to prevent single or multiple ports failure in an aggregation channel. 5. Support load sharing mechanisms： - Source MAC, destination MAC, VLAN, EtherType, - Source IP, destination IP, source + destination IP - TCP/UDP ports | IEEE 802.3ad IEEE 802.3AX | R4.00 |
| Mirroring | 1. Port Mirroring - Support 1 mirroring group - One-to-one port mirroring - Many-to-one port mirroring - Port mirroring for Tx/Rx/Both 2. Flow-based (ACL) mirroring |  | R4.00 |
|  | 3. RSPAN |  | TBD |
| L2 Protocol Tunneling (L2PT) | Support tunnels for the following protocols across the service provider's networks - GVRP - STP - Protocol MAC Type: 01000CCCCCCC, 01000CCCCCCD |  | R4.00 |
| L2 Multicasting |  |  |  |
| L2 multicast table |  |  | R4.00 |
| L2 Multicast Filtering | Support L2 multicast filtering modes as below - Forward all groups - Forward all unregistered groups - Filter all unregistered groups |  | R4.00 |
| IGMP snooping | 1. Support IGMP v1, v2, v3 awareness 2. 1024 IGMP snooping groups (shared with MLD Snooping Group and static L2 multicast address) 3. Be able to forward the multicast traffic from the multicast-enabled router only 4. Be able to forbid multicast traffic on specific ports (can config forbid_mc_router) 5. Per VLAN IGMP snooping 6. Support port/host-based IGMP Snooping fast leave 7. Support Report Suppression. When IGMP report suppression is enabled, the switch sends the first IGMP report from all hosts for a group to all the multicast routers. The switch does NOT send remaining IGMP reports for the group to the multicast routers. This feature prevents duplicate reports from being sent to the multicast devices 8. Support Data Driven Learning so the switch will build up the entry in its multicast table when sniffing the multicast traffic from the media server. Typically used in an environment where the multicast server is connected to the switch directly. 9. Support the configuration of maximum allowed multicast groups. 10. Support static IGMP snooping groups 11.Support max 24 Profile ID 12. Support max 128 group Range Number in a Profile ID 13. Support max 24 profile per port 14. Support max 1024 multicast address per port | RFC1112 RFC2236 RFC3376 | R4.00 |
| IGMP Authentication | 1. Check the validity of client's MAC address, Switch IP , Port, and Multicast group IP 2. Only the authenticated port can join IGMP group |  | R4.00 |
| Limited IP Multicast (IGMP Filtering) | 1. Filter IGMP join packets 2. Support 24 profiles per system 3. Support port/VLAN based filtering. Each port/VLAN can bind up to 24 profiles independently. 4. Support up to 128 IP multicast group ranges per profile 5. Support IPv4 6. Support replace or drop action while exceed the max multicast group |  | R4.00 |
|  | 7. Support IPv6 |  | TBD |
| Proxy Reporting | 1. Proxy-reporting for multicast VLAN 2. Support IGMP/MLD proxy reporting |  | TBD |
| MLD snooping | 1. Support MLD v1, v2 awareness 2. 1024 MLD snooping groups. (shared with IGMP Snooping Group and static L2 multicast address) 3. Be able to forward the multicast traffic from the multicast-enabled router only 4. Be able to forbid multicast traffic to specific ports (can config forbid_mc_router) 5. Per VLAN MLD snooping 6. Support host-based MLD fast leave 7. Support Data Driven Learning so the switch will build up the entry in its multicast table when sniffing the multicast traffic from the media server. Typically used in an environment where the multicast server is connected to the switch directly. | RFC4541 | R4.00 |
| VLAN |  |  |  |
| VLAN Group | 1. Total 4094 VLAN groups; 2. Configurable vid from 1~4094 3. Configurable TPID |  | R4.00 |
| GVRP | 1. Support 255 dynamic VLAN groups 2. GVRP advertisement both on dynamic and static VLAN 3. GVRP can be enabled/disabled by per port/VLAN/system basis |  | R4.00 |
| 802.1Q | Support 802.1Q 2005 edition |  | R4.00 |
| Port-based VLAN | 1. Support VLAN tagging based on PVID 2. Support VLAN classification based on default port priority |  | R4.00 |
| 802.1v Protocol VLAN | 1. Support up to 16 protocol-based VLAN 2. Support 16 protocol-based VLAN per physical port 3. Be able to configure 802.1Q and 802.1v untagged port on same physical port 4. Support multiple VLANs for each protocol 5. Support VLAN classification based on ether type | RFC4363 | R4.00 |
| Double VLAN (Q in Q) | 1. Support Port-based Q-in-Q 2. Add SVLAN tag to either untagged or single tagged packets and generate double tagged packets 3. Support Selective Q-in-Q. Add SVLAN based on - MAC address - PVID (for untagged packets only) - CVLAN tag - 802.1p priority - IP Address - Ether type -IPv4/IPv6 Address - TCP/UDP port number 4. Support untagged / single tagged / double tagged at same physical port UNI Port:  For packets ingress from UNI port, the packet can be untagged, or CVLAN tagged  Based on user configuration, SVLAN tag can be added to the packet, or CVLAN tag be replaced with SVLAN tag  The learning and forwarding is based on SVLAN. NNI Port:  For packets ingress from NNI, it can be untag, or single SVLAN tag, or double tags. The switch's learning and forwarding is based on SVLAN. (CVLAN tag is identified by CVALN TPID. SVLAN tag is identified by SVLAN TPID.) 5. Insert CVLAN tag if the packet is recognized as without CVLAN tag. 6. Per port configurable SVLAN TPID 7. Per port configurable CVLAN TPID 8. Support 4 SVLAN TPID |  | R4.00 |
| VLAN Translation | 1. Single tag Replace VLAN tag based on - Physical port - ACL - Original VLAN tag 2. Double tag a. Support CVLAN translation based on - Physical port - ACL b. Support SVLAN translation based on - Physical port - ACL - CVLAN tag 3. Support up to 1024 entries/rules |  | R4.00 |
| Voice VLAN | Support D-Link Voice VLAN v2.0 |  | TBD |
| MAC-based VLAN | Support up to 1K entries |  | R4.00 |
| ISM-VLAN | 1. Support up to 3 dedicated ISM VLANs 2. Be able to filter unknown multicast packets per ISM VLAN 3. Support tagged and untagged members while the port still belongs to: - Port-based VLAN - 802.1Q VLAN whose VID is different from ISM VLAN 4. The IGMP snooping function in ISM VLAN is independent from global IGMP snooping 5. Support IGMP v1/v2/v3 6. Support up to 80 ISM profiles per system, each ISM VLAN can bind up to 16 profiles 7. Each ISM profile can support up to 32 address ranges 8. Source ports can be untagged 9.Support the modification of ISM VLAN's priority level for better traffic control 10. Support the configuration of multiple ISM VLAN source ports on the switch. When working with STP , it will allow the forwarding of multicast stream using backup 11. Configurable Multicast VLAN priority 12. Tag/Untagged source and member ports 13. Support the configuration of multiple ISM VLAN source ports |  | R4.00 |
|  | 14. Support ISM VLANv6 |  | TBD |
| Private VLAN |  |  | TBD |
| VLAN Trunking | Allow a physical network interface to carry and pass multiple tagged VLAN traffic so different VLAN information can be exchanged between switches |  | R4.00 |
| L3 Feature |  |  |  |
| ARP | 1. Max 1024 ARP entries 2. Support 255 static ARP entries 3. Support Gratuitous ARP 4. Support IPv4 Address |  | R4.00 |
|  | 5. Support IPv6 Address |  | TBD |
| IPv6 Neighbor Discovery (ND) | 1. Max 512 ND entries 2. Support up to 128 static ND entries |  | R4.00 |
| Default Route | Support IPv4/v6 address(Static route) |  | R4.00 |
| QoS (Quality Of Service) |  |  |  |
| Priority Queue Mapping | Be able to prioritize packets according to: - 802.1p Priority |  | R4.00 |
|  | - Differentiated Services Code Point (DSCP) | RFC2475 |  |
| Number of Queues | 8 Queues per port |  | R4.00 |
| Queue handling | Support following mode - Strict - Weighted Round Robin (WRR) - Strict + WRR |  | R4.00 |
|  | Support following mode - Deficit Round Robin (DRR) |  | TBD |
|  | - Configurable default 802.1p priority on each port - Scheduling mechanisms (strict or WRR) can be configured on a per port basis |  | R4.00 |
| Class of Service | Be able to classify packets according to following contents: - 802.1p priority - VLAN ID - MAC address - Ether type - IPv4/v6 address - TOS - DSCP - Protocol type - TCP/UDP port number - IPv6 Traffic Class - IPv6 flow label - User defined packet content | RFC2475, RFC2598 | R4.00 |
| QoS Flow Actions | Support following actions for flows - Remark 802.1p priority tag - Remark TOS/DSCP tag - Bandwidth Control | RFC2597 RFC2598 | R4.00 |
| Three Color Marker | 1. Support Two Rate Three Color Marker (trTCM) - CIR/PIR minimum granularity: 8 kbps - Support CBS/PBS 2. Support single rate three color marker (srTCM) - CIR minimum granularity: 8 kbps - Support CBS/EBS | RFC2697, RFC2698 | R4.00 |
| Congestion control | Support following congestion control mechanism: - Simple Random Early Detection (SRED) |  | TBD |
| Bandwidth Control | 1. Port-based bandwidth control - Minimum granularity: 8 Kb/s - Ingress / Egress bandwidth control 2. Flow-based bandwidth control - Minimum granularity: 8 Kb/s - Ingress / Egress bandwidth control 3. Per queue bandwidth control - Minimum granularity: 8 Kb/s - Per egress queue bandwidth control - Per egress queue bandwidth guarantee |  | R4.00 |
| Time-based QoS |  |  | R4.00 |
| ACL (Access Control List) |  |  |  |
| Maximum ACL Entries | 1. Max 4 ingress ACL profiles, 1024 ingress ACL rules in best case scenario 2. Each rule can be associated to a single port, multiple ports |  | R4.00 |
| MAC Access List | 1.Support maximum of 3 MAC ACLs 2. Support max 256 rules per MAC ACL 3. Support following packet contents: - 802.1p priority - VID - Source/Destination MAC address - Ethernet type |  | R4.0 |
| IP Access List | 1. Support maximum of 3 IP ACL 2. Support max 256 rules per IP ACL 3. Support standard and extended IP ACL configuration 4. Support following packet contents: - Source/Destination IP Address (Standard/extended) - DSCP (Extended) - Protocol type (Extended) - TCP/UDP port number (Extended) 5. Standard IP ACL is able to bind different protocol types. |  | R4.0 |
| IPv6 Access List | 1. Support 2 IPv6 ACL 2. Support max 256 rules per IPv6 ACL 3. Support following packet contents: - Source/Destination IP address - DSCP - Protocol type - TCP/UDP port number - IPv6 Traffic Class - IPv6 flow label |  | R4.0 |
| User-defined Access List | 1. Support 1 User-defined ACL 2. Support max 256 rules per User-defined ACL 3. Lookup any 16 Bytes among the first 128 bytes of packets. |  | R4.0 |
| ACL Policy | Support following ACL policy packet contents: - Switch port - MAC address - 802.1p priority - VLAN ID - TOS - DSCP - IPv4/v6 address - Protocol type - TCP/UDP port number - User Defined Packet Content - IPv6 traffic class: It can be available in profile type 3, 4, 5, 9, 10 (see Table1) - IPv6 flow label : It can be available in profile type 3, 4, 5 (see Table 1) <Table 1> Supported Profile Types Profile Type; Fields. 1; MAC DA, MAC SA, Ethernet Type, Outer VLAN Tag. 2; Outer VLAN Tag, Source IPv4, Destination IPv4, DSCP, Protocol ID, TCP/UDP Source Port, IGMP type, ICMP type/code, TCP flags, Outer VLAN Tag. 3; Source IPv6 Address, Class, Flow Label, IPv6 Protocol (Next Header). 4; Destination IPv6 Address, Class, Flow Label, IPv6 Protocol (Next Header). 5; Class, Flow Label, IPv6 Protocol (Next Header), TCP/UDP source port, ICMP type/code, Outer VLAN Tag. 6; MAC SA, Ethernet Type, Source IPv4/ARP sender IP, Outer VLAN Tag. 7; LLC Header/SNAP Header, Outer VLAN Tag. 8; LLC Header/SNAP Header, Outer VLAN Tag. 9; Source IPv6 Address, Class, IPv6 Protocol (Next Header), Outer VLAN Tag. 10; Source IPv6 Address, Class, IPv6 Protocol (Next Header), Outer VLAN Tag. |  | R4.00 |
| ACL Statistics | 1. Support max. 512 counters per system for ACL/QoS statistics 2. Support 29 bit counter |  | R4.00 |
| Time-based ACL | Support 64 instance of time_range table |  | R4.00 |
| CPU interface filtering | Support 5 profiles, 100 rule per profile |  | R4.00 |
| Security |  |  |  |
| SSH | 1. Support SSH v1, v2 2. Support IPv4 protocols 3. Allows the configuration of SSH port to any desired port number 4. Support 8 management session simultaneously |  | R4.00 |
|  | 5. Support IPv6 protocol |  | TBD |
| SSL | 1. Support v1/v2/v3 2. Support IPv4 protocol | RFC2246 | R4.00 |
|  | 3. Support IPv6 protocol |  | TBD |
| Port Security | 1.Support 64 MAC addresses per port 2.Support 3328 MAC addresses per device |  | R4.00 |
| Unicast/Broadcast/Multicast Storm Control | 1. Be able to set up a storm threshold. When broadcast / multicast / DLF unicast or multicast traffic hit the defined threshold, the switch will start blocking or discarding packets which would otherwise 'flood' the network. The blocked port will be activated again only when the broadcast/ multicast traffic falls below the defined threshold. 2. Min granularity: 8 kbps 3. Allows the configuration of 5 minutes recovery timer for shutdown ports |  | R4.00 |
| Traffic Segmentation |  |  | R4.00 |
| IP-MAC-Port Binding | 1. Support D-Link IMPB v3.9 2. Support 510 address binding entries per device 3. ARP mode - ARP Packet Inspection 4. ACL mode - ARP Packet Inspection - IP Packet Inspection 5. Support IPv4 DHCP Snooping |  | R4.00 |
|  | 6. IMPB supports IPv6 address 7. Support IPv6 DHCP Snooping 8. Support IPv6 ND Snooping |  | TBD |
| D-Link Safeguard Engine | 1. System will send the trap and log the event when CPU utilization hits the defined threshold 2. Protect CPU from Broadcast / Multicast / Unicast flooding 3. System will send the trap and log the event when CPU utilization hits the defined threshold 4. Protect CPU from Broadcast / Multicast / Unicast flooding |  | R4.00 |
| NetBIOS/NetBEUI filtering | Support IP & non-IP (802.3) mode (Default : IP mode) |  | R4.00 |
| DHCP Server Screening | 1.Support max. 100 Permit DHCP Server/Client entry 2.Support IPv4 address |  | R4.00 |
|  | 3. Support IPv6 address |  | TBD |
| DHCP client filtering | 1.Support IPv4 address |  | R4.00 |
|  | 2.Support IPv6 address |  | TBD |
| ARP Spoofing Prevention | - Support 256 entries - Support IPv4 address |  | R4.00 |
|  | - Support IPv6 address |  | TBD |
| BPDU Attack Protection |  |  | R4.00 |
| DoS Attack Prevention |  |  | R4.00 |
| L3 Control Packet Filtering | - Allow designation of a port or range of ports as being forbidden to multicast-enabled routers. Can sblock DVMRP, PIM, IGMP Querier, OSPF, RIP, or VRRP traffics. - Support IPv4 address |  | R4.00 |
|  | - Support IPv6 address |  | TBD |
| AAA |  |  |  |
| 802.1X | 1. Support local/RADIUS database - Maximum 448 MACs per port when using local database - Maximum 448 MACs per device when using local database - Maximum 128 entries can be added into local database 2. Support Port-based Access Control 3. Support Host-based Access Control 4. Be able to configure 802.1X packet transparency / filtering when 802.1X is disabled 5. Support EAP, OTP, TLS, TTLS and PEAP 6. Support MD5 authentication 7. Be able to force the 802.1X client to be off-line 8. Support 802.1X session timeout attribute 9. Identity driven policy assignment: Can assign VLAN, ingress/egress bandwidth control, ACL and 802.1p default priority to the port based on the attributes dispatched from the RADIUS server. 10. Support dynamic VLAN assignment by VLAN Name 11. Support max. 3 Radius Server | RFC3580 | R4.00 |
| Web-based Access Control (WAC) | 1. Support local/RADIUS database - Maximum TBD MACs per port when using local database - Maximum TBD MACs per device when using local database - Maximum TBD MACs per port when using RADIUS database - Maximum TBD MACs per device when using RADIUS database 2. Support logout and logout timer functions 3. Support Port-based authentication 4. Support Host-based authentication 5. Identity driven policy assignment: Can assign VLAN, ingress/egress bandwidth control, ACL and 802.1p default priority to the port according to the attributes dispatched from RADIUS server. 6. WAC 2.0 - Support HTTPS - Detailed port auth_state description |  | TBD |
|  | 7. Support IPv6 address |  | TBD |
| MAC-based Access Control (MAC) | 1. Support local/RADIUS database - Maximum 128 MACs per port when using local database - Maximum 128 MACs per device when using local database - Maximum 512 MACs per port when using RADIUS database - Maximum 512 MACs per device when using RADIUS database 2. Support Port-based authentication 3. Support Host-based authentication 4. Identity driven policy assignment: Can assign VLAN, ingress/egress bandwidth control, ACL and 802.1p default priority to the port according to the attributes dispatched from RADIUS server. |  | R4.00 |
| Guest VLAN | Support following authentication methods: - 802.1X - MAC-based Access Control |  | R4.00 |
| Microsoft® NAP | 1. Support IPv4 2. Support 802.1X NAP 3. Support DHCP NAP |  | R4.00 |
|  | 4. Support IPv6 |  | TBD |
| Authentication for Mgmt. Access | Support following authentication methods: - Local database - RADIUS - TACACS - XTACAS - TACACS+ | RFC2865, RFC2618 only for 1x RFC1492 | R4.00 |
| RADIUS | 1.Support IPv4 RADIUS Server | RFC2138, 2139, 2865, 2618 | R4.00 |
|  | 2. Support IPv6 RADIUS Server |  | R5.00 |
| TACACS |  | RFC1492 | R4.00 |
| XTACACS |  |  | R4.00 |
| TACACS+ |  | draft-grant-tacacs-02.txt | R4.00 |
| RADIUS Accounting | 1. Support Network accounting (for 802.1x user) 2. Support EXEC accounting (for Mgm Access) 3. Support System accounting (for Mgm Access) | RFC2866 | R4.00 |
| User Account Privilege for Mgmt Access | Support 4 level user account - User: allowed to execute ‘show’ and some troubleshooting commands (like ping) - Power user: allowed to execute all commands except the ones related to account management and service (telnet/SSH/Web/SNMP/RMON/sFlow etc.) enablement/disablement. - Operator: allowed to execute all commands except the ones related to account management - Administrator: can execute all switch commands. |  | R4.00 |
| Authentication Database failover | Allows to configure the switch to check local database or bypass authentication when configured RADIUS server fails - Support 802.1X, MAC |  | R4.00 |
|  | Allows to configure the switch to check local database or bypass authentication when configured RADIUS server fails - Support WAC |  | TBD |
| Trusted Host | 1. Support up to 10 trusted host IPv4 ranges 2. Be able to enable trusted host by application - Telnet - SNMP - HTTP - HTTPS - SSH - Ping |  | R4.00 |
|  | 2. Support up to 10 trusted host IPv6 ranges |  | TBD |
| OAM |  |  |  |
| Non-volatile configuration | The updated configuration will be automatically reloaded after system cold/warm restart |  | R4.00 |
| Factory Reset | Support factory reset function; reset the system to its default configuration |  | R4.00 |
| Loopback Diagnostics | Support loopback detection/diagnostics on the link and physical layers of each switch network interface |  | R4.00 |
| Cable Diagnostics | Support cable diagnostic function to check the status of connected RJ-45 cables. |  | R4.00 |
| 802.3ah Ethernet Link OAM | 1. Support 802.3ah link layer remote loopback and discovery 2. 802.3ah D-Link extension: D-link Unidirectional Link Detection (DULD) 3. Support following dying gasp PDUs and traps: - Power failure - Device reboot | IEEE 802.3ah | R4.00 |
| Connectivity Fault Management (CFM) | 1.Support 802.1ag 2. Support up to 8 MD, 64 MIPs and 64 MEPs 3. Support following functions for customer levels: - Per port and per VLAN basis MIP and MEP - LinkTrace and loopback reply for each MIP - Loopback Message (LBM) function that can generate LBM toward broadband access network - Loopback reply (LBR) function towards its peer MEP(s), in response to a unicast or multicast LBM 4. Support following functions for operator levels: - Per port and per VLAN basis MIP and MEP - Linktrace and loopback reply for each MIP - Be able to receive a LBM from its peer MEPs and initiate the associated LBR, for the MEP on the network interface - Be able to receive a LinkTrace Message (LTM) from its peer MEP and initiate the associated LinkTrace Reply (LTR) for the MEP on the network interface - Be able to receive a Continuity Check Message (CCM) from its peer MEPs and initiate the CCM, for the MEP on the network interface - Be able to initiate the LBM, LTM and CCM on a per-network interface/per-VLAN basis | IEEE 802.1ag | R4.00 |
| Y.1731 Ethernet OAM | Support following Y.1731 functions - Continuity Check Message (CCM) - Unicast/Multicast Loopback Message (LBM), Loopback Reply (LBR) - Link Trace Message (LTM), LinkTrace Reply (LTR) | ITU Y.1731 | TBD |
| Optical Transceiver Digital Diagnostic Monitoring (DDM) | 1. Comply with SFF-8472 real time access to optical transceiver operating parameters 2. Support configurable warning and auto shut down threshold for following events - Internally measured transceiver temperature - Internally measured transceiver supply voltage - Measured TX bias current in uA - Measured TX output power in mW - Measured RX received optical power in mW 3. Support trap/log for both thresholds | SFF-8472 | R4.00 |
| Port Status Monitoring | Support the following port status display functions - Administration and operation status - Source MAC address information, - Display Tx/Rx traffic counter - The link state, full duplex or half duplex communication mode, and automatic negotiation function enabled or disabled |  | R4.00 |
| Device Status Monitoring | Support the following equipment status display functions - Plug-in module status - System loading display (CPU, Memory, Fan, power…), CPU utilization ratio and memory usage ratio - System Resource display (Scheduling, Queue, Policy....) |  | R4.00 |
| Traffic Monitoring | 1. Support the following traffic information display functions on a per-VLAN basis. - Packet count - Byte count - Byte distribution of ingress subscriber interfaces - Broadcast, multicast and unicast count - Transmission rate of ingress subscriber interface |  | R4.00 |
| IGMP Statistics | Support IGMP statistics, including: - IGMP join, IGMP leave, and current number of active multicast groups per port and per ISM VLAN - Current number of active join ports per multicast group |  | R4.00 |
| Switch Performance Monitoring | 1. Support following performance monitoring: - Error/Unknown packets - Discarded packets - Unicast/Multicast/ Broadcast packets - CPU utilization ratio and memory usage ratio |  | R4.00 |
|  | 2. The monitoring interval of performance counters can be set between 15 minutes to 24 hours. The monitoring counters can be used to provide the performance information of network interfaces. 3. Support minimum granularity 30secs for 15-minutes, 30-mins for 24-hour counter 4. Use local database to store the historical performance data 5. Be able to configure the warning threshold so as to trigger the performance event traps |  | TBD |
| Green |  |  |  |
| Advanced Power Saving | 1. Power saving by Time-based PoE - PoE ports can be turned on/off by port or system through schedule or admin control (enable/disable forever) via GUI or command. |  | R4.00 |
|  | 2. Power Saving by LED Shut-Off: - Powered LEDs can be turned on/off by port or system through schedule or admin control (enable/disable forever) via GUI or command. 3. Power Saving by Port Shut-Off: - Each port on the system can be turned on/off by schedule or admin control (enable/disable forever) via GUI or command. 4. Power Saving by System Hibernation: - System enters hibernation by schedule or admin control (enable/disable forever) via GUI and command. In this mode, switches get most power-saving figures since main chipsets (both MAC and PHY) are disabled for all ports, and energy required to power the CPU is minimal. The port can be turned on via command. |  | TBD |
| IPv6 |  |  |  |
| IPv6 Neighbor Discovery |  | RFC2461, 4861 | R4.00 |
| Default Route |  |  | R4.00 |
| ICMPv6 | Include IPv6 Ping and IPv6 traceroute |  | R4.00 |
| DHCPv6 Client |  |  | TBD |
| DHCPv6 Relay Agent |  |  | TBD |
| IPv6 Ready Management |  |  | R4.00 |
| IPv6 ACL/QoS |  |  | R4.00 |
| IPv4/v6 Dual Stack |  | RFC2893, 4213 | R4.00 |
| DHCPv6 / NDP Snooping |  |  | TBD |
| Management |  |  |  |
| Web-based GUI | 1. Compatible with following Web Browsers: - IE5.5 or later versions (IPv4) - Netscape (IPv4) - Mozilla (IPv4) - Firefox (IPv4) - Safari (IPv4) 2. Support max 8 management session simultaneously (shared with CLI) | RFC2068 RFC2616 | R4.00 |
|  | 3. Compatible with following Web Browsers: - IE5.5 or later versions (IPv6) - Netscape (IPv6) - Mozilla (IPv6) - Firefox (IPv6) - Safari (IPv6) - Chrome (IPv4/v6) |  | TBD |
| CLI | - Support Multiple Commands in one line - Support max 8 management session simultaneously (shared with Web-based GUI) |  | R4.00 |
| Telnet Server | 1.Support IPv4 2.Support 8 management session simultaneously | RFC854 | R4.00 |
|  | Support IPv6 |  | TBD |
| Telnet Client | Support IPv4 |  | R4.00 |
|  | Support IPv6 |  | TBD |
| TFTP client | - Support IPv4 - Pre-config TFTP Setting | RFC783 RFC1350 | R4.00 |
|  | Support IPv6 |  | TBD |
| FTP client | 1.Support IPv4 |  | R4.00 |
|  | 2.Support IPv6 |  | TBD |
| Remote Copy Protocol (RCP) | 1.Support IPv4/v6 address |  | TBD |
| ZModem |  |  | R4.00 |
| Command logging | Command logging with account message |  | R4.00 |
| Greeting Message | Support 24 lines, each line support 80 bytes |  | R4.00 |
| Debug Command | Support “Show tech support” |  | R4.00 |
| SNMP | SNMP v1/v2c/v3 | RFC1157, RFC1901, RFC1908, RFC2570, RFC2574, RFC2575 | R4.00 |
|  | SNMP over IPv6 |  | TBD |
| SNMP Trap | SNMP supports following trap events: - New MAC Notification - Power and fan errors/recovery - Cold / Warm start-up - Link state change - STP state change - SNMP authentication failure - Mgm access violation - IP-MAC-Port Binding access violation - Port Security violation - Loopback occurring and recovery - DHCP Server Screening - Gratuitous ARP - While the firmware upgrade via SNMP finishesb |  | R4.00 |
| System Log | 1. Support IPv4 log server 2. Support WAC/MAC log information 3. Support the log for storm control block mode 4. Support Max 4 Management Host | RFC3164 | R4.00 |
|  | 5. Support IPv6 log server |  | TBD |
| Trap/ Alarm/ Log Severity Control | 1. Be able to divide the events into 3 severity levels and trigger corresponding logging activities based on the defined severity levels. 2. Support 8-level severity log |  | R4.00 |
| SMTP | 1. Transmitting critical events to an e-mail address 2. Support IPv4 address 3. Support the following events - System start(reboot) - Port link up or link down - SNMP Authentication Failure - Config or log saving by user - Config reset by user - TFTP update fw status | RFC2821 | R4.00 |
|  | 4. Support IPv6 address |  | TBD |
| RMON v1 | 1.Support 4 groups | RFC2819 | R4.00 |
|  | 2.Support IPv6 Address |  | TBD |
| RMON v2 | 1.Support Probeconfig group |  | R4.00 |
|  | 2.Support IPv6 Address |  | TBD |
| LLDP | 1.Support 802.1AB |  | R4.00 |
|  | 2.LLDP-MED(Media Endpoint Discovery) |  | TBD |
| BootP/DHCP client | - Support IPv4 Address | RFC951 RFC1542 RFC2131 | R4.00 |
|  | - Support IPv6 Address |  | TBD |
| DHCP Auto-configuration |  |  | R4.00 |
| DHCP relay | - Support IPv4 DHCP relay - Support keep, replace and drop action for option82 field in IPv4 DHCP relay - Support DHCP Relay option 82 |  | R4.00 |
|  | - Support DHCP local relay function that can insert option 82 - Support keep, replace and drop action for option82 field in IPv4 DHCP local relay | RFC3046 | R4.00 |
|  | - Support DHCP Relay Per VLAN |  | R4.00 |
|  | - Support IPv6 DHCP relay(support Basic DHCPv6 Relay functions only) |  | TBD |
|  | - Support IPv6 DHCP Local relay(TBD) |  | TBD |
| DHCP option 12 |  |  | R4.00 |
| PPPoE Circuit Insertion | Insert/delete Circuit ID tag to/from PPPoE packet |  | R4.00 |
| Flash file system |  |  | R4.00 |
| Port Description |  |  | R4.00 |
| Editable Login Banner |  |  | R4.00 |
| Editable System Prompt |  |  | R4.00 |
| CPU Monitoring | Be able to monitor the utilization of CPU via Web/ CLI/ SNMP |  | R4.00 |
| Memory Monitoring | Be able to monitor the utilization of Memory via Web/ CLI/ SNMP |  | R4.00 |
| DNS Relay | Support attributes: - IP, Domain (relay-only, static, cache mode) |  | TBD |
|  | - MX, NS, CNAME, AAAA, TXT, SOA, ANY (relay-only) |  | TBD |
| Debug Command | Support “Show tech support” |  | R4.00 |
| Time Setting | - SNTP IPv4 - Support max 2 SNTP Server | RFC1769 (SNTP) | R4.00 |
|  | - SNTP IPv6 |  | TBD |
| Password Recovery | - Support enable/disable password recovery function - CLI Command for password recovery |  | R4.00 |
| ICMPv6 |  |  | R4.00 |
| Password Encryption |  |  | R4.00 |
| MTU Setting | Allow the ability to modify MTU settings | RFC1191 | TBD |
| Microsoft® NLB (Network Load Balancing) Support | 1. Used to support Microsoft server load balancing application where multiple servers can share the same IP and MAC address. The request from clients will be forwarded to all servers, but will only be processed by one of them. 2. Support IPv4 NLB |  | R4.00 |
|  | 3. Support IPv6 NLB |  | TBD |
| Ping | Support IPv4/v6 ping |  | R4.00 |
| Traceroute | 1. Support IPv4 |  | R4.00 |
|  | 2. Support IPv6 |  | R4.00 |
| Multiple IP Interface | Multiple IP Interface support for debug usage via ping/traceroute/telnet/SSH/WEB/TFTP |  | R4.00 |
| Standard MIB |  |  |  |
| MIB Structure |  | RFC1065, RFC1066, RFC1155, RFC1156, RFC2578 | R4.00 |
| Concise MIB Definitions |  | RFC1212 | R4.00 |
| MIBII |  | RFC1213 | R4.00 |
| MIB Traps Convention |  | RFC1215 | R4.00 |
| Bridge MIB |  | RFC1493,4188 | R4.00 |
| SNMP MIB |  | RFC1157, 2571-2576 | R4.00 |
| SNMPv2 MIB |  | RFC 1901-1908, 3418, 3636, 1442, 2578 | R4.00 |
| RMON MIB |  | RFC271, 1757, 2819 | R4.00 |
| RMONv2 MIB |  | RFC2021 | R4.00 |
| Ether-like MIB |  | RFC1398, 1643, 1650, 2358, 2665, 3635 | R4.00 |
| 802.1p MIB |  | RFC2674,4363 | R4.00 |
| IF MIB |  | RFC2233, 2863 | R4.00 |
| RADIUS Authentication Client MIB |  | RFC2618 | R4.00 |
| MIB for TCP |  | RFC4022 | R4.00 |
| MIB for UDP |  | RFC4113 | R4.00 |
| MIB for Diffserv. |  | RFC3298 | R4.00 |
| 802.3 MAU MIB |  | RFC2668 | R4.00 |
| RADIUS Accounting Client MIB |  | RFC2620 | R4.00 |
| Ping & Traceroute MIB | Can configure the frequency of ping | RFC2925 | R4.00 |
| Running configuration writes and backup |  |  | R4.00 |
| TFTP uploads and downloads |  |  | R4.00 |
| Trap MIB |  |  | R4.00 |
| IPv6 MIB |  | RFC2465 | R4.00 |
| ICMPv6 MIB |  | RFC2466 | R4.00 |
| IPv6 SNMP Mgmt Interface MIB |  | RFC4293 | R4.00 |
| DDM MIB |  |  | R4.00 |
| Private MIB |  |  | R4.00 |
| DIFFSERV MIB |  | RFC3289 | R4.00 |
| Entity MIB |  | RFC2737 | R4.00 |
| Private MIB |  |  |  |
| D-Link Private MIB |  |  | R4.00 |
| D-Link Zone Defense MIB |  |  | R4.00 |
| Standard compliance |  |  |  |
| UDP |  | RFC768 | R4.00 |
| IP |  | RFC791 | R4.00 |
| ICMP | ICMPv4 | RFC792 | R4.00 |
|  | ICMPv6 | RFC2463, RFC4443 | R4.00 |
|  | Extended ICMP to Support Multi-Part Messages | RFC4884 | R4.00 |
| TCP |  | RFC793 | R4.00 |
| ARP | 1 | RFC826 | R4.00 |
| CIDR |  | RFC1338,1519 | R4.00 |
| Definition of the DS Field in the IPv4 and IPv6 Headers |  | RFC2474, 3168, 3260 | R4.00 |
| Extensible Authentication Protocol (EAP) |  | RFC 1321, 2284, 2865, 2716, 1759, 3580, 3748 | R4.00 |
| SNMP | SNMP Framework | RFC2571 | R4.00 |
|  | SNMP Message Processing and Dispatching | RFC2572 | R4.00 |
|  | SNMP Applications | RFC2573 | R4.00 |
|  | User-based Security Model for SNMPv3 | User-based Security Model for SNMPv3 | R4.00 |
| IPv6 | Path MTU Discovery for IPv6 | RFC1981 | R4.00 |
|  | IPv6 | RFC2460 | R4.00 |
|  | Neighbor Discovery for IPv6 | RFC2461, RFC4861 | R4.00 |
|  | IPv6 Stateless Address Auto-configuration | RFC2462, RFC4862 | R4.00 |
|  | IPv6 Neighbour Discovery over Ethernet and definition | RFC2464 | R4.00 |
|  | IPv6 Addressing Architecture | RFC3513, RFC4291 | R4.00 |
|  | IPv4/IPv6 dual stack function | RFC2893, RFC4213 | R4.00 |
| IPv6 Ready Logo | Comply with IPv6 Ready Logo phase 2 |  | R4.00 |

# Mechanical & ID design

## Mechanical

2.1.1 DES-3200-28P C1

Dimension

■ 19” Metal Case □ 11” Metal Case □Others (9” Metal Case) Metal Case: 441(W) x 308(D) x 44(H) mm, 1U Rack-Mount size

2.1.2 DES-3200-52 C1

Dimension

■ 19” Metal Case □ 11” Metal Case □ Others Metal Case: 441(W) x 308(D) x 44(H) mm, 1U Rack-Mount size

2.1.3 DES-3200-52P C1

Dimension

■ 19” Metal Case □ 11” Metal Case □ Others Metal Case: 441(W) x 308(D) x 44(H) mm, 1U Rack-Mount size

## ID Design

2.2.1 DES-3200-28P C1

2.2.2 DES-3200-52 C1

2.2.3 DES-3200-52P C1

## AC Power Inlet Design

The AC power inlet must locate at right side of rear panel. Detail dimensions are as follows:

3.4

## New Ear Mount Design

This Ear Mount Design is the same as DES-3800 and can be hung in the front, and rear and four angels.

      - Below ear mount design is for DES-3200-28P/52/52P C1 device when hung on 19 inch rack mount.

## LED indicators

| Location | LED Indicative | Color | Status | Description |
| --- | --- | --- | --- | --- |
| Per Device | Power | Green | Solid Light | Device Power On (Note: Power LED can’t be shut down when enable Power Saving by LED Shut-Off mode ) |
|  |  |  | Light off | Device Power Off |
|  | Console | Green | Solid Light | Console on |
|  |  |  | Blinking | POST is in progress |
|  |  |  | Light off | Console off |
|  | Fan(only for devices with fan. DES-3200-28P/52/52P, | - Red | - Blinking | - When any of the fans has failed |
|  |  |  | - Light off | - When all fans work normally |
|  | Port LED Mode Indicator | Only for 10/100Mbps on DES-3200-28P/52P: (Link/Act Mode / PoE mode can’t be shut off together) |  |  |
|  |  | Green | - Solid Light | - When 10/100 port is in Link/Act Mode |
|  |  | Green | - Solid Light | - When 10/100 port is in PoE Mode |
| LED Per 10/100 Mbps Port | Link/Act | Green | Solid Light | When there is a secure connection (or link) at any of the ports. |
|  |  | Green | Blinking | When there is reception or transmission (i.e. Activity—Act) of data occurring at a port. |
|  |  | Green | Light off | No link |
|  | PoE Mode | - Green | - Solid Light | - Power feeding |
|  |  | - Green | - Blinking | - Error Condition |
|  |  | - Off | - Light off | - No Power feeding |
| LED Per 10/100/1000 Mbps Port | Link/Act/Speed | Green | Solid Light | When there is a secure 1000Mbps connection at the port. |
|  |  |  | Blinking | When there is reception or transmission occurring at the port. |
|  |  | Amber | Solid Light | When there is a secure 10/100Mbps connection at the port. |
|  |  |  | Blinking | When there is reception or transmission occurring at the port. |
|  |  | off | Light off | No link |
| LED Per 100/1000 SFP Port | Link/Act/Speed | Green | Solid Light | When there is a secure 100/1000Mbps connection at the port. |
|  |  |  | Blinking | When there is reception or transmission occurring at the port. |
|  |  | Amber | Solid Light | When there is a secure 10/100Mbps connection at the port. |
|  |  |  | Blinking | When there is reception or transmission occurring at the port. |
|  |  | off | Light off | No link |

# Certifications/Test Reports

## Software Certification

NA

## Surge/Lightening Protection

Support surge/lightening protection on following switch interfaces.

| Interface | IEC 61000-4-5 Class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| AC Power Inlet (DES-3200-28P/52/52P) |  |  |  | V (2KV/1KV) |  |  |  |
| DC Power Inlet (DES-3200-52) |  |  | V 1KV |  |  |  |  |
| All Ethernet Ports (Include non-PoE,PoE,PoE+ port) PS :PoE+ without PD attached, restart is acceptable |  |  |  |  |  |  | 1,2/50us – 8/20us kV 10/700us 6 kV V 6KV |

## EMC/RF Certificates and Test Reports

| Required | EMI/EMC Test Report | Class A | Class B | Region & Country requirements |
| --- | --- | --- | --- | --- |
| V | CE Report (89/336/EEC(EN55022/24), 2004/108/EC) | V |  | EU |
| V | FCC report(FCC CFR 47 Part 15 B) | V |  | US |
| V | IC report(ICES-003) | V |  | Canada |
|  | IC report(ICES-006) | V |  | Canada |
| V | C-Tick Report(AS/NZS CISPR 22) | V |  | New Zealand & Australia |
| V | VCCI Report(CISPR 22) | V |  | JAPAN |
|  | MIC report |  |  | KOREA |
|  | Anatel |  |  | Brazil |
| V | BSMI | V |  | Taiwan |
|  | CCC |  |  | China |
|  | Other’s |  |  |  |

## Safety Certificates and Test Reports

| Required | Certifications | Standards | Region & Country requirements |
| --- | --- | --- | --- |
| V | CE LVD report (LVD:73/23/EEC) | EN60950-1: 2001 | European Union |
| V | UL/cUL Listed Mark | UL/CSA 60950-1 | US / Canada |
|  | CSA International Mark | IEC60950-1 UL60950-1 CSA60950-1 EN 60950-1 | World US Canada European Union |
| V | CB Report | IEC60950-1: 2001 EN60950-1 :2001 | World European Union |
|  | Other’s requirements |  |  |

## Reliability Test Reports

| Required | Test Items | Refer to Standards |
| --- | --- | --- |
| V | MTBF of Prediction Report | Bell-core TR-332, Issue 6 or Telcordia SR-332, Issue 1 |
| V | MTBF of Endurance test Report | D-Link SPEC |
| Optional | MTBF of Demonstration test Report | D-Link SPEC |
| V | Free Fall(Drop)Test Report | IEC 60068-2-32 |
| V | Random Vibration Test Report | IEC 60068-2-34: 1973 |
| V | Storage Test Report | IEC 60068-2-48 |
| V | Operation Cold(low temperature) Test | IEC 60068-2-1 |
| V | Operation Dry Heat(High temperature) Test | IEC 60068-2-2 |
| V | Operation Temperature Cycles Test | IEC 60068-2-14 |
| V | Thermal Shock Test | IEC 60068-2-14 |
| V | Damp Heat Steady State test | IEC 60068-2-78 |
| V | Thermal Profile Test | D-Link SPEC |
| V | ESD Simulation Test report | IEC 61000-4-2 Air Discharge: ±8KV Contact Discharge: ±4KV |
| V | High / Low Temperature Start Test | D-Link SPEC |
| V | Parts on/off & Insert/pulling Test | D-Link SPEC |
| V | Acoustic Noise test | EN 27779 & ISO 7779 |

## RoHS Requirement

### Level A of Substance limitation requirements table (for appliances & accessories)

| Level | D-Link No. | Description |  | Limitation/ ppm |
| --- | --- | --- | --- | --- |
| A | A1 | 鎘及其化合物 | Cadmium/ Cadmium Compounds | 80 |
|  | A2 | 六價鉻及其化合物 | Hexavalent Chromium/ Hexavalent Chromium Compounds | 800 |
|  | A3 | 鉛及其化合物 | Lead/ Lead Compounds | 800 |
|  | A4 | 汞及其化合物 | Mercury/ Mercury Compounds | 800 |
|  | A5 | 聚溴聯苯 | Polybrominated Biphenyls (PBBs) | 800 |
|  | A6 | 聚溴聯苯醚 | Polybrominated Diphenylethers (PBDEs) | 800 |

### Level A of Substance limitation requirements table (for Packing)

| Level | D-Link No. | Description |  | Limitation/ ppm |
| --- | --- | --- | --- | --- |
| A | A1 ~ A4 | 鎘, 六價鉻, 鉛, 汞 | Cadmium Chromium VI, Lead and Mercury compounds | 100 |

### Level A of Substance limitation requirements table (for battery)

| Level | D-Link No. | Description |  | Limitation/ ppm |
| --- | --- | --- | --- | --- |
| A | A1 | 鎘及其化合物 | Cadmium/ Cadmium Compounds(Cd) | 2500 (0.025%) |
|  | A3 | 鉛及其化合物 | Lead/ Lead Compounds(Pd) | 400 (0.4%) |
|  | A4 | 汞及其化合物 | Mercury/ Mercury Compounds(Hg) | 5 |