Model Name:

DGS-1008A Rev.B1

8-Port 10/100/1000BASE-T

Ethernet Gigabit Unmanaged Switch

Specification Version

1.0

By: Abbie Lin

Ethernet Switch Products Dept.

2011/06/20

This document contains confidential proprietary information and is the property of D-Link Corporation. The contents of this document may not be disclosed to unauthorized persons without the written consent of D-Link Corporation.

DGS-1008A Specification Revision History

| Version | Revised Date | # Person Name | # Content Revised |
| --- | --- | --- | --- |
| 1.0 | 2011/06/20 | Abbie Lin | First version |

Product Descriptions

This DGS-1008A, 8-Port 10/100/1000BASE-T Gigabit Ethernet Switch, is designed for the departmental and enterprise connection, Powerful and versatile, it eliminates network bottlenecks while giving users the capability to fine-tune performance.

The DGS-1008A is particularly designed for emerging countries and project-based with cost-demanding. The DGS-1008A is a cost-effective edition with economical specification and package content.

Desktop size is especially suitable for departmental environment, providing users the best performance/price ratio. Clear, at-a-glance per-device/per-port LED indicators make users easier to understand and control network status.

DGS-1008A enables IEEE 802.3az Energy-Efficient Ethernet (EEE) standard and Power-Saving Technology. IEEE 802.3az Energy-Efficient Ethernet (EEE) is the first standard in the history of Ethernet to address proactive reduction in energy consumption for networked devices. IEEE 802.3az Energy-Efficient Ethernet (EEE) is enlisted into one of D-Link’s Green Technology to re-emphasizes D-Link’s strong commitment to making its products more energy efficient.

- Product Specifications:

  - Hardware Specifications:

| Key Components |  |
| --- | --- |
| Dimension | 128 x 68.5 x 25.4 mm (5.0 x 2.7 x 1.0 inches) |
| Port packet Filtering/ Forwarding Rate | 14,880 pps for 10M 148,800 pps for 100M 1,488,000 pps for 1000M |
| Network Data Transfer Rate | Ethernet; 10Mbps; Half-Duplex. 20Mbps; Full-Duplex. Fast Ethernet; 100Mbps; Half-Duplex. 200Mbps; Full-Duplex. Gigabit Ethernet; 2000Mbps; Full-Duplex. |
| Number of Ports | 8-Port 10/100/1000Base-T |
| LAN | Compliant to following standards, - IEEE 802.3 10BASE-T - IEEE 802.3u 100BASE-TX - IEEE 802.3ab 1000BASE-T - D-Link’s Green Technology - IEEE 802.3az Energy-Efficient Ethernet (EEE) - Power Saving Technology: - Power saving by link status - Power saving by cable length - Support Auto-Negotiation for each port - Support Half/Full-Duplex operations at 10/100Mbps. - Support Full duplex transfer mode for 1000Mbps - Support back pressure at Half-Duplex operation - IEEE 802.3x Flow Control support. - Auto MDI/MDIX - Wire speed reception and transmission - Store-and-Forward switching method - supports 8K MAC addresses - Supports 128 KBytes RAM for data buffering - Support Jumbo Frames: 9216Bytes - Switching Capacity 10Gbps - RoHS Compliant. - ErP Compliant |

  - D-Link’s Green Technology:

- IEEE 802.3az Energy-Efficient Ethernet (EEE):

It is the first standard in the history of Ethernet to address proactive reduction in energy consumption for networked devices. The IEEE 802.3 EEE standard defines mechanisms and protocols intended to reduce the energy consumption of network links during periods of low utilization, by transitioning interfaces into a low-power state without interrupting the network connection.

- D-Link Innovative “Green Ethernet” Technology implements special power-saving features under speed at 1000Mbps that detect cable length and link status and adjust power usage accordingly. Green Ethernet Technology saves energy in two specific ways:

- Method 1: If there is no link on a port, such as when there is no computer connected to the port or the connected computer is powered off, Green Ethernet Technology will enter a "sleep mode", drastically reducing power used for that port.

- Method 2: D-Link Green Ethernet detects the length of connected Ethernet cable and adjusts power usage accordingly without affecting performance.  This way, a port connected to a 20m cable only uses as much power as it needs, instead of using full power, which is only needed for 100m cables.

2. Mechanic & ID Design

  - Nameplate Design

  - LED

| - LED Indicative | - Color | - Status | - Description |
| --- | --- | --- | --- |
| - Power - (per system) | - Green | - Solid Light | - Power On |
|  |  | - Light off | - Power Off |
| - Link/Act - (per port) | - Green | - Solid Light | - When there is a connection (or link) to device at any of the ports. |
|  |  | - Blinking | - When there is reception or transmission (i.e. Activity--Act) of data occurring at a port. |
|  |  | - Off | - No link. |

3. Physical & Environment

3.1 External Power Adapter

- Switching 5V/1A Level “V” Power Adapter

3.2 Power Consumption:

- AC side: 3.9 Watts

- DC side: 3.06 Watts

3.3 MTBF:

- 412,933 Hours

3.4 Operation Temperature:

- 0 C ~ 40 ºC (32F ~ 104F)

3.5 Storage Temperature:

- -10 ~ 70°C (14 ~ 158º F)

3.6 Humidity

- Operating: 10% ~ 90%,

- Storage: 5% ~ 90%

4. Power Consumption of D-Link’s Green Technology [AC Input]

| Mode | Power Consumption [220V AC Input] |
| --- | --- |
| Link Down | 0.89 Watts |
| Link Up with traffic | 4.2 Watts |
| Link Up without traffic (EEE mode) | 1.2 Watts |

- Emission (EMI) and Safety Certifications:

  - EMI: CE class B, FCC class B, ICES-003 Class B, VCCI class B, C-Tick, Anatel, CCC

  - Safety: LVD, cUL/UL