Model Name：DGS-1100-08 ver. A1

8 Port 10/100/1000Mbps EasySmart Switch

External Specifications

Version 1.00

By: Samantha Su

Ethernet Switch Product Div.

Date: 2011/10/26

This document contains confidential proprietary information and is the property of D-Link Corporation.

The contents of this document may not be disclosed to unauthorized persons without the written consent of

D-Link Corporation.

DGS-1100-16/24 Specification Revision History

| Version | Revised Date | # Person Name | # Content Revised |
| --- | --- | --- | --- |
| 1.00 | 2011/10/23 | Samantha Su | First Edition |

This 8-Port Gigabit Ethernet EasySmart Switch DGS-1100-08 is designed for SOHOs or home. It incorporates essential L2 features, such as 802.1p QoS for traffic prioritization, IGMP Snooping for multicast application, and 802.1Q VLAN for traffic segmentation. Besides, Voice VLAN and Surveillance give specific packets higher priority to secure transmission quality on selected ports. A user-friendly Web-based management is supported for easy configuration and flexibility for different network requirements. The palm size is especially suitable for home and departmental environment.

- Product Specifications:

  - Hardware Specifications

| General Features | Detailed Description |  |
| --- | --- | --- |
| Dimension | 171mm x 98 mm x 28mm |  |
| Weight | TBD kg |  |
| Switching Capacity | 16Gbps |  |
| Maximum Forwarding Rate | 11.9Mpps |  |
| MAC Address | 8K |  |
| Forwarding Mode | Store and Forward |  |
| Packet Buffer memory | 256KB |  |
| CPU | Embedded MIPS32 (80MHz) |  |
| Flash Memory | 2MBytes (SPI Flash) |  |
| Power Consumption | Maximum Power Consumption: (Power measurement from AC side) 4.58 Watts Standby Power Consumption: 0.62 Watts |  |
| Heat Dissipation | 2.936 BTU/hr |  |
| MTBF | TBD hours |  |
| Reset | A factory reset button |  |
| Kensington slot |  |  |
| Hardware Protection | Grounding design in the back side of case |  |
| Port Functions |  |  |
| LAN |  | 8 10/100/1000BaseT ports Compliant to following standards: - IEEE 802.3 compliance - IEEE 802.3u compliance - IEEE 802.3ab compliance - Support Auto-Negotiation for each port - Support Half/Full-Duplex operations at 10/100Mbps - Support Full-Duplex operations at 1000Mbps - Support back pressure at Half-Duplex operation - IEEE 802.3x Flow Control support - Head-of-line blocking prevention 10. Auto-MDI/MDIX for each port |

  - Product Features:

Metal Case

DGS-1100-08 □19" Metal Case □ 11" Metal Case ■ Others

  - Product Features:

| Feature | Detailed Description | IETF Standard | Release Schedule |
| --- | --- | --- | --- |
| L2 features |  |  |  |
| IGMP snooping | 1. Enabled and disabled. Supports IGMP v1/v2 2. IGMP Per Port 3. Supports 32 IGMP snooping groups |  | FCS |
| Static trunk | Support max 2 groups per device and 2-4 ports per group |  | FCS |
| Flow Control | Enabled and disabled |  | FCS |
| Port mirroring | 1. Support one-to-one port mirroring 2. Support TX,RX and Both mode |  | FCS |
| Loopback Detection | Auto disable port when loop was detected |  | FCS |
| Statistics | Supported the following packets: TxOk, TxError, RxOk RxError |  | FCS |
| VLAN |  |  |  |
| 802.1Q | Supports IVL mode. |  | FCS |
| VLAN Group | 1. Support 32 static VLAN groups 2. Be able to configure VID from 1-4094 |  | FCS |
| Port-based VLAN |  |  | FCS |
| Surveillance VLAN | By adding specific ports into the pre-defined VLAN, the switch can automatically detect MAC addresses or OUI, and give higher priority to the packet sent by matched devices to secure the transmission. |  | FCS |
| Voice VLAN | Voice VLAN spec 2.0 |  | FCS |
| QoS |  |  |  |
| 802.1p support | 4 Outbound Queues |  | FCS |
| Number of Queues per Port | 1. 4 Outbound Queues; 2. Support Strict/WRR mode in queue handling 3. 4 Classes mapping to 802.1p value: Class 0 (Low queue): 1,2 Class 1 (Middle queue): 0,3 Class 2 (High queue): 4,5 Class 3 (Highest queue): 6,7 |  | FCS |
| Security |  |  |  |
| Static MAC | 1. Select from Dynamic Forwarding Table 2. Manually enter static MAC 3. Can disable auto learning 4. Support 32 Static MAC entries Only allow the packets with SA matched configured static MACs |  | FCS |
| Storm Control | When Broadcast / Multicast / Unknown Unicast hit the definable threshold, the switch starts blocking or discarding packets which would otherwise 'flood' the network. The blocked port will be activated again only when the broadcast/ multicast traffic falls under the defined threshold by user. The threshold can be defined as 1 – 1000000pps. |  | FCS |
| Bandwidth Control | 1. Support Port based bandwidth control 2. Ingress, minimum granularity 8 Kbps (8/16/32/64/128/256/512kbps, 1/2/4/8/16/32/64/128/256/512M) 3. Egress, minimum granularity 8 Kbps (8/16/32/64/128/256/512kbps, 1/2/4/8/16/32/64/128/256/512M) |  | FCS |
| Management |  |  |  |
| Web-based GUI | 1. D-Link style 2. Compatible with following browsers - IE6 or later version - Firefox 3.0 or later version - Chrome 5.0 or later version - Safari 4.0 or later version - Netscape 8 or later version - Opera 10 or later version 3. Supports IPv4 |  | FCS |
| Password Access Control |  |  | FCS |
| Port Setting | Can set Speed, Duplex mode, flow control |  | FCS |
| Configuration File Upload/Download | Via HTTP |  | FCS |
| Firmware File Backup/Upgrade | Via HTTP |  | FCS |
| Reboot device | System reboot |  | FCS |
| Switch Reset | Switch will be reset to its factory default setting and then reboot |  | FCS |
| BootP/DHCP Client |  | RFC951, 1542 | FCS |
| Cable Diagnostics | Support cable diagnostic function to check the status of connected RJ-45 cables. |  | FCS |
| SmartConsole Utility | - Basic Utility Settings- log, trap and save monitor and list. - Device Configurations - device configuration, password setting, link to web GUI, and DHCP refresh - Device List - display all discovered smart devices on the network. All devices in the Device List are monitored. - Upgrade DGS-1100 switch firmware |  | FCS |
| Default IP | 10.90.90.90 |  | FCS |
| Green |  |  |  |
| EEE Support | Based on draft 2.4 of 802.3az compliance Low Power Idle Mode Activate at 10/100/1000Mpbs |  | FCS |
| Power Saving | Power Saving by cable length(short cable <=10m) |  | FCS |

2. Mechanism & ID Design:

2.1 Name Plate Design

2.2 Rear Plate Design

  - 2.3 LED Indicator

| - Location | - LED Indicative | - Color | - Status | - DGScription |
| --- | --- | --- | --- | --- |
| - Per Device | - Power | - Green | - Solid Light | - Power On |
|  |  |  | - Light off | - Power Off |
| - LED Per 10/100/1000 Mbps Port | - Link/Act/Speed Mode | - Green | - Solid Light | - When there is a secure connection (or link) to 1000Mbps Ethernet device at any of the ports. |
|  |  |  | - Blinking | - When there is reception or transmission of data occurring at 1000Mbps. |
|  |  | - Off | - Light off | - No link |
|  |  | - Amber | - Solid Light | - When there is a secure connection (or link) to 10/100Mbps Ethernet device at any of the ports. |
|  |  |  | - Blinking | - When there is reception or transmission of data occurring at 10/100Mbps. |
|  |  | - Off | - Light off | - No link |

###### 3. Physical & Environment

3.1 AC input

- External power adapter

3.2 Operation Temperature

- 0-40°C

3.3 Storage Temperature

- -10-70°C

3.4 Humidity

- Operation: 10%-95% RH

- Storage: 5% ~ 95% RH

4. Emission (EMI) and Safety Certifications:

4.1 EMC/RF Certificates and Test Reports

| Request | EMI/EMC Test Report | Class A | Class B | Regional & Country Requirements |
| --- | --- | --- | --- | --- |
| V | CE Report (89/336/EEC(EN55022/24), 2004/108/EC) | V |  | European Union |
| V | FCC Report (FCC CFR 47 Part 15 B) | V |  | United States |
| V | IC Report (ICES-003) | V |  | Canada |
|  | IC Report (ICES-006) |  |  | Canada |
| V | C-Tick Report (AS/NZS CISPR 22) | V |  | New Zealand & Australia |
| V | VCCI Report (CISPR 22) | V |  | Japan |
|  | MIC Report |  |  | Korea |
|  | Anatel |  |  | Brazil |
|  | BSMI |  |  | Taiwan |
|  | CCC |  |  | China |

4.2 Safety Certificates and Test Reports

| Request | Certifications | Standards | Regional Requirements |
| --- | --- | --- | --- |
| V | CE LVD Report (LVD:73/23/EEC) | EN60950-1: 2001 | European Union |
| V | UL/cUL Listed Mark | UL/CSA 60950-1 | United States / Canada |
|  | CSA International Mark | IEC60950-1 UL60950-1 CSA60950-1 EN 60950-1 | World United States Canada European Union |
|  | CB Report | IEC60950-1: 2001 EN60950-1 :2001 | World European Union |
|  | Other Requirements |  |  |

4.3 Reliability Test Reports

| No. | Required | Test Items | Refer to Standards |
| --- | --- | --- | --- |
| 1 | V | MTBF of Prediction Report | Bell-core TR-332, Issue 6 or Telcordia SR-332, Issue 1 |
| 2 | V | Endurance Test Report | D-Link SPEC |
| 3 |  | MTBF of Demonstration Test Report | D-Link SPEC |
| 4 | V | Free Fall (Drop) Test Report | IEC 60068-2-32 |
| 5 | V | Random Vibration Test Report | IEC 60068-2-34: 1973 |
| 6 | V | Storage Test Report | IEC 60068-2-48 |
| 7 | V | Operation Cold (Low Temperature) Test | IEC 60068-2-1 |
| 8 | V | Operation Dry Heat (High Temperature) Test | IEC 60068-2-2 |
| 9 | V | Operation Temperature Cycle Test | IEC 60068-2-14 |
| 10 | V | Thermal Shock Test | IEC 60068-2-14 |
| 11 | V | Damp Heat Stability Test | IEC 60068-2-78 |
| 12 | V | Thermal Profile Test | D-Link SPEC |
| 13 | V | ESD Simulation Test Report | IEC 61000-4-2 Air Discharge: ±8KV Contact Discharge: ±4KV |
| 14 | V | High / Low Temperature Start Test | D-Link SPEC |
| 15 |  | Parts On/Off & Insert/Extract Test | D-Link SPEC |
| 16 | V | Acoustic Noise Test | EN 27779 & ISO 7779 |

4.4 RoHS Requirement

| Level | D-Link No. | Description |  | Limitation/ ppm |
| --- | --- | --- | --- | --- |
| ### A | A1 | 鎘及其化合物 | Cadmium/ Cadmium Compounds | 80 |
|  | A2 | 六價鉻及其化合物 | Hexavalent Chromium/ Hexavalent Chromium Compounds | 800 |
|  | A3 | 鉛及其化合物 | Lead/ Lead Compounds | 800 |
|  | A4 | 汞及其化合物 | Mercury/ Mercury Compounds | 800 |
|  | A5 | 聚溴聯苯 | Polybrominated Biphenyls (PBBs) | 800 |
|  | A6 | 聚溴聯苯醚 | Polybrominated Diphenylethers (PBDEs) | 800 |