D-Link Corporation

ShareCenter Quattro

4-Bay SATA Network Storage 
DNS-345 A1 (v1.0)

External Product Spec

Version History:

| Date | Version | Person | Description |
| --- | --- | --- | --- |
| 2011/4/25 | 0.9 | Eric Jen | First draft release |
| 2011/7/4 | 1.0 | Eric Jen | First release version |

## I.	Product Features:

- Equipped two 10/100/1000 Mbps auto-MDIX Gigabit Ethernet LAN ports, 1000BASE-T support Half duplex and Full duplex mode

- Embedded four hot-swappable SATA II 3.5” HDD interface, support 3TB or above Hard Drives

- Supports 128x32 OLED Display

- 4-bay SATA Hard Drive with standard, JBOD, RAID 0/1/5/5 + Hot Spare/10 support

- Equipped Ultra Cooler (Aluminum housing + FAN speed control)

- Tool-less hard drive installation

- Supports Real time clock (RTC)

- Network Protocols

    - DHCP Client or Static IP

    - NTP Server

    - Windows 7, Vista x32/64 PnP-X/LLTD

    - Dynamic DNS

    - Bonjour

    - VLAN (tag)

    - Link Aggregation/ Fail-over and Fail-back for 2 Gigabit Ethernet ports

    - UPnP Port Forwarding

- Network File Services

    - Supports Windows XP/ Vista/ 7, Mac OS X 10.5, Linux, Windows Server 2003/2008

    - CIFS/SMB for Windows and Mac OS X

    - NFSv2/v3 for Linux and UNIX

    - HTTP and HTTP/S for web browsers

    - AFP3.1 for Mac OS X

    - DFS (Microsoft DFS)

    - WebDAV

- Security

    - Active Directory support for Windows Server 2003/ 2008

    - ADS support Quota

    - Block / Allow IP list

    - Volume Encryption

- File System

    - EXT 4 for internal HDD

    - FAT16/32, NTFS (Read/ Write) for USB external Storage

- File System Management

    - Unicode Support for both Samba and FTP server

    - File sharing: Windows/ Mac/ Linux

- FTP server

    - FTP over explicit SSL/ TLS mode (FTPES)

    - FTP bandwidth and connection control

    - Supports FXP

- Download Management

    - HTTP/FTP schedule download

    - Torrent download, aMule download support

- Disk Management

    - RAID: Standard, JBOD, 0/1/5/10/5 + hot spare support

    - Disk Status Monitoring (S.M.A.R.T.)

    - RAID migration:

- Standard to RAID 1

- Standard to RAID 5

- RAID 1 to RAID 5

    - HDD S.M.A.R.T test

    - Scandisk

    - Reformat HDD: EXT4

    - AES based volume encryption

    - Support Advanced Format HDD

    - Disk roaming

    - RAID roaming

- Power Management

    - Power Saving (Disk idle spin-down)

    - Automatic power recovery (with UPS)

    - Schedule power on/off

    - Auto-offload system on UPS low battery

    - Smart FAN Control

    - Network UPS

- Folder Management

    - Share Folder Level Permission

    - Public Folder

    - Supports ISO Mount

- Remote File Sharing

    - FTP Server

    - WebDAV

    - AjaXplorer

    - Web File Server

- User/Group Management

    - User/group assignment for Network sharing and FTP server

    - Quota for user and group

    - Users can assign to multiple Groups

    - Batch User Creation

- Backup Management

    - Schedule Backup from PC to NAS

    - Remote Network Backup (Rsync, Remote Snapshot)

    - Local Backup

    - Apple Time Machine support

    - Cloud Storage Backup (Amazon S3)

    - USB Copy

- System Management

    - OLED display to show - RAID status, System temperature, Disk Information, USB Copy/ un-mount

    - D-Link Storage Utility for device discovery, IP configuration, and drive mounting

      - OS support: Windows XP, Vista, 7, Mac OS X, Windows Server 2003/2008

    - Two access modes supported: Open (Share) mode and Account (User) mode, no setup necessary in open mode

    - Support Configuration file save/load

    - System status

    - Email/ SMS notifications

    - Network Recycle bin

    - System/FTP Log (Syslog Client)

    - Rescue F/W image

    - SNMPv2

    - Resource Monitor

- USB port support:

    - Print Server

    - UPS monitoring

    - External Storage device

    - MTP/PTP

- Media Streaming

    - UPnP AV server:

- DLNA v1.5 certified

- Support PS3/Xbox 360

- Support UPnP AV file > 4GB

- D-Link new Media Player (including Boxee) compatible

    - iTunes Server/ FireFly

    - Logitech Squeeze Center support

    - Audio Streamer (icestation)

- Windows Applications

    - Yahoo Widget support

- Add-ons Packages

    - Multilingual GUI Package

    - Photo Station

    - Blog (WordPress)

    - P2P downloader/ aMule

    - Audio Streamer (icestation)

    - AjaXplorer

    - Logitech Squeeze Center

- Web Browsers Supported

    - Internet Explorer 7+

    - Mozilla Firefox 3+

    - Apple Safari 4+

    - Google Chrome 3+

    - Opera 10+

## II.	Hardware specifications:

2.1 H/W Specifications:

| Key Component | Detailed Description |
| --- | --- |
| Storage Network Processor | SoC 1.6 GHz |
| OS | Linux 2.6.31 |
| SDRAM | 512MB |
| NAND Flash | 128MB |
| Hard Disk Bays | Support four standard SATA II drives |
| Network | 2 x Ethernet 10/100/1000M |
| Thermal Sensor | Yes |
| Power Supply | Detachable power supply |
| OLED Display | 128 x 32 |
| FAN | 2 cooling FANs with 3 control modes: - Auto - Off / Low / High - Auto - Low / High - Manual - Always on High |
| Ports | Detailed Description |
| USB connector | 1 USB 2.0 (Type A) connector: USB Host port for USB external storage device, USB printer, and UPS monitoring |
| Power receptacle | Connect to the power supply |

2.2 OLED Display:

2.3 Button:

| Button Indicative | Status | Description |
| --- | --- | --- |
| Power | Push | Power on; to confirm USB Backups function |
|  | Push over 5 seconds | Power off |
| Next | Push | OLED to display the next status page; to cancel the USB Backups function |
| Reset | Push over 5 seconds | Reset configuration to default |

## III.	Software specifications:

| OS | Linux 2.6.31 |
| --- | --- |
| Network file Protocols | - FTP - SMB - NFS - AFP |
| Standards | - IEEE 802.3/u/ab/ad - SATAI, II. - DDR3 - UPnP AV 1.0 |
| Network Service Protocols | - DHCP Client - SMB - SMTP/S - HTTP/S - DLNA HNv1.5 |
| Network Client Type | - Microsoft: XP, Vista, 7, Windows Server 2003/2008 - Linux - Mac OSX 10.5+ |
| File System | Internal HDD: EXT4 External HDD: FAT16/32, NTFS |
| File Sharing (TBD) | - Max User Accounts: 512 (non-ADS)/ 800 (ADS) - Max Groups: 64 (non-ADS)/ 200 (ADS) - Max Shared Folders: 128 (without P2P) - Max Concurrent Connections: 64(Samba)/10(FTP) |
| Media Type (Supported streaming media format) | For UPnP AV Server Audio Type - MP3, MPA, WAV/PCM, WMA, AIF/AIFF, OGG, FLAC, APE, AAC, M4A, MKA, LPCM Video Type - VOB, AVI, MPEG2, MP4, MPEG4, WMV9, MOV, MKV, ISO, DIVX, IFO, M2TS, DAT, TS Subtitle Type - SRT PlayList Type - M3U, PLS, WPL Photo Type - JPG, BMP, GIF, TIF, PNG For iTunes Server Audio Type - MP3, M4P, M4A For PS3 Audio Type - MP3, WAV, WMA Video Type - MPEG2, MPEG4, VOB, WMV9 Photo Type - JPG, BMP, GIF, TIF, PNG For Xbox 360 Audio Type - MP3, WMA Video Type - WMV Photo Type - JPG |
| HDD | - Reformat Disk - ScanDisk - S.M.A.R.T |
| Two RESET modes | - Reset firmware GUI button - Reset by HW reset button (press over 5 seconds) |
| Drive Management | - User/Group Quota management |
| Device Management | - HTTP and HTTPS Web browser - Easy Search utility - E-mail Alert |
| Power Management | - Hard Drive Hibernation - Auto Power Recovery - Schedule Power On/ Off - Smart FAN Control - D-Link Green Ethernet |
| Multiple language support | - Samba: Unicode - FTP Client: Unicode, Croatian, Cyrillic (Kyrgyz Republic), Czech, Danish, Dutch, English, Finnish, French, German, Greek, Hungarian, Italian, Norwegian, Polish, Portuguese, Romanian, Russian, Simplified Chinese, Slovenian, Spanish, Swedish, Traditional Chinese, Turkish, Korean, Hebrew |

## IV.	ID and Mechanical Design

Dimension

+ Item (W x D x H): 128 x 200 x 178mm (5” x 7.9” x 7” )

V.	Technical Specifications

Standards

+ IEEE 802.3                       + IEEE 802.3ab

+ IEEE 802.3u                      + IEEE 802.3ad

+ TCP/IP						  + CIFS/SMB

+ AFP		                     + NFS

+ UPnP AV                         + Bonjour

+ DDNS                            + NTP

+ FTP over SSL/TLS, FXP           + HTTP/HTTPS

+ WebDAV                         + USB 2.0

+ DHCP Client					  + PnP -X

+ LLTD

Supported Hard Drive Type

+ SATA I/II – Support hard drive capacity up to 3TB

+ 3.5” Internal

Ports

+ 2 10/100/1000 Gigabit Ethernet Ports with LED indicators

+ 1 USB 2.0 Host Port

+ Power

LEDs

+ LAN (on LAN ports)

Display

+ 128 x 32 OLED Display Screen

Disk Management

+ Multiple Hard Drive Configurations: RAID 0/ 1/ 5/ 10/ 5 + Hot Spare, JBOD, Standard

+ RAID migration from Non-RAID to RAID 1/ 5, RAID 1 to RAID 5

+ Hard Drive Format: Internal : EXT4;   External : FAT16/32, NTFS

+ Scandisk

+ S.M.A.R.T.

+ AES based volume encryption

+ Disk roaming

+ RAID roaming

Account Management

+ Windows Active Directory service support for Windows Server 2003/ 2008

+ User account management

+ Group account management

+ User/Group Quota management

+ Network Share management

+ ISO Mount management

+ Batch User Creation

Download management

+ HTTP/FTP schedule download

+ Peer to peer (P2P), aMule

Backup Management

+ Schedule Backup from PC to NAS

+ Schedule Local Backups

+ Schedule Remote Backups (Rsync, Remote Snapshot)

+ Apple Time Machine support

+ USB Backups

+ Cloud Storage Backup (Amazon S3)

Device Management

+ Internet Explorer 7+, Mozilla Firefox 3+ , Apple Safari 4+ or Google Chrome 3+, Opera 10+

+ Easy Search Utility

+ E-Mail Notifications

+ SMS Notifications

+ System/FTP log

+ Yahoo! Widget

+ SNMPv2

+ Resource Monitor

+ Add-ons Packages

Power Management

+ Power Saving mode

+ Auto Power Recovery

+ Schedule Power on/ off

+ Smart Fan control

+ D-Link Green Ethernet

+ Auto-offload system on UPS low battery

+ Network UPS

Remote File Sharing

+ Web File Server

+ FTP Server

+ WebDAV

+ AjaXplorer

File Sharing

+ Max. User Account: 512 (non-ADS)/ 800 (ADS)

+ Max. Group: 64 (non-ADS)/ 200 (ADS)

+ Max. Shared Folder: 128 (without P2P)

+ Max. Con-current Connection: 64 (Samba) / 10 (FTP)

Power Supply

+ External Power Supply

+ DC 19V/4.74A Switching

Power Consumption

+ Normal mode: 53W

+ Sleep mode: 13.5W

Temperature

+ Operating: 0˚ - 40˚ C (32˚ - 104˚ F)

+ Storage: -20˚ - 70˚ C (-4˚ - 158˚ F)

Operating Humidity

+ 5% ~ 90% (Non-condensing)

Dimensions

+ Item (W x D x H): 128 x 200 x 178mm (5” x 7.9” x 7”)

+ Packaging (W x D x H): 242 x 278 x 255mm (9.52” x 10.94” x 10.04”)

Weight

+ Item: 2.7kg (5.95bs)

+ Packaging: 4.02kg (8.86lbs)

Multilingual Support:

+ Samba: Unicode

+ FTP Server/ iTunes Server:	Unicode, Korean, Turkish, Croatian, Cyrillic (Kyrgyz Republic), Czech, Danish, Dutch, English, Finnish, French, German, Greek, Hungarian, Italian, Norwegian, Polish, Portuguese, Romanian, Russian, Simplified Chinese, Slovenian, Spanish, Swedish, Traditional Chinese, Turkish, Korean, Hebrew