DGS-1210-10P v.B1

8-ports 10/100/1000Mbps PoE + 2 Combo 1000Base-T/SFP ports Web Smart Switch

DGS-1210-20 ver. B1

16 Ports 10/100/1000Mbps + 4 SFP Web Smart Switch

DGS-1210-28 ver. B1

24 Ports 10/100/1000Mbps + 4 SFP Web Smart Switch

DGS-1210-52 ver. B1

48 Ports 10/100/1000Mbps + 4 SFP Web Smart Switch

DGS-1210-28P ver. B1

24 Ports 10/100/1000Mbps PoE + 4 SFP Web Smart Switch

External Specification

V1.0

By: Samantha Su

Ethernet Switch Products Division

2011/11/18

DGS-1210 Series Specification Revision History

| Version | Revised Date | # Person Name | # Content Revised |
| --- | --- | --- | --- |
| 1.00 | 2011/11/24 | Samantha Su | First Edition |

Product Description

The DGS-1210 series is the new generation of Web Smart series. It provides a variety of port counts- 16, 24 or 48 10/100/1000Mbps ports plus 4 SFP ports. Besides, the series offers 2 PoE models with 8 or 24 10/100/1000Mbps PoE ports plus 2 Combo ports or 4 SFP ports.

As the new generation of DGS-1210 models, they provide rich L2 functions and various management tools. They also offer IPv6 ready core logo phase II to have investment protection.

This new DGS-1210 series implements D-Link Innovative “3nd-Gen. Green Ethernet” technology, it not only support latest 802.3az EEE standard to save more power while linkup and enter standby mode if no traffic, but also supports scheduling to shut-off LED, ports and enter hibernation mode automatically.

1. Product Specifications

  - Hardware Specifications

| General Feature | Detailed Description |
| --- | --- |
| Dimension | DGS-1210-10P: 330mm x TBD mm x 44mm DGS-1210-20: 440mm x 210 mm x 44mm DGS-1210-28: 440mm x 250 mm x 44mm DGS-1210-52: 440mm x 310 mm x 44mm DGS-1210-28P: 440mm x 250 mm x 44mm |
| Reset pinhole | A factory reset pinhole in the front panel |
| Switching Capacity | DGS-1210-10P: 20Gbps DGS-1210-20: 40Gbps DGS-1210-28: 56Gbps DGS-1210-52: 104Gbps DGS-1210-28P: 56Gbps |
| 64 Bytes max. packet forwarding rate | DGS-1210-10P: 14.88Mpps DGS-1210-20: 29.8Mpps DGS-1210-28: 41.7Mpps DGS-1210-52: 77.4Mpps DGS-1210-28P: 41.7Mpps |
| Forwarding Mode | Store and Forward |
| Packet Buffer memory | DGS-1210-10P: 1MBytes DGS-1210-20: 1MBytes DGS-1210-28: 1MBytes DGS-1210-52: 1MBytes DGS-1210-28P: 1Mbytes |
| DDRII for CPU | 128Mbytes |
| Flash Memory | 16Mbytes |
| Power Consumption | Maximum Power Consumption: (Power measurement from AC side) DGS-1210-10P: xxx Watts (PoE on), xxx Watts (PoE off) DGS-1210-20: TBD Watts DGS-1210-28: TBD Watts DGS-1210-52: TBD Watts DGS-1210-28P: TBD Watts (PoE on), TBD Watts (PoE off) Standby Power Consumption: DGS-1210-10P: xxx Watts (PoE on), xxx Watts (PoE off) DGS-1210-20: TBD W/100V, TBD W/240V DGS-1210-28: TBD W/100V, TBD W/240V DGS-1210-52: TBD W/100V, TBD W/240V DGS-1210-28P: TBD W/100V , TBD W/240V |
| Heat dissipation | DGS-1210-10P: xxx BTU/hr DGS-1210-20: TBD BTU/hr DGS-1210-28: TBD BTU/hr DGS-1210-52: TBD BTU/hr DGS-1210-28P: TBD BTU/hr |
| MTBF | DGS-1210-10P: xxx hours DGS-1210-20: TBD hours DGS-1210-28: TBD hours DGS-1210-52: TBD hours DGS-1210-28P: TBD hours |
| Acoustic | DGS-1210-10P: 0 dB(A) DGS-1210-20: 0 dB(A) DGS-1210-28: 0 dB(A) DGS-1210-52: TBD dB(A) DGS-1210-28P: TBD dB(A) |
| Fan number | DGS-1210-10P: 0 pcs DGS-1210-20: 0 pcs DGS-1210-28: 0 pcs DGS-1210-52: TBD pcs DGS-1210-28P: TBD pcs |
| Smart Fan (DGS-1210-28P/52) | 1. Default : Low speed 2. When ambient temperature detected by the sensor goes over 33℃, the fan speed will automatically change to high speed. 3. When ambient temperature detected by the sensor goes below 30℃, the fan speed will automatically change to low speed. |
| Weight | DGS-1210-10P: TBD kg DGS-1210-20: TBD kg DGS-1210-28: TBD kg DGS-1210-52: TBD kg DGS-1210-28P: TBD kg |
| Port Functions (For DGS-1210-20/28/52) |  |
| 16/24/48 10/100/1000Mbps Ports in the front | Compliant to following standards: - IEEE 802.3 compliance - IEEE 802.3u compliance - IEEE 802.3ab compliance - IEEE 802.3az compliance - Support Half/Full-Duplex operations - Auto-negotiation - Auto MDI/MDIX - IEEE 802.3x Flow Control support for Full-Duplex mode |
| 4 SFP ports in the front panel | DGS-1210-20/28 SFP Transceivers Supported: - DGS-712 1000BASE-T Copper SFP Transceiver - DEM-310GT (1000Base-LX, Single-mode, 10km) - DEM-311GT (1000ase-SX, Mutli-mode, 550m) - DEM-312GT2 (1000Base-SX, Multi-mode, 2km) - DEM-314GT (1000BASE-LHX, Single-mode, 50km) - DEM-315GT (1000BASE-ZX, Single-mode, 80km) - DEM-330T/R (Gigabit WDM transceiver, Single-Mode 10km) - DEM-331T/R (Gigabit WDM transceiver, Single-Mode 40km) Compliant to following standards: - IEEE 802.3z compliance - Support Auto-negotiation for Full-Duplex/ flow control operations DGS-1210-52 SFP Transceivers Supported: - DGS-712 1000BASE-T Copper SFP Transceiver - DEM-310GT (1000Base-LX, Single-mode, 10km) - DEM-311GT (1000ase-SX, Mutli-mode, 550m) - DEM-312GT2 (1000Base-SX, Multi-mode, 2km) - DEM-314GT (1000BASE-LHX, Single-mode, 50km) - DEM-315GT (1000BASE-ZX, Single-mode, 80km) - DEM-330T/R (Gigabit WDM transceiver, Single-Mode 10km) - DEM-331T/R (Gigabit WDM transceiver, Single-Mode 40km) - DEM-211 (100Base-FX, Multi-mode, 2km) - DEM-210 (100Base-FX, Single-mode, 15km) - DEM-220T (100Base-FX WDM transceiver, Single-mode, 20km) - DEM-220R (100Base-FX WDM transceiver, Single-mode, 20km) Compliant to following standards: - IEEE 802.3z compliance - IEEE 802.3u compliance - Support Auto-negotiation for Full-Duplex/ flow control operations |
| Port Functions (For DGS-1210-10P/28P) |  |
| 10 or 24 10/100/1000Mbps Ports in the front | Compliant to following standards: - IEEE 802.3 compliance - IEEE 802.3u compliance - IEEE 802.3ab compliance - Support Half/Full-Duplex operations - Auto-negotiation - Auto MDI/MDIX - IEEE 802.3x Flow Control support for Full-Duplex mode - IEEE 802.3af compliance - IEEE 802.3at compliance - IEEE 802.3az compliance |
| 2 or 4 SFP ports in the front | DGS-1210-10P SFP Transceivers Supported: - DGS-712 (10/100/1000Base-T) - DEM-310GT (1000Base-LX, Single-mode, 10km) - DEM-311GT (1000ase-SX, Mutli-mode, 550m) - DEM-312GT2 (1000Base-SX, Multi-mode, 2km) - DEM-314GT (1000BASE-LHX, Single-mode, 50km) - DEM-315GT (1000BASE-ZX, Single-mode, 80km) - DEM-330T/R (Gigabit WDM transceiver, Single-Mode 10km) - DEM-331T/R (Gigabit WDM transceiver, Single-Mode 40km) - DEM-211 (100Base-FX, Multi-mode, 2km) - DEM-210 (100Base-FX, Single-mode, 15km) - DEM-220T (100Base-FX WDM transceiver, Single-mode, 20km) - DEM-220R (100Base-FX WDM transceiver, Single-mode, 20km) Compliant to following standards: - IEEE 802.3z compliance - IEEE 802.3u compliance - Support Auto-negotiation for Full-Duplex/ flow control operations DGS-1210-28P SFP Transceivers Supported: - DGS-712 1000BASE-T Copper SFP Transceiver - DEM-310GT (1000Base-LX, Single-mode, 10km) - DEM-311GT (1000Base-SX, Mutli-mode, 550m) - DEM-312GT2 (1000Base-SX, Multi-mode, 2km) - DEM-314GT (1000BASE-LHX, Single-mode, 50km) - DEM-315GT (1000BASE-ZX, Single-mode, 80km) - DEM-330T/R (WDM transceiver, Single-Mode 10km) - DEM-331T/R (WDM transceiver, Single-Mode 40km) Compliant to following standards: - IEEE 802.3z compliance - Support Auto-negotiation for Full-Duplex/ flow control operations |

  - PoE Features:

| PoE Features (For DGS-1210-10P/28P) |  |  |  |  |
| --- | --- | --- | --- | --- |
| PoE standard | DGS-1210-10P port 1~8 Support IEEE 802.3af and IEEE802.3at DGS-1210-28P Port 1~4 support IEEE 802.3at and port 5~24 Support IEEE 802.3af |  |  |  |
| PoE Capable Ports | DGS-1210-10P Port 1~8 can output up to 30W DGS-1210-28P Port 1~4 can output up to 30W , Port 5~24 can output up to 15.4W. |  |  |  |
| Power budget | DGS-1210-10P: At least 78Watts DGS-1210-28P: At least 185Watts |  |  |  |
| PoE Specification | - IEEE 802.3af and 802.3at compliance - Auto discover the connection of PD and immediately provides power - Auto disable port if the output power is over 16.2W (+/- 2%) in 802.3af, or over 31.2W (+/- 2%) in 802.3at. - The maximum power used by power devices is defined by the following classification. When Port works in Auto Mode, the output port power limit will be associated with PD Classification Value. If the PD supports LLDP-MED that claims itself as 802.3at compliant, then Port will output power up to 30W automatically. |  |  |  |
|  | Class | Usage | Minimum Power Levels Output at the PSE | Maximum Power Levels at the Powered Device |
|  | 0 | Default | 15.4W | 0.44 to 12.95W |
|  | 1 | Optional | 4.0W | 0.44 to 3.84W |
|  | 2 | Optional | 7.0W | 3.84 to 6.49W |
|  | 3 | Optional | 15.4W | 6.49 to 12.95W |
|  | 4 | Optional | 30W | 12.95W to 25.5W |
|  | Note: Class 4 only for 802.3at - Feeding power over pin 1, 2, 3, 6 (PSE Alternative A) of 8 wires of UTP cables |  |  |  |

  - Product Features:

| Features |  | Detailed Description |  |  |  |  | IETF Standard |  | Release Schedule |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L2 Features |  |  |  |  |  |  |  |  |  |
| MAC Address |  | 1. 16K MAC Address Table 2. Supports 256 static MAC 3. MAC Learning Control - Minimum granularity 1 MAC - Based on port, VLAN |  |  |  |  |  |  | R3.0 |
| Flow Control |  | 1. 802.3x when full duplex 2. Back Pressure when half duplex 3. Head-of-line blocking prevention |  |  |  |  |  |  | R3.0 |
| Spanning Tree |  | 1. Support 802.1D STP 2004 edition 2. Support 802.1w RSTP 3. Support Root Restriction(defined in 802.1Q-2005) |  |  |  |  |  |  | R3.0 |
| Loopback Detection |  | 1. D-Link LBD v.4.03 2. STP Independent 3. Support per port/per VLAN shutdown, max 8 VLAN |  |  |  |  |  |  | R3.0 |
| IEEE 802.3ad Link Aggregation |  | DGS-1210-10P Supports max 5 groups per device and 8 ports per group DGS-1210-20 Supports max 10 groups per device and 8 ports per group DGS-1210-28/28P Supports max 14 groups per device and 8 ports per group DGS-1210-52 Supports max 26 groups per device and 8 ports per group |  |  |  |  |  |  | R3.0 |
| Mirroring |  | 1. Port Mirroring 2. Support one mirroring group 3. Mirroring supports following mode: - One-to-one mode - Many-to-one mode 4. Supports Mirroring for Tx/Rx/Both |  |  |  |  |  |  | R3.0 |
| Jumbo Frame |  | Up to 10,000Bytes |  |  |  |  |  |  | R3.0 |
| LLDP |  |  | LLDP allows the switch to advertise its identification, configuration, and capabilities to neighboring devices. |  |  |  |  |  | R3.0 |
| LLDP-MED |  |  |  |  |  |  |  |  | R3.0 |
| IPv6 Neighbor Discovery (ND) |  |  | 1. Max TBD ND entries 2. Support up to TBD static ND entries |  |  |  |  |  | R3.10 |
| L2 Multicast |  |  |  |  |  |  |  |  |  |
| L2 Multicast Filtering |  |  | Support L2 multicast filtering modes as below: - Forward all unregistered groups - Filter all unregistered groups |  |  |  |  |  | R3.0 |
| IGMP Snooping |  |  | 1. Supports IGMP v1,v2 snooping 2. Supports 256 IGMP snooping groups 3. Supports at least 64 static multicast groups 4. Per VLAN IGMP snooping 5. Be able to forward the multicast traffic from the multicast-enabled router 6. Support VLAN based IGMP snooping fast leave 7. Supports IGMP snooping querier |  |  |  |  |  | R3.0 |
| VLAN |  |  |  |  |  |  |  |  |  |
| 802.1Q |  | Support 802.1Q 2005 edition |  |  |  |  |  |  | R3.0 |
| VLAN Group |  | 1. Total 256 VLAN groups 2. Configurable VID from 1~4094 3. TPID: 0x8100 |  |  |  |  |  |  | R3.0 |
| Management VLAN |  |  |  |  |  |  |  |  | R3.0 |
| Asymmetric VLAN |  |  |  |  |  |  |  |  | R3.0 |
| Voice VLAN |  | Support Auto Voice VLAN v2.1 |  |  |  |  |  |  | R3.0 |
| Auto Surveillance VLAN |  | Support Auto Surveillance VLAN v1.3 |  |  |  |  |  |  | R3.0 |
| ACL (Access Control List) |  |  |  |  |  |  |  |  |  |
| Maximum ACL entries |  | ACL Entries: - DGS-1210-10P/20/28/28P: Max 50 ingress ACL profiles, 200 ingress ACL rules. Each rule can be associated to a single port, or multiple ports. - DGS-1210-52: Max 50 ingress ACL profiles, 450 ingress ACL rules |  |  |  |  |  |  | R3.0 |
| ACL Policy |  | Support following ACL policy packet contents: - MAC address - Ethernet Type - IPv4 address - ICMP - IGMP - TCP/UDP port number |  |  |  |  |  |  | R3.0 |
| ACL Actions |  | The ACL supports following actions after analyzing packets: - Permit - Deny |  |  |  |  |  |  | R3.0 |
| QoS(Quality Of Service) |  |  |  |  |  |  |  |  |  |
| Number of Queue |  | Four Queues: Class 0 is Low queue Class 1 is Middle queue (Default) Class 2 is High queue Class 3 is Highest queue |  |  |  |  |  |  | R3.0 |
| Priority Queue Mapping |  | 1. Be able to prioritize packets according to: - 802.1p priority - Differentiated Services Code Point (DSCP) 3. Four Classes mapping to 802.1p value: Class 0: 1,2 Class 1: 0,3 (Default) Class 2: 4,5 Class 3: 6,7 |  |  |  |  |  |  | R3.0 |
| Queue Handling |  | Support following mode - Strict - Weighted Round Robin (WRR) |  |  |  |  |  |  | R3.0 |
| Bandwidth Control |  | 1. Supports Port-based bandwidth control 2. Minimum granularity for 10/100Mbps ports: 64Kb/s 3. Minimum granularity for 10/100/1000Mbps ports: 64Kb/s 4. Ingress / Egress bandwidth control |  |  |  |  |  |  | R3.0 |
| AAA |  |  |  |  |  |  |  |  |  |
| 802.1X |  | 1. Support 1 RADIUS server 2. Support Port-based Access Control |  |  |  |  |  |  | R3.0 |
| Security |  |  |  |  |  |  |  |  |  |
| Broadcast/Multicast/Unicast Storm Control |  | 1. Be able to set up a storm threshold. When broadcast /multicast/unicast or multicast traffic hit the defined threshold, the switch will start blocking or discarding packets, otherwise, the packets will 'flood' the network. The blocked port will be activated again only when the broadcast/multicast/unicast traffic falls below the defined threshold. 2. Min granularity: 64Kb/s |  |  |  |  |  |  | R3.0 |
| Traffic Segmentation |  |  |  |  |  |  |  |  | R3.0 |
| D-Link Safeguard Engine |  | Protects CPU from Broadcast / Multicast / Unicast flooding |  |  |  |  |  |  | R3.0 |
| Port Security |  | - Support 64 MACs per port |  |  |  |  |  |  | R3.0 |
| DHCP Server Screening |  | Able to configure up to 5 DHCP server IP addresses. |  |  |  |  |  |  | R3.0 |
| ARP Spoofing Prevention |  | The maximum 64 entries |  |  |  |  |  |  | R3.0 |
| Smart Binding (IP-MAC-Port Binding, IMPB) |  | 1. Support D-Link IMPB v3.91 2. Binding address per device: 256 for ARP inspection, 128 for IP inspection. 3. Support ARP inspection as default, IP inspection as option. 4. Support DHCP Snooping 5. able to scan connected devices in a range of IP address or VLAN, max 256 devices block list : 500 entries |  |  |  |  |  |  | R3.0 |
| SSL |  | Supports v1/v2/v3 |  |  |  |  |  |  | R3.0 |
|  |  | Support IPv6 |  |  |  |  |  |  | R3.10 |
| OAM |  |  |  |  |  |  |  |  |  |
| Cable Diagnostics |  |  |  | Support cable diagnostic function to check the status of connected RJ-45 cables. |  |  |  |  | R3.0 |
| Factory Reset |  |  |  | Support factory reset function; reset the system to its default configuration |  |  |  |  | R3.0 |
| Management |  |  |  |  |  |  |  |  |  |
| Web-based GUI |  | - Support following 10 languages - English (Default) - Simplified-Chinese - Traditional-Chinese - French - Italian - Japanese - Russian - Spanish - Portuguese - German - Compatible with following browsers - IE6 or later version - Firefox 3.0 or later version - Chrome 5.0 or later version - Safari 4.0 or later version - Netscape 8 or later version - Opera 10 or later version |  |  | RFC2068, 2616 |  |  |  | R3.0 |
|  |  | - Support IPv6 |  |  |  |  |  |  | R3.10 |
| CLI |  | Supports following 12 commands: - Ping: Sending Echo message - Show switch: Display the device information - Save: Save the device’s configurations - Configures IP Address - Show IPIF - Upload firmware and configuration file via TFTP - Download firmware and configuration file via TFTP - Reset config: Reset the device to factory default - Reboot: Power cycles the device - Logout: Log out from current session - Password: Change admin’s password - Debug info command |  |  |  |  |  |  | R3.0 |
| Telnet Server |  | Maximum connection is 4 |  |  | RFC854 |  |  |  | R3.0 |
|  |  | Support IPv6 |  |  |  |  |  |  | R3.10 |
| TFTP Client |  | Support IPv4 |  |  | RFC783 |  |  |  | R3.0 |
|  |  | Support IPv6 |  |  |  |  |  |  | R3.10 |
| Configurable MDI/MDIX |  |  |  |  |  |  |  |  | R3.0 |
| SNMP |  | Supports v1/v2c/v3 |  |  | RFC1157, 1901, 1908, 2570, 2575 |  |  |  | R3.0 |
| SNMP Trap |  | SNMP Trap supports following events: - SNMP Authentication Traps - Device Bootup - Port Link Up / Link Down - RSTP Port State Change - Firmware Upgrade State - PoE Power On / Off - PoE Power Error - PoE over max power budget |  |  |  |  |  |  | R3.0 |
| Trap for SmartConsole Utility |  | SmartConsole Utility can receive the following trap events: - Illegal Login - Device Bootup - Port Link Up / Link Down - RSTP Port State Change - Firmware Upgrade State - PoE Power On / Off - PoE Power Error - PoE over max power budget |  |  |  |  |  |  | R3.0 |
| System Log |  | Max 500 log Entries |  |  |  |  |  |  | R3.0 |
|  |  | Support IPv6 |  |  |  |  |  |  | R3.10 |
| BootP/DHCP Client |  | - Support IPv4 |  |  | RFC951, 1542 |  |  |  | R3.0 |
|  |  | - Support IPv6 |  |  |  |  |  |  | R3.10 |
| SNTP |  | Time setting |  |  |  |  |  |  | R3.0 |
|  |  | Support IPv6 |  |  |  |  |  |  | R3.10 |
| User Account |  | Maximum 4 concurrent session |  |  |  |  |  |  | R3.0 |
| Trusted Host |  | Support up to 10 trusted host ranges |  |  |  |  |  |  | R3.0 |
|  |  | Support IPv6 |  |  |  |  |  |  | R3.10 |
| Upload/Download Configuration file |  | Via HTTP/TFTP |  |  |  |  |  |  | R3.0 |
| Backup/Upgrade firmware |  | Via HTTP/TFTP |  |  |  |  |  |  | R3.0 |
| Smart Wizard |  | Pop up a setting wizard page while first time login |  |  |  |  |  |  | R3.0 |
| DHCP Auto-Configuration |  |  |  |  |  |  |  |  | R3.0 |
| ICMPv6 |  |  |  |  |  |  |  |  | R3.10 |
| RMON |  | Support RMON v1 group 1, 2, 3, 9 |  |  |  |  |  |  | R3.0 |
| IPv4/IPv6 Dual Stack |  |  |  |  |  |  |  |  | R3.10 |
| MIB |  |  |  |  |  |  |  |  |  |
| MIB II |  |  |  |  |  |  | RFC1213 |  | R3.0 |
| Bridge MIB |  |  |  |  |  |  | RFC1493 |  | R3.0 |
| SNMPv2 MIB |  |  |  |  |  |  | RFC1907 |  | R3.0 |
| MIB Traps Convention |  |  |  |  |  |  | RFC1215 |  | R3.0 |
| Interface Group MIB |  |  |  |  |  |  | RFC2233 |  | R3.0 |
| Private MIB |  |  |  |  |  |  |  |  | R3.0 |
| POWER-ETHERNET-MIB |  | For DGS-1210-10P/28P |  |  |  |  | RFC3621 |  | R3.0 |
| LLDP-MIB |  |  |  |  |  |  | IEEE 802.1AB-2005 |  | R3.0 |
| Green |  |  |  |  |  |  |  |  |  |
| Intuitive Power Saving (D-Link Green v1.0) | 1. Support power saving function on all Gigabit RJ-45 ports 2. Support 2 power saving mode: - Power Saving Link status - Power Saving Cable length |  |  |  |  |  |  | R3.0 |  |
| Advanced Power Saving (D-Link Green v3.0) | Support 5 power saving features - Power saving by Time-based PoE: PoE ports can be turned on/off by port or system through schedule - Power Saving by LED Shut-Off: Powered LEDs can be turned on/off by system through schedule - Power Saving by Port Shut-Off: Each port on the system can be turned on/off by schedule. - Power Saving by Port Standby: Each port on the system enters sleep state by schedule. - Power Saving by System Hibernation: System enters hibernation by schedule. In this mode, switches get most power-saving figures since main chipsets (both MAC and PHY) are disabled for all ports, and energy required to power the CPU is minimal. |  |  |  |  |  |  | R3.0 |  |
| EEE (Energy Efficient Ethernet) |  |  |  |  |  | IEEE 802.3az |  | R3.0 |  |
| 3rd party certification |  |  |  |  |  |  |  |  |  |
| IPv6 Core Ready Logo phase II |  |  |  |  |  |  |  | R3.10 |  |

2. Mechanism & ID Design:

2.1 Name Plate Design

DGS-1210-10P

DGS-1210-20

DGS-1210-28

DGS-1210-52

DGS-1210-28P

2.2 Rear Panel Design

DGS-1210-10P

TBD

DGS-1210-20

DGS-1210-28

DGS-1210-52

DGS-1210-28P

TBD

  - LED Indicator (For DGS-1210-20/28/52)

| - Location | - LED Indicative | - Color | - Status | - Description |
| --- | --- | --- | --- | --- |
| - Per Device | - Power | - Green | - Solid light | - Power On |
|  |  |  | - Blink | - Switch is performing a system self-test. |
|  |  |  | - Light off | - Power Off |
|  | - Fan - (DGS-1210-28P/52) | - Green | - Solid light | All diagnostics pass. The fan is operation well. |
|  |  | - Red | - Solid light | - The fan has runtime failure and is brought offline. |
| - LED Per 10/100/1000 Mbps Port | - Link/Act/Speed Mode | - Green | - Solid light | - When there is a secure connection (or link) to 1000Mbps Ethernet device at any of the ports. |
|  |  |  | - Blinking | - When there is reception or transmission of data occurring at 1000Mbps. |
|  |  | - Amber | - Solid light | - When there is a secure connection (or link) to 10/100Mbps Ethernet device at any of the ports. |
|  |  |  | - Blinking | - When there is reception or transmission of data occurring at 10/100Mbps. |
|  |  | - Off | - Light off | - No link |
| - LED Per SFP Port | - Link/Act | - Green | - Solid light | - When there is a secure connection (or link) to 1000Mbps Ethernet device at any of the ports. |
|  |  |  | - Blinking | - When there is reception or transmission of data occurring at 1000Mbps. |
|  |  | - Amber | - Solid light | - When there is a secure connection (or link) to 100Mbps Ethernet device at any of the ports. |
|  |  |  | - Blinking | - When there is reception or transmission of data occurring at 100Mbps. |
|  |  | - Off | - Light off | - No link |

  - LED Indicator (For DGS-1210-10P/28P)

| - Location | - LED Indicative | - Color | - Status | - Description |
| --- | --- | --- | --- | --- |
| - Per Device | - Power | - Green | - Solid light | - Power On |
|  |  |  | - Blink | - Switch is performing a system self-test. |
|  |  |  | - Light off | - Power Off |
|  | - PoE Max | - Red | - Solid light | - When the total PoE output power achieve max power budget. |
|  |  |  | - Light off | - System still have spare power for new PD |
|  | - Port LED Indicator | A LED select button to switch two mode in turn for all 10/100/1000Mbps ports on DGS-1210-10P/28P: - Link/Act/Speed Mode - - PoE Mode |  |  |
| - LED Per 10/100/1000 Mbps Port | - Link/Act/Speed Mode | - Green | - Solid light | - When there is a secure connection (or link) to 1000Mbps Ethernet device at any of the ports. |
|  |  |  | - Blinking | - When there is reception or transmission of data occurring at 1000Mbps. |
|  |  | - Amber | - Solid light | - When there is a secure connection (or link) to 10/100Mbps Ethernet device at any of the ports. |
|  |  |  | - Blinking | - When there is reception or transmission of data occurring at 10/100Mbps. |
|  |  | - Off | - Light off | - No link |
|  | - PoE Mode | - Green | - Solid light | - Power feeding |
|  |  | - Amber | - Solid light | - Error condition |
|  |  | - Off | - Light off | - No Power feeding |
| - LED Per SFP Port | - Link/Act | - Green | - Solid light | - When there is a secure connection (or link) to 1000Mbps Ethernet device at any of the ports. |
|  |  |  | - Blinking | - When there is reception or transmission of data occurring at 1000Mbps. |
|  |  | - Amber | - Solid light | - When there is a secure connection (or link) to 100Mbps Ethernet device at any of the ports. |
|  |  |  | - Blinking | - When there is reception or transmission of data occurring at 100Mbps. |
|  |  | - Off | - Light off | - No link |

###### 3. Physical & Environment

3.1 AC input

100-240 VAC, 50/60Hz, Internal universal power supply

- DGS-1210-10P: AC LINE 100-240VAC 50/60HZ TBD MAX

- DGS-1210-20: AC LINE 100-240VAC 50/60HZ TBD MAX

- DGS-1210-28: AC LINE 100-240VAC 50/60HZ TBD MAX 
DGS-1210-52: AC LINE 100-240VAC 50/60HZ TBD MAX

- DGS-1210-28P: AC LINE 100-240VAC 50/60HZ TBD MAX

3.2 Operation Temperature

- DGS-1210-20/28/52/28P: -5 - 50°C

- DGS-1210-10P: 0-40°C

3.3 Storage Temperature

-20-70°C

3.4 Humidity

- Operation: 0% - 95% RH

- Storage: 0% ~ 95% RH

- Environment Requirement

  - Surge/Lightening Protection

The device supports circuit protection in AC power side which complies with the standard of IEC 61000-4-5 Class 3.

- Product Development Guideline

  - Emissions & Safety

- 5.1.1 EMI/EMC Certificates and Test Reports

| Required | EMI/EMC Test Report | Class A | Class B | Region |
| --- | --- | --- | --- | --- |
| V | CE Report (EMC Directive of 2004/108/EC) for ITE EN 55022 & EN 55024 EN 61000-3-2 EN 61000-3-3 EN 61000-4-2/3/4/5/6/8/11(ESD/RS/FET/Surge/CS/Magentic/Dip) | V |  | European Union |
| V | FCC report FCC CFR 47 Part 15 Subpart B & ANSI C63.4 (including ITE & Home Plug) IC report ICES-003 (ITE) | V |  | US Canada |
| V | C-Tick Report AS/NZS CISPR 22 (ITE) | V |  | New Zealand & Australia |
| V | VCCI Report V-3 & V-4 | V |  | JAPAN |
| V | BSMI | V |  | Taiwan |
| V | CCC (DGS-1210-20/28/52) | V |  | China |

  - Safety Certificates and Test Reports

| Required | Certifications | Standard No. | Region |
| --- | --- | --- | --- |
| V | cUL Listed Mark UL file number should be under vendor own | UL/CSA 60950-1 | North/ South America |
| V | CE (LVD report) | EN60950-1 | European Union |
| V | CCC (DGS-1210-20/28/52) | GB4943-2001, GB9254-1998, GB17625.1-2003 | China |

  - Reliability Test

| No. | Required | Test Items |
| --- | --- | --- |
| 1. | V | MTBF of Prediction Report |
| 2. | V | Endurance test Report |
| 3. | Option | MTBF of Demonstration test Report |
| 4. | V | Free Fall(Drop)Test Report |
| 5. | V | Random Vibration Test Report |
| 6. | V | Storage Test Report |
| 7. | V | Operation Cold(low temperature) Test |
| 8. | V | Operation Dry Heat(High temperature) Test |
| 9. | V | Operation Temperature Cycles Test |
| 10. | V | Thermal Shock Test |
| 11. | V | Damp Heat Steady State test |
| 12. | V | Thermal Profile Test |
| 13. | V | ESD Simulation Test report |
| 14. | V | High / Low Temperature Start Test |
| 15. | Option | Parts on/off & Insert/pulling Test |
| 16. | V | Acoustic Noise test of Sound Pressure (with fan product) |
|  | V | Acoustic Noise test of Sound Quality (with fan product) |
|  | V | Acoustic Noise test for (Fan-less Product) |

  - RoHS

    - Level A of Substance limitation requirements table (for appliances & accessories)

| Level | D-Link No. | Description |  | Limitation/ ppm |
| --- | --- | --- | --- | --- |
| ### A | A1 | 鎘及其化合物 | Cadmium/ Cadmium Compounds | 80 |
|  | A2 | 六價鉻及其化合物 | Hexavalent Chromium/ Hexavalent Chromium Compounds | 800 |
|  | A3 | 鉛及其化合物 | Lead/ Lead Compounds | 800 |
|  | A4 | 汞及其化合物 | Mercury/ Mercury Compounds | 800 |
|  | A5 | 聚溴聯苯 | Polybrominated Biphenyls (PBBs) | 800 |
|  | A6 | 聚溴聯苯醚 | Polybrominated Diphenylethers (PBDEs) | 800 |

    - Level A of Substance limitation requirements table (for Packing)

| Level | D-Link No. | Description |  | Limitation/ ppm |
| --- | --- | --- | --- | --- |
| A | A1 ~ A4 | 鎘, 六價鉻, 鉛, 汞 | Cadmium Chromium VI, Lead and Mercury compounds | 100 |

    - Level A of Substance limitation requirements table (for battery)

| Level | D-Link No. | Description |  | Limitation/ ppm |
| --- | --- | --- | --- | --- |
| A | A1 | 鎘及其化合物 | Cadmium/ Cadmium Compounds(Cd) | 2500 (0.025%) |
|  | A3 | 鉛及其化合物 | Lead/ Lead Compounds(Pd) | 400 (0.4%) |
|  | A4 | 汞及其化合物 | Mercury/ Mercury Compounds(Hg) | 5 |