DMC-805X Rev.A1

Date: Feb. 2012

| Product Name | # 10G CX4 to 10G SFP+ Media Converter |  |  |
| --- | --- | --- | --- |
| Revision | A1 |  |  |
| Picture: |  |  |  |
| Why Sell This Product | The DMC-805X comes with a 10Gbps SFP+ port and a 10Gbps CX4 ports which can be used as Stand-Alone converter or installed in rack-mount chassis (DMC-1000) as module type converter. It is a high performance 10G media converter which is flexible for distance extension on a server equipped with CX4 NIC or previous 10G switch built with 10G CX4 ports. Selling Points: - Converts 10G CX4 signals to 10G fiber signals - Can be used as Stand-Alone converter or installed in rack-mount chassis (DMC-1000) as module type converter. - Small & portable in size - Wall-mountable - Silent fan-less design - Adopts Level V power adapter - ErP & RoHS compliance |  |  |
| Key Features | Key Features: Feature; Detailed Description. DIP Switch; Bit 1: Off: Auto mode(default) On: Force mode(for 10G fiber) Bit 2: Off: LLR disable(default) On: LLR enable. LLR; Default setting is enabled Bit2 of DIP switch turn to on, LLR enable. LLCF; Always enable. Module type support; Yes. The DMC-805X can be plugged in 16 slot chassis (DMC-1000) and managed by a management module (DMC-1002).. Environment: Operation Temperature: - 0 C ~ 40 ºC (32F ~ 104F) Storage Temperature: - -10 ~ 70°C (14 ~ 158º F) Humidity - Operating: 10% ~ 90%, - Storage: 5% ~ 90%. |
| Technical Detail | General Feature; Detailed Description. Housing; 120 x 88 x 25mm. Power; DC 12V/0.5A. Fan; Fanless design.. MTBF; Follow D-Link’s regulation. Switching Capacity; 20Gbps. Max. Forwarding Rate; 10GBASE- SR/ER/LR/LRM/LR: 1,488,0000 pps per port. Forwarding Mode; Store and Forward. Port Functions. LAN; 1 x 10GE CX4 port Compliant to following standards: - IEEE 802.3ak compliance - Support Full-Duplex operations. Fiber; 1 x 10GE SFP+ port Compliant to following standards: - IEEE-802.3ae compliance - IEEE-802.3aq compliance. |
| EMI/EMC Report |  FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  ICES-003 No.  Others |  |  |
| Safety |  TUV No.  UL No.  CSA No.  LVD No.  CE  CDRH  CB |  |  |
| Packaging Style | Clamshell  White Box  Gift Box Bulk  Brown Box Others |  |  |
| Packaging Contents | - One DMX-805X - One external power adapter - Quick Installation Guide |  |  |
| Country of Origin | China |  |  |
| UPC Code | UPC; Part Number. 790069; 377907; IMC805XZ.....A1G. 790069; 377532; TMC805XA.....A1G. 790069; 377525; NMC805XC.....A1G. 790069; 377501; EMC805XB.....A1G. 790069; 377518; EMC805XE.....A1G. 790069; 377549; IMC805XA.....A1G. 790069; 377556; IMC805XB.....A1G. 790069; 377563; IMC805XE.....A1G. 790069; 377570; IMC805XN.....A1G. 790069; 377495; BMC805XA.....A1. 790069; 377587; RMC805XA1A...A1G. |
| Units Per Carton: | For WW | For EU | For USA |
|  | 10pcs/Carton | 10pcs/Carton | 5pcs/Carton |
| Cartons Per Pallet: | For WW | For EU | For USA |
|  | 330pcs/Carton(Air) 440pcs/Carton(sea) | 440pcs/Carton(Air) 440pcs/Carton(sea) | 260pcs/Carton(Air) 390pcs/Carton(sea) |
| Dimensions and Weight: | Box:  237x168x36mm (Type A)  168x237x53mm (Type A53)  225x150x75mm (Type B) 150x225x55mm (Type B1)  269x162x65mm (Type C)  274x208x68mm (Type D)  208x274x95mm(Type D95)  208x137x95mm (Type D95 half)  274x208x118mm (Type D1)  274x208x138mm (Type D2)  208x274x190mm (Type D4)  288x245x66mm (Type E)  245x285x82mm(Type E1) 245x285x87mm(Type E2)  256x356x97mm (Type F) 137x 208x 36mm (Type G)  208x250x68mm (Type H) 253x 274x247mm (Type I) Others (138x75x50mm) Others (120 x 88 x 25mm) Device (mm / g): Device Depth; Device Width; Device Height; Device Weight. DMC-805X; 119; 880; 250; 0.285. Carton (cm/ kg): Carton; Carton Width; Carton Height; Carton Gross Weight. Depth. DMC-805X; 370; 491; 452; 7.391. Pallet (cm/ kg): Carton per Air Pallet; Air Pallet PCS; Air Pallet Length; Air Pallet Width; Air Pallet Height. DMC-805X; 33; 330; 110; 120; 194.3. Carton per Sea Pallet; Sea Pallet PCS; Sea Pallet Length; Sea Pallet Width; Sea Pallet Height. DMC-805X; 44; 440; 110; 120; 194.3. DMC-805X INTRASTAT weight: 0.2950 kg Packing material weight (Box): 0.6690kg - Paper: 0.3620kg - Plastic: 0.0110 kg - Mixed: 0.0010 kg |