Model Name:

DES-1005P Rev.A1

5-Port 10/100BASE-TX with one port PoE Unmanaged Switch

# [Metal Case]

Version 1.0

By: Abbie Lin

Ethernet Switch Product Dept.

2011/10/07

This document contains confidential proprietary information and is the property of D-Link Corporation. The contents of this document may not be disclosed to unauthorized persons without the written consent of D-Link Corporation.

DES-1005P Specification Revision History

| Version | Revised Date | # Person Name | # Content Revised |
| --- | --- | --- | --- |
| 1.00 | 2011/10/07 | Abbie Lin | First release |

Product Descriptions

DES-1005P, 5 10/100 Switch with One PoE port is a new member of D-Link Unmanaged PoE switches. DES-1005P features One 10/100BASE-TX port that supports IEEE 802.3af PoE protocol.  It supplies up to 15.4 Watts allowing users to attach an 802.3af compliant device to the DES-1005P without requiring additional power. SOHO users not only can enjoy the main benefits having a 5 port network switch, but with 1 of those 5 ports being POE enabled they can easily create a home surveillance system with minimal clutter even with limited power sockets. The US market has not seen many products of this nature and will allow us to tap into a demographic that wants more than just a regular 5 port switch. Adding one port POE to a 5 port switch also provides SMB users an alternative to larger, more expensive switches, without sacrificing the need to power POE devices such as AP’s and IP surveillance.

- Product Specifications:

  - Hardware Specifications:

| General Feature | Detailed Description |  |
| --- | --- | --- |
| Dimension | 140mm(W) x 85mm(D) x 28mm(H) (5.5 x 3.3 x 1.2 inches) |  |
| Switching Capacity | 1Gbps |  |
| Max. Forwarding Rate | 10BASE-T : 14,880 pps per port 100BASE-TX : 148,800 pps per port |  |
| Forwarding Mode | Store and Forward |  |
| MAC Address | 2K |  |
| Packet Buffer memory | 64KBytes |  |
| QoS | - 802.1p QoS 4 queues 802.1p priority to COS mapping table: Priority 2 & 1 = COS0 Priority 3 & 0 = COS1 Priority 5 & 4 = COS2 Priority 7 & 6 = COS3 DSCP to COS mapping table: DSCP 0-15 = CoS0 DSCP 16-31 = CoS1 DSCP 32-47 = CoS2 DSCP 48-63 = CoS3 Port-Based QoS. The PoE port is assigned to highest priority Strict Priority Mode: COS3 > COS2 > COS1 > COS0 If a packet contains 802.1p and DSCP, DSCP packet has higher priority than 802.1p. |  |
| Eco Design | - D-Link Green: - Power Saving by Link Status - Power Saving by Cable Length - EEE: IEEE 802.3az EEE |  |
|  | EuP compliance - On/Off switching on device |  |
| Acoustic | 0 dBs |  |
| Fan | Fan-less |  |
| Spec. of Reserved Multicast addresses | 01-80-C2-00-00-03 - IEEE 802.1X PAE address | Pass Through |
|  | 01-80-C2-00-00-00 - BPDU | Pass Through |
|  | 01-80-C2-00-00-0E - LLDP | Pass Through |
|  | 01-80-C2-00-00-02 - 802.3ad LACP | Pass Through |
| Others | IPv6 | Pass Through |
| Port Functions |  |  |
| 5 10/100 Ports | Compliant to following standards: - IEEE 802.3 compliance - IEEE 802.3u compliance - IEEE 802.3az EEE compliance - IEEE 802.3af for PoE port 1 - Support Half/Full-Duplex operations - Auto-negotiation - Auto MDI/MDIX - IEEE 802.3x Flow Control support for Full-Duplex mode, back pressure when Half-Duplex mode, and Head-of-line blocking prevention |  |

  - PoE Features:

| PoE Features |  |
| --- | --- |
| PoE standard | Supports IEEE 802.3af |
| PoE Capable Ports / PoE Watts per port | Port one. |
| Watts available for PoE (Power Budget) | 15.4 Watts |
| PoE Specification | - Supplies power to PD device, meeting IEEE802.3af standards. It is able to provide power to PD devices more sufficiently. - Auto discovery feature, automatically recognize the connection of PD device and immediately sends power to it - Active circuit protection, automatically disables the port if there is a short while other ports remain active - Auto disable port via two different methods as below: a. Over Current Protection - 802.3af mode: if the current is over 375mA while other ports remain active, the port will be auto disabled. b. Over Output Power Limit - 802.3af mode: if the port is over 15.4W while other ports remain active, the lower priority port will be auto disabled. - DES-1005P should follow the standard PSE pin-out standard of Alternative A which is sending out power over number 1,2,3,6 pins of 8 wires of CAT5 UTP cable - DES-1005P works with all D-Link 802.3af devices - DES-1005P works with all non-802.3af capable D-Link AP, IP Cam and IP phone via DWL-P50 |

2. Mechanic & ID Design

  - Nameplate Design

  - LED Indicators

| - LED Indicative | - Color | - Status | - Description |
| --- | --- | --- | --- |
| - Power - (per device) | - Green | - Solid Light | - Power On |
|  |  | - Light off | - Power Off |
| - Link/Act - (per port) | - Green | - Solid Light | - When there is a connection (or link) to device at any of the ports. |
|  |  | - Blinking | - When there is reception or transmission (i.e. Activity--Act) of data occurring at a port. |
|  |  | - Off | - No link. |

  - PoE LED Indicators

| Location | LED Indicative | Color | Status | Description |
| --- | --- | --- | --- | --- |
| PoE Port | PoE Status | Green | Solid Light | When the PoE powered device (PD) is connected and the port supplies power successfully. |
|  |  | Green | Blinking | Overload/Short circuit occurred. PSE will turn off power and blink the LED 2 seconds. After 2s, LED will be off. PSE will auto-recovery within 5s after turn off power. |
|  |  | Off | Light Off | No Powered Device (PD) connected. |

3. Physical & Environment

3.1 External Power Adapter

- Switching 48V/0.5A Level “V” Power Adapter

3.2 Power Consumption: [Max. Power Consumption, which is measured at 240V input]

     Power On (Standby):

     DC input: 0.9 Watts

     AC input: 1.8 Watts

     Maximum: (PoE off. No Powered Devices connected)

     DC input: 2.05 Watts

     AC input: 2.7 Watts

     Maximum: (PoE on. While consuming all 15.4W power budget)

     DC input: 17.29 Watts

     AC input: 20.2 Watts

3.3 Heat Dissipation: [Max. heat dissipation, which is measured at 240V input]

- Power On (Standby):

- AC input: 6.14 BTU/h

- Max.:

- AC input: 9.08 BTU/h

- AC input with PoE on: 68.92 BTU/h

3.4 MTBF:

- 324,616 hours

3.5 Operation Temperature:

- 0 - 40°C

3.6 Storage Temperature:

- - 40 ~ 70°C

3.7 Humidity

- Operating: 0% ~ 95%,

- Storage: 0% ~ 95%

- Emission (EMI) and Safety Certifications:

- EMI: FCC Class B, ICES-003 Class B

- Safety: cUL/UL