D-Link Corporation

External Specification

Full HD Vandal-Proof Fixed Dome Network Camera

DCS-6112V ver. A1

Version 0.91

Date: 2011/9/16

DCS-6112V External Spec Revision History

| Version | Revised Date | # Person Name | # Content Revised |
| --- | --- | --- | --- |
| 0.91 |  | Carrie Lee | First edition |

- Introduction

- DCS-6112V is an indoor Full HD Vandal-Proof Fixed Dome Network Camera for professional surveillance and security. This camera adopts 2 mega-pixel progressive scan CMOS sensor; the video stream covers a larger scene and better quality than an analog CCTV camera. This is a very important consideration in video surveillance applications, where a high resolution image can enable a criminal to be identified.

- Using an ARM SoC provides excellent real-time video quality. Multiple video streams with different configurations can be activated simultaneously to address the different requirements for live viewing and recording. Multicast video/audio stream helps to reduce the network transmission load. When multicast-enabled router is used, the network camera can serve multiple clients at the same time.

- The DCS-6112V includes digital input and output ports for connectivity to external devices such as IR sensors, switches, and alarm relays. It also incorporates Power over Ethernet (PoE), allowing it to be easily installed in a variety of locations without the need for supplemental power cabling.

- The included D-Link D-ViewCam™ is sophisticated software which allows users to manage up to 32 network cameras, set e-mail alert notifications, create recording schedules, and use motion detection to record directly to a hard drive. D-ViewCam™ also allows users to upload a floor plan to create a realistic layout of the premises where cameras are located, further simplifying the management process.

- Product Features

  - Features:

      - 1/2.7” megapixel CMOS sensor

      - Real-time H.264/ MPEG-4/ MJPEG

      - Max. resolution 1920x1080 at 15 fps, 720p at 30fps

      - Four (4) simultaneous streams

      - Built-in 4.0 mm, F1.5 fixed lens

      - ePTZ for virtual PTZ operation

      - Tamper detection

      - Motion detection

      - Alarm in/out

      - Privacy mask

      - Two Way audio support

      - Advanced event management

      - Advanced web user interface

      - HTML code example of streaming

      - IPv6, 802.1x, QoS, CoS

      - HTTPS secured streaming

      - 3GPP mobile surveillance

      - Samba client for NAS

      - Multiple access lists

      - Windows Vista LLTD

      - Power over Ethernet (PoE)

      - Equipped with MicroSD card slot

      - Vandal-proof IK10 criterion

      - Unified SDK

      - ONVIF compliant

  - H/W Features

    - Specification

| Key Components / Performance |  |
| --- | --- |
| Video Codec | H.264 / MPEG4 / MJPEG |
| PoE | Supported IEEE 802.3af standard |
| Sensor | 1/2.7” Full HD progressive CMOS sensor |
| SDRAM | 256 Mbytes |
| Flash Memory | 128 Mbytes |
| ICR | - |
| Lens | 4.0mm fixed lens , F1.5 |
| Camera View angle | Horizontal 77.4 degrees |
| IR LED | - |
| Port Functions |  |
| LAN | 10/100 BaseT ports x1 Compliant to following standards: IEEE 802.3 compliance IEEE 802.3u compliance Support Full-Duplex operations Supported IEEE 802.3af standard (PoE) |
| I/O connector | 1 digital input (DI), 1 digital output (DO) |
| External Microphone Input | Yes |
| Audio Output | Yes |
| Video System Switch | Select from NTSC / PAL for TV system |
| Reset Button | Reset to factory default |
| RS-485 | - |
| Power Consumption | Max. 4.4W |
| Power Supply | PoE, DC adaptor (input: 100~240AC, 50/60Hz, Output: 12VDC, 1.25A) |
| Dimension (WxHxD) | Ø130 xx 99.4mm |
| Weight | (TBC) |

    - LED indicators

| Location | LED Indicative | Color | Status | Description |
| --- | --- | --- | --- | --- |
| Per Device | Link/Act | Green | Off | Link Down |
|  |  |  | On | Link Up |
|  |  |  | Blinking | Traffic |
|  | Power | Red | on | power on |

  - Software Features

| Feature |  | Detailed Description |
| --- | --- | --- |
| Networking | Protocol | - IPv4, IPv6 - TCP/IP, UDP, ICMP - DHCP Client - NTP Client (D-Link) - DNS Client, DDNS Client (D-Link) - SMTP Client - FTP Client - HTTP / HTTPS - Samba Client - PPPoE - UPnP Port Forwarding - RTP / RTSP/ RTCP - IP filtering - 3GPP - LLTD - CoS - QoS - SNMP - IGMP - 802.1x |
|  | Ethernet | 10/100M BaseT Fast Ethernet auto negotiation |
| Video | Algorithm supported | H.264/MPEG4/MJPEG dual format compression simultaneously MJPEG for still image |
|  | Resolution | 1920x1080 (16:9) @15fps 1280x1024 (4:3) @20fps 1280x720 (16:9) @30fps 640x480 @30fps 320x240 @30fps 176x144 @30fps |
|  | AGC | Yes, on/off, adjustable |
|  | AWB | Yes, on/off, adjustable |
|  | AES | Yes, on/off, adjustable |
|  | BLC | Yes, on/off, adjustable |
|  | Electronic shutter | 1/5 ~ 1/32000 sec. |
|  | Motion Detection | 3 configurable motion detection windows |
|  | Tamper Detection | Yes |
|  | Privacy Mask | 5 configurable privacy masks |
|  | ePTZ | Virtual PTZ function |
|  | Digital Zoom | 4x |
|  | Image adjustment | Configurable Brightness, Saturation, Contrast |
|  | Flip | Vertical, Horizontal, Diagonal |
| SDK | NIPCA | D-Link unified SDK |
|  | ONVIF | Worldwide standard |

  - SYSTEM REQUIREMENTS

Operating System: Windows 2000, Windows XP, Windows Vista (32/64 bit), Windows 7(32/64 bit)

Browser: Internet Explorer 6 or above, Chrome, Firefox and Safari

  - 3GPP MOBILE SURVEILLANCE

Handsets with 3GPP player with following software support:

Packet Video Player 3.0 or above

QuickTime 6.5 or above

Real Player 10.5 or above

- Physical & Environment

  - Power

Power input

DC power adaptor (D-Link standard): Input: 100-240VAC, 50/60Hz; Output: 12VDC, 1.25A 24V AC

PoE, IEEE 802.3af standard

  - Operation Temperature

0 ~ 40℃ (32 to 104 ℉)

  - Storage Temperature

-20 ~ 70℃ (-4 to 158 ℉)

  - Humidity

90% RH non-condensing

- Emission (EMI), Safety & Other Certifications:

- CE (class A)

- CE LVD(EN60965-1)

- FCC (class A)

- ICES-003

- C-Tick

- Box Included

- Network Camera x 1

- 12V 1.25A External Power Adapter x 1

- CAT5 Ethernet cable x 1

- AV/Power connecting cable x 1

- Quick Installation Guide x 1

- Master CD x 1

- Screw driver

- Alignment sticker