# DSL-306D/DSL-306L External Specification    Ver.1

The two product difference is DSL-306D for G.dmt standard and DSL-306L for G.Lite standard. All specification is same. I will write the external specification together.

H/W specification

- Support 6 10 Base-T port.

- Support one RJ-11 interface for ADSL access.

- Use external power adapter (18VDC, 750ma)

- Support DB-9 connector for console and telnet.

- Total 11 LEDs: Power*1, System test *1, ADSL Link *1 ADSL Act * 1

LAN col*1(Org.), Link/Act per port *6. All LEDs are green

Except Col LED.

- Support 1 M flash for storing system Firmware.

- Total 6M SDRAM. 4M for CPU, 2M for DMT controller.

- Dimensions: 215mm x 163mm x 45mm

- Operating Temperature: 0 to 50 deg; C

- Relative Humidity: 5 to 95% non-condensing

- EMI: CE B,FCC class B

- Safety: UL/CSA,

Software Specification

- IP over ATM

- Bridge

- PPP over ATM

- RFC1483

- SNMP

- Flash-FS

- Bootp

- Support ATM signaling UNI 3.1