- URLFilter CLI commands

This chapter describes URLFilter CLI commands.

Summary

The URLFilter module when enabled, will check all outgoing packets with TCP protocol and destination port is either 80 or 8080. The URLs with other protocols, like ftp, news, etc. are not filtered in this module.

URL (Uniform Resource Locator) syntax used for HTTP defined in RFC1738 as below:

http://host[:port]/directory/file

The string that URLFilter is going to check the sub-string right after first period (‘.’). For example, an URL, http://www.website.com/index, the URLFilter will compare ‘website.com/index’ with the defined rules. In the following text, we called this sub-string as SS.

There are two rule types: one is domain rule and the other is keyword rule. The difference of types is how to match the token in SS. If the token is belonging to domain rule, it must compare the token from the first character of SS. While the token is in keyword rule, any occurrence of token is SS, in despite of position, is called matched.

Moreover, all of URLs are classified into three categories: trusted, blocked, and unknown. When a URL match a domain rule with ‘pass’ action, it is trusted. It matched a domain rule with ‘block’ action, it is ‘blocked’. Except thest two cases, it is ‘unknown’.

The checking sequence of URLFilter is as below:

Start to check if ‘urlfilter enable’

Check domain rules if ‘urlfilter domain enable’

match ‘pass’, then pass;

If ‘blockall enable’, then drop unconditionally;

If match domain rule with ‘block’, then drop;

Check keyword rules if ‘urlfilter keyword enable’

If match keyword block rule, then drop;

Otherwise, pass

    - URLFilter CLI commands

The table below lists the URLFilter commands provided by the CLI:

| Command | Reference |
| --- | --- |
| urlfilter enable¦disable | urlfilter enable¦disable |
| urlfilter domain add rule | urlfilter domain add rule |
| urlfilter domain enable¦disable | urlfilter domain enable¦disable |
| urlfilter domain delete rule | urlfilter domain delete rule |
| urlfilter domain list | urlfilter domain list |
| urlfilter keyword add | urlfilter keyword add |
| urlfilter keyword enable¦disable | urlfilter keyword enable¦disable |
| Urlfilter keyword delete | urlfilter keyword delete |
| urlfilter keyword list | urlfilter keyword list |
| urlfilter blockall enable¦disable | urlfilter blockall enable¦disable |
| urlfilter blocklog enable¦disable | urlfilter blocklog enable¦disable |
| urlfilter time clock_begin | urlfilter time clock_begin |
| urlfilter time day_begin | urlfilter time day_begin |
| urlfilter time enable¦disable | urlfilter time enable¦disable |
| urlfilter time show | urlfilter time show |
| urlfilter status | urlfilter status |

urlfilter enable|disable

    - Syntax

urlfilter {enable|disable}

    - Description

This command enables or disable the whole URL Filtering function.

    - Example

prompt> urlfilter enable

    - See also

urlfilter domain

urlfilter keyword

urlfilter status

urlfilter domain add rule

    - Syntax

urlfilter domain add rule <name> {block|pass} <token>

    - Description

This command create a domain rule that checks the remaining portion of an URL right after first period (‘.’). If token is matched with starting of SS, this URL will be blocked or passed depending on the action.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Name | The name of domain rule. | N/A |
| Token | The token or pattern you want to compare with the SS. The maximum length is 80 characters. | N/A |

    - Example

prompt> urlfilter domain add rule r1 pass search

<This rule will allow the URL contains ‘search’ passed.

So, www.search.com, www.search.edu, www.search.org, are passed or trusted; but, www.asearch.com is unknow>

prompt> urlfilter domain add rule r1 block xxx

<This rule will block all URL that contains ‘xxx’>

    - See also

urlfilter enable|disable

urlfilter domain enable|disable

urlfilter domain enable|disable

    - Syntax

urlfilter domain {enable|disable}

    - Description

This command enables (or disables) the URLFilter to check URL according to domain rules.

    - Example

prompt> urlfilter domain enable

    - See also

urlfilter domain add rule

urlfilter domain list

urlfilter domain delete rule

    - Syntax

urlfilter domain delete rule <name>

    - Description

- This command deletes an existing domain rule.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of domain rule. | N/A |

    - Example

prompt> urlfilter domain delete rule r1

    - See also

urlfilter domain add rule

urlfilter domain list

urlfilter domain list

    - Syntax

urlfilter domain list

    - Description

This command displays the defined domain rules. The following details are displayed:

• ID and Rule name

• Searched token

• Action when match

    - Example

prompt> urlfilter domain list

URL Filtering Accesses:

ID | Name | Rule            | Action

-------------------------------------

1 | r1   | search          | pass

-------------------------------------

    - See also

urlfilter domain add rule

urlfilter domain enable|disable

urlfilter keyword add rule

    - Syntax

urlfilter keyword add rule <name> block <token>

    - Description

This command define a keyword rule. If the token can be located in SS, the URL will be blocked..

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of keyword rule. | N/A |
| token | The token or pattern you want to locate in HTTP GET packet content. The maximum length is 80 characters. | N/A |

    - Example

prompt> urlfilter keyword add rule k1 block sex

<The following URLs will be blocked:

http://www.sex.com, http://www.shared.sex.com, http://www.shared.com/sex>

    - See also

urlfilter enable|disable

urlfilter keyword enable|disable

urlfilter keyword list

urlfilter keyword enable|disable

    - Syntax

urlfilter keyword {enable|disable}

    - Description

This command enables (or disables) the filtering function according to keyword rules.

    - Example

prompt> urlfilter keyword enable

    - See also

urlfilter keyword add

urlfilter keyword list

urlfilter keyword delete rule

    - Syntax

urlfilter keyword delete rule <name>

    - Description

- This command deletes an existing keyword rule.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of keyword rule. | N/A |

    - Example

prompt> urlfilter keyword delete rule k1

    - See also

urlfilter keyword add rule

urlfilter keyword list

urlfilter keyword list

    - Syntax

urlfilter keyword list

    - Description

This command displays the defined keyword rules. The following details are displayed:

• ID and Rule name

• Keyword

    - Example

prompt> urlfilter keyword list

URL Filtering Accesses:

ID | Name | Block Keyword

----------------------------

1 | k1   | sex

----------------------------

    - See also

urlfilter keyword add rule

urlfilter keyword enable|disable

urlfilter blockall enable|disable

    - Syntax

urlfilter blockall {enable|disable}

    - Description

When ‘blockall’ enabled, the URL filter will block all of URLs except the trusted web site. The trusted URL is the URL matched with any ‘pass’ domain rule.

    - Example

prompt> urlfilter blockall enable

    - See also

urlfilter domain

urlfilter keyword

urlfilter blocklog enable|disable

    - Syntax

urlfilter blocklog {enable|disable}

    - Description

This command enables (or disables) the logging of filtering function. When enabled, the blocked information will store in system Event Log buffer and you can check the content with system log dump command.

    - Example

prompt> urlfilter blocklog enable

    - See also

urlfilter enable|disable

urlfilter domain

urlfilter keyword

urlfilter time clock_begin

    - Syntax

urlfilter time clock_begin <h1>:<m1> end <h2>:<m2>

    - Description

- This command specifies the URLFilter schedule within a day. It is valid only when the scheduling is enabled using urlfilter time enable command. If time schedule is disabled, the URLFilter is functioned all the time.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| h1 | The starting hour of schedule. Valid range is 0 to 23. | N/A |
| m1 | The starting minute of schedule. Valid range is 0 to 59. | N/A |
| h2 | The ending hour of schedule. Valid range is 0 to 23. | N/A |
| m2 | The ending minute of schedule. Valid range is 0 to 59. | N/A |

    - Example

prompt> urlfilter time clock_begin 7 : 00 end 18 :00

    - See also

urlfilter time day_begin

urlfilter time enable|disable

urlfilter time day_begin

    - Syntax

urlfilter time day_begin <d1> end <d2>

    - Description

- This command specifies the URLFilter schedule within a week. It is valid only when the scheduling is enabled using urlfilter time enable command. If time schedule is disabled, the URLFilter is functioned all the time.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| d1 | The starting day of schedule in a week. Valid range is 0 to 6 where 0 is meant for Sunday, 1 is Monday, and so on. | N/A |
| d2 | The ending day of schedule in a week. Valid range is 0 to 6. | N/A |

    - Example

prompt> urlfilter time day_begin 1 end 5

    - See also

urlfilter time clock_begin

urlfilter time enable|disable

urlfilter time enable|disable

    - Syntax

urlfilter time {enable|disable}

    - Description

This command enables (or disables) the time schedule of URLFilter.

    - Example

prompt> urlfilter time enable

    - See also

urlfilter time clock_begin

urlfilter time day_begin

urlfilter time show

urlfilter time show

    - Syntax

urlfilter time show

    - Description

This command displays the scheduling setting of URLFilter.

    - Example

prompt> urlfilter time show

URL filter time range:

Begin day  : Mon.     End day  : Fri.

Begin time : 07:00    End time : 18:00

    - See also

urlfilter time clock_begin

urlfilter time day_begin

urlfilter status

    - Syntax

urlfilter status

    - Description

This command displays the whole settings of URLFilter.

    - Example

prompt> urlfilter status

Url Filter Status

URL Filter    : Enable

URL Filter Log: Disable

All Block     : Disable

Domain filter : Enable

Keyword block : Disable

Time Range    : Disable

    - See also

urlfilter enable|disable

urlfilter domain enable|disable

urlfilter keyword enable|disable

urlfilter blockall enable|disable

urlfilter blocklog enable|disable

urlfilter time enable|disable