- Mailcheck CLI commands

This chapter describes Mailcheck CLI commands.

Summary

    - Mailcheck CLI commands

The table below lists the Mailcheck commands provided by the CLI:

| Command | Reference |
| --- | --- |
| mailcheck add | mailcheck add |
| mailcheck clear | mailcheck clear |
| mailcheck delete | mailcheck delete |
| mailcheck list | mailcheck list |
| mailcheck reset | mailcheck reset |
| mailcheck set autoconnect | mailcheck set autoconnect |
| mailcheck set enabled¦disabled | mailcheck set enabled¦disabled |
| mailcheck set interval | mailcheck set interval |
| mailcheck set mail | mailcheck set mail |
| mailcheck set waninterface | mailcheck set waninterface |
| mailcheck show | mailcheck show |

mailcheck add

    - Syntax

mailcheck add <name> waninterface <interface>

    - Description

This command adds a mail account object that the mail checker can check periodically. During creating the object, you need to specify the IP interface that checker used to send POP3 protocol packets. The mailcheck set commands are used to configure the mail account objects.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of mail account object. | N/A |
| interface | The IP interface used to transfer POP3 protocol packet. The ip list interfaces command will list all existing IP interfaces. | N/A |

    - Example

prompt> mailcheck add mailman waninterface ipwan

    - See also

mailcheck set

mailcheck clear

mailcheck delete

mailcheck list

ip list interfaces

mailcheck clear

    - Syntax

mailcheck clear

    - Description

This command deletes all of mail account objects created via mailcheck add command.

    - Example

prompt> mailcheck delete

    - See also

mailcheck add

mailcheck delete

mailcheck list

mailcheck delete

    - Syntax

mailcheck delete <name>

    - Description

This command delete a specific mail account object that is created using mailcheck add command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of mail account object. | N/A |

    - Example

prompt> mailcheck delete mailman

    - See also

mailcheck add

mailcheck clear

mailcheck list

mailcheck list

    - Syntax

mailcheck list <name>

    - Description

This command displays the existing mail account objects. The following details are displayed:

• ID and Name

• Mail username and POP3 Server

• WAN interface

• Enabled or Disabled

    - Example

prompt> mailcheck list

Mail Check Accounts:

ID | Name    | Mail UserName/POP3 ServerAddress | WAN interface | Enabled

---|---------|----------------------------------|---------------|---------

1 | mailman | manname / email.com              | ipwan         | false

--------------------------------------------------------------------------

    - See also

mailcheck add

mailcheck clear

mailcheck delete

mailcheck set

mailcheck reset

    - Syntax

mailcheck reset <name>

    - Description

This command clears the email statistics stored in the system.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of mail account object. | N/A |

    - Example

prompt> mailcheck reset mailman

    - See also

mailcheck set

mailcheck set autoconnect

    - Syntax

mailcheck set <name> autoconnect {enabled|disabled}

    - Description

This command specifies the permission for the system to dial out to get an WAN interface in order to check emails. By default, it is disabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of mail account object. | N/A |
| enabled | Allows to dial out automatically to get an WAN interface up. | disabled |
| disabled | Do not permit to dial out automatically to check emails. |  |

    - Example

prompt> mailcheck set mailman autoconnect enabled

    - See also

mailcheck add

mailcheck set enabled|disabled

    - Syntax

mailcheck set <name> {enabled|disabled}

    - Description

This command enables the system to check the mail account object or not. When an mail account object created, it is put in disabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of mail account object. | N/A |
| enabled | Allows to check this email account. | disabled |
| disabled | Do not need to check this email account. |  |

    - Example

prompt> mailcheck add mailman enabled

    - See also

mailcheck set

mailcheck set interval

    - Syntax

mailcheck set <name> interval <time>

    - Description

This command specifies how long the system will check the email statistics of a specific mail account.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of mail account object. | N/A |
| time | The time (in minutes) to refresh the email statistics from the POP3 server. | 60 |

    - Example

prompt> mailcheck set mailman interval 5

    - See also

mailcheck add

mailcheck set mail

    - Syntax

mailcheck set <name> mail {username <user>|password <pwd>|pop3server <server>}

    - Description

This command configures the actual email account information, including the user name and password to login into POP3 server and the POP3 server Domain Name or IP address.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of mail account object. | N/A |
| user | The user name to login the email account of a specific POP3 server.. | N/A |
| pwd | The password to login the email account of a specific POP3 server.. | N/A |
| server | The domain name of IP address of the specific POP3 server.. | N/A |

    - Example

prompt> mailcheck set mailman mail username user1

prompt> mailcheck set mailman mail password pwd2login

prompt> mailcheck set mailman mail pop3server email.com

prompt> mailcheck set mailman mail pop3server 210.39.27.1

    - See also

mailcheck add

mailcheck show

mailcheck set waninterface

    - Syntax

mailcheck set <name> waninterface <interface>

    - Description

This command re-configure the IP interface that a mail account object sends POP3 protocol packets.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of mail account object. | N/A |
| interface | The IP interface used to transfer POP3 protocol packet. The ip list interfaces command will list all existing IP interfaces. | N/A |

    - Example

prompt> mailcheck set mailman waninterface ipwan2

    - See also

mailcheck add

mailcheck show

ip list interfaces

mailcheck show

    - Syntax

mailcheck show <name>

    - Description

This command displays the information of a mail account object. The following details are displayed:

• Mail account object name

• Login username, password and POP3 server

• WAN interface

• Polling interval

• Autoconnect

• Enable to check or not

• Current email count

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Name | The name of mail account object. | N/A |

    - Example

prompt> mailcheck show mailman

MailCheck: mailman

E-mail Username: user1

E-mail POP3Server: email.com

E-mail Password: *********

WAN interface: ipwan

Interval: 60 (minutes)

Autoconnect: false

Enabled: false

Status: No mail

    - See also

mailcheck add

mailcheck set