- PPTP CLI commands

This chapter describes the PPTP CLI commands.

Summary

    - PPTP CLI commands

The table below lists the CLI commands for manipulating PPTP tunnels:

| Command | Reference |
| --- | --- |
| pptp add tunnel | pptp add tunnel |
| pptp attach | pptp attach |
| pptp clear tunnels | pptp clear tunnels |
| pptp delete tunnel | pptp delete tunnel |
| pptp detach | pptp detach |
| pptp list tunnels | pptp list tunnels |
| pptp set listening | pptp set listening |
| pptp set localaddr | pptp set localaddr |
| pptp set tunnel remoteip | pptp set tunnel remoteip |
| pptp set tunnel type | pptp set tunnel type |
| pptp show tunnel | pptp show tunnel |
| pptp show | pptp show |

    - PPTP Console commands

The table below lists the pptp console commands and, if available, their CLI equivalent command:

| Command | CLI Equivalent |
| --- | --- |
| pptp bind | Replaced by CLI command pptp set listening . |
| pptp <tunnel> connect | Replaced by CLI command pptp attach |
| pptp <tunnel> create | Replaced by CLI command pptp add tunnel |
| pptp <tunnel> delete | Replaced by CLI command pptp delete tunnel |
| pptp <tunnel> disconnect | Replaced by CLI command pptp detach |
| pptp <tunnel> event | Blacklisted command, see <tunnel> event |
| pptp <tunnel> info | Blacklisted command, see <tunnel> info |
| pptp list | Usable command, see list |
| pptp version | Usable command, see version |

pptp add tunnel

    - Syntax

pptp add tunnel <name>

    - Description

This command creates a PPP tunnel.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the tunnel. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |

    - Example

prompt> pptp add tunnel tunnel1

    - See also

pptp attach

pptp list tunnels

pptp attach

    - Syntax

pptp attach {<name>|<number>} <ppp-transport>

    - Description

This command attaches an existing tunnel to an existing PPP transport to allow data to be tunnelled via PPP.

Only one tunnel can be attached to a PPP transport. If you use this command when there is already a tunnel attached to the transport, the previous tunnel is replaced by the new one.

This command implicitly enables the PPP transport being attached.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing tunnel. To display tunnel names, use the pptp list tunnels command. | N/A |
| number | A number that identifies an existing tunnel. To display tunnel numbers, use the pptp list tunnels command. | N/A |
| ppp-transport | A name that identifies an existing PPP transport. To display transport names, use the pppoa/pppoe list transports command. | N/A |

    - Example

prompt> pptp attach tunnel1 pppoa1

    - See also

pptp add tunnel

pptp list tunnels

pppoa list transports

pppoe list transports

pptp clear tunnels

    - Syntax

pptp clear tunnels

    - Description

This command deletes all tunnels that were created using the pptp add tunnel command.

    - Example

prompt> pptp clear tunnels

    - See also

pptp delete tunnel

pptp delete tunnel

    - Syntax

pptp delete tunnel {<name>|<number>}

    - Description

This command deletes a single PPTP tunnel.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing tunnel. To display tunnel names, use the pptp list tunnels command. | N/A |
| number | A number that identifies an existing tunnel. To display tunnel numbers, use the pptp list tunnels command. | N/A |

    - Example

prompt> pptp delete tunnel tunnel1

    - See also

pptp list tunnels

pptp detach

    - Syntax

pptp detach {<name>|<number>}

    - Description

This command detaches a single tunnel that was attached to a PPP transport using the pptp attach tunnel command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing tunnel. To display tunnel names, use the pptp list tunnels command. | N/A |
| number | A number that identifies an existing tunnel. To display tunnel numbers, use the pptp list tunnels command. | N/A |

    - Example

prompt> pptp detach tunnel1

    - See also

pptp list tunnels

pptp list tunnels

    - Syntax

pptp list tunnels

    - Description

This command lists all PPTP tunnels that have been created using the pptp add tunnel command. It displays the following information about PPTP tunnels:

- tunnel ID number

- tunnel name

- type (dialin or dialout)

- remote IP address (if applicable)

- whether the tunnel is connected to a named PPP transport

- name of attached PPP transport (if applicable)

    - Example

prompt> pptp list tunnels

PPTP Tunnels:

ID  |   Name    |    Type    |    Remote IP   | Connected |  Attached

-----|-----------|------------|----------------|-----------|--------------

1 | tunnel1   | dialin     | 192.168.102.3  | true      | pppoa1

2 | tunnel2   | dialin     | 0.0.0.0        | false     | Not attached

--------------------------------------------------------------------------

    - See also

pptp show tunnel

pptp set listening

    - Syntax

pptp listening {enabled|disabled}

    - Description

This command determines whether PPTP tunnels can accept incoming connections from a remote PPTP Network Server (PNS).

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enabled | Incoming connections are accepted by PPTP tunnels. | enabled |
| disabled | Incoming connections are not accepted by PPTP tunnels. |  |

    - Example

prompt> pptp set listening disabled

pptp set localaddr

    - Syntax

pptp set localaddr <ip address>

    - Description

This command sets the IP address of the local system running the PPTP Access Concentrator (PAC). This is used for incoming and outgoing tunnel connections.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ip address | The IP address of the local system running the PAC, displayed in the following format: 192.168.102.3 | 0.0.0.0 |

    - Example

prompt> pptp set localaddr 192.168.102.3

    - See also

pptp set tunnel remoteip

pptp set tunnel remoteip

    - Syntax

pptp set tunnel {<name>|<number>} remoteip <ip address>

    - Description

This command sets the IP address of the remote end of the tunnel. The remote IP address is for a system running PNS (PPTP Network Server).

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing tunnel. To display tunnel names, use the pptp list tunnels command. | N/A |
| number | A number that identifies an existing tunnel. To display tunnel numbers, use the pptp list tunnels command. | N/A |
| ip address | The IP address of the remote system running the PNS, displayed in the following format: 192.168.102.3 | 0.0.0.0 |

    - Example

prompt> pptp set tunnel1 remoteip 192.168.103.2

    - See also

pptp set localaddr

pptp set tunnel type

    - Syntax

pptp set tunnel {<name>|<number>} type {<dialin>|<dialout>}

    - Description

This command sets the type of connection (dialin or dialout) that an existing tunnel can use.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing tunnel. To display tunnel names, use the pptp list tunnels command. | N/A |
| number | A number that identifies an existing tunnel. To display tunnel numbers, use the pptp list tunnels command. | N/A |
| dialin | This allows incoming calls (dialin) to be tunnelled to a specific PNS. | dialin |
| dialout | This allows PNS-capable machines on a local network to dialout. |  |

    - Example

prompt> pptp set tunnel tunnel1 type dialout

    - See also

pptp list tunnels

pptp show tunnel

    - Syntax

pptp show tunnel {<name>|<number>}

    - Description

This command displays the following information about an existing tunnel:

- name

- connection type (dialin or dialout)

- remote IP address (if set)

- transport connection status

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing tunnel. To display tunnel names, use the pptp list tunnels command. | N/A |
| number | A number that identifies an existing tunnel. To display tunnel numbers, use the pptp list tunnels command. | N/A |

    - Example

prompt> pptp show tunnel tunnel1

PPTP Tunnel: tunnel1

Type: dialin

Remote IP: 192.168.103.2

Connected: true

    - See also

pptp show

pptp show

    - Syntax

pptp show

    - Description

This command displays the following information about your PPTP setup:

- local IP address (if set)

- listening status

    - Example

prompt> pptp show

Global PPTP Configuration:

Local address: 192.168.102.3

Listening: true

    - See also

pptp show tunnel

pptp set localaddr

pptp set listening