- NAT CLI commands

This chapter describes the NAT (Network Address Translation) CLI commands.

The NAT module is a child module in the Security package.

Summary

    - NAT CLI commands

The table below lists the NAT commands provided by the CLI:

| Command | Reference |
| --- | --- |
| nat enable | nat enable |
| nat disable | nat disable |
| nat add globalpool | nat add globalpool |
| nat clear globalpools | nat clear globalpools |
| nat delete globalpool | nat delete globalpool |
| nat iketranslation | nat iketranslation |
| nat list globalpools | nat list globalpools |
| nat show globalpool | nat show globalpool |
| nat add resvmap globalip tcp¦udp | nat add resvmap globalip tcp¦udp |
| nat add resvmap globalip | nat add resvmap globalip |
| nat add resvmap interfacename tcp¦udp | nat add resvmap interfacename tcp¦udp |
| nat add resvmap interfacename | nat add resvmap interfacename |
| nat clear resvmaps | nat clear resvmaps |
| nat delete resvmap | nat delete resvmap |
| nat list resvmaps | nat list resvmaps |
| nat show resvmap | nat show resvmap |
| nat status | nat status |
| nat set translation tcp-timeout | nat set translation tcp-timeout |
| nat set translation udp-timeout | nat set translation udp-timeout |
| nat set translation timeout | nat set translation timeout |
| nat show translation | nat show translation |

nat enable

    - Syntax

nat enable <name> <interfacename> {internal|dmz}

    - Description

This command enables NAT between an existing security interface and a network interface type. NAT is enabled between the security interface and all the interfaces that belong to the chosen network interface type.

Note - You must enable the Security package using the command security on Security CLI if you want to use the NAT module to configure security for your system.

An interface is either an inside or outside interface. The network attached to an inside interface needs to be protected from the network attached to an outside interface. For example, the network attached to an internal interface (inside) needs to be protected from the network attached to a DMZ (outside). Also, you can only enable NAT between two different interface types. For example, if interfacename is an external interface type, you can enable NAT between the interfacename and the internal or the DMZ interface type, but not the external interface type. The following interface combinations are the only ones that you can use:

- external (outside) and internal (inside)

- external (outside) and DMZ (inside)

- DMZ (outside) and internal (inside)

The existing security interface must be an outside interface. NAT translates packets between the outside interface and the inside interface type. In this way, the IP address of a host on a network attached to an inside interface is hidden from a host on a network attached to an outside interface.

If you want to map an outside interface to an individual host on an inside interface type, you can use the command nat add resvmap interfacename .

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies a NAT object enabled between a security interface and an interface type. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| interfacename | The name of an existing security interface (external or DMZ) that was added to the Security package using the security add interface command. To display security interfaces, use the security list interfaces command. | N/A |
| internal | Allows NAT to be enabled/disabled between the interfacename and all interfaces that belong to the internal interface type. | N/A |
| dmz | Allows NAT to be enabled/disabled between the interfacename and all interfaces that belong to the DMZ interface type. The interfacename must be an external interface type. | N/A |

    - Example

prompt> nat enable nat1 extinterface internal

    - See also

nat disable

nat status

security list interfaces

security add interface

nat add resvmap interfacename

nat disable

    - Syntax

nat disable <name>

    - Description

This command disables a NAT object that was previously enabled between an existing security interface and a network interface type using the nat enable command. NAT is disabled between the security interface and all the interfaces that belong to the chosen interface type.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of an existing NAT object created between a security interface and an interface type using the nat enable command. To display enabled NAT objects, use the nat status command. | N/A |

    - Example

prompt> nat disable nat1

    - See also

nat enable

nat status

nat add globalpool

    - Syntax

nat add globalpool <name> <interfacename> {internal|dmz} <ipaddress> {subnetmask <mask>|endaddress <address>}

    - Description

Note - Before you can add a global address pool, you must enable a NAT object using the command nat enable.

The nat enable command creates an IP address for the outside security interface, however, you may want to use more than one outside IP address. For example, if your ISP provides multiple IP addresses, you might want to map an outside address to an inside interface that is your web server, and map another outside address to an inside interface that is your mail server.

This command creates a pool of outside network addresses. A network address pool is a range of IP addresses that is visible outside your network. NAT translates packets between the outside addresses and the inside interfaces that each address is mapped to.

There are two ways to specify a range of IP addresses:

- specify the interfacename IP address and a subnet mask address

- specify the interfacename IP address that represents the first address in the range, then specify the last address in the range

If you want to map IP addresses to individual hosts on an inside interface type, you can use the command nat add resvmap globalip.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies a global network address or pool of addresses. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| interfacename | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |
| internal | Maps the IP addresses to the internal interface type inside the network. | N/A |
| dmz | Maps the global addresses to the DMZ interface type inside the network. | N/A |
| ipaddress | The IP address of the interfacename that is visible outside the network. | N/A |
| mask | The subnet mask of the network IP address. | N/A |
| endaddress | The last IP address in the range of addresses that make up the global address pool. | N/A |

    - Examples

Example 1

This example creates a network address pool that allows NAT to translate packets between the external interface and the DMZ interface type.

First, NAT is enabled between the external interface and the DMZ interface type:

prompt> nat enable n1 extinterface dmz

Then the IP address and subnet mask is created:

prompt> nat add globalpool gp1 extinterface dmz 192.168.102.3 subnetmask 255.255.255.0

Example 2

This example creates a network address pool that allows NAT to translate packets between the external interface and the internal interface type.

First NAT is enabled between the external interface and the internal interface type:

prompt> nat enable n2 extinterface internal

Then the address range is created:

prompt> nat add globalpool gp2 extinterface internal 192.168.103.2 endaddress 192.168.103.50

    - See also

nat enable

nat status

security list interfaces

Once you have created an address pool, packets received on a specific IP address can be mapped to individual hosts inside the network. See nat add resvmap globalip.

nat clear globalpools

    - Syntax

nat clear globalpools <interfacename>

    - Description

This command deletes all address pools that were added to a specific outside interface using the nat add globalpool command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| interfacename | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |

    - Example

prompt> nat clear globalpools extinterface

    - See also

nat add globalpool

security list interfaces

nat delete globalpool

    - Syntax

nat delete globalpool <name> <interfacename>

    - Description

This command deletes a single address pool that was added to a specific outside interface using the nat add globalpool command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing global IP address. To display global IP addresses, use the nat list globalpools command. | N/A |
| interfacename | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |

    - Example

prompt> nat delete globalpool gp1 extinterface

    - See also

nat add globalpool

nat list globalpools

security list interfaces

nat iketranslation

    - Syntax

nat iketranslation {cookies | ports}

    - Description

This command supports NAT IPSec traversal. It allows you to specify how Internet Key Exchange (IKE) packets are translated.

IKE establishes a shared security policy and authenticates keys for services that require keys, such as IPSec. Before any IPSec traffic can be passed, each router/firewall/host must verify the identity of its peer. This can be done by manually entering pre-shared keys into both hosts or by a CA service.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| cookies | Source port will not be translated for IKE packets; IKE cookies are used to identify IKE sessions. | ports |
| ports | Source port will be translated for IKE packets. |  |

    - Example

prompt> nat iketranslation cookies

nat list globalpools

    - Syntax

nat list globalpools <interfacename>

    - Description

This command lists the following NAT address pool information for a specific outside interface:

- Address pool identification number

- Address pool name

- Type of inside interface (internal or DMZ)

- Subnet status (true or false)

- IP address - the outside network IP address or the first address in the range of network pool addresses

- Mask/End Address - the outside subnet mask of the outside network IP address or the last address in the range of network pool addresses

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| interfacename | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |

    - Example

prompt> nat list globalpools extinterface

NAT global address pool:

ID |    Name    |   Type   | Subnet |    IP address    | Mask/End Address

---------------------------------------------------------------------------

1 | gp1        | dmz      | true   | 192.168.102.3    | 255.255.255.0

2 | g2         | internal | false  | 192.168.103.2    | 192.168.103.50

---------------------------------------------------------------------------

    - See also

security list interfaces

nat show globalpool

nat show globalpool

    - Syntax

nat show globalpool <name> <interfacename>

    - Description

This command displays information about a single network address pool that has been added to an outside interface:

- Type of inside interface (internal or DMZ)

- Subnet configuration status (true if the network pool was set using a subnet mask, false if it was set using a range of IP addresses)

- IP address - the outside network IP address or the first address in the range of addresses

- Subnet Mask or End Address - the subnet mask of the outside network IP address or the last address in the range of addresses

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing global IP address. To display global IP addresses, use the nat list globalpools command. | N/A |
| interfacename | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |

    - Example

prompt> nat show globalpool gpl extinterface

NAT global address pool: gp1

Interface type: dmz

Subnet configuration: true

IP address: 192.168.102.3

Subnet mask or End Address: 255.255.255.0

    - See also

nat list globalpools

security list interfaces

nat add resvmap globalip tcp|udp

    - Syntax

nat add resvmap <name> globalip <interfacename> <globalip> <internalip> {tcp|udp} <portno> [<2ndportno> [<localportno> [2ndlocalportno]]]

    - Description

Note - Before you can add reserved mapping, you must enable a NAT object using the command nat enable .

This command maps an IP address from a global pool (created using the nat add globalpool command) to an individual IP address inside the network. NAT translates packets between the outside IP address and the individual host based on the transport information (TCP or UDP) given in this command.

You  can define reserved mappings for a range of ports and/or translating port numbers.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies a reserved mapping configuration. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| interfacename | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |
| Globalip | The IP address of an outside interface set using the nat add globalpool command. | N/A |
| internalip | The IP address of an individual host inside the network (internal or DMZ interface type). | N/A |
| portno | Either a single TCP or UDP port number that you want to use in your reserved mapping configuration, or the first port number in the range of ports | N/A |
| 2ndportno | The second TCP or UDP port number in the range that started with the port specifed in portno. | N/A |
| localportno | Either a single TCP or UDP port number, or the first port number in the range of internal ports | N/A |
| 2ndlocalportno | The second internal TCP or UDP port number in the range that started with the port specifed in localportno. | N/A |

    - Example

prompt> nat add resvmap rm1 globalip extinterface 192.168.68.68 10.10.10.10 tcp 25

    - See also

nat enable

nat list globalpools

nat status

security list interfaces

nat add resvmap globalip

    - Syntax

nat add resvmap <name> globalip <interfacename> <globalip> <internalip> {icmp|igmp|ip|egp|rsvp|ospf|ipip|all}

    - Description

Note - Before you can add reserved mapping, you must enable a NAT object using the command nat enable .

This command maps an IP address from a global pool (created using the nat add globalpool command) to an individual IP address inside the network. NAT translates packets between the outside IP address and the individual host based on the transport information given in this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies a reserved mapping configuration. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| interfacename | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |
| globalip | The IP address of an outside interface set using the nat add globalpool command. | N/A |
| internalip | The IP address of an individual host inside the network (internal or DMZ interface type). | N/A |
| icmp | Internet Control Message Protocol (ICMP) is set as the transport type. ICMP messages are used for out-of-band messages related to network operation or mis-operation. See http://www.ietf.org/rfc/rfc0792.txt. | N/A |
| igmp | Internet Group Management Protocol (IGMP) is set as the transport type. Allows Internet hosts to participate in multicasting. See http://www.ietf.org/rfc/rfc1112.txt. | N/A |
| ip | Internetwork Protocol (IP). Provides all of the Internet’s data transport services. http://www.ietf.org/rfc/rfc791.txt and http://www.ietf.org/rfc/rfc919.txt. | N/A |
| egp | Exterior Gateway Protocol (EGP). Protocol for exchanging routing information between autonomous systems. See http://www.ietf.org/rfc/rfc904.txt. | N/A |
| rsvp | Resource Reservation Protocol (RSVP) is set as the transport type. Supports the reservation of resources across an IP network. See http://www.ietf.org/rfc/rfc2205.txt. | N/A |
| ospf | Open Shortest Path First (OSPF) is set as the transport type. A link-state routing protocol. See http://www.ietf.org/rfc/rfc1583. | N/A |
| ipip | IP-within-IP Encapsulation Protocol. Encapsulates an IP datagram within a datagram. See http://www.ietf.org/rfc/rfc2896.txt. | N/A |
| all | All traffic is translated between the global IP address and the specified inside address that it is mapped to. | N/A |

    - Example

prompt> nat add resvmap rm1 globalip extinterface 192.168.68.68 10.10.10.10 icmp

    - See also

nat enable

nat list globalpools

nat status

security list interfaces

nat add resvmap interfacename tcp|udp

    - Syntax

nat add resvmap <name> interfacename <interfacename> <internalip> {tcp|udp} <portno> [<2ndportno> [<localportno> [2ndlocalportno]]]

    - Description

Note - Before you can add reserved mapping, you must enable a NAT object using the command nat enable .

This command maps an outside IP security interface (enabled as a NAT object using the nat enable command) to an individual IP address inside the network. NAT translates packets between the outside IP address and the individual host based on the transport information (TCP or UDP) given in this command. A range of external ports can be translated to a single local port if required.

You can define reserved mapping for a range of ports and/or translating port numbers.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies a reserved mapping configuration. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| interface name | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |
| internalip | The IP address of an individual host inside the network (internal or DMZ interface type). | N/A |
| portno | Either a single TCP or UDP port number that you want to use in your reserved mapping configuration, or the first port number in the range of ports. | N/A |
| 2ndportno | The second TCP or UDP port number in the range that started with the port specified in portno. | N/A |
| localportno | Either a single TCP or UDP port number, or the first port number in the range of internal ports. | N/A |
| 2ndlocalportno | The second internal TCP or UDP port number in the range that started with the port specified in localportno. | N/A |

    - Example

The example below forwards TCP port 25 requests on the WAN interface to 10.10.10.10 port 80:

prompt> nat add resvmap rm1 interfacename extinterface 10.10.10.10 tcp 25 25 80 80

The example below forwards TCP port 80 to 90 requests on the WAN interface to 10.10.10.10 port 8080 to 8090. Note that the first range must be the same size as the second range:

prompt> nat add resvmap rm1 interfacename extinterface 10.10.10.10 tcp 80 90 8080 8090

    - See also

nat enable

security list interfaces

nat add resvmap interfacename

    - Syntax

nat add resvmap <name> interfacename <interfacename> <internalip> {icmp|igmp|ip|egp|rsvp|ospf|ipip|all}

    - Description

Note - Before you can add reserved mapping, you must enable a NAT object using the command nat enable .

This command maps an outside IP security interface (enabled as a NAT object using the nat enable command) to an individual IP address inside the network. NAT translates packets between the outside IP address and the individual host based on the transport information given in this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies a reserved mapping configuration. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| interface name | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |
| internalip | The IP address of an individual host inside the network (internal or DMZ interface type). | N/A |
| icmp | Internet Control Message Protocol (ICMP) is set as the transport type. ICMP messages are used for out-of-band messages related to network operation or mis-operation. See http://www.ietf.org/rfc/rfc0792.txt. | N/A |
| igmp | Internet Group Management Protocol (IGMP) is set as the transport type. Allows Internet hosts to participate in multicasting. See http://www.ietf.org/rfc/rfc1112.txt. | N/A |
| ip | Internetwork Protocol (IP). Provides all of the Internet’s data transport services. http://www.ietf.org/rfc/rfc791.txt and http://www.ietf.org/rfc/rfc919.txt. | N/A |
| egp | Exterior Gateway Protocol (EGP). Protocol for exchanging routing information between autonomous systems. See http://www.ietf.org/rfc/rfc904.txt. | N/A |
| rsvp | Resource Reservation Protocol (RSVP) is set as the transport type. Supports the reservation of resources across an IP network. See http://www.ietf.org/rfc/rfc2205.txt. | N/A |
| ospf | Open Shortest Path First (OSPF) is set as the transport type. A link-state routing protocol. See http://www.ietf.org/rfc/rfc1583. | N/A |
| ipip | IP-within-IP Encapsulation Protocol. Encapsulates an IP datagram within a datagram. See http://www.ietf.org/rfc/rfc2896.txt. | N/A |
| all | Traffic is translated between the global IP address and the inside address that it is mapped to. | N/A |

    - Example

prompt> nat add resvmap rm1 interfacename extinterface 10.10.10.10 icmp

    - See also

nat enable

security list interfaces

nat clear resvmaps

    - Syntax

nat clear resvmaps <interfacename>

    - Description

This command deletes all NAT reserved mappings that were added to an outside security interface using the nat add resvmap commands.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| interfacename | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |

    - Example

prompt> nat clear resvmaps extinterface

    - See also

nat delete resvmap

security list interfaces

nat delete resvmap

    - Syntax

nat delete resvmap <name> <interfacename>

    - Description

This command deletes a single NAT reserved mapping that was added to an outside security interface using the nat add resvmap commands.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing global IP address. To display global IP addresses, use the nat list resvmaps command. | N/A |
| interfacename | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |

    - Example

prompt> nat delete resvmap rm1 extinterface

    - See also

nat enable

nat list resvmaps

security list interfaces

nat list resvmaps

    - Syntax

nat list resvmaps <interfacename>

    - Description

This command lists the following reserved mapping information for a specific outside security interface:

- Reserved mapping identification number

- Reserved mapping name

- Global address - the IP address of the outside interface that is mapped to the inside IP address

- Internal address - the IP address inside the network that the outside security interface IP address is mapped to

- Transport type (IGMP, IPIP etc.)

- Port - TCP or UDP port used by the transport type. If a non-TCP/UDP protocol is used, the port is set to 0.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| interfacename | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |

    - Example

prompt> nat list resvmaps extinterface

NAT reserved mappings:

ID |    Name      |  Global Address | Internal Address | Type  | Port

-----------------------------------------------------------------------

1 |  rm2         | 192.168.103.2   | 10.10.10.10      | tcp   | 25

2 |  rm1         | 192.168.103.15  | 20.20.20.20      | udp   | 21

-----------------------------------------------------------------------

    - See also

security list interfaces

nat show resvmap

    - Syntax

nat show resvmap <name> <interfacename>

    - Description

This command displays the following information about a single reserved mapping configuration that has been added to an outside security interface:

- Global IP address

- Internal IP address

- Transport type

- Port number

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing global pool. To display global pool names, use the nat list resvmaps command. | N/A |
| interfacename | The name of an existing security interface (external or DMZ) created and connected to an inside interface (DMZ or internal) using the nat enable command. To display security interfaces, use the security list interfaces command. | N/A |

    - Example

prompt> nat show resvmap rm1 extinterface

NAT reserved mapping: rm1

Global IP address: 192.168.103.15

Internal IP address: 20.20.20.20

Transport type: tcp

Port number: 25

    - See also

nat list resvmaps

security list interfaces

nat status

    - Syntax

nat status

    - Description

This command lists the outside security interfaces and inside interface types that NAT is currently enabled between. It displays the following information:

- NAT object identification number

- NAT object name

- Outside security interface name

- Inside interface type

    - Example

prompt> nat status

NAT enabled on:

ID |    Name    | Interface  |  Type

------------------------------------------

1 | n2         | ip2        | internal

2 | n1         | if1        | internal

------------------------------------------

    - See also

nat enable

nat set translation tcp-timeout

    - Syntax

nat set translation tcp-timeout <time>

    - Description

This command changes the TCP session timeout value The system maintains a session table for recording all NAT/PAT information. The table is a limited resource. This timeout value is used to delete the inactive session entries that keep idle for specified time.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| time | The maximum inactive time (in seconds) that a TCP session is treated as idle. | 1800 |

    - Example

prompt> nat set translation tcp-timeout 3600

    - See also

nat show translation

nat set translation udp-timeout

    - Syntax

nat set translation udp-timeout <time>

    - Description

This command changes the UDP session timeout value The system maintains a session table for recording all NAT/PAT information. The table is a limited resource. This timeout value is used to delete the inactive session entries that keep idle for specified time.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| time | The maximum inactive time (in seconds) that a UDP session is treated as idle. | 120 |

    - Example

prompt> nat set translation udp-timeout 1800

    - See also

nat show translation

nat set translation timeout

    - Syntax

nat set translation timeout <time>

    - Description

This command changes the non-TCP/UDP session timeout value The system maintains a session table for recording all NAT/PAT information. The table is a limited resource. This timeout value is used to delete the inactive session entries that keep idle for specified time.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| time | The maximum inactive time (in seconds) that a non-TCP/UDP session (e.g. ICMP) is treated as idle. | 60 |

    - Example

prompt> nat set translation timeout 360

    - See also

nat show translation

nat show translation

    - Syntax

nat show translation

    - Description

This command displays the current session timeout value for TCP, UDP and others.

    - Example

prompt> nat show translation

NAT session timeout:

TCP Session Timeout : 1800 Secs

UDP Session Timeout : 120 Secs

Other Session Timeout : 60 Secs

    - See also

nat set translation tcp-timeout

nat set translation udp-timeout

nat set translation timeout

nat set lowmemorythroshold

    - Syntax

nat set lowmemorythreshold <value>

    - Description

This command adjusts the threshold value of low memory indicator for maintaining session table. If the free session memory size is lower than the threshold , the system will start to perform special housekeeping.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| value | The low watermark of session memory (in bytes). | 5000 |

    - Example

prompt> nat set lowmemorythreshold 0

    - See also