10.Classifier CLI commands

This chapter describes the Classifier CLI

commands

| 10.1 Summary |  |
| --- | --- |
|  | 10.1.1 Classifier CLI commands |

The table below lists the Classifier commands provided by the CLI:

| Command | Reference |
| --- | --- |
| classifier add profile | classifier add profile |
| classifier clear profiles | classifier clear profiles |
| classifier delete profile | classifier delete profile |
| classifier list profiles | classifier list profiles |
| classifier profile add rule | classifier profile add rule |
| classifier profile clear rules | classifier profile clear rules |
| classifier profile delete rule | classifier profile delete rule |
| classifier profile list rules | classifier profile list rules |
| classifier profile set rule droppri | classifier profile set rule droppri |
| classifier profile set rule dscp | classifier profile set rule dscp |
| classifier profile set rule dstaddr | classifier profile set rule dstaddr |
| classifier profile set rule dstport | classifier profile set rule dstport |
| classifier profile set rule mark dscp | classifier profile set rule mark dscp |
| classifier profile set rule meterid | classifier profile set rule meterid |
| classifier profile set rule priority | classifier profile set rule priority |
| classifier profile set rule protocol | classifier profile set rule protocol |
| classifier profile set rule srcaddr | classifier profile set rule srcadd |
| classifier profile set rule srcport | classifier profile set rule srcport |
| classifier profile set rule tos | classifier profile set rule to |
| classifier show profile | classifier show profile |

| 10.2 | classifier add profile |  |
| --- | --- | --- |
|  | 10.2.1 | Syntax |
|  | classifier add profile <name> |  |
|  | 10.2.2 | Description |
|  | This command creates a classifier profile. You can add rules to the |  |
|  | profile using the classifier profile add rule command. You can then set |  |
|  | the profile to work on an ATM transport or transports, using the |  |
|  | transport set classifier profile command. |  |

| 10.2.3 | Options |
| --- | --- |
|  | The following table gives the range of values for each option which can |
|  | be specified with this command and a default value for each option (if |
|  | applicable). |

| Option | Description | Default value |
| --- | --- | --- |
|  | An arbitrary name that identifies the |  |
| name | profile. It can be made up of one or more letters or a combination of letters and | N/A |
|  | digits, but it cannot start with a digit. |  |

| 10.2.4 | Example prompt> classifier add profile pr1 |
| --- | --- |
| 10.2.5 | See also |
|  | classifier profile add rule |
|  | transports set classifier profile |

| 10.3 | classifier clear profiles |
| --- | --- |
|  | 10.3.1 Syntax |
|  | classifier clear profiles |
|  | 10.3.2 Description |
|  | This command deletes all classifier profiles that were created using the classifier add profile command. Any rules associated with the profiles are also deleted by this command. |
|  | 10.3.3 Example |
|  | prompt> classifier clear profiles |
|  | 10.3.4 See also |
|  | classifier add profile |

| 10.4 | classifier delete profile |  |
| --- | --- | --- |
|  | 10.4.1 | Syntax |
|  | classifier delete profile {<name>¦<number>} |  |
|  | 10.4.2 | Description |
|  | This command deletes a single profile that was created using the |  |
|  | classifier add profile command. Any rules associated with the profile |  |
|  | are also deleted by this command. |  |
|  | 10.4.3 | Options |
|  | The following table gives the range of values for each option which can |  |
|  | be specified with this command and a default value (if applicable). |  |

| Option | Description | Default value |
| --- | --- | --- |
|  | A name that identifies an existing profile. |  |
| name | To display profile names, use the | N/A |
|  | classifier list profiles command. |  |
|  | A number that identifies an existing |  |
| number | profile. To display profile numbers, use | N/A |
|  | the classifier list profiles command. |  |

| 10.4.4 | Example prompt> classifier delete profile pr1 |
| --- | --- |
| 10.4.5 | See also |
|  | classifier add profile |

| 10.5 | classifier list profiles |  |  |
| --- | --- | --- | --- |
|  | 10.5.1 Syntax |  |  |
|  | classifier list profiles |  |  |
|  | 10.5.2 Description |  |  |
|  | This command lists all classifier profiles that have been created using |  |  |
|  | the classifier add profile command. It displays the following |  |  |
|  | information about classifier profiles: |  |  |
|  | • profile ID number |  |  |
| • profile name |  |  |  |
| 10.5.3 |  | Examples |  |
|  |  | prompt> classifier list profiles |  |
|  |  | Classifier Profiles: |  |
|  |  | ID ¦ Name |  |
|  |  | ----¦--------------- |  |
|  |  | 1 | ¦ pr1 |
|  |  | 2 | ¦ pr2 |
|  |  | 3 | ¦ pr3 |
|  |  | -------------------- |  |
| 10.5.4 |  | See also |  |
|  |  | classifier add profile |  |

| 10.6 | classifier profile add rule |
| --- | --- |
|  | 10.6.1 Syntax |
|  | classifier profile {<name>¦<number>} add rule <rule name> |
|  | 10.6.2 Description |
|  | This command adds a rule to an existing profile. Once you have created a rule, you can configure it using the classifier profile set rule commands. |
|  | 10.6.3 Options |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |
| rule name | An arbitrary name that identifies the rule. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |

| 10.6.4 | Example prompt> classifier profile pr1 add rule r1 |
| --- | --- |
| 10.6.5 | See also |
|  | classifier list profiles |

| 10.7 | classifier profile clear rules |  |
| --- | --- | --- |
|  | 10.7.1 | Syntax |
|  | classifier profile {<name>¦<number>} clear rules |  |
|  | 10.7.2 | Description |
|  | This command deletes all of the rules that were added to an existing |  |
|  | profile using the classifier profile add rule command. |  |
|  | 10.7.3 | Options |
|  | The following table gives the range of values for each option which can |  |
|  | be specified with this command and a default value for each option (if |  |
|  | applicable). |  |

| Option | Description | Default value |
| --- | --- | --- |
|  | A name that identifies an existing profile. |  |
| name | To display profile names, use the | N/A |
|  | classifier list profiles command. |  |
|  | A number that identifies an existing |  |
| number | profile. To display profile numbers, use | N/A |
|  | the classifier list profiles command. |  |

| 10.7.4 | Example prompt> classifier profile pr1 clear rules |
| --- | --- |
| 10.7.5 | See also |
|  | classifier profile add rule |
|  | classifier list profiles |

| 10.8 | classifier profile delete rule |
| --- | --- |
|  | 10.8.1 Syntax |
|  | classifier profile {<name>¦<number>} delete rule {<rule name>¦ <rule number>} |
|  | 10.8.2 Description |
|  | This command deletes a single rule that was added to an existing profile |
|  | using the classifier profile add rule command. |
|  | 10.8.3 Options |
|  | The following table gives the range of values for each option which can |
|  | be specified with this command and a default value for each option (if |
|  | applicable). |

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |
| rule name | A name that identifies an existing rule. To display rule names, use the classifier profile list rules command. | N/A |
| rule number | A number that identifies an existing rule. To display rule numbers, use the classifier profile list rules command. | N/A |

| 10.8.4 | Example prompt> classifier profile pr1 delete rule r1 |  |
| --- | --- | --- |
| 10.8.5 | See also |  |
|  | classifier list profiles |  |
|  | classifier profile add rule |  |

| 10.9 | classifier profile list rules |  |  |
| --- | --- | --- | --- |
|  | 10.9.1 Syntax |  |  |
|  | classifier profile list rules |  |  |
|  | 10.9.2 Description |  |  |
|  | This command displays the following information about the rules previously added to an existing profile: |  |  |
| • |  | rule ID number |  |
| • |  | rule name |  |
| • Test details; each packet is tested against the following configurable fields: |  |  |  |
| • |  |  | the source IP address and netmask |
| • |  |  | the destination IP address and netmask |
| • the Differentiated Services Code Point (DSCP) |  |  |  |
| • the protocol name or number |  |  |  |
| • the source port range |  |  |  |
| • the destination port range |  |  |  |
| If a packet matches the configured fields, the packet is assigned a specific QoSC. If a DSCP is specified by the classifier profile set rule mark command, a DSCP is also assigned. The assigned fields are also displayed in the Test details column. |  |  |  |
| 10.9.3 |  | Options |  |
|  |  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |

| Option | Description | Default value |
| --- | --- | --- |
|  | A name that identifies an existing profile. |  |
| name | To display profile names, use the | N/A |
|  | classifier list profiles command. |  |
|  | A number that identifies an existing |  |
| number | profile. To display profile numbers, use | N/A |
|  | the classifier list profiles command. |  |

| 10.9.4 | Example |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | prompt> classifier profile pr1 list rules |  |  |  |  |  |  |  |  |
|  | Classifier profile: pr1 |  |  |  |  |  |  |  |  |
|  | ID ¦ Name ¦ Test details |  |  |  |  |  |  |  |  |
|  | ----¦-------------------------------------------------------- |  |  |  |  |  |  |  |  |
|  | 1 ¦ r1 | ¦ Set QOSC 1 |  | Mark DSCP none |  |  |  |  |  |
|  | ¦ | ¦ Src Addr 1.1.1.1 Src Mask |  |  | 255.255.255.0 |  |  |  |  |
|  | ¦ | ¦ Dst Addr 2.2.2.2 Dst Mask |  |  | 255.255.255.255 |  |  |  |  |
|  | ¦ | ¦ DSCP any Protocol |  |  | UDP |  |  |  |  |
|  | ¦ | ¦ Src Port 2727-2727 Dst Port |  |  | 0 -0 |  |  |  |  |
| ------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| 10.9.5 | See also |  |  |  |  |  |  |  |  |
|  | classifier list profiles |  |  |  |  |  |  |  |  |

| 10.10 classifier profile set rule droppri |  |  |
| --- | --- | --- |
| 10.10.1 | Syntax |  |
|  | classifier profile {<name>¦<number>} set rule {<rule name>¦<rule number>} droppri <droppri> |  |
| 10.10.2 | Description |  |
|  | This command configures the specified drop priority to be assigned to packets when the rule is matched. The drop priority of the packet is read by the algorithmic dropper module. |  |
|  | If the algorithmic dropper is not in use (for example, the scheduler device is not included in the build), this setting has no effect. |  |
| 10.10.3 | Options |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |
| rule name | A name that identifies an existing rule. To display rule names, use the classifier profile list rules command. | N/A |
| rule number | A number that identifies an existing rule. To display rule numbers, use the classifier profile list rules command. | N/A |
| droppri | The drop priority set as either 0, 1 or 2: O = the lowest drop priority (green) 1 = the medium drop priority (yellow) 2 = the highest drop priority (red) | N/A |

| 10.10.4 | Example prompt> classifier profile pr1 set rule r1 droppri 0 |  |  |
| --- | --- | --- | --- |
| 10.10.5 | See also |  |  |
|  |  | classifier list profiles classifier profile list rules ALD CLI commands |  |

| 10.11 classifier profile set rule dscp |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 10.11.1 | Syntax |  |  |  |  |  |
|  | classifier profile {<name>¦<number>} set rule {<rule name>¦<rule number>} dscp <dscp> |  |  |  |  |  |
| 10.11.2 | Description |  |  |  |  |  |
|  | This command configures a classifier rule to test the Differentiated Services Code Point (DSCP) field in the IP header of incoming packets. |  |  |  |  |  |
|  | The DSCP field is also known as the Type of Service (TOS) field. See the classifier profile set rule tos command, which provides an alternative method of testing the same IP header field. |  |  |  |  |  |
| 10.11.3 | Options |  |  |  |  |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |  |  |  |  |
|  |  | Option | Description | Default value |  |  |
|  |  | name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |  |  |
|  |  | number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |  |  |
|  |  | rule name | A name that identifies an existing rule. To display rule names, use the classifier profile list rules command. | N/A |  |  |
|  |  | rule number | A number that identifies an existing rule. To display rule numbers, use the classifier profile list rules command. | N/A |  |  |
|  |  | dscp | The Differentiated Service Code Point (DSCP). This value is specified as 6 binary digits (NNNNNN). Set to anyto cancel the dscp setting. | any |  |  |

| 10.11.4 | Example |  |  |
| --- | --- | --- | --- |
|  | prompt> classifier profile pr1 set rule r1 dscp 101110 |  |  |
| 10.11.5 |  | See also |  |
|  |  | classifier list profiles classifier profile list rules classifier profile set rule tos |  |

| 10.12 classifier profile set rule dstaddr |  |  |
| --- | --- | --- |
| 10.12.1 | Syntax |  |
|  | classifier profile {<name>¦<number>} set rule {<rule name>¦<rule number>} dstaddr <addr> <mask> |  |
| 10.12.2 | Description |  |
|  | This command configures a classifier rule to test the destination IP address of incoming packets. The netmask address is compulsory and allows you to match either a whole subnet or just one host using the mask 255.255.255.255. |  |
|  | Set the dstaddr values to 0.0.0.0 if you want to cancel this test and set the rule to match any destination IP address. |  |
| 10.12.3 | Options |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |
| rule name | A name that identifies an existing rule. To display rule names, use the classifier profile list rules command. | N/A |
| rule number | A number that identifies an existing rule. To display rule numbers, use the classifier profile list rules command. | N/A |
| addr | The destination IP address, displayed in the following format: 192.168.102.3 | 0.0.0.0 |
| mask | The destination subnet mask address, displayed in the following format: 255.255.255.0 | 0.0.0.0 |

| 10.12.4 | Example prompt> classifier profile pr2 set rule r1 dstaddr 192.168.86.93 255.255.255.255 |
| --- | --- |
| 10.12.5 | See also |
| classifier list profiles classifier profile list rules |  |

| 10.13 classifier profile set rule dstport |  |  |
| --- | --- | --- |
| 10.13.1 | Syntax |  |
|  | classifier profile {<name>¦<number>} set rule {<rule name>¦<rule number>} dstport <min> <max> |  |
| 10.13.2 | Description |  |
|  | This command configures a classifier rule to test the destination port of incoming packets. You can specify a range of ports by configuring different minimum and maximum port numbers, or specify a single port by configuring the same port number for both port values. |  |
|  | Set the dstport min and max values to 0if you want to cancel this test and set the rule to match any destination port. |  |
|  | Note- This rule is only meaningful for TCP and UDP protocols. If you have set a destination port rule, you must also specify which protocol (TCP or UDP) should be tested by the rule. See classifier profile set rule protocol. |  |
| 10.13.3 | Options |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable) |  |

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |
| rule name | A name that identifies an existing rule. To display rule names, use the classifier profile list rules command. | N/A |
| rule number | A number that identifies an existing rule. To display rule numbers, use the classifier profile list rules command. | N/A |
| min | The start of the destination port range for a TCP or UDP protocol. | 0 |
| max | The end of the destination port range for a TCP or UDP protocol. | 0 |

| 10.13.4 | Example prompt> classifier profile pr1 set rule r1 dstport 2727 2727 |  |
| --- | --- | --- |
| 10.13.5 | See also |  |
| classifier list profiles classifier profile list rules classifier profile set rule protocol |  |  |

| 10.14 classifier profile set rule mark dscp |  |  |
| --- | --- | --- |
| 10.14.1 | Syntax |  |
|  | classifier profile {<name>¦<number>} set rule {<rule name>¦<rule number>} mark dscp <dscp> |  |
| 10.14.2 | Description |  |
|  | This command determines what happens if a rule is matched. You can configure the classifier to mark packets with a specific Differentiated Services Code Point (DSCP). If set, the DSCP field in the IP header will be changed to the value set here. |  |
| 10.14.3 | Options |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |
| rule name | A name that identifies an existing rule. To display rule names, use the classifier profile list rules command. | N/A |
| rule number | A number that identifies an existing rule. To display rule numbers, use the classifier profile list rules command. | N/A |
| dscp | The Differentiated Service Code Point (DSCP) that matched packets are marked with. This can be in hexadecimal format (0xNN) or as 6 binary digits (NNNNNN). Set to none to cancel the dscp setting. | none |

|  | 10.14.4 |  | Example |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | prompt> classifier profile pr1 set rule r1 mark dscp 101110 |  |  |
| 10.14.5 |  | See also |  |  |  |
|  | classifier list profiles classifier profile list rules |  |  |  |  |

| 10.15 classifier profile set rule meterid |  |  |
| --- | --- | --- |
| 10.15.1 | Syntax |  |
|  | classifier profile {<name>¦<number>} set rule {<rule name>¦<rule number>} meterid <meterid> |  |
| 10.15.2 | Description |  |
|  | This command determines what happens if a rule is matched. You can configure the classifier to assign a meter Id to the packet matching a rule. |  |
|  | Once you have set which meter Id to assign, you need to create an association between the classified stream and the meter instance using the transports set meter instance command. |  |
|  | If the meter device is not in use, this setting has no effect. |  |
| 10.15.3 | Options |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |
| rule name | A name that identifies an existing rule. To display rule names, use the classifier profile list rules command. | N/A |
| meterid | A number that represents a meter instance. The meter instance and profile is set on a specific transport using the transports set meter instance command. | N/A |

| 10.15.4 | Example |
| --- | --- |
|  | prompt> classifier profile pr1 set rule r1 meterid 1 |

| 10.15.5 | See also |  |
| --- | --- | --- |
| classifier list profiles classifier profile list rules transports set meter instance profile |  |  |

| 10.16 classifier profile set rule priority |  |  |
| --- | --- | --- |
| 10.16.1 | Syntax |  |
|  | classifier profile {<name>¦<number>} set rule {<rule name>¦<rule number>} priority <priority> |  |
| 10.16.2 | Description |  |
|  | This command determines what happens if a rule is matched. You can configure the classifier to assign a priority to the packet matching a rule. The priority is mapped to a scheduling priority value or queue ID (configured using the scheduler profile set queue weight command) in order for the scheduler to identify different priority streams and provide appropriate scheduling behavior. |  |
|  | Multiple traffic streams may map to the same scheduling priority. |  |
| 10.16.3 | Options |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |
| rule name | A name that identifies an existing rule. To display rule names, use the classifier profile list rules command. | N/A |
| priority | The scheduling priority value or queue ID value. This can be any number between 1 and 7 (inclusive). To display scheduling priority values, use the scheduler show profile queues command. | N/A |

| 10.16.4 | Example |
| --- | --- |
|  | prompt> classifier profile pr1 set rule r1 priority 1 |

| 10.16.5 | See also |
| --- | --- |
|  | classifier list profiles |
|  | classifier profile list rules |
|  | scheduler profile set queue weight |

| 10.17 classifier profile set rule protocol |  |  |  |
| --- | --- | --- | --- |
| 10.17.1 | Syntax |  |  |
|  | classifier profile {<name>¦<number>} set rule {<rule name>¦<rule number>} protocol <protocol> |  |  |
| 10.17.2 | Description |  |  |
|  | This command configures a classifier rule to test the protocol of incoming packets. You can specify the following protocols by name or decimal protocol number: |  |  |
|  | • TCP (6) |  |  |
|  | • UDP (17) |  |  |
|  | • | ICMP (1) |  |
|  | • | GRE (47) |  |
|  | All other protocols can only be specified by their decimal protocol number. |  |  |
| 10.17.3 | Options |  |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |  |

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |
| rule name | A name that identifies an existing rule. To display rule names, use the classifier profile list rules command. | N/A |
| rule number | A number that identifies an existing rule. To display rule numbers, use the classifier profile list rules command. | N/A |
| protocol | The name or decimal number of the supported protocol. Set to any to cancel the test and allow the rule to match any protocol. | any |

| 10.17.4 | Example prompt> classifier profile pr1 set rule r1 protocol udp |
| --- | --- |
| 10.17.5 | See also |
|  | classifier list profiles |
|  | classifier profile list rules |

| 10.18 classifier profile set rule srcaddr |  |  |
| --- | --- | --- |
| 10.18.1 | Syntax |  |
|  | classifier profile {<name>¦<number>} set rule {<rule name>¦<rule number>} srcaddr <addr> <mask> |  |
| 10.18.2 | Description |  |
|  | This command configures a classifier rule to test the source IP address of incoming packets. The netmask address is compulsory and allows you to match either a whole subnet or just one host using the mask 255.255.255.255. |  |
|  | Set the srcaddrvalues to 0.0.0.0if you want to cancel this test and set the rule to match any source IP address. |  |
| 10.18.3 | Options |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |
| rule name | A name that identifies an existing rule. To display rule names, use the classifier profile list rules command. | N/A |
| rule number | A number that identifies an existing rule. To display rule numbers, use the classifier profile list rules command. | N/A |
| addr | The source IP address, displayed in the following format: 192.168.102.3 | 0.0.0.0 |
| mask | The source subnet mask address, displayed in the following format: 255.255.255.0 | 0.0.0.0 |

| 10.18.4 | Example prompt> classifier profile pr2 set rule r1 srcaddr 192.168.101.1 255.255.255.255 |
| --- | --- |
| 10.18.5 | See also |
|  | classifier list profiles classifier profile list rules |

| 10.19 classifier profile set rule srcport |  |  |
| --- | --- | --- |
| 10.19.1 | Syntax |  |
|  | classifier profile {<name>¦<number>} set rule {<rule name>¦<rule number>} srcport <min> <max> |  |
| 10.19.2 | Description |  |
|  | This command configures a classifier rule to test the source port of incoming packets. You can specify a range of ports by configuring different minimum and maximum port numbers, or specify a single port by configuring the same port number for both port values. |  |
|  | Set the srcport value to 0.0.0.0if you want to cancel this test and set the rule to match any source port. |  |
|  | Note- This rule is only meaningful for TCP and UDP protocols. If you have set a source port rule, you must also specify which protocol (TCP or UDP) should be tested by the rule. See classifier profile set rule protocol . |  |
| 10.19.3 | Options |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable) |  |

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |
| rule name | A name that identifies an existing rule. To display rule names, use the classifier profile list rules command. | N/A |
| rule number | A number that identifies an existing rule. To display rule numbers, use the classifier profile list rules command. | N/A |
| min | The start of the source port range for a TCP or UDP protocol. | 0 |
| max | The end of the source port range for a TCP or UDP protocol. | 0 |

| 10.19.4 | Example prompt> classifier profile pr1 set rule r1 srcport 2727 2727 |  |
| --- | --- | --- |
| 10.19.5 | See also |  |
|  | classifier list profiles classifier profile list rules classifier profile set rule protocol |  |

| 10.20 classifier profile set rule tos |  |  |
| --- | --- | --- |
| 10.20.1 | Syntax |  |
|  | classifier profile {<name>¦<number>} set rule {<rule name>¦<rule number>} tos <tos> |  |
| 10.20.2 | Description |  |
|  | This command configures a classifier rule to test the Type of Service (ToS) field in the IP header of incoming packets. |  |
|  | The ToS field is also known as the DSCP field. See also the classifier |  |
|  | profile set rule dscp command which provides an alternative method of testing the same IP header field. |  |
| 10.20.3 | Options |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |
| rule name | A name that identifies an existing rule. To display rule names, use the classifier profile list rules command. | N/A |
| rule number | A number that identifies an existing rule. To display rule numbers, use the classifier profile list rules command. | N/A |
| tos | The Type of Service (ToS). This value is specified in hexadecimal format (0xNN) between the ranges 0x00 and 0xfc. The 2 least significant bits are ignored. Set to any to cancel the tos setting. | any |

| 10.20.4 | Example prompt> classifier profile pr1 set rule r1 tos 0x04 |  |
| --- | --- | --- |
| 10.20.5 | See also |  |
|  | classifier list profiles classifier profile list rules classifier profile set rule dscp |  |

| 10.21 classifier show profile |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 10.21.1 | Syntax |  |  |  |  |  |
|  | classifier show profile {<name>¦<number>} |  |  |  |  |  |
| 10.21.2 | Description |  |  |  |  |  |
|  | This command displays the following information about the rules previously added to an existing profile: |  |  |  |  |  |
|  | • rule ID number • rule name • Test details; each packet is tested against the following configurable fields: |  |  |  |  |  |
|  | • |  | the source IP address and netmask |  |  |  |
|  | • |  | the destination IP address and netmask |  |  |  |
|  |  | • the Differentiated Services Code Point (DSCP) |  |  |  |  |
|  |  | • the protocol name or number |  |  |  |  |
|  |  | • the source port range |  |  |  |  |
|  |  | • the destination port range |  |  |  |  |
|  | If a packet matches the configured fields, the packet is assigned a specific QoSC. If a DSCP is specified by the classifier profile set rule mark command, a DSCP is also assigned. The assigned fields are also displayed in the Test details column. |  |  |  |  |  |
| 10.21.3 | Options |  |  |  |  |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |  |  |  |  |

| Option | Description | Default value |
| --- | --- | --- |
|  | A name that identifies an existing profile. |  |
| name | To display profile names, use the | N/A |
|  | classifier list profiles command. |  |
|  | A number that identifies an existing |  |
| number | profile. To display profile numbers, use | N/A |
|  | the classifier list profiles command. |  |

| 10.21.4 |  | Example |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | prompt> classifier show profile pr1 |  |  |  |  |  |  |  |
|  |  | Classifier profile: pr1 |  |  |  |  |  |  |  |
|  |  | ID ¦ Name ¦ Test details |  |  |  |  |  |  |  |
|  |  | ----¦------------------------------------------------------- |  |  |  |  |  |  |  |
|  |  | 1 ¦ r1 |  | ¦ Set QOSC 1 | Mark DSCP none |  |  |  |  |
|  | ¦ |  | ¦ Src Addr 1.1.1.1 Src Mask |  |  | 255.255.255.0 |  |  |  |
|  | ¦ |  | ¦ Dst Addr 2.2.2.2 Dst Mask |  |  | 255.255.255.255 |  |  |  |
|  | ¦ |  | ¦ DSCP any Protocol |  |  | UDP |  |  |  |
|  | ¦ |  | ¦ Src Port 2727-2727 Dst Port |  |  | 0 -0 |  |  |  |
|  |  | ------------------------------------------------------------ |  |  |  |  |  |  |  |
| 10.21.5 |  | See also |  |  |  |  |  |  |  |
|  |  | classifier list profiles classifier profile list rules classifier profile set rule protocol |  |  |  |  |  |  |  |