- DDNS CLI commands

This chapter describes CLI support for Dynamic DNS.

Summary

    - DDNS CLI commands

The table below lists the Dynamic DNS commands provided by the CLI:

| Command | Reference |
| --- | --- |
| ddns enable¦disable | ddns enable¦disable |
| ddns list servers | ddns list servers |
| ddns set hostdomainname | ddns set hostdomainname |
| ddns set ipinterface | ddns set ipinterface |
| ddns set password | ddns set password |
| ddns set refreshevery | ddns set refreshevery |
| ddns set server-id | ddns set server-id |
| ddns set timeunit | ddns set timeunit |
| ddns set username | ddns set username |
| ddns show status | ddns show status |

ddns enable|disable

    - Syntax

ddns <enable|disable>

    - Description

This command enables/disables the DDNS service. Before enabling the DDNS service, you should configure the DDNS settings. When enabled, the system will try to update its Dynamic DNS information to DDNS server. By default, the DDNS service is disabled.

    - Example

prompt> ddns enable

    - See also

ddns set

ddns list servers

    - Syntax

ddns list servers

    - Description

This command lists all DDNS servers supported in the system and its corresponding server-ID. The server-ID is used in ddns set server-id command to specify the DDNS server will connect to.

    - Example

prompt> ddns list servers

Supported Dynamic DNS Server List:

1: www.dyndns.org (custom)

2: www.dyndns.org (dynamic)

3: www.dyndns.org (static)

4: www.zoneedit.com

5: www.orgdns.org

6: www.dhs.org

7: www.dyns.cx

8: www.minidns.net

9: www.no-ip.com

    - See also

ddns set server-id

ddns enable|disable

ddns set hostdomainname

    - Syntax

ddns set hostdomainname <name>

    - Description

This command configure the Host Domain Name that will used to register to DDNS server. When the registration succeeded, somebody else can locate this device through this domain name.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The Host Domain Name or FQDN (Full Qualified Domain Name), like ddnstest.dyndns.org. | N/A |

    - Example

prompt> ddns set hostdomainname ddnstest.dyndns.org

    - See also

ddns enable|disable

ddns set ipinterface

    - Syntax

ddns set ipinterface <interface>

    - Description

The system may contains more than one IP interfaces. This command configure the IP interface you want to use to send out updating packets to register to DDNS server. The ip list interfaces  command displays the existing IP interfaces in the system.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| interface | The IP interface name of the system. | N/A |

    - Example

prompt> ddns set hostdomainname ipwan

    - See also

ddns enable|disable

ip list interfaces

ddns set password

    - Syntax

ddns set password <password>

    - Description

This command configures the password embedded in updating packets for DDNS server to authenticate the updating packet content. This information is obtained when you make registration to a specific DDNS server.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| password | The password for authenticating purpose.. | N/A |

    - Example

prompt> ddns set password pwd

    - See also

ddns set username

ddns set refreshevery

    - Syntax

ddns set refreshevery <count>

    - Description

This command configure how frequent to send out updating packet to DDNS server. It is used accompanied with ddns set timeunt command that specifies the time unit is either hour or day. Please note, some DDNS servers cannot accept updating packets too frequent if the dynamic IP address has not changed.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| count | The tick number of time out to send a new updating packet. Range is 1-99. | 28 |

    - Example

prompt> ddns set refreshevery 1

    - See also

ddns set timeunt

ddns set server-id

    - Syntax

ddns set server-id <id>

    - Description

This command specifies the default DDNS server. The available DDNS server list and its corresponding ID can be found in ddns list servers command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| id | The ID of DDNS server. Now, it is ranged from 1 to 9. | N/A |

    - Example

prompt> ddns set server-id 2

    - See also

ddns enable|disable

ddns list servers

ddns set timeunit

    - Syntax

ddns set timeunit <unit>

    - Description

This command configure the time unit for refresh operation. The actual refresh interval is (refreshevery * timeunit).

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| unit | The time unit, either 1 (in hours) or 2 (in days). | 2 |

    - Example

prompt> ddns set timeunit 1

    - See also

ddns enable|disable

ddns set refreshevery

ddns set username

    - Syntax

ddns set username <name>

    - Description

This command configures the username embedded in updating packets for DDNS server to authenticate the updating packet content. This information is obtained when you make registration to a specific DDNS server.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The registration name for a specific DDNS server. | N/A |

    - Example

prompt> ddns set username simon.home

    - See also

ddns enable|disable

ddns set password

ddns show status

    - Syntax

ddns show status

    - Description

This command displays the current configurations and status of DDNS service. The information contains:

- DDNS service is enabled or disabled

- Have updated successfully or not

- IP interface

- DDNS server selected

- User’s domain name

- Username and password for login to DDNS server

- Refresh interval

    - Example

prompt> ddns list servers

Global Dynamic DNS Service Configuration: DDns

This Function: disabled

Dynamic DNS Updated: false

IP Interface Name: ipwan

Current DDNS Server: 2, www.dyndns.org (dynamic)

Host Domain Name:

Login User Name:

Login Password:

Refresh Period:

    - See also

ddns enable|disable

ddns set