### DE-220P

Ethernet Adapter

for ISA Bus

#### Installation Guide

### Introduction

Installation of a DE-220P Series Ethernet adapter requires these three steps:

¨ Insert the adapter into an available expansion slot.

¨ Connect network cable to the adapter.

¨ For non-PnP systems, configure the adapter by running the supplied setup program, setup . exe. (For PnP-compliant systems, this step is fully automated and requires no action . )

#### 1st Step – Instert the adapter into an extension slot

- Shut down the computer, unplug its power cord, and remove the chassis cover.

- Insert the DE-220P Series card into the connector of any open ISA expansion slot. Ascertain that the card is fully seated in the expansion slot connector.

- Install the bracket screw which secures the card to the computer chassis.

- Replace the computer’s chassis cover.

- For non-PnP systems, configure the DE-220P Series adapter by running the supplied setup program, setup.exe. (For PnP-compliant sys-tems, this step is fully automated and requires no action.) to disable Plug and Play fonction , give an IRQ address and I/E address. Then install the driver available on the disket.

2nd Step -  Connect the cable

Before connecting a network cable to the DE-220PCT adapter, ascertain that the network cable medium is Twisted-Pair or Thin Coaxial cable. Then refer to one of the two sections following, according to your network cable type.

TWISTED-PAIR CABLE

1. Ascertain that you have a straight 8 twisted-pair

cable not more than 100 meters (328 feet)

in length for the network connection.

2. The cable will have an RJ-45 plug on each

end. Plug one end into the RJ-45 receptacle of

the DE-220P Series adapter, and plug the

other end into an available port of a 10Base-T

hub. Push each RJ-45 plug into its receptacle

until its retainer snaps into place.

THIN-COAXIAL CABLE

1. Connect a BNC T-connector onto the BNC recep-tacle of the DE-220P Series card.

2. Connect the network feed cable onto either open

end of the BNC T-connector.

3a. If the cable continues on to an additional station

or stations, then connect the continuation cable

onto the remaining open end of the BNC T-connector.

3b. If the network cable does not continue to addi-tional stations (your computer is at the end of the

cable), then connect a 50-ohm terminator to the

remaining open end of the BNC T-connector.