DFE-520TX

Date: March 15, 2005

| Product name: | DFE-520TX |  |
| --- | --- | --- |
| Product Group & Type: |  |  |
| Revision: | A1 |  |
| Available: |  |  |
| Picture: |  |  |
| Why Sell This Product: | - This network card is the lowest cost network card offered by D-Link. It provides the basic functions for desktop Ethernet connection (no WOL, no boot ROM). It’s target at users needing just the basic Ethernet connection. Selling Points - Extremely low price. - Single-chip solution for high hardware reliance. - Small size, light weight. |  |
| Key Features: | - 802.3 10BASE-T/802.3u 100Base-TX standard - Auto-detection of 10Mbps and 100Mbps network speeds - Supports 802.1x function (MD5 only) - 32-bit PCI Local Bus Master high-speed operation - Full/half duplex operation - Complies with Advanced Configuration and Power Interface specification v.1.0 - Complies with PCI Power Management v.1.1 - Supports 802.3x Flow Control - Supports PCI clock speed up to 33Mhz independent of network clock - On-board FIFO buffer, no external memory on host required - Low Ethernet command processing overhead |  |
| Technical Details: | General Standards - IEEE 802.3 10BASE-T Ethernet - IEEE 802.3u 100BASE-TX Fast Ethernet - IEEE 802.3 NWay auto-negotiation - IEEE 802.3x Flow Control - PCI local bus 2.2 specification Topology Star Protocol CSMA/CD Network Data Transfer Rates - 10BASE-T: 10Mbps (half-duplex) 20Mbps (full-duplex) - 100BASE-TX: 100Mbps (half-duplex) 200Mbps (full-duplex) Network Cables - 10BASE-T: UTP Cat.3, 4, 5 (100 m max.) EIA/TIA-568 STP (100 m max.) - 100BASE-TX: UTP Cat. 5 (100 m max.) EIA/TIA-568 STP (100 m max.) Diagnostic LED Report - Link - Activity Physical & Environmental Power Consumption 0.4 watts max. Dimensions 120 x 20 mm Weight 106 grams Operating Temperature 0 to 40 ℃ Storage Temperature -10 to 70 ℃ Operating Humidity 10% to 90% RH Storage Humidity 5% to 90% RH Emission - FCC Class B - CE Class B - C-Tick O.S. Driver Support Microsoft Windows 98 SE, 2000, ME, XP |  |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |  |
| Safety |  TUV No.  CUL No.  CSA No.  Others |  |
| Packaging Style |  Clamshell  White Box Gift Box  Bulk  White Box with D-Link Logo  Others |  |
| Packaging Contents: | - DFE-520TX NIC card - One CD Rom for Manual/QIG/Driver - One hard copy QIG |  |
| UPC Code: | EFE520TX..A1; 790069275562. IFE520TX..A1; 790069275579. TFE520TX..A1; 790069275586. NFE520TX..A1; 790069275593. BFE520TX..A1; 790069275609. |
| Units Per Carton: | 60 |  |
| Cartons Per Pallet: | For USA Air:12, Sea: 14 | For WW Air:16, Sea: 16 |
| Dimensions and Weight: | Box: mm,Others 160mm x 182mm x 20mm, 0.106 kg Carton: 590x457x480 mm, 10.01 kg Pallet: (by Air) 1220x1020x1453 mm, 136.2 kg (for USA) 1200x1100x2055 mm, 178.66 kg (for WW) (by Sea) 1220x1020x2050 mm, 176.16 kg (for USA) 1200x1100x2055 mm, 178.66 kg (for WW) INTRASTAT weight: 0.058 kg (Box) Packing material weight (Box): - Paper: 0.022 kg - Plastic: 0.017 kg - Mixed: 0.019 kg |  |

| PMA |  |  |
| --- | --- | --- |
| Product Class |  |  |
| Item Group |  |  |
| Sub Group |  |  |
| Intelligence |  |  |
| Tech.(Features) |  |  |
| Layer |  |  |
| Application |  |  |
| Q'ty. |  |  |
| Interface |  |  |
| Net Interface |  |  |
| Port NO. |  |  |