DFE-550FX

#### Fiber-optics 100 Mbps Adapter.

#### Installation Guide

First Edition (Apr. 2000)

6FE550FXML01

Printed In Taiwan

RECYCLABLE

### Introduction

This Quick Installation Guide gives step-by-step instructions for setting up the D-Link DFE-550FX Fiber-optics 100 Mbps Adapter. For more detailed information about the adapter, making network connections, and technical specifications, please refer to the User’s Guide included with your adapter.

#### Step 1 – Unpacking

Please make sure the following items are present and undamaged.

- Network Adapter

- Driver Diskette

- User’s Guide

#### Step 2 – Hardware Installation

2-1 Shut down the computer & unplug the power cord.

2-2 Remove the chassis cover.

2-3 Insert the card into an empty PCI slot.

2-5 Install the bracket screw to secure the card to the computer chassis.

2-6 Replace the computer chassis cover.

2-7 Reconnect the power cord and switch the computer power on.

#### Step 3 – Connecting Network Cables

Multimode (62.5/125m or 50/125m) fiber cabling with an IEEE recommended SC connector is required for the adapter. Observe the correct relationship between the receive (Rx) and transmit (Tx) ports on the adapter and repeater.  If there is a need to monitor the adapter’s faceplate LEDs, make the rear of the PC or workstation accessible and leave sufficient clearance for cabling and service.  The maximum length of fiber cable between any station and its supporting hub is 2,000 meters at full-duplex and 412 meters at half-duplex.

#### Step 4 – Software Installation

On account of the great variety of network environments in which the DFE-550FX may be used, the instructions for software installation are given as README files on the DFE-550FX Software Diskette. Review the root directory and locate the subdirectory containing the appropriate software for use with your network operating system. Access the README file for overview information. For full installation details, see the README file and the referenced instruction files in the sub-directory.

#### Additional Information

If you are encountering problems setting up your adapter, please refer to the User’s Guide that came with the adapter. It contains many more rules, charts, explanations, and examples to help you install your adapter.

Additional help is available online at http://www.dlink.com for the United States and http://www.dlink.co.uk for Great Britain.  URLs for D-Link Websites in other countries are contained in the list of D-Link Offices at the back of the User’s Guide.