### DFE-570TX

#### Fast Ethernet

#### 4-port Server Adapter

#### Installation Guide

Rev. 01 (Apr. 2000)

6FE570TXML01

Printed In Taiwan

RECYCLABLE

### Introduction

This Quick Installation Guide gives step-by-step instructions for setting up the D-Link DFE-570TX 4-port Server Adapter. This adapter is used to provide a super high capacity network connection for Windows NT servers. For more detailed information about the adapter, making network connections, and technical specifications, please refer to the User’s Guide included with your adapter.

#### Step 1 – Unpacking

Please make sure the following items are present and undamaged.

1. Network Adapter	2. Driver Diskette	 3. User’s Guide

#### Step 2 – Hardware Installation

2-1 Shut down the computer & unplug the power cord.

2-2 Remove the chassis cover.

2-3 Insert the card into an empty PCI slot.

2-5 Install the bracket screw to secure the card to the computer chassis.

2-6 Replace the computer chassis cover.

2-7 Reconnect the power cord and switch the computer power on.

#### Step 3 – Connecting Network Cables

100 Mbps Fast Ethernet connections require either a Category 5 UTP cable or an STP cable.  These cables can be up to 100 meters long.

10 Mbps Ethernet connections require a Category 3 or better UTP cable.  These cables can also be up to 100 meters long.

It is recommended that you use Category 5 cabling for all connections, in order to make it easier to transition to 100Mbps.

#### Step 4 – Software Installation

4-1 Turn on your server and start Windows.

4-2 After your computer boots up, double-click on the Add New Hardware icon in the Control Panel.

4-3 When prompted ‘Do you want Windows to search for your new hardware?’, choose No. Click on the Next button.

4-4 When prompted ‘Select the type of hardware you want to install’, highlight Network Adapters and click on the Next button.

4-5 Insert the floppy diskette that came included with the adapter into the floopy drive of the server and choose Have Disk.

4-6 Choose the drive letter for the floppy drive and click OK.

The driver software will now be installed on the server. When prompted, Reboot the server. Installation is now complete.

#### Additional Information

If you are encountering problems setting up your adapter, please refer to the User’s Guide that came with the adapter. It contains many more rules, charts, explanations, and examples to help you install your adapter.

Additional help is available online at http://www.dlink.com for the United States and http://www.dlink.co.uk for Great Britain.  URLs for D-Link Websites in other countries are contained in the list of D-Link Offices at the back of the User’s Guide.