# DFL-200/700/1100 Firmware Release Note

Firmware: 1.33.00-SU1 ( Security Update 1)

Hardware: DFL-200-700(A1) / 1100 (A2)

Date: Nov 23 , 2005

Bug fixes

# Vulnerabilities in IKE v1 Implementation (CERT-FI 7710; 273756/NISCC/ISAKMP)

CERT-FI and NISCC published a joint vulnerability advisory on 14 Nov 2005 about several vulnerabilities in various IKEv1 protocol implementations. The vulnerabilities were found with PROTOS ISAKMP Test Suite for IKEv1 Phase 1, developed by Oulu University Security Programming Group (OUSPG)

=============================================================

Firmware: 1.33.00

Hardware: DFL-200-700(A1) / 1100 (A2)

Date: May 24, 2005

## Bug fixes

#2700 - Possible crash when an active L2TP tunnel was removed from configuration.

Affects dfl-1.30 and up.

#2791 - Some E100 interfaces don’t report the correct link status.

Affects dfl-1.00 and up (DFL-1100 only).

#2822 - Advanced settings for the DHCP client changed to work with the European

ISP Telia.

Affects dfl-1.00 and up.

#2831 - A statically configured host could prevent a DHCP enabled host with

Windows as host OS from receiving an "valid" IP.

Affects dfl-1.00 and up.

#2834 - High DHCPRelayer load can cause a transaction recycle bug resulting

in the relayer stalling.

Affects dfl-1.00 and up.

#2839 - An username or password containing some special characters could not

log in to a HTTPS session.

Affects dfl-1.00 and up.

Minor bug fixes

#2826 - PPTP and L2TP are now logging the authenticated user and assigned IP after

completed ppp negotiation.

Affects dfl-1.30 and up.

## Changes

- Rebuilt IDS signature database.

#2850 - Restart timers were the same for DFL-200/700/1100. The timers are now

modified to suit the models different startup times better.

#2859 - Warning added on the firmware upload page.

Firmware: 1.32.00

Hardware: DFL-200-700(A1) / 1100 (A2)

Date: Apr 14, 2005

Bug fixes

#2320 - Unable to login via serial console if a serial console password had

been set.

Affects dfl-1.30.

#2332 - PPPoE tunnels failing to connect would not be closed properly, leading

eventually to the system's tunnel limit being reached, if failures

occurred often. Reboot would be required to resume operation.

Affects dfl-1.30.

#2300 - PPTP and L2TP tunnels not usable in HA setups. Would cause warnings

and cause the slave unit refusing to forward traffic.

Affects DFL-1100, dfl-1.30.

Note: The only traffic likely to work outbound through PPTP and L2TP

client interfaces on a HA cluster is dynamically NATed traffic.

Without NAT, the server on the other end will likely not know where

to send return traffic.

#2381 - Configuring a signature rule that used more than 2000 signatures

would result in a buffer overflow during configuration.

Affects dfl-1.30

#2403 - ARP incompatibility with Microsoft NLB.

Affects dfl-1.00 and up.

#2447 - PPTP idle timeout was interpreted as seconds while the webUI said

minutes.

Affects dfl-1.30.

#2588 - TCP MSS option was sometimes parsed wrong for TCP traffic directly to

or from the firewall.

Affects DFL-200, v1.00 and up.

#2595 - PPTP setup messages were sent with erroneous values in some fields.

Affects dfl-1.30.

#2409 - Routes automatically added for IPsec were in some cases not removed when

tunnels were taken down.

Affects dfl-1.00 and up.

## Minor bug fixes

#2316 - JavaScript error when editing schedules.

Affects dfl-1.30.

#2388 - PPP license limit exceeded log format did not adhere to standard

logging format. (Hard to parse for software)

Affects dfl-1.30.

## Changes

#2301 - Support for DSA certificates in IPsec

#2349 - Added memlog for DFL-700

#2376 - Improved IDS Signatures format

Firmware: 1.30.00

Hardware: DFL-200-700(A1) / 1100 (A2)

Date: Jan 18, 2005

Bug fixes

#2031 - Sustained 100 Mbit/s small-packet flood could cause improper

operation (packet loss, high RTTs) of DFL-700 interfaces and

DFL-200 WAN interface until next restart.

Affects: DFL-200, DFL-700, v1.00 and up.

#2273 - RADIUS server mode (PAP/CHAP) not properly saved.

Editing a configuration with the mode set to CHAP causes it

to become PAP (unless explicitly set to CHAP each time).

Affects: v1.00 and up

#2237 - Read-Only Admin Access Settings would not obey the IP span configured

but rather use the IP span configured for full admin access.

Note however that per-interface filtering worked as expected.

Or, put another way: read-only access would not work unless full

admin access was also granted from that network.

Affects: DFL-700, DFL-1100, v1.00 and up.

Minor bug fixes

#2158 - Routes: Local network shown wrong for WAN interface when using DHCP.

In the route listing, the local network route for the WAN interface

would always be shown as "0.0.0.0/0" when using DHCP.

As of v1.30, it is shown as "(dynamic)".

#2178 - VPNs and VLANs with configured Admin Access could not be deleted

without first deleting the Admin Access and activating the configuration.

As of v1.30, Admin Access settings associated with an interface

are automatically deleted when the interface is deleted.

## Changes

#2094 - Implemented L2TP and PPTP clients and servers.

#2051 - Traffic shaping now configurable for VPN tunnels.

#2206 - IDS/IDP signatures are now automatically updated from public

D-Link servers if IDS/IDP is in use.  Signature packages are

digitally signed to prevent outside manipulation.

#2186 - Routes may now be manually configured through VPN tunnels.

Previously, only the Remote Network got routed across

the IPsec tunnel.

Note that for IPsec, the "Remote Network" must still cover

the route added. This is requirement in the IPsec protocol.

#2205 - Two or more interfaces may now share the same DHCP pool

by configuring the DHCP server settings for one interface

or VPN tunnel to relay the request to another interface

with an already-configured DHCP server.

#2025 - New up/down arrows in policy list allows rules to be moved

up and down one step at a time.

#2095 - User database format changed.

Due to requirements for L2TP and PPTP (specifically, the CHAP

and MSCHAP authentication modes) the user database format

was changed from irreversible cryptography to reversible.

Previously-existing database entries remain in the old format

until a new password is set, at which point it is stored in

the new format and the account becomes usable in L2TP and

PPTP servers.

#2014 - Changes to Schedules:

- Warning added if no hours were checked; such a schedule

never triggers

- All hours/weekdays now checked by default

- Per-day checkboxes added that enable all hours of a day

#2135 - Remote network now configurable on a per-user basis.

Used primarily by L2TP and PPTP servers.

#1874 - IPsec Interop: Full draft-beaulieu-ike-xauth-02 support

Although support for authenticating IPsec clients via XAuth has

existed for some time, XAuthInit* notifications were not sent to

clients. Some IPsec clients have been confused by this behavior.

XAuthInit* notifications are now sent.

#2166 - (DFL-200 ONLY) "Clear log" button added for in-memory log

#2146 - Dynamic DNS re-registration intervals increased.

dyndns.org: 28 days (previously daily)

cjb.net: 28 days (previously daily)

#2026 - IPsec Settings: Increase maximum length of Local and Remote Net

editboxes from 128 to 1024 characters. These boxes can contain

an arbitrarily long list of networks.

#2007 - IPsec Settings: Explained what the "Allowed:" part of e.g.

"AES-128 Allowed: 128-256" means in the page itself as well

as in the online help.

Firmware: 1.21.00

Hardware: A1

Date: Aug 31, 2004

## Bug fixes

#1903 - Instability (crash) problem in HTTP ALG resolved.

Affects: v1.00 -- v1.20.

#1939 - User database is not restored when restoring system backups.

Affects: v1.20

Additional info: the user db is actually momentarily restored,

but when activating the restored configuration, the user db

gets overwritten.  Also note that previous to v1.20, the user db

was not part of the backup dump at all.

#1942 - Editing a user without requesting a password change corrupts

the user's password.

Affects: v1.00 -- v1.20.

#1990 - VLANs interfaces could not be deleted if there were rules

defined for traffic to or from them.

Affects: v1.10 -- v1.20 (DFL-1100 only)

Rules regarding an interface being deleted are now automatically

deleted (the same is now done for VPN interfaces that have rules).

#1996 - PPPoE would not detect link failure and reconnect if the

remote end closed the connection.

This was especially problematic since there are a few ISPs

that close all existing PPPoE connections during the night.

Affects: v1.00 -- v1.20.

## Minor bug fixes

#1935 - HTTP ALG erroneously rejects some pages using "chunked encoding".

Affects: v1.00 -- v1.20.

Classified as minor due to the fact that the problem does not

trigger for the vast majority of pages using chunked encoding.

Also note that static HTML and images never use chunked encoding.

#1961 - Could not disable traffic shaping for a rule simply by unchecking

the "traffic shaping" checkbox. One would need to delete the

limits in the edit boxes.

Affects: v1.00 -- v1.20.

#1894 - Editing "Custom" DynDNS URLs transforms existing "&" to "&amp;".

Affects: v1.00 -- v1.20.

Classified as minor due to the fact that entering the URL

works - it is the display that is wrong.  Also due to the fact

that most DynDNS users use the pre-defined providers.

#1941 - Arrows missing around "Swap This/Shared" option in HA wizard.

Affects: v1.10 -- v1.20 (DFL-1100 only)

#1944 - HTTP ALG log format did not adhere to standard logging format.

(Hard to parse for software)

Affects: v1.00 -- v1.20.

#1954 - Adding a VLAN with an ID that already exists on the same

interface would result in a crash.

Classified as minor due to the fact that having two VLANs

on the same interface is not possible to begin with -- the

crash occured while displaying the error message resulting

from this action.

Affects: v1.10 -- v1.20 (DFL-1100 only)

## Changes

#1922 - Policies for VPN tunnels may now be edited.

This behavior is controlled by a setting in the "Global Policy

Options" page, and defaults to behaving as before.

A well-visible note about this is displayed in the policy

selection page (Firewall -> Policy).

#1938 - Status -> Users page added.

Shows logged-on users, and also information about on which

interfaces user authentication via HTTP(S) is currently

available. Also allows administrator to forcibly log users out.

#1943 - ARP responses from multicast MAC addresses are now accepted.

Some load balancing clusters work by associating a balanced

IP address with a layer 2 multicast MAC address.

As of v1.21, the firewall will accept such ARP responses.

#1949 - Dyndns.org re-post interval changed from daily to monthly.

Dyndns.org apparently does not approve of daily re-posts.

#1775 - More information shown for DHCP interfaces under

Status -> Interfaces: information for the current lease,

and reasons for failure.

#1781 - Tools->Ping will now retain the number of packets and packet

size settings when Submit is pressed (and the results are

displayed).

#1913 - Warning message now displayed if auditors (read-only users) are

added, but not actually allowed to access the firewall through

any interface (under system -> admin settings).

#1914 - Warning message now displayed if Content Filtering settings

are edited, but no rule actually makes use of HTTP content

filtering.

#1936 - VERY visible warning message now displayed on every page of the

web GUI if the firewall enters "lockdown" state.  (May happen if

something is severely broken, or if license limits are exceeded.)

#1973 - Warning message now displayed if HTTP ALG (HTTP Content Filtering)

is selected for port 443. No application layer logic can be

applied to HTTPS, since it is encrypted.

#1980 - License limitations (and current usage) is now displayed under

Status->System.