ENGLISH

# DSA-5100

Wireless Service Gateway Balancer

This product can be set up using any current web browser, i.e., Internet Explorer 6 or Netscape Navigator 6.2.3.

## Before You Begin

This Quick Installation Guide (QIG) provides instruction for installing the DSA-5100 and Ethernet configuration for other PCs. The former part is going to show how to connect DSA-5100 with other Network equipments and Network configuration of DSA-5100. The latter part is a description for configuring Network setting of other PCs on Ethernet to make sure the normal function of whole Network.

Check Your Package Contents CCCCONCContents

- DSA-5100 Wireless Service Gateway Balancer

- CD-ROM (Containing Manual and Warranty)

- 4 Ethernet cable

- CAT5 UTP(Straight-through, white) ,

- 1 CAT5 UTP Crossover cable (blue)

- RS232 Console Cable

Using a power cord

with a different voltage

rating will damage and void the

warranty of this product.

- Power Cord

If any of the above items are missing, please contact your reseller.

©2003 D-Link System, Inc. All rights reserved. Trademarks or registered trademarks are the property of their respective holders. Software and specifications subject to change without notice. DSA-5100. 05192003

Part No. IG-BDSA-5100

Product overview (Front)

POWER LED - Solid light: Power is on

light indicates a proper

connection to the power

supply.

WAN LED – a solid light

indicates connection on the

WAN port. The LED below

a connection from the Local

Network port for a Private

Network. The LED below blinks

WAN1 port - can be connected to the Internet or Intranet by Switch.

CONSOLE Port – For resetting to

factory defaults, or reconfiguring the device. For Advanced users only!

WAN2 port - can be connected to the Internet or Intranet by Switch.

Link LEDs – stands for being connected to RJ45 connectors

light indicates a proper

connection to the power

supply.

WAN LED – a solid light

indicates connection on the

WAN port. The LED below

a connection from the Local

Network port for a Private

Network. The LED below blinks

Public LAN port - can be connected to the open network environment managed by the system or WLAN , and requires authentication before logging on to the Internet.

ACT LEDs – stands for transmitting data.

AUX Port – Reserved for future usage

Private LAN port – can be connected to the trustful Intranet environment by Switch or Ethernet without authentication for logging on to the Internet.

Status LED – After boot up: Black: run BIOS.

Blink: run OS.

Solid light: System ready.

light indicates a proper

connection to the power

supply.

WAN LED – a solid light

indicates connection on the

WAN port. The LED below

a connection from the Local

Network port for a Private

Network. The LED below blinks

Power Socket:

Power Cord Plug

Product overview (Back)

Connecting the DSA-5100 to Your Network

C

CCCONCContents

A. Ensure that the DSA-5100 is powered ON.

B. Public LAN Port Connection.

Please use a Straight-through Ethernet cable to connect the DSA-5100 to a switch

or a wireless access point to provide public access to your broadband

connection. The Link LED light will illuminate to indicate a proper

connection. If the Link LED does not illuminate, please check the

device’s physical connections.

C. Insert an Ethernet cable to the Private LAN Port on the front

panel of the DSA-5100. Connect the other end of the Ethernet cable to a switch.

If the Link LED illuminates, it means you have a proper connection.

D. Insert an Ethernet cable into the WAN1 port on the front panel of the DSA-5100.

Connect the other end to a router or directly to a high speed cable or DSL

modem. If the connection is live, then the Link LED will illuminate.

E. ( Optional ) Insert an Ethernet cable into the WAN2 port on the front panel of the DSA-5100. Connect the other end to a router or directly to a high speed cable. If the connection is live, then the Link LED will illuminate.

Using The Setup Wizard

C

CCCONCContents

Open your Web browser and

type “https://192.168.0.40” into

the URL address box. Then press the Enter or Return key.

Type “admin” for both the username and the password

Click Enter

Click Run Wizard

Once you have logged in, the Home screen will appear.

2 . The setup Wizard (Continued)

Click next

You will see the following screen

Click next

Set up your new password.

You have the option to establish a password.

Click next

Select the appropriate time zone for your location

Click next

Set System Information in the

fields provided to you. The

Succeed Page is the URL that

you are directed to when you

have been successfully

Authenticated. The Time

Server and Domain Name

Server (DNS) must be required.

Click next

Select an Option

At this point, the setup wizard

will ask you for your Internet

connection type. If you have a Dynamic or PPPoE connection,you will be brought to the corresponding page.

Click next

For Static IP Address :

Please enter the IP ,Netmask, and Gateway of WAN1 Port. After this setup is completed, click “Next” to continue or “Exit” to exit.

Click next

Click next

Configure Public LAN. You will be prompted to enter the settings for the Managed “Public” Network. You may also switch the DHCP Server ON or OFF.

Click Restart

If you selected Enable DHCP Server, this screen will

appear:

Enter the settings in the fields provided to setup the distribution

of IP Addresses to clients on your network.

After Restarting the DSA-5100,

the basic setup for the unit is

complete. Please refer to the

User Manual for other advanced settings.

Verify if local user could pass the authentication, and then use the online Network:

- To add a local user account, please refer to manual for instructions.

2. After register a user account, please enter username and password in the User Login Page.

Example: Suppose a local user account has both username and password as “test”, then type “test” in respective field, then click “Enter”.

Check login status.

Technical Support

You can find software updates and user documentation on the D-Link website.

D-Link provides free technical support for customers within the United States and within Canada for the duration of the warranty period on this product.

U.S. and Canadian customers can contact D-Link technical support through our website, or by phone.

Tech Support for customers within the United States:

D-Link Technical Support over the Telephone:

(877) 453-5465

24 hours a day, seven days a week

D-Link Technical Support over the Internet:

http://support.dlink.com

email: support@dlink.com

Tech Support for customers within the Canada:

D-Link Technical Support over the Telephone:

(800) 361-5265

Monday to Friday 8:30am to 9:00pm EST

D-Link Technical Support over the Internet:

http://support.dlink.ca

email: support@dlink.ca