D-Link Corporation

DHN-520 HomePNA Adapter

-- 10Mbps Phoneline Network Interface Card

Product Specification Draft 0.90

Prepared by Andre Lu/PPD1

December, 1999

DHN-520  10Mbps HomePNA Card

- Introduction

With more and more homes typically having two or more computers, it is already becoming common practice for households to share a Internet connection with other computers in the house and it makes great sense to be able to share printers and files as well.

The HomePNA ( Home Phone Networking Alliance) was formed in early 1998 by many of the major players in the PC and networking industries. The list includes Compaq, Intel, AMD, IBM, 3Com, HP and many others. Currently HomePNA 1.0 is a standard HomePNA product to link up to 25PC’s at 1Mbps speed on an ordinary house hold phone line and each one of the 25PC’s can be up to 500 feet apart, daisy chained off of each other.

The new HomePNA Spec 2.0 is expected to be released in Q4 1999 and promises up to 10Mbps speeds using the same phone line connections. The DHN-520 was designed around the proposed iLine10 standard as accepted by the HomePNA on July 27, 1999 as the basis for HomePNA 2.0.

The DHN-520 brings the instant power of connectivity to almost every household- without the addition of any new wiring. It takes a major step towards networking the entire home, sharing computers and other devices in the house, and will be an easy, cost-effective solution for file, printer and Internet connection sharing with full 10Mbps speed.

DHN-520  10Mbps HomePNA Card

- Features

- HomePNA pre 2.0 specification compliant

- Backward compatible with 1 Mb HomePNA 1.0 certified hardware

- Operates in a PCI Bus Master slot of a Pentium/ Pentium-Pro/ Pentium-II computer, independent of CPU speed.

- PCI Bus Master memory access, for high throughput and low CPU demand.

- Plug and Play installation.

- 10 Mbps throughput over standard Home Phonelines

- Three LED indicators: Link, Activity

- Driver for Windows 95 OSR2, Windows 98, Windows NT, and beta drivers for Windows 2000.

- External Specification

| NIC Controller | BCM4210 iLine10TM Controller |
| --- | --- |
| AFE | BCM4100 iLine10TM Analog Front End |
| Network Type | 10M Home Phoneline |
| Host Interface | PCI 2.2 Specification Compliant |
| Media Interface | Two built-in RJ-11 connectors |
| Auto-Negotiation |  |
| Power Consumption | 1.1 Watts (maximum) |
| Storage Temp. | -25°C ~ 55°C (-13°F ~ 131°F) |
| Operating Temp. | 0°C ~ 50°C ( 32°F ~ 122°F) |
| Humidity | 10% ~ 90% non-condensing |
| PCB Layer | 2 Layers |
| EMI | FCC B, CE B, VCCI B, C-Tick |
| Software Driver | Microsoft Windows NT3.51,NT4.0 |
|  | Microsoft Windows95(OSR2), 98 |
|  | Microsoft Windows 2000 |