| Firmware Version: | V1.04 |
| --- | --- |
| Prom Code Version: | N/A |
| Published Date: | 2016/3/14 |

Content:

Upgrading Instructions:	2

New Features:	2

Problems Fixed:	2

Known Issues:	2

Related Documentation:	2

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| V1.00 | 2015/05/19 | DCS-4701E | A1 |
| V1.01 | 2015/06/30 | DCS-4701E | A1 |
| V1.02 | 2015/10/05 | DCS-4701E | A1 |
| V1.03 | 2015/10/12 | DCS-4701E | A1 |
| V1.04 | 2016/03/14 | DCS-4701E | A1 |

# Upgrading Instructions:

Follow the steps below to upgrade the firmware:

1. Download the latest firmware file to your hard drive from the D-Link website.

2. Log-in camera web UI and go to Maintenance/Firmware web page upgrade

3. Locate the file (upgraded firmware) on your hard drive by clicking Choose File.

4. Click Upload to upgrade the firmware on your camera.

# New Features:

| Fireware Version | New Features |
| --- | --- |
|  | N/A |

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.01.02 | - Fixed image color performance when WDR level setting >6 - Fixed flicking image when anti-flicker setting =50Hz - Remove the SD card information on Event Setup help hints page. |
| V1.02.02 | 1.Fixed the security issue of tunneling RTSP streaming over HTTP 2.Fixed the Cross-Site Scripting(XSS) issue 3.Fixed the Cross-Site Request Forgery (CSFR) issue |
| V1.03.00 | 1. Upgrade to new signing keys for fixing Code-Signing Key issue |
| V1.04.00 | - Enhanced image performance - Enhancement for security issue: a. Support TLS 1.1/1.2. b. CSRF protection default enable and follow DHQ CSRF response rule. . c. Fixed CSRF and Referer issue. d. Cross Site Scripting(XSS) issue. - Fixed video drop frame issue while recording by Windows Samba. - Fixed Curl issue -mail server SMTP authentication fail. - Fixed motion detection is disable when change IR-LED or ICR setting. - Fixed “Image Setup” page can’t be shown in Safari browser. - Fixed D-Track #HQ20151027000002 [DEUR20151023000013] New Mail Client in firmware is not RFC 2821 conform |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
|  | N/A |

# Related Documentation:

N/A