DAS4192 IP-DSLAM

System Configuration Guide

| Class: | User Guide |
| --- | --- |
| Product Name: | DAS4192 IP-DSLAM |
| Product Version: | S.W. v2.0.2@R134, H.W. NC-V5, ALC-V3 |
| Doc. No.: | BCD3-TM-E-230502 |
| Doc. Version: | 1.8 |
| Publish Date: | 2007-October-18 |

(Build 1328)

THE SPECIFICATIONS AND INFORMATION REGARDING THE PRODUCTS IN THIS MANUAL ARE SUBJECT TO CHANGE WITHOUT NOTICE. ALL STATEMENTS, INFORMATION, AND RECOMMENDATIONS IN THIS MANUAL ARE BELIEVED TO BE ACCURATE BUT

ARE PRESENTED WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED. USERS MUST TAKE FULL RESPONSIBILITY FOR THEIR APPLICATION OF ANY PRODUCTS.

THE SOFTWARE LICENSE AND LIMITED WARRANTY FOR THE ACCOMPANYING PRODUCT ARE SET FORTH IN THE INFORMATION PACKET THAT SHIPPED WITH THE PRODUCT AND ARE INCORPORATED HEREIN BY THIS REFERENCE.

NOTWITHSTANDING ANY OTHER WARRANTY HEREIN, ALL DOCUMENT FILES AND SOFTWARE OF THESE SUPPLIERS ARE PROVIDED “AS IS” WITH ALL FAULTS. PRODUCT AND THE ABOVE-NAMED SUPPLIERS DISCLAIM ALL WARRANTIES, EXPRESSED OR IMPLIED, INCLUDING, WITHOUT LIMITATION, THOSE OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT OR ARISING FROM A COURSE OF DEALING, USAGE, OR TRADE PRACTICE.

IN NO EVENT SHALL PRODUCT OR ITS SUPPLIERS BE LIABLE FOR ANY INDIRECT, SPECIAL, CONSEQUENTIAL, OR INCIDENTAL DAMAGES, INCLUDING, WITHOUT LIMITATION, LOST PROFITS OR LOSS OR DAMAGE TO DATA ARISING OUT OF THE USE OR INABILITY TO USE THIS MANUAL, EVEN IF PRODUCT OR ITS SUPPLIERS HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

DAS4192 IP-DSLAM

System Configuration Guide

Text Part Number: 2305-0217

Table of Contents

Chapter 1	Preface	1

Purpose	1

Organization	1

Conventions	1

Revision History	3

Chapter 2	DAS4192 User Interface	5

User Interface Mode	5

Access via the Console Port	5

Access using the Telnet Session	6

Managing the Session Login Account	6

Command Syntax and Operating Regulation	8

Syntax Notation Conventions	8

Structure of a CLI Command	8

Command Syntax and Context Sensitive Help	9

Command History and Editing Features	9

Ending a Session	10

Chapter 3	Initialing the NE	11

Port Interface Indication	11

Constructing the NE Objects	12

Planning the System Card Type	12

Verifying Current Software and Hardware Versions	13

Managing the Boot Section	15

Configuring the System Information	15

Configuring the UGE Negotiation Mode	16

Configuring the System Date and Time	16

Configuring the Internet Time Server	17

Configuring the SNMP Manager	18

Configuring the SNMP Community	18

Configuring the Trap Station IP Address	19

Configuring the DNS Server	19

Configuring the Management Interface	20

Setting the Management Ethernet (NME) Interface IP Address	21

Setting the in-band Interface (UGE) IP Address	22

Configuring the Default Gateway	22

Configuring the Secured Host	23

Storing the Active System Configuration	24

Backup and Restore the Active System Configuration	24

File System Configuration	26

NE Firmware Upgrade	26

SHDSL Firmware Upgrade	28

Ambient Temperature	29

Checking the SFP module information	29

Chapter 4	Managing the System Profiles	31

Overview of System Profile	31

Managing the ADSL Performance Alarm Profile	31

Managing the SHDSL Performance Alarm Profile	33

Managing the ADSL Port Connection Profile	34

Managing the SHDSL Port Connection Profile	38

Managing the IP Traffic Profile	40

Managing the Traffic Policing Profile	41

Managing the Multicast Service Profile	41

Multicast Channel Profile Setting	42

Multicast Service Profile Setting	42

Chapter 5	Managing the Subscriber Interface	45

Configuring the ADSL Line Port	45

Monitoring the ADSL Connection Status	47

Configuring the SHDSL Line Port	47

Monitoring the SHDSL Connection Status	48

Subscriber Interface Administrating	48

Chapter 6	Managing the Network Interface	51

Configuring the Subtending	51

Configuring the RSTP	52

Configuring the LACP	54

Network Interface Administrating	56

Chapter 7	Managing the Connection Services	59

VC-to-VLAN Connection Management	59

Configuring a VC-to-VLAN Connection for the VC of RFC2684 Bridged Mode	60

Configuring a VC-to-VLAN Connection for the VC of RFC2684 Routed Mode	61

Configuring the Multicast Services	62

Monitoring the VC-to-VLAN Connection Status	63

Configuring the System Services	63

MAC Aging for Bridged Services	63

DHCP Broadcast Control	64

DHCP Relay Setting	65

DHCP Relay Option 82 Setting	66

IGMP Snooping/Proxy Setting	66

Configuring the PPPoE Suboption	67

Managing the Subscriber Access Services	68

Configuring the Access Control List	69

Configuring the VLAN MAC Limitation	70

Monitoring the VLAN Group	70

Monitoring the IGMP Snoopy/Proxy Information	71

Monitoring the Subscriber MAC	72

Filtering the NetBIOS and NetBEUI	72

Configuring the MAC Spoofed	73

Chapter 8	Managing the System Operations	75

Card Module Operations	75

Defining the Line Card Operation Mode	75

System Administrating	77

Reset the Line Card and Port	77

Reboot the System	78

Session Logout	78

Alarm Definition and Relay Setting	78

Configuring the Alarm Definition	78

Configuring the System Relay-In Alarm	80

Configuring the CoS Traffic Mapping	80

Configuring the Differentiated Service	81

Chapter 9	Diagnostic and Performance Monitoring	83

Performance Monitoring on System and Network Interface	83

Performance Monitoring on ADSL Subscriber Interface	84

Performance Monitoring on SHDSL Subscriber Interface	85

Monitoring System Alarms	85

OAM and Loop Diagnostic Test on Subscriber Interface	87

OAM F5 VC Diagnostic	87

ADSL Loop Diagnostic	87

ADSL Link Monitoring	89

SELT Link Monitoring	89

Network Ping Test	90

Monitoring the System Environment	90

Monitoring the System Performance	91

List of Figures

Figure 3-1	DAS4192 Port Addressing Diagram	11

Figure 6-1	Daisy-Chain Topology for MCI2031 NC Card	51

Figure 6-2	Spanning Tree Active Topology	52

Figure 6-3	Typical GE-Channel Configuration	55

Figure 7-1	VC-to-VLAN Mapping Illustrate	59

Figure 8-1	DiffServ Field	82

.

List of Tables

Table 2-1	DAS4192 Console Management Setting	5

Table 2-2	DAS4192 Default Login Account Index	6

Table 2-3	User Account Management	7

Table 2-4	Syntax Notation of CLI Ex	8

Table 2-5	Structure of CLI Ex Mode	8

Table 2-6	CLI Ex Syntax Help	9

Table 2-7	Command History and Editing	9

Table 3-1	Port Interface Indication Format	12

Table 3-2	Planning the system card type	12

Table 3-3	Retrieve the software and hardware information of NC card	13

Table 3-4	Software and Firmware Verify of LC on-board card	13

Table 3-5	Managing the Boot Section	15

Table 3-6	System Information Configuration	15

Table 3-7	Configuring the UGE Negotiation Mode	16

Table 3-8	System Date Time Configuration	16

Table 3-9	Internet Time Server Setting	17

Table 3-10	SNMP Community Setting	18

Table 3-11	SNMP Trap Station Setting	19

Table 3-12	DNS Server Setting	19

Table 3-13	Management Interface IP Address Setting	20

Table 3-14	Secured Host Configuration	23

Table 3-15	Store the Active System Configuration	24

Table 3-16	File System Configuration	26

Table 3-17	SHDSL Firmware Upgrade	28

Table 3-18	Configuring Ambient Temperature	29

Table 3-19	Checking the SFP module information	29

Table 4-1	ADSL Performance Alarm Profile Configuration	32

Table 4-2	SHDSL Performance Alarm Profile Configuration	33

Table 4-3	ADSL Port Connection Profile Configuration	35

Table 4-4	SDSL Port Connection Profile Configuration	38

Table 4-5	IP Traffic Profile Configuration	40

Table 4-6	Traffic Policing Profile Configuration	41

Table 4-7	Multicast Channel Profile Configuration	42

Table 4-8	Multicast Service Profile Configuration	43

Table 5-1	ADSL Port Interface Configuration	45

Table 5-2	ADSL Connection Status Monitor	47

Table 5-3	SHDSL Port Interface Configuration	48

Table 5-4	SHDSL Connection Status Monitor	48

Table 5-5	Subscriber Service Administration	48

Table 6-1	Subtending Configuration	51

Table 6-2	RSTP Switch Configuration	52

Table 6-3	RSTP Port Configuration	53

Table 6-4	LACP Configuration	55

Table 6-5	The NE behavior when configuring NC and ADSL LC with various Tagged modes and VTP parameters.	57

Table 6-6	Network Services Administration	57

Table 7-1	Bridged Services Configuration	60

Table 7-2	Routed Services Configuration	61

Table 7-3	Multicast Services Configuration	62

Table 7-4	Unicast Connection Status Monitor	63

Table 7-5	Bridged Services Setting	64

Table 7-6	DHCP Broadcast Control	64

Table 7-7	DHCP Relay Setting	65

Table 7-8	DHCP Relay Option 82 Setting	66

Table 7-9	IGMP Snooping/Proxy Setting	66

Table 7-10	PPPoE Suboption Setting	67

Table 7-11	Access Services Configuration	68

Table 7-12	Access Control List Configuration	69

Table 7-13	VLAN MAC Limiting Configuration	70

Table 7-14	Viewing Subscriber VLAN Group	71

Table 7-15	Viewing IGMP Proxy Information	71

Table 7-16	VC MAC Learning Table	72

Table 7-17	NetBIOS and NetBEUI Filter	72

Table 7-18	MAC Spoofed Configuring	73

Table 8-1	Plan the Line Card Slot	76

Table 8-2	Line Card and Port Reset Command	77

Table 8-3	System Reboot Command	78

Table 8-4	Session Logout Command	78

Table 8-5	Alarm Definition Configuration	78

Table 8-6	System Relay-In Alarm Configuration	80

Table 8-7	CoS Traffic Mapping	81

Table 8-8	Configuring the DiffServ	81

Table 8-9	Precedence Level	82

Table 8-10	DSCP Class Relationship	82

Table 9-1	Performance Monitoring on System and Network Interface	83

Table 9-2	Performance Monitoring on ADSL Subscriber Interface	84

Table 9-3	Performance Monitoring on SHDSL Subscriber Interface	85

Table 9-4	Viewing the System Alarm	85

Table 9-5	OAM F5 VC Diagnosis Test	87

Table 9-6 	ADSL Loop Diagnostic Test	88

Table 9-7 	ADSL Link Monitoring	89

Table 9-8 	SELT Link Monitoring	89

Table 9-9	Network Ping Test	90

Table 9-10	System Environment Monitoring	90

Table 9-11	System Performance Monitoring	91

..…

Table A-1	Abbreviations and Acronyms Table	A-1

# Preface

This preface discusses the following topics:

- Purpose

- Organization

- Conventions

## Purpose

The purpose of this guide is to provide detailed information and description of DAS4192 IP-DSLAM, which includes software configuration and other specific features.

## Organization

This guide contains the following information:

- Preface

- DAS4192 User Interface

- Initialing the DAS4192

- Managing the System Profiles

- Managing the Subscriber Interface

- Managing the Network Interface

- Managing the Unicast Services

- Managing the Multicast Services

- Configuring the System Functions

- Diagnosis and Performance Monitoring

- Appendix

## Conventions

This section describes the conventions used in this guide.

The DAS4192 IP-DSLAM is the Next-Generation xDSL Broadband Access Network comprises a Gigabit Ethernet and a number of ATU-Rs, STU-Rs, and POTS splitter to construct a broadband access network between central office and customer premises. The DAS4192 IP-DSLAM uses statistically multiplexing and ATM over xDSL technologies to provide the broadband data communication services, such as high speed Internet access and multimedia services, across existing twisted pair telephone line.

NE/NEs hereinafter referred as DAS4192 medium capacity IP-DSLAM, unless specifically indicated.

ADSL mention in this document covers ADSL, ADSL2, and ADSL2+, unless specifically indicated.

xDSL hereinafter referred as ADSL, unless specifically indicated.

The xDSL specified in this document compliance with ITU-T Rec. G.992.1, G.992.2, G.992.3 and G.992.5 for ADSL.

CLI Ex – The command line management with a local console or Telnet through in-band or out-of-band IP interface for CIT (Craft Interface Terminal) connection.

AMS – AM Management System (AMS), a complete centralized SNMP base NMS (Network Management System) provides GUI operation under Client-Server architecture through in-band or out-of-band IP interface to carrying out day of day operation, administration, maintenance, and configuration functions of the NE.

- AMS Client – Software system for Network Management System (NMS), it’s in Client-Server architecture and has ability to provide controlling and management for the whole network through GUI interface to collocate with AMS Server.

- AMS Server – The server station provides multiple NEs management and Database in order to perform reliability, stability, and flexibility to entire network management.

AMS LCT – AMS Local Craft Terminal (LCT), a stand-along host with SNMP base EMS (Element Management System) provides GUI operation under single section through in-band or out-of-band IP management interface.

|  | This sign indicates the NOTICE. A note contains helpful suggestions or reference relay on the topical subjects. |
| --- | --- |

|  | This sign indicates the TIP. Performing the information described in the paragraph will help you solve a problem. The tip information might not be troubleshooting or even an action, but could be useful information. |
| --- | --- |

|  | This sign indicates the CAUTION. In this situation, you might do something that could result in equipment damage or loss of data. |
| --- | --- |

|  | This sign indicates the DANGER. You are in situation that could cause bodily injury. Before you work on any equipment, you must be aware of the hazards involved with electrical circuitry and be familiar with standard practices for preventing accidents. |
| --- | --- |

## Revision History

- Revision Date: 2007/10/9

Release version:1.7.1

Author: William Fu

Summary of Change(s):

(1)	Modify Example 15 and Example 16 to show “226- CAUTION: Please wait for 120 seconds” instead of “226- CAUTION: Please wait for 120 seconds or check the flash LED”.

- Revision Date: 2007/10/11

Release version:1.8

Author: Tim Yeh

Summary of Change(s):

Rewrite Section “Configuring the Subtending”, Section “Defining the Line Card Operation Mode” and Section “Verifying Current Software and Hardware Versions” to add LC/NC tagged/untagged related CLI commands to support the IEEE 802.1Q function and hence add/modify the following table and related descriptions.

- Table 6-44 is new created for describing the NE behavior when configuring NC and ADSL LC with various Tagged mode and VTP parameters.

- In “Table 6-45 Network Services Administration”, the new command “set tagged-mode” is added and the usage of its parameter “tagged-only | untagged-only” is described.

- In “Table 8-64 Plan the Line Card Slot”, the new command “set tagged-mode” is added and the usage of its parameter “tagged-only | untagged-only” is described.

- Example 3 and Example 4 are modified to reflect the software changes.

Rewrite Section “Configuring the Subtending” to add new CLI commands to support the subtend port function of the NC and hence add/modify the following table and related descriptions.

- In “Table 6-45 Network Services Administration”, the new corresponding commands “add”, “clear”, “del”, “subtend” and parameters “subtend-vid”, “disabled | enabled”are added.

Add new CLI commands to support the configuration of telnet timeout and hence add/modify the following table and related descriptions.

- In “Table 3-13 System Information Configuration”, the “CLI(config systeminfo)#” is replaced by “CLI(config sys-info)#”.

- In “Table 3-13 System Information Configuration”, the new command “set timeout” is added and the usage of its parameter “min” is described.

Add new CLI commands to support the monitoring of the PM parameters on NC and hence add/modify the following table and related descriptions.

- In “Table 9-75 Performance Monitoring on ADSL Subscriber Interface”, the new command “show nc” is added.

- In Section “Verifying Current Software and Hardware Versions” of Chapter 3, the command “card show” is changed to “nc show” (and “lc show”), the related description is modified accordingly.

- In Chapter 3, a new section “Backup and Restore the Active System Configuration ”is added to depict the process.

- Example 5, Example 8, Example 9,Example 22, Example 29, Example 30, Example 32, Example 34, Example 36, Example 37, Example 40, Example 43 are changed due to the CLI displayed messages are changed.

- In “Table 3-16 Internet Time Server Setting”, the “CLI(config timeservice)#” is replaced by “CLI(config time-service)#”.

- In “Table 3-25 Configuring Ambient Temperature”, the “CLI(config sensorthreshold)#” is replaced by “CLI(config hw-sensor)#”.

- In”Table 7-46 Bridged Services Configuration”,”Table 7-47 Routed Services Configuration”,”Table 7-49 Unicast Connection Status Monitor” the “CLI(config unicast)#” is replaced by “CLI(config ucast)#”.

- In “Table 4-33 Multicast Channel Profile Configuration”, the corresponding value of the parameter “rate” is changed due to software change.

- In “Table 5-35 ADSL Port Interface Configuration”, the new command “set cpe-wireless” is added and the usage of its parameters “port-range” and “wireless-status” are described.

- In “Table 6-42 RSTP Port Configuration”, the corresponding parameter “uge-list” is changed to “uge-range”.

- In “Table 7-47 Routed Services Configuration”, the parameter “<vlan-id>” of the command “add nexthop” is replaced to “<vid>”

- In “Table 7-54 IGMP Snooping/Proxy Setting”, the corresponding value of command “proxy-stateful” is changed to “stateful”.

- In “Table 7-58 VLAN MAC Limiting Configuration”, the default value of parameter “maclimit” is setting to 43 and the range of vlan id is changed to 1~4094.

- In “Table 7-59 Viewing Subscriber VLAN Group”, the new commands “show vlan-id” and “show uge” are added and the usage of its parameter “vid” and “uge-id” are described.

- In “Table 9-79  ADSL Loop Diagnostic Test”, the “CLI(diag loopdiag)#” is replaced by “CLI(diag delt)#”.

- Recapture the process of uploading NC/ADSL LC Firmware to Flash Memory through FTP in Example 15.

# DAS4192 User Interface

This chapter describes the DAS4192 user interface, the instructions describe how to using the command-line interface, and also describes the command editing and command history features that enable you to recall previous command entries and edit previously entered commands.

- User Interface Mode

- Access via the Console Port

- Access using the Telnet Session

- Managing the Session Login Account

- Command Syntax and Operating Regulation

## User Interface Mode

The DAS4192 provides the user access mode to allow user to login, it requires a password with remote Telnet and Console access, this user configuration interface mode is called CLI Ex mode.

## Access via the Console Port

Access to CLI Ex mode via the Console Port can be done using a VT100-compatile terminal directly connected.

To access the CLI Ex mode via a direct VT100-compatible terminal connection to the Console Port, use the following procedures:

Step 1	Set the communication parameters of a VT100-compatible terminal as follows:

Table 2-1	DAS4192 Console Management Setting

| Parameter | Setting |
| --- | --- |
| Baud rate | 9600 |
| Data bits | 8 |
| Parity | None |
| Start bits | 1 |
| Stop bits | 1 |
| Flow control | None |

Step 2	Connect the VT100-compatible terminal to the Console Port of the DAS4192 front panel.

Step 3	Press <Enter> a number of times until the “Login:” is display on the screen.

The Console session required a username and password to access, the default administration username and password is list in below (case sensitive):

Login: admin

Password: admin

See “Chapter 2 Managing the Session Login Account” for detail information.

## Access using the Telnet Session

Access the CLI Ex mode by establishing a Telnet session onto the assigned IP address of the Network uplink interface.

If the IP address is changed during configuration and the change are saved, your Telnet session onto the system will be broken. You can then Telnet to the new IP address assigned during the configuration.

The IP address assigned on Network uplink interface must not be in use with another device on the same network segment or a conflict may occur. Refer to “Chapter 3 Configuring the Management Interface” for more information.

If the assign IP has been changed and forgotten, locally access the system via Console port with below command syntax to retrieve the IP address assign to the system, it also shown the system default IP address for WAN (In-band, UGE) and Management IP Interface (NME) as well as the default gateway.

- Display the system management IP addresses

- CLI# config ip show

UGE

IP Address     : 192.168.3.90

Subnet Mask    : 255.255.255.0

MAC Address    : 00:01:02:02:01:cf

UGE VLAN ID    : 4092

NME

IP Address     : 10.5.1.242

Subnet Mask    : 255.255.0.0

MAC Address    : 00:01:0a:01:cf:cf

Gateway

IP address     : 192.168.3.254

The Telnet session required a username and password to access, the default administration username and password is list in below (case sensitive):

User Name: admin

Password: admin

Telnet supports maximum of 12 sessions simultaneously login to a single NE, but only one of admin account user is allow at all time (Console access included), guest account for the rest of session limiting, the default “admin” account user is with administrator privilege level, see “Chapter 2 Managing the Session Login Account” for detail information.

### Managing the Session Login Account

For security reason, the CLI Ex mode provides two levels of account user privileges, “admin” and “guest”. Admin level has full access privileges while guest level has only the browse privileges. Table 2-2 shows detail system default login account and session information.

Table 2-2	DAS4192 Default Login Account Index

| Group | Default Account | Login Mode | Session | Session Timeout |
| --- | --- | --- | --- | --- |
| Admin | Username: admin Password: admin | Console, Telnet | Single session occupying on either Console access or Telnet access. | Console: limitless Telnet: 120 Seconds |
| Guest | Username: guest Password: guest | Console, Telnet | 1 session for Console access, up to 12 sessions for Telnet access. | Console: limitless Telnet: 120 Seconds |

Enter to the “config mgt” sub-group directory to create and delete the user account.

CLI# config mgt

CLI(config mgt)#

Table 2-3	User Account Management

| Use this command to create the account user and it group privileges, while valid user name was defined, the password prompt will appear |  |
| --- | --- |
| CLI(config mgt)# add user <name> [<group>] |  |
| Use this command to delete a user login |  |
| CLI(config mgt)# del user <name> |  |
| Use this command to change the user password |  |
| CLI(config mgt)# password <user> |  |
| Use this command to change the user group privileges |  |
| CLI(config mgt)# group <name> [<group>] |  |
| Use this command to identify the VLAN ID for in-band management traffics. |  |
| CLI(config mgt)# set vlan <vlan-id> |  |
| Use this command to display information of all the users. Password information is not included. |  |
| CLI(config mgt)# show |  |
| Parameters | Task |
| <name> | This specifies the user name and password to be created. Type: Mandatory Valid values: String of up to 16 characters (‘A’ – ‘Z’, ‘a’ – ‘z’, ‘0’ – ‘9’, ‘-’, ‘_’, ‘.’, ‘@’) |
| <user> | This specifies the current user name. |
| <group> | This specifies group privilege of the name user. Type: Option Default value: guest Valid values: admin, guest |
| <vlan-id> | Assign the in-band interface to the proper VLAN (make sure the VLAN is associated with the network to which the IP address belongs). Type: Mandatory Valid values: 1 ~ 4094 |

The below example shown how to generate a new account user and join to the admin group.

- Create a new user account

- CLI(config mgt)# add user abc

Enter password (up to 16 characters):

Confirm password:

OK

CLI(config mgt)# group abc admin

OK

CLI(config mgt)# show

management VLAN    : 4092

user    : guest (guest)

user    : admin (admin)

user    : abc (admin)

## Command Syntax and Operating Regulation

This section describes how to configure and display the syntax notation, structure, context-sensitive, command history features, and command syntax help.

### Syntax Notation Conventions

CLI Ex command syntax using different bracket form to display syntax notation, Table below lists the notation information.

Table 2-4	Syntax Notation of CLI Ex

| Notation | Descriptions |
| --- | --- |
| keyword | Keywords in a command that you must enter exactly as shown. |
| <Parameter> | Parameter values must be specified. |
| [<Parameter>] | Parameter values are optional. |
| [Parameter 1 ¦ Parameter 2 ¦ .. ¦ Parameter n] | Parameter values are enclosed in “[ .. ¦ .. ] ” when you optional use one of the values specified. |
| {Parameter 1 ¦ Parameter 2 ¦ .. ¦ Parameter n} | Parameter values are enclosed in “{ .. ¦ .. }” when you must use one of the values specified. |

### Structure of a CLI Command

The CLI Ex commands conform to the following structure in group base. Each group contains sub-group directory or action command that can be use directly with proper syntax.

CLI# {[<Group-A> | <Action-A>] | [<Group-B> | <Action-B>] | [<Group-C> | <Action-C>] | <Action-D>}

or

CLI# [<Group-A> | <Action-A>]

CLI(Group-A)# [<Group-B> | <Action-B>]

CLI(Group-B)# [<Group-C> | <Action-C>]

CLI(Group-C)# <Action-D>

The command structure can complete in a single sentence or access into specific group directories.

Table 2-5	Structure of CLI Ex Mode

| Keyword | Descriptions |
| --- | --- |
| <Group-#> | This is the group directory of a CLI Ex command which contains relative keywords under. It indicates the type of group to be performed. “config” is an example of the group directory. |
| <Action-#> | This is the keyword of a CLI Ex command. It indicates the type of operation to be performed. “ping” is an example of this action keyword. |
| Command | Descriptions |
| exit | Jump to the upper group directory. |
| exit all | Jump to the root directory CLI# |
| clear | Clear the screen. |
| Press Enter / Return | Execute the command. |

### Command Syntax and Context Sensitive Help

Fully utilize the “ ? ” command to assist your task; this command can be use to brows command and to be assistants on the command keywords or arguments.

To get help specific to a command, a keyword, or argument, perform one of these tasks:

Table 2-6	CLI Ex Syntax Help

| Command | Task |
| --- | --- |
| ? | To list all command available of CLI Ex mode. |
| Command ? | To list the associated keywords and arguments for a command. |
| Abbreviated-command-entry <Tab> | Complete a partial command or group directory name. |

To list the command keywords, enter a question mark “ ? “ to complete the command keywords and arguments. Include a space before the ?. This form of help is called command syntax help.

The CLI Ex mode provides an error announce that appears in which you have entered an incorrect or incomplete command, syntax, keyword, or argument.

If you have entered the correct command but invalid syntax or a wrong keyword parameters, the CLI Ex will automatic prompt the error messages and reprint the command with cursor indexed on wrong syntax.

### Command History and Editing Features

By default, the system records ten command lines in its history buffer. To recall commands from the history buffer, perform one of these commands:

Table 2-7	Command History and Editing

| Command | Task |
| --- | --- |
| Press the Up arrow key | To recall commands in the history buffer. Beginning with the most recent commands. Repeat the key sequence to recall the older commands. |
| Press the Down arrow key | To return to more recent commands in the history buffer. Repeat the key sequence to recall the more recent commands. |
| Press the left arrow key | To move the cursor back one character. |
| Press the right arrow key | To move the cursor forward one character. |
| Press Backspace | To erase the character to the left of the cursor. |
| Press Q | To quite the print listing on the console screen. |

This CLI Ex mode includes an editing feature. You can move cursor around on the command line to insert or delete the character.

|  | The arrow keys function only on ANSI-compatible terminals such as VT100s. |
| --- | --- |

### Ending a Session

If you access using the Telnet session, you can type “logout” command to terminate the Telnet session instantly, while in Console access will quit the CLI Ex and stay in login state.

Console port will stay in life until you close the terminal session.

Define the timeout seconds and move this section behind “Manage the Session Login Account”.

# Initialing the NE

This chapter describes how to initially configure the DAS4192 IP-DSLAMs, and includes these sections:

- Port Interface Indication

- Verifying Current Software and Hardware Versions

- Managing the Startup Boot Section

- Configuring the System Information

- Planning the System Card Type

- Configuring the UGE Negotiation Mode

- Configuring the System Date and Time

- Configuring the Internet Time Server

- Configuring the SNMP Manager

- Configuring the DNS Server

- Configuring the Management Interface

- Storing the Active System Configuration

- File System configuration

- NE Firmware Upgrade

- SHDSL Firmware Upgrade

- Ambient Temperature

- Checking the SFP module information

## Port Interface Indication

The DAS4192 is designed in single shelf and five slots, 1 for NC and 4 for xDSL LC, each xDSL LC contain maximum of 48 ports.

The interface indicate parameters <slot-id>, <port-id>, <slot-range>, and <port-range> are use to identify the particular slot/port interface or its configuration slot/port range inside the CLI Ex mode.

Figure 3-1 shows the shelf, slot, and port addressing outward on DAS4192.

Figure 3-1	DAS4192 Port Addressing Diagram

The CLI Ex mode using “.” notation to differentiate between shelf, slot, and port at interface indicate parameters. The format of “[shelf_#] . slot_#” identify upon slot base for <slot-id> and <slot-range> parameters while the format of “[shelf_#] . [slot_#] . port_#” identify as the port base for <port-id> and <port-range> parameters. If shelf_# and/or slot_# is not defined, CLI_Ex will apply the default value automatically to those syntax.

The <slot-range> and <port-range> parameters using “-” notation to identify the continuously range, the range configuration has ability to stride over slot or shelf.

Table 3-8	Port Interface Indication Format

| Parameters | Descriptions |
| --- | --- |
| <slot-id> | Format: [shelf_#] . slot_# Valid values: shelf_# (1), slot_# (1 ~ 4) Default value: shelf_# (1) |
| <port-id> | Format: [shelf_#] . [slot_#] . port_# Valid values: shelf_# (1), slot_# (1 ~ 4), port_# (1 ~ 48) Default value: shelf_# (1), slot_# (1) |
| <slot-range> | Format (Continuously): [shelf_#] . slot_# - [shelf_#] . slot_# Format (Individually): [shelf_#] . slot_# Valid values: shelf_# (1), slot_# (1 ~ 4) Default value: shelf_# (1) |
| <port-range> | Format (Continuously): [shelf_#] . [slot_#] . port_# - [shelf_#] . [slot_#] . port_# Format (Individually): [shelf_#] . [slot_#] . port_# Valid values: shelf_# (1), slot_# (1 ~ 4), port_# (1 ~ 48) Default value: shelf_# (1), slot_# (1) |

## Constructing the NE Objects

As the DAS4 Series IP-DSLAM provides the flexibility to be equipped with various card modules such as ADSL-LC (Line Card) and SHDSL-LC, constructing the NE board type of card module is the first task you need to perform.

### Planning the System Card Type

Enter to the “config nc” sub-group directory to planning the NC (Network Control) card.

CLI# config nc

CLI(config nc)#

Enter to the “config lc” sub-group directory to planning the LC (Line Card) card.

CLI# config lc

CLI(config lc)#

Table 3-9	Planning the system card type

| Use this command to modify the planning NC card type. |  |
| --- | --- |
| CLI(config nc)# set planned-type <nc-id> {none ¦ cpu} |  |
| Use this command to modify the planning LC card type. |  |
| CLI(config lc)# set planned-type <lc-range> <card-type> |  |
| Parameters | Task |
| <nc-id> | Identify the slot range of the NC card Type: Mandatory Valid values: 1 ~ 2 (value = 2 is only on DAS4672) |
| {none ¦ cpu} | Identify the NC type. |
| <lc-range> | Identify the slot range of the Line card. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <card-type> | Identify the line card type Valid values: none, adsl, shdsl |

### Verifying Current Software and Hardware Versions

Follow the commands to display the inventory information of NE software/ hardware version, card serial number, card type and so on

Use the “nc show” or “lc show” command under the “status” group directory to display the system H/W and S/W version of each plug-in card module and slot planning type.

Enter to the “status” group directory to verify the software and hardware versions.

CLI# status

CLI(status)#

Table 3-10	Retrieve the software and hardware information of NC card

Using this command to display the system on-board card version and plugging status.

CLI(status)# nc show

- Monitoring the NC board-level information

CLI(status)# nc show

NC

current card type    : CPU Module

planned card type    : CPU Module

role                 : active

hardware version     : DAS-4672-10

software version     : 1.0v2.0.2@R134

serial number        : MCI2021-11613000028

oper status          : up

system up time       : 0day / 1hr / 33min / 33sec

tagged mode          : untagged-only

Table 3-11	Software and Firmware Verify of LC on-board card

| The following command is to display the LC card version and plugging status. |  |
| --- | --- |
| CLI(status)# lc show [<lc-range >] |  |
| Parameters | Task |
| <lc-range> | Identify the slot range of the Line card. Type: Mandatory Valid values: See “Chapter 3 Port Interface Indication” |

- Monitoring the LC board-level information

CLI(status)# lc show

LC1

current card type                     : ADSL

planned card type                     : ADSL

hardware version                      : MLA2031

software version                      : 6.5.7_2.4.0

serial number                         : MLA2031

oper status                           : up

system up time                        : 0day / 0hr / 34min / 18sec

RFC2684 encapsulation                 : LLC

tagged mode (configured)              : untagged-only

tagged mode (run-time)                : untagged-only

VLAN tag pass through (configured)    : disabled

VLAN tag pass through (run-time)      : disabled

|  | It is noted that the NE will drop the tagged Ethernet frames of VLAN-ID not configured by the VC-to-VLAN setting (see Table 6-44) in the following case. NC tagged mode = Tagged LC tagged mode Run-Time Status = Tagged LC VTP Run-Time Status = Enabled |
| --- | --- |

|  | The tagged mode (run-time) indicates the operational status of tagged mode. Tagged-only: LC (or NC) only forwards the tagged Ethernet frame and drops the untagged Ethernet frame. Untagged-only: LC (or NC) only forwards the untagged Ethernet frame and drops the tagged Ethernet frame. It is noted that the value of configured Tagged mode and its Run-Time Status may be different. Please refer to Table 6-44 for the NE behavior when configuring NC and ADSL LC with various Tagged mode and VTP parameters. |
| --- | --- |

## Managing the Boot Section

The system has two boot sections ‘opCodeA’ and ‘opCodeB’, each of it contains the firmware necessary for the system. You can manually assign the startup section for next booting.

Use the command “boot-device” to managing the boot section of your system.

CLI# boot-device

Table 3-12	Managing the Boot Section

Use this command to identify the startup boot section.

CLI# boot-device set {opCodeA ¦ opCodeB}

Use this command to display the current boot device and firmware file.

CLI# boot-device show

## Configuring the System Information

The system information contains system name, location, and contact person info, enter the ‘sys-info’ sub-group under ‘config’ group directory to manage your NE.

Enter to the “sys-info” sub-group directory to configure the system information.

CLI# config sys-info

CLI(sys-info)#

Table 3-13	System Information Configuration

| Use this command to modify the system location. |  |
| --- | --- |
| CLI(sys-info)# set location <string> |  |
| Use this command to modify the system contact information. |  |
| CLI(sys-info)# set contact <string> |  |
| Use this command to modify the system name. |  |
| CLI(sys-info)# set name <string> |  |
| Use this command to set the telnet time out of the system. |  |
| CLI(sys-info)# set time-out <min> |  |
| Use this command to set the telnet time out of the system. |  |
| CLI(sys-info)# show |  |
| Parameters | Task |
| <string> | This specifies the textual identification of the information on the given field Type: Mandatory Valid values: String of up to 255 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |
| <min> | This specifies the telnet time out of the system Type: Mandatory Valid values: 1~1440 minutes. Default values: 2 minutes |

- Modifying the name of system information

CLI(sys-info)# set name IP_DSLAM

OK

CLI(sys-info)# show

System Name       : IP_DSLAM

System Contact        : No_Contact_Information

System Description    : IP-DSLAM

System Location       : No_Physical_Location

Telnet time-out       : 2 min (2 min)

## Configuring the UGE Negotiation Mode

Enter to the “config nc” sub-group directory to planning the NC (Network Control) card.

CLI# config nc

CLI(config nc)#

Table 3-14	Configuring the UGE Negotiation Mode

| Use this command to modify the UGE negotiation mode. |  |
| --- | --- |
| CLI(config nc)# set autoneg <uge-id> {off ¦ on} |  |
| Parameters | Task |
| {off ¦ on} | Identify the auto negotiation mode of specified UGE port. Type: Mandatory Valid values: off ¦ on |
| <uge-id> | Identify the slot range of the UGE port Type: Mandatory Valid values: 1 ~ 2 |

## Configuring the System Date and Time

You can set the date and time parameters as part of the initial system configuration.

Set the system date and time using the “datetime” command at the prompt for CLI#.

|  | The date and time will be reset every times due to system reboot, synchronize will be perform if managed under EMS Server. |
| --- | --- |

Table 3-15	System Date Time Configuration

| Use this command to set the system date time. |  |
| --- | --- |
| CLI# datetime set <date> <time> |  |
| Use this command to set the GMT time zone for system. |  |
| CLI# datetime timezone <zone> |  |
| Use this command to monitor the current system time. |  |
| CLI# datetime show |  |
| Parameters | Task |
| <date> | Identify the year, month, and date. Type: Mandatory Valid values: yyyy-mm-dd |
| <time> | Identify the time in hour, minute, and second. Type: Mandatory Valid values: hh:mm:ss |
| {zone} | Identify the GMT time zone. Type: Mandatory Valid values: -12 ~ +13 |

- Configure the system date and time

CLI# datetime set 2005-03-10 10:38:00

OK

CLI# datetime timezone +8

OK

CLI# datetime show

datetime: 2005-03-10 10:38:11 GMT+8

## Configuring the Internet Time Server

Enable the time server to allow the IP-DSLAM clock to synchronize with an Internet time server.

Enter to the “config time-service” sub-group directory to configure the Internet time server.

Table 3-16	Internet Time Server Setting

| Use this command to enable the time server IP address or domain name. |  |
| --- | --- |
| CLI(config time-service)# servers set <server1 ¦ server2 ¦ server3> <address> |  |
| Use this command to disable the time server. |  |
| CLI(config time-service)# servers delete <server1 ¦ server2 ¦ server3> |  |
| Use this command to define the synchronization protocol. |  |
| CLI(config time-service)# set protocol <none ¦ sntp> |  |
| Use this command to define the synchronization time period. |  |
| CLI(config time-service)# set timezone <zone-value> |  |
| Use this command to define the synchronization time period. |  |
| CLI(config time-service)# set period <time> |  |
| Use this command to display the time server configuration information. |  |
| CLI(config time-service)# show |  |
| Use this command to manually synchronize with time server. |  |
| CLI(config time-service)# update |  |
| Parameters | Task |
| <address> | This specifies the network IP address or domain name for Internet time server. Type: Mandatory Valid values: Any valid class A/B/C IP address or domain name |
| <zone-value> | Identify the GMT time zone. Type: Mandatory Valid values: -12 ~ +13 |
| <time> | This specifies the automatic synchronizing time period Type: Mandatory Valid values: 1 ~ 1440 Minutes |

## Configuring the SNMP Manager

SNMP (Simple Network Management Protocol) is an application-layer protocol that facilitates the exchange of management information between a NE and LCT. SNMP enables the administrators to manage the NE by the LCT. In the term of SNMP, the NE plays the role of SNMP agent and the LCT serves as the SNMP server. This section describes how to configure the SNMP on the NE.

|  | Beware of the SNMP community setting, this will affects the communication between the AMS LCT (or AMS server) and NE, re-login the AMS LCT if the SNMP community has been modified. |
| --- | --- |

### Configuring the SNMP Community

The SNMP community is a string representing the password to access the MIB of NE with the associated privilege. The NE supports two levels of privilege (Permission) as follows.

- Read / Write / Create – Allow the SNMP server to read and write all objects in the MIB, as well as the community strings.

- Read-only – Only allow the SNMP server to read all objects in the MIB except the community strings.

|  | The community string definitions on your AMS LCT (or AMS Server) must match at least one of those community string definitions on the NE. Otherwise, the LCT is not allowed to access the NE. |
| --- | --- |

The SNMP Community setting allows you to assign the community privilege levels. Two privilege levels are support, read-only and read-write.

Enter to the “config snmp” sub-group directory to configure the SNMP community.

CLI# config snmp

CLI(config snmp)#

Table 3-17	SNMP Community Setting

| Use this command to create new SNMP community information, system allows up to 8 community settings. |  |
| --- | --- |
| CLI(config snmp)# add community <name> {rw ¦ ro} |  |
| Use this command to delete the SNMP community information. |  |
| CLI(config snmp)# del community <name> |  |
| Use this command to monitor the status of SNMP community sets (Community Table). |  |
| CLI(config snmp)# show community |  |
| Parameters | Task |
| <name> | This specifies the community name Type: Mandatory Valid values: String of up to 20 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |
| {rw ¦ ro} | This specifies the access permissions given to managers with this community name. ‘ro’ implies read only permissions and ‘rw’ implies read-write permissions. Type: Mandatory |

- Add a new SNMP community to system

CLI(config snmp)# add community xxx ro

OK

CLI(config snmp)# show community

Community Table:

Community       Permission

-------------------- ----------

"public"  read-only

"xxx"  read-only

### Configuring the Trap Station IP Address

SNMP Trap Manager records the hosts (any SNMP server, like LCT, AMS Server, and so on) to be notified whenever the NE encounters abnormalities. When a trap condition happens to the NE, the NE sends the corresponding SNMP trap message to the hosts (SNMP server) specified in the SNMP Manager IP Address List.

Enter to the “config snmp” sub-group directory to configure the Trap station.

CLI# config snmp

CLI(config snmp)#

Table 3-18	SNMP Trap Station Setting

| Use this command to create a new trap station, system allows up to 8 of trap station in maximum. |  |
| --- | --- |
| CLI(config snmp)# add trapstation <ip-addr> <community-name> |  |
| Use this command to delete the trap station information. |  |
| CLI(config snmp)# del trapstation <ip-addr> |  |
| Use this command to monitor the status of trap stations (Trap Station Table). |  |
| CLI(config snmp)# show trapstation |  |
| Parameters | Task |
| <ip-addr> | This indicates the IP address (Server / Host IP) of SNMP Manager. Type: Mandatory |
| <community-name> | This specifies the SNMP trap community of NE (Send Trap). Type: Mandatory |

- Add a new Trap station

CLI(config snmp)# add trapstation 192.168.1.1 xxx

OK

CLI(config snmp)# show trapstation

Trap Station Table:

IP Address         Community       Version

--------------- -------------------- -------

192.168.1.1             "xxx"     v2c

## Configuring the DNS Server

The DNS Server is used for the resolution of domain name. In other words, the DNS replies the corresponding IP address to the URL.

Follow the commands to configure the DNS Server.

Enter to the “config dns” sub-group directory to configure the DNS server.

CLI# config dns

CLI(config dns)#

Table 3-19	DNS Server Setting

| Use this command to define the DNS server IP address. |  |
| --- | --- |
| CLI(config dns)# set {dns1 ¦ dns2 ¦ dns3} <ip-addr> |  |
| Use this command to delete the DNS server. |  |
| CLI(config dns)# del {dns1 ¦ dns2 ¦ dns3} |  |
| Use this command to display the DNS server. |  |
| CLI(config dns)# show |  |
| Parameters | Task |
| <ip-addr> | This specifies the DNS server IP address. Type: Mandatory |

- Add a new DNS server to system

CLI(config dns)# set dns1 168.95.1.1

OK.

CLI(config dns)# set dns2 168.95.1.88

OK.

CLI(config dns)# show

DNS server IP

dns1             dns2             dns3

---------------  ---------------  ---------------

168.95.1.1      168.95.1.2           0.0.0.0

## Configuring the Management Interface

This section explains how to configure an IP address on the network management Ethernet interface (nme) and uplink Network interface (uge) present on the NC (Network Control) card. The nme is an out-of-band management Ethernet port on the system engine. Packets received on this interface will never reach the switching fabric and there is no access to the nme interface except through the Management Ethernet port on the NC card.

The uge is an in-band management interface connected to the switching fabric present on the uplink gigabit Ethernet port which has ability to join the VLAN membership.

Enter to the “ip” sub-group directory to configure the management interface IP address.

CLI# config ip

CLI(config ip)#

Enter to the “config mgt” sub-group directory to configure the uge in-band interface VLAN membership.

CLI# config mgt

CLI(config mgt)#

Table 3-20	Management Interface IP Address Setting

| Use this command to assign the IP address and subnet mask for management Ethernet interface (nme). |  |
| --- | --- |
| CLI(config ip)# set nme <ip-addr> <netmask> <gatewayip> |  |
| Use this command to assign the IP address and subnet mask for uplink Network interface (uge). |  |
| CLI(config ip)# set uge <ip-addr> <netmask> <gatewayip> |  |
| Use this command to assign the default gateway. The DAS4192 sends all off-network IP traffic to the default gateway. |  |
| CLI(config ip)# set gateway <ip-addr> |  |
| Use this command to monitor the management interface information. |  |
| CLI(config ip)# show |  |
| Use this command to identify the VLAN ID for in-band management traffics. |  |
| CLI(config mgt)# set vlan <vlan-id> |  |
| Parameters | Task |
| <ip-addr> | This specifies the network IP address for nme and uge interface, this IP address use only for system management. Type: Mandatory Valid values: Any valid class A/B/C address Default value: None |
| <gatewayip> | This specifies the gateway IP address for system, this gateway IP address use only for system management. Type: Mandatory Valid values: Any valid class A/B/C address |
| <netmask> | This specifies the subnet mask configured for the interface. Type: Mandatory Valid values: 255.0.0.0 ~ 255.255.255.255 |
| <vlan-id> | Assign the in-band interface to the proper VLAN (make sure the VLAN is associated with the network to which the IP address belongs). Type: Mandatory Valid values: 1 ~ 4094 |

### Setting the Management Ethernet (NME) Interface IP Address

Before you can Telnet to the DAS4192 or use SNMP to manage the DAS4192, you must assign an IP address to either the in-band (uge) interface or the management Ethernet (nme) interface.

You can specify the subnet mask (netmask) in dotted decimal format.

To set the management Ethernet (nme) interface IP address, perform this procedure in CLI Ex mode:

Step 1	Assign an IP address and subnet mask to the management Ethernet (nme) interface.

Step 2	Verify the default gateway, if necessary.

The example below shows how to assign an IP address and subnet mask to the management Ethernet (nme) interface and how to verify the interface configuration.

- Setup the out-of-band management interface

CLI(config ip)# set nme 172.16.1.1 255.255.0.0 172.16.1.254

OK

CLI(config ip)# show

UGE

IP address     : 100.168.3.97

subnet mask    : 255.255.0.0

MAC address    : 00:11:f5:dc:7a:17

UGE VLAN ID    : 4092

NME

IP address     : 172.16.1.1

subnet mask    : 255.255.0.0

MAC address    : 00:11:f5:dc:7a:16

Gateway

IP address     : 172.16.1.254

### Setting the in-band Interface (UGE) IP Address

Before you can Telnet to the DAS4192 or use SNMP to manage the DAS4192, you must assign an IP address to either the in-band (uge) interface or the management Ethernet (nme) interface.

You can specify the subnet mask (netmask) in dotted decimal format.

To set the IP address and VLAN membership of the in-band (uge) management interface, perform the following procedures in CLI Ex mode:

Step 1	Assign an IP address and subnet mask to the in-band (uge) management interface.

Step 2	Verify the default gateway, if necessary.

Step 3	Assign the in-band interface to the proper VLAN.

This example shows how to assign an IP address, specify the subnet mask, and specify the VLAN assignment for the in-band (uge) interface.

- Setup the in-band management interface

CLI(config ip)# set uge 192.168.100.1 255.255.255.0 192.168.100.254

OK

CLI(config ip)# show

UGE

IP address     : 192.168.100.1

subnet mask    : 255.255.255.0

MAC address    : 00:11:f5:dc:7a:17

UGE VLAN ID    : 4092

NME

IP address     : 10.12.3.97

subnet mask    : 255.255.248.0

MAC address    : 00:11:f5:dc:7a:16

Gateway

IP address     : 192.168.100.254

CLI(config ip)# exit

CLI# config mgt

CLI(mgt)# set vlan 10

OK

CLI(config mgt)# show

management VLAN    : 10

user    : guest (guest)

user    : admin (admin)

user    : abc (admin)

### Configuring the Default Gateway

The DAS4192 sends IP packets destined for other IP subnets to the default gateway (typically a router interface in the same network or subnet as the switch IP address). The DAS4192 does not use the IP routing table to forward traffic from connected devices, only IP traffic generated by the DAS4192 itself (for example, Telnet, TFTP, and ping).

The switch sends all off-network IP traffic to the primary default gateway. Both the in-band (uge) and management Ethernet (nme) interfaces are specified with common default gateway, the system forward traffics automatically determines through which interface the default gateway can be reached.

### Configuring the Secured Host

The security host mechanism protects the IP-DSLAM against unauthorized access from untrustful host. This feature allows you to specify the trusted host IPs and authorized services (e.g. SNMP, TELNET, and FTP)

Enter to the “config secure” sub-group directory to configure the secured host IP address.

CLI# config secure

CLI(config secure)#

Table 3-21	Secured Host Configuration

| Use this command to specify the secured host with all permission services. |  |
| --- | --- |
| CLI(config secure)# allow <index> all |  |
| Use this command to specify the secured host with none permission services. |  |
| CLI(config secure)# allow <index> none |  |
| Use this command to specify the secured host in a specifics service. |  |
| CLI(config secure)# allow <index> <snmp,telnet,ftp,tftp> |  |
| Use this command to enable the secured host feature. |  |
| CLI(config secure)# enable |  |
| Use this command to disable the secured host feature. |  |
| CLI(config secure)# disable |  |
| Use this command to specify the secured host IP range. |  |
| CLI(config secure)# set <index> <from-ip> [<to-ip>] |  |
| Use this command to displays the information of secured host. |  |
| CLI(config secure)# show [<index>] |  |
| Parameters | Task |
| <index> | This specifies the entry number of secured host list. Valid values: 1 ~ 10 |
| <snmp,telnet,ftp,tftp> | This indicates the services (any combination of SNMP, TELNET, FTP and TFTP) the specified secured hosts are allowed. Valid values: snmp, telnet, ftp, tftp |
| <from-ip> | This indicates the beginning of the IP address range of the secured hosts. Valid values: 0.0.0.0 ~ 255.255.255.255 |
| <to-ip> | This indicates the end of the IP address range of the secured hosts. Valid values: 0.0.0.0 ~ 255.255.255.255 |

## Storing the Active System Configuration

Configuration modified under “config” directory without saving (storing) will be lost due to hardware reboot.

Use “save” command under “config file” sub-group directory to save your active configuration in system flash, DAS4192 will load the new setting whenever the system reboot.

Enter to the “config file” sub-group directory to operation.

CLI# config file

CLI(config file)#

Table 3-22	Store the Active System Configuration

Use this command to save current configuration and backup old configuration.

CLI(config file)# save

Use this command to remove all save configuration files.

CLI(config file)# erase

Use this command to show configuration information.

CLI(config file)# ls

System configures saving takes around 15 seconds to finish in normal.

- Save the system configuration

CLI(config file)# save

OK

CLI(config file)# ls

Listing directory [cfg:]

-rwxrwxrwx  1 0       0           133630 Oct 10 12:45 default.cfg

-rwxrwxrwx  1 0       0               32 Oct 10 12:45 default.md5

-rwxrwxrwx  1 0       0               37 Oct 10 00:11 mac.cfg

## Backup and Restore the Active System Configuration

NE provides NC/ADSL LC backup and restore related CLI commands to backup or restore the NE configuration via FTP. The backup procedures are as following:

- Step 1	  Open the DOS prompt window (or environment) on personal computer (PC).

- Step 2    Go to the directory where the backup file is saved, and then login the

- DAS4192 by FTP

- Step 3    Get the configuration file from NE to the target partition via FTP by following

- commands:

- ftp> cd cfg:

- ftp> get default.cfg

- or

- ftp> put default.cfg

|  | It is noted that login device via FTP must be used the read-write authorization. The default username/password is admin/admin. |
| --- | --- |

|  | It is noted that the NE configuration is saved in “default.cfg” on the NE. The operator can backup the “default.cfg” and save it with a different filename on the local host. However, the operator has to restore (by the ftp “put” command) the NE configuration with filename of “default.cfg”. |
| --- | --- |

- Example 13 and Example 14 show the process to backup the configurations and restore the configurations, respectively..

- Backup the configurations from the NE via FTP.

- D:\>ftp 10.12.3.160 - Connected to 10.12.3.160. - 220-====================================================================- - 220- Welcome to the IP-DSLAM FTP Server - - 220- - - 220- CAUTION: It's your responsibility to use the FTP service correctly - - 220- , please put the right files into the right file system. - - 220 ====================================================================- - User (10.12.3.160:(none)): admin - 331 Password required - Password: - 230 User logged in - ftp> cd cfg: - 250 Changed directory to "cfg:/" - ftp> get default.cfg D:\DSLAM-TPE-4.txt - 200 Port set okay - 150 Opening BINARY mode data connection - 226 Transfer complete - ftp: 152231 bytes received in 0.45Seconds 335.31Kbytes/sec. - ftp> bye - 221 Bye...see you later - D:\>

- Restore the configurations to the NE via FTP.

- D:\>ftp 10.12.3.160 - Connected to 10.12.3.160. - 220-====================================================================- - 220- Welcome to the IP-DSLAM FTP Server - - 220- - - 220- CAUTION: It's your responsibility to use the FTP service correctly - - 220- , please put the right files into the right file system. - - 220 ====================================================================- - User (10.12.3.160:(none)): admin - 331 Password required - Password: - 230 User logged in - ftp> cd cfg: - 250 Changed directory to "cfg:/" - ftp> put DSLAM-TPE-4.cfg default.cfg - 200 Port set okay - 150 Opening BINARY mode data connection - 226- CAUTION:Please wait for 120 seconds - - 226 Transfer complete - ftp: 152231 bytes sent in 0.80Seconds 191.01Kbytes/sec. - ftp> bye - 221 Bye...see you later

## File System Configuration

Enter to the “filesystem” sub-group directory to operation.

CLI# filesystem

CLI(filesystem)#

Table 3-23	File System Configuration

| Use this command to delete file. |  |
| --- | --- |
| CLI(filesystem)# del {opCodeA ¦ opCodeB ¦ cfg} <filename> |  |
| Use this command to format file system partition. |  |
| CLI(filesystem)# format {opCodeA ¦ opCodeB ¦ cfg} |  |
| Use this command to list files inside file system partition. |  |
| CLI(filesystem)# ls [opCodeA ¦ opCodeB ¦ cfg] |  |
| Parameters | Task |
| {opCodeA ¦ opCodeB ¦ cfg} | This specifies the partition of NC on-board flash. opCodeA/opCodeB: the partition to store the NE firmware and LC firmware. cfg: the partition to store the configuration file. |
| <filename> | This specifies the file name of file to be stored in the partition of NC on-board flash. Type: Mandatory Valid values: String ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |

## NE Firmware Upgrade

This Section depicts the “NE Firmware Upgrade” to upgrade the NC/ADSL LC firmware image to NE through FTP.

NE provides NC/ADSL LC Firmware Upgrade related commands to transfer the new firmware to the NE on-board flash (non-violent memory) by FTP. The upgrade firmware procedure is as fllow.

  - Check the current boot partition via the CLI command:

        - CLI# boot-device show

- current boot device    : opCodeB:xxxxx.enc

- next boot device       : opCodeB:xxxxx.enc

  - Open the DOS prompt window (or environment) on personal computer (PC).

  - Go to the directory where the new firmware is saved, and then login the DAS4192 by FTP

  - Upload the new firmware to the target partition via FTP by following commands:

        - ftp> cd opCodeA: (or ftp> cd opCodeB: )

        - ftp> bin

        - ftp> put xxxxx.enc

  - Change the next boot partition to be rebooted to execute the new image (refer to Table 3-12 Managing the Boot Section).

Example 15 shows how to upload firmware to memory.

|  | It is noted that login device via FTP must be used the read-write authorized. The default username/password is admin/admin. |
| --- | --- |

- Upload NC/ADSL LC Firmware to Flash Memory through FTP

Upgrade image file of NC root@redhat9:/tmp> ftp 10.5.2.100 Connected to 10.5.2.100 (10.5.2.100). Name (10.5.2.100:axdsl): admin 331 Password required Password: 230 User logged in ftp> cd opCodeA: 250 Changed directory to "opCodeA:" ftp> bin 200 Type set to I, binary mode ftp> put am0031.enc 200 Port set okay 150 Opening BINARY mode data connection 226- CAUTION:Please wait for 120 seconds or check the Flash LED - 226 Transfer complete ftp: 3126797 bytes sent in 6.91Seconds 452.70Kbytes/sec. ftp> bye 221 Bye...see you later

Upgrade the DSP code of LC root@redhat9:/tmp> ftp 10.5.2.100 Connected to 10.5.2.100 (10.5.2.100). Name (10.5.2.100:axdsl): admin 331 Password required Password: 230 User logged in ftp> cd opCodeA: 250 Changed directory to "opCodeA:" ftp> bin 200 Type set to I, binary mode ftp> put mla2031fw.enc 200 Port set okay 150 Opening BINARY mode data connection 226- CAUTION:Please wait for 120 seconds or check the Flash LED - 226 Transfer complete ftp: 457808 bytes sent in 1.03Seconds 444.04Kbytes/sec ftp> bye 221 Bye...see you later

Upgrade the Booter of LC root@redhat9:/tmp> ftp 10.5.2.100 Connected to 10.5.2.100 (10.5.2.100). Name (10.5.2.100:axdsl): admin 331 Password required Password: 230 User logged in ftp> cd opCodeA: 250 Changed directory to "opCodeA:" ftp> bin 200 Type set to I, binary mode ftp> put mla2031br.enc 200 Port set okay 150 Opening BINARY mode data connection 226- CAUTION:Please wait for 120 seconds or check the Flash LED - 226 Transfer complete ftp: 32892 bytes sent in 0.11Seconds 299.02Kbytes/sec ftp> bye 221 Bye...see you later

|  | Make sure the source image file that you select is accordant to the NE model; else the NE may not run well with the upgraded firmware image after rebooting. |
| --- | --- |

## SHDSL Firmware Upgrade

This section explains the procedures to upgrade the SHDSL line card’s firmware version; the higher version will bring new features and function of SHDSL LCs.

CLI employs a NE SHDSL Firmware Upgrade utility to transfer the new code files to the memory of NC card by FTP, and then upgrade this new version from memory to SHDSL line card, follow the below procedures to update your SHDSL line card if necessary.

First, open the DOS prompt window and go to the directory where the new firmware is, then upload the new SHDSL firmware to flash memory through FTP.

Example 16 shows how to upload firmware to memory.

The username/password of FTP is admin/admin.

Second, use the commands described in Table 3-24 to upgrade the new firmware to SHDSL line card. Wait for the state of upgrade becomes “finished”.

- Upload SHDSL Firmware to Flash Memory through FTP

D:\image\SHDSL Firmware>ftp 10.12.3.97

Connected to 10.12.3.97.

220-====================================================================-

220-                Welcome to the IP-DSLAM FTP Server                  -

220-                                                                    -

220- CAUTION: It's your responsibility to use the FTP service correctly -

220-          , please put the right files into the right file system.  -

220 ====================================================================-

User (10.12.3.97:(none)): admin

331 Password required

Password:

230 User logged in

ftp>

ftp> cd shdsl:

250 Changed directory to "shdsl:/"

ftp> put TEImage.bin.gz

200 Port set okay

150 Opening BINARY mode data connection

226- CAUTION:Please wait for 120 seconds -

226 Transfer complete

ftp: 1834196 bytes sent in 1.30Seconds 1414.18Kbytes/sec.

ftp>

ftp> bye

221 Bye...see you later

Table 3-24	SHDSL Firmware Upgrade

| Use this command to upgrade SHDSL firmware from flash memory to SHDSL line card. |  |
| --- | --- |
| CLI# shdsl-fw-upgrade start <lc-range> |  |
| Use this command to show the upgrade status. |  |
| CLI# shdsl-fw-upgrade show |  |
| Parameters | Task |
| <lc-range> | This specifies the slot index of target SHDSL line card. Type: Mandatory Valid values: 1 ~ 4 |

- Upload SHDSL Firmware to Line Card through FTP

CLI# shdsl-fw-upgrade start 4

OK: Please reset LC after "finished" state

CLI# shdsl-fw-upgrade show

SHDSL firmware upgrade state

LC  type               state

--  -----  ------------------------------

1   ADSL                             n/a

2   ADSL                             n/a

3   ADSL                             n/a

4  SHDSL  transmission of firmware image

## Ambient Temperature

Use “hw-sensor” command under “config” group directory to set the temperature threshold of hardware sensor of DAS4192.

Enter to the “config hw-sensor” sub-group directory to operation.

CLI# config hw-sensor

CLI(hw-sensor)#

Table 3-25	Configuring Ambient Temperature

| Use this command to set the temp threshold of whole system. |  |
| --- | --- |
| CLI(hw-sensor)# set temp <temp-high> <temp-low> |  |
| Use this command to show the current setting. |  |
| CLI(hw-sensor)# show |  |
| Parameters | Task |
| <temp-high> | This specifies the high temperature to trigger the alarm transmit. Type: Mandatory Valid values: -20 ~ 100 (degrees centigrade) |
| <temp-low> | This specifies the low temperature to trigger the alarm transmit. Type: Mandatory Valid values: -20 ~ 100 (degrees centigrade) |

## Checking the SFP module information

DAS4192 IP-DSLAM support up to 2 SFP (Small Form Pluggable) Mini-GBIC module.

Use the “gbic show” command under the “status” group directory to display the SFP information, each plug-in SFP module.

Enter to the “status” group directory to verify the SFP module information.

CLI# status

CLI(status)#

Table 3-26	Checking the SFP module information

| Using this command to display the system plugged SFP mini GBIC module. |  |
| --- | --- |
| CLI(status)# gbic show <uge-id> |  |
| Parameters | Task |
| <uge-id> | This specifies the index of UGE. Type: Mandatory Valid values: 1 ¦ 2 |

# Managing the System Profiles

This chapter explains how to use profile feature to create and manage profiles for your DAS4192 IP-DSLAM.

This chapter contains the following sections:

- Overview of System Profile

- Managing the ADSL Performance Alarm Profile

- Managing the SHDSL Performance Alarm Profile

- Managing the ADSL Port Connection Profile

- Managing the SHDSL Port Connection Profile

- Managing the IP Traffic Profile

- Managing the Traffic Policing Profile

- Managing the Multicast Service Profile

## Overview of System Profile

The system profile feature allows you to create and manage of ADSL PM Alarm profile, ADSL port connection profile, IP traffic profile, and multicast group profile for your DAS4192 IP-DSLAM. These profiles enable you to use on the different ADSL line port and data connection, each of which requires different configuration settings. For example, you may want to set up profiles for using your ADSL line port at the home user, small office user, and in enterprise customer such as bank. They may require the different line speed and performance monitoring to secure the service quality. Once the profiles are created, you can easily assign or modify between them without having to reconfigure your ADSL line port each time you face a new connection.

A profile is a named list of configuration parameters with a value assigned to each parameter. When you delete a profile you will affect the change on all ports using that profile. If you want to change a single port or a subset of ports, you can create another profile with desired parameters, and then assign the new profile to the desired port.

Two types of profiles are enclosing with the ADSL line port, which are Performance Alarm Profile and Port Connection Profile. The Multicast Group Profile is used with the multicast service which contains management of channel profile and services profile, while IP traffic profile manages the traffic bandwidth of Unicast connection.

## Managing the ADSL Performance Alarm Profile

| Use this command to create the GE Network Uplink Interface parameters. |  |
| --- | --- |
| #create ethernet intf ifname interface-name [ip ip-address] [mask net-mask] [usedhcp true ¦ false] [speed {auto ¦ 100BT ¦ 1000BT}] [type uplink ¦ downlink] [enable ¦ disable] [pkttype Mcast ¦ Bcast ¦ UnknownUcast ¦ All ¦ None] [orl decvalue] [duplex half ¦ full ¦ auto] [class0thrshld class0thrshld] [class1thrshld class1thrshld] [class2thrshld class2thrshld] [class3thrshld class3thrshld] [class4thrshld class4thrshld] [class5thrshld class5thrshld] [class6thrshld class6thrshld] [class7thrshld class7thrshld] [profilename profilename] [mgmtvlanid mgmtvlanid] [priority priority] |  |
| Use this command to modify the GE Network Uplink Interface parameters. |  |
| #modify ethernet intf ifname interface-name [ip ip-address] [mask net-mask] [usedhcp true ¦ false] [speed {auto ¦ 100BT ¦ 1000BT}] [type uplink ¦ downlink] [enable ¦ disable] [pkttype Mcast ¦ Bcast ¦ UnknownUcast ¦ All ¦ None] [orl decvalue] [duplex half ¦ full ¦ auto] [class0thrshld class0thrshld] [class1thrshld class1thrshld] [class2thrshld class2thrshld] [class3thrshld class3thrshld] [class4thrshld class4thrshld] [class5thrshld class5thrshld] [class6thrshld class6thrshld] [class7thrshld class7thrshld] [profilename profilename] [mgmtvlanid mgmtvlanid] [priority priority] |  |
| Use this command to delete the GE Network Uplink Interface parameters. |  |
| #delete ethernet intf ifname interface-name |  |
| Parameters | Task |
| interface-name |  |
| ip-address |  |
| net-mask |  |
| true ¦ false |  |
| {auto ¦ 100BT ¦ 1000BT} |  |
| uplink ¦ downlink |  |

| Parameters | Task |
| --- | --- |
| enable ¦ disable |  |
| auto ¦ half ¦ full |  |
| Mcast ¦ Bcast ¦UnknownUcast ¦ All ¦ None |  |
| decvalue |  |
| class0thrshld, class1thrshld, class2thrshld, class3thrshld, class4thrshld, class5thrshld, class6thrshld, class7thrshld |  |
| profilename |  |
| mgmtvlanid |  |
| priority |  |

Use performance monitoring (PM) parameter to gather, store, threshold, and alarm report performance data for early detection of ADSL line port problems.

Create a performance alarm profile to define the Near-End and Far-End threshold conditions in 15 minutes and 1 day interval. The thresholds are used to set error levels for each PM. You can program PM threshold ranges such as ES (Errored Seconds), SES (Severely Errored Seconds), and UAS (Unavailable Seconds).

During the accumulation cycle, if the current value of a performance monitoring parameter reaches or exceeds its corresponding threshold value, a threshold crossing alert (TCA) is generated by the system and sent to trap station. TCAs provide early detection of performance degradation. When a threshold is crossed, the ADSL line port continues to count the errors during a given accumulation period. If 0 is entered as the threshold value, the PM parameter is disabling.

A of profile corresponds to a particular set of parameters, and can be referenced to by separate ADSL line port.

Enter to the “config profile adsl-alarm” sub-group directory to manage the ADSL performance alarm profile.

CLI# config profile adsl-alarm

CLI(config profile adsl-alarm)#

Table 4-27	ADSL Performance Alarm Profile Configuration

| Use this command to generate a new ADSL performance alarm profile. |  |
| --- | --- |
| CLI(config profile adsl-alarm)# add <profile-name> |  |
| Use this command to remove the specific ADSL performance alarm profile. |  |
| CLI(config profile adsl-alarm)# del <profile-name> |  |
| Use this command to activate the specific ADSL performance alarm profile. |  |
| CLI(config profile adsl-alarm)# enable <profile-name> |  |
| Use this command to deactivate the specific ADSL performance alarm profile. |  |
| CLI(config profile adsl-alarm)# disable <profile-name> |  |
| Use this command to modify the performance ADSL alarm profile parameters at Near-End and Far-End. |  |
| CLI(config profile adsl-alarm)# set <profile-name> <15min-es> <15min-ses> <15min-uas> <1day-es> <1day-ses> <1day-uas> [near ¦ far] |  |
| Use this command to monitor the ADSL performance alarm profile information. |  |
| CLI(config profile adsl-alarm)# show [<profile-name>] |  |
| Parameters | Task |
| <profile-name> | This specifies the performance alarm profile name Type: Mandatory Valid values: String of up to 32 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |
| <15min-es> | The 15 minutes Errored Seconds (ES), this indicates that a count of 1-second intervals with one or more CRC-8 anomalies summed over all received bearer channels, or one or more LOS defects, or one or more SEF defects, or one or more LPR defects. Type: Mandatory Valid values: 0 ~ 900 Default value: 0 (due to profile generated) |
| <15min-ses> | The 15 minutes Severely Errored Seconds (SES), this indicates that a count of 1-second intervals contains 18 or more CRC-8 anomalies summed over all received bearer channels, or one or more LOS defects, or one or more SEF defects, or one or more LPR defects. Type: Mandatory Valid values: 0 ~ 900 Default value: 0 (due to profile generated) |
| <15min-uas> | The 15 minutes Unavailable Seconds (UAS), this indicates that a count of 1-second intervals for which the ADSL line is unavailable. The ADSL line becomes unavailable at the onset of 10 contiguous SES. The 10 SES are included in unavailable time. Once unavailable, the ADSL line becomes available at the onset of 10 contiguous seconds with no SES. The 10 seconds with no SES are excluded from unavailable time. Type: Mandatory Valid values: 0 ~ 900 Default value: 0 (due to profile generated) |
| <1day-es> | The 1 day Errored Seconds (ES), this indicates that a count of 1-second intervals contains one or more CRC-8 anomalies summed over all received bearer channels, or one or more LOS defects, or one or more SEF defects, or one or more LPR defects. Type: Mandatory Valid values: 0 ~ 86400 Default value: 0 (due to profile generated) |
| <1day-ses> | The 1 day Severely Errored Seconds (SES), this indicates that a count of 1-second intervals contains 18 or more CRC-8 anomalies summed over all received bearer channels, or one or more LOS defects, or one or more SEF defects, or one or more LPR defects. Type: Mandatory Valid values: 0 ~ 86400 Default value: 0 (due to profile generated) |
| <1day-uas> | The 1 day Unavailable Seconds (UAS), this indicates that a count of 1-second intervals for which the ADSL line is unavailable. The ADSL line becomes unavailable at the onset of 10 contiguous SES. The 10 SES are included in unavailable time. Once unavailable, the ADSL line becomes available at the onset of 10 contiguous seconds with no SES. The 10 seconds with no SES are excluded from unavailable time. Type: Mandatory Valid values: 0 ~ 86400 Default value: 0 (due to profile generated) |
| [near ¦ far] | Identify the given PM value in Near-End or Far-End side, CLI Ex will apply the same PM value for Near-End and Far-End if not specify. Type: Optional Valid values: near, far |

This example shows how to generate a new performance alarm profile with corresponding PM values, use “enable” command to activate the given profile.

- Add a new performance alarm profile with correspond PM values

CLI(config profile adsl-alarm)# add bank_pm

OK

CLI(config profile adsl-alarm)# set bank_pm 10 15 20 30 40 50

OK

CLI(config profile adsl-alarm)# enable bank_pm

OK

CLI(config profile adsl-alarm)# show

profile [bank_pm]: enabled

side-end  15min-es  15min-ses  15min-uas  1day-es  1day-ses  1day-uas

--------  --------  ---------  ---------  -------  --------  --------

near end        10         15         20       30        40        50

far end        10         15         20       30        40        50

Attaching the performance alarm profile to the proper ADSL line port can be task at “config port” sub-group directory, refer to the “Chapter 5 Configuring the ADSL Line Port”.

## Managing the SHDSL Performance Alarm Profile

| Use this command to create the GE Network Uplink Interface parameters. |  |
| --- | --- |
| #create ethernet intf ifname interface-name [ip ip-address] [mask net-mask] [usedhcp true ¦ false] [speed {auto ¦ 100BT ¦ 1000BT}] [type uplink ¦ downlink] [enable ¦ disable] [pkttype Mcast ¦ Bcast ¦ UnknownUcast ¦ All ¦ None] [orl decvalue] [duplex half ¦ full ¦ auto] [class0thrshld class0thrshld] [class1thrshld class1thrshld] [class2thrshld class2thrshld] [class3thrshld class3thrshld] [class4thrshld class4thrshld] [class5thrshld class5thrshld] [class6thrshld class6thrshld] [class7thrshld class7thrshld] [profilename profilename] [mgmtvlanid mgmtvlanid] [priority priority] |  |
| Use this command to modify the GE Network Uplink Interface parameters. |  |
| #modify ethernet intf ifname interface-name [ip ip-address] [mask net-mask] [usedhcp true ¦ false] [speed {auto ¦ 100BT ¦ 1000BT}] [type uplink ¦ downlink] [enable ¦ disable] [pkttype Mcast ¦ Bcast ¦ UnknownUcast ¦ All ¦ None] [orl decvalue] [duplex half ¦ full ¦ auto] [class0thrshld class0thrshld] [class1thrshld class1thrshld] [class2thrshld class2thrshld] [class3thrshld class3thrshld] [class4thrshld class4thrshld] [class5thrshld class5thrshld] [class6thrshld class6thrshld] [class7thrshld class7thrshld] [profilename profilename] [mgmtvlanid mgmtvlanid] [priority priority] |  |
| Use this command to delete the GE Network Uplink Interface parameters. |  |
| #delete ethernet intf ifname interface-name |  |
| Parameters | Task |
| interface-name |  |
| ip-address |  |
| net-mask |  |
| true ¦ false |  |
| {auto ¦ 100BT ¦ 1000BT} |  |
| uplink ¦ downlink |  |

| Parameters | Task |
| --- | --- |
| enable ¦ disable |  |
| auto ¦ half ¦ full |  |
| Mcast ¦ Bcast ¦UnknownUcast ¦ All ¦ None |  |
| decvalue |  |
| class0thrshld, class1thrshld, class2thrshld, class3thrshld, class4thrshld, class5thrshld, class6thrshld, class7thrshld |  |
| profilename |  |
| mgmtvlanid |  |
| priority |  |

Enter to the “config shdsl profile alarm” sub-group directory to manage the SHDSL performance alarm profile.

CLI# config shdsl profile alarm

CLI(config shdsl profile alarm)#

Table 4-28	SHDSL Performance Alarm Profile Configuration

| Use this command to generate a new SHDSL performance alarm profile. |  |
| --- | --- |
| CLI(config shdsl profile alarm)# add <name> |  |
| Use this command to remove the specific SHDSL performance alarm profile. |  |
| CLI(config shdsl profile alarm)# del <name> |  |
| Use this command to activate the specific SHDSL performance alarm profile. |  |
| CLI(config shdsl profile alarm)# enable <name> |  |
| Use this command to deactivate the specific SHDSL performance alarm profile. |  |
| CLI(config shdsl profile alarm)# disable <name> |  |
| Use this command to modify the SHDSL performance alarm profile parameters at Near-End. |  |
| CLI(config shdsl profile alarm)# set <name> [atte <atte> snr <snr> es <es> ses <ses> crc <crc> losws <losws> uas <usa>] |  |
| Use this command to monitor the SHDSL performance alarm profile information. |  |
| CLI(config shdsl profile alarm)# show <name> |  |
| Parameters | Task |
| <name> | This specifies the performance alarm profile name Type: Mandatory Valid values: String of up to 32 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |
| <atte> | This identifies the attenuation threshold. Type: Mandatory Valid values: 1 ~ 127 |
| <es> | The 15 minutes Errored Seconds (ES), this indicates that a count of 1-second intervals with one or more CRC-8 anomalies summed over all received bearer channels, or one or more LOS defects, or one or more SEF defects, or one or more LPR defects. Type: Mandatory Valid values: 0 ~ 900 Default value: 0 (due to profile generated) |
| <ses> | The 15 minutes Severely Errored Seconds (SES), this indicates that a count of 1-second intervals contains 18 or more CRC-8 anomalies summed over all received bearer channels, or one or more LOS defects, or one or more SEF defects, or one or more LPR defects. Type: Mandatory Valid values: 0 ~ 900 Default value: 0 (due to profile generated) |
| <crc> | This identifies the CRC error threshold. Type: Mandatory Valid values: 0 ~ 44100 |
| <losws> | This identifies the LOSWS error threshold. Type: Mandatory Valid values: 0 ~ 900 |
| <losws> | This identifies the LOSWS error threshold. Type: Mandatory Valid values: 0 ~ 900 |
| <usa> | The 15 minutes Unavailable Seconds (UAS), this indicates that a count of 1-second intervals for which the ADSL line is unavailable. The ADSL line becomes unavailable at the onset of 10 contiguous SES. The 10 SES are included in unavailable time. Once unavailable, the ADSL line becomes available at the onset of 10 contiguous seconds with no SES. The 10 seconds with no SES are excluded from unavailable time. Type: Mandatory Valid values: 0 ~ 900 Default value: 0 (due to profile generated) |

## Managing the ADSL Port Connection Profile

The ADSL port connection profile indicates the expected overall physical parameters of the ADSL line port. This profile describes the communication at the ADSL layer. A number of parameters will be specified such as fast/interleaved, rate adaptation mode, noise margin, power spectrum density, and transmit rate.

A profile corresponds to a particular set of parameters, and can be referenced to by separate ADSL line port.

Enter to the “config profile adsl-conf” sub-group directory to manage the ADSL port connection profile.

CLI# config profile adsl-conf

CLI(config profile adsl-conf)#

Table 4-29	ADSL Port Connection Profile Configuration

| Use this command to generate a new ADSL port connection profile. |  |
| --- | --- |
| CLI(config profile adsl-conf)# add <profile-name> |  |
| Use this command to remove the specific ADSL port connection profile. |  |
| CLI(config profile adsl-conf)# del <profile-name> |  |
| Use this command to activate the specific ADSL port connection profile. |  |
| CLI(config profile adsl-conf)# enable <profile-name> |  |
| Use this command to deactivate the specific ADSL port connection profile. |  |
| CLI(config profile adsl-conf)# disable <profile-name> |  |
| Use this command to modify the profile rate mode to adaptive with desired parameters. |  |
| CLI(config profile adsl-conf)# set adaptive-rate <profile-name> <us-min-rate> <us-max-rate> <ds-min-rate> <ds-max-rate> |  |
| Use this command to modify the profile rate mode to dynamic with desired parameters. |  |
| CLI(config profile adsl-conf)# set dynamic-rate <profile-name> <us-min-rate> <us-max-rate> <ds-min-rate> <ds-max-rate> <us-down-shift> <us-up-shift> <ds-down-shift> <ds-up-shift> |  |
| Use this command to modify the profile rate mode to fix with desired parameters. |  |
| CLI(config profile adsl-conf)# set fixed-rate <profile-name> <us-rate> <ds-rate> |  |
| Use this command to modify the profile line mode to interleaved path with latency. |  |
| CLI(config profile adsl-conf)# set line-mode <profile-name> interleave <max-us-latency> <max-ds-latency> <min-us-inp> <min-ds-inp> |  |
| Use this command to modify the profile line mode to fast path. |  |
| CLI(config profile adsl-conf)# set linemode <profile-name> fast |  |
| Use this command to modify the profile PSD (Power Spectrum Density) with desired parameters. |  |
| CLI(config profile adsl-conf)# set psd <profile-name> <us-psd> <ds-psd> |  |
| Use this command to modify the SNR (Signal Noise Ratio) margin of downstream. |  |
| CLI(config profile adsl-conf)# set snr <profile-name> ds <target-snr-mgn> <min-snr-mgn> <max-snr-mgn> |  |
| Use this command to modify the SNR (Signal Noise Ratio) margin of upstream. |  |
| CLI(config profile adsl-conf)# set snr <profile-name> us <target-snr-mgn> <min-snr-mgn> <max-snr-mgn> |  |
| Use this command to modify the downstream shelf SNR margin due to dynamic rate mode. |  |
| CLI(config profile adsl-conf)# set shift-snr <profile-name> ds <down-shift-snr> <up-shift-snr> |  |
| Use this command to modify the upstream shelf SNR margin due to dynamic rate mode. |  |
| CLI(config profile adsl-conf)# set shift-snr <profile-name> us <down-shift-snr> <up-shift-snr> |  |
| Use this command to modify the ADSL2/ADSL2+ power management for L2 state. |  |
| CLI(config profile adsl-conf)# set pwr-mgt <profile-name> l2 <mode> <l2-min-rate> <l2-max-rate> <l2-low-rate-time> <l0-time> |  |
| Use this command to modify the ADSL2/ADSL2+ power management for L3 state. |  |
| CLI(config profile adsl-conf)# set pwr-mgt <profile-name> l3 <denied ¦ accepted> |  |
| Use this command to monitor the ADSL port connection profile information. |  |
| CLI(config profile adsl-conf)# show [<profile-name>] |  |
| Parameters | Task |
| <profile-name> | This specifies the ADSL port connection profile name Type: Mandatory Valid values: String of up to 32 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |
| <us-min-rate> | Defines upstream minimum transmit rate, this parameter is available for adaptive and dynamic rate mode. Type: Mandatory Valid values: 64 ~ 2976 (multiple of 32 kbps) Default value: 64 kbps (due to profile generated) |
| <us-max-rate> | Defines upstream maximum transmit rate, this parameter is available for adaptive and dynamic rate mode. Type: Mandatory Valid values: 64 ~ 2976 (multiple of 32 kbps) Default value: 64 kbps (due to profile generated) |
| <ds-min-rate> | Defines downstream minimum transmit rate, this parameter is available for adaptive and dynamic rate mode. Type: Mandatory Valid values: 64 ~ 29984 (multiple of 32 kbps) Default value: 64 kbps (due to profile generated) |
| <ds-max-rate> | Defines downstream maximum transmit rate, this parameter is available for adaptive and dynamic rate mode. Type: Mandatory Valid values: 64 ~ 29984 (multiple of 32 kbps) Default value: 64 kbps (due to profile generated) |
| <us-rate> | Defines upstream transmit rate, this parameter is available for fixed rate mode. Type: Mandatory Valid values: 64 ~ 2976 (multiple of 32 kbps) Default value: 64 kbps (due to profile generated) |
| <ds-rate> | Defines downstream transmit rate, this parameter is available for fixed rate mode. Type: Mandatory Valid values: 64 ~ 29984 (multiple of 32 kbps) Default value: 64 kbps (due to profile generated) |
| < us-down-shift > | Defines upstream down-shift margin time interval due to dynamic rate of seamless adaptation action. Type: Mandatory Valid values: 0 ~ 16384 (seconds) Default value: 0 sec (due to profile generated) |
| < us-up-shift > | Defines upstream up-shift margin time interval due to dynamic rate of seamless adaptation action. Type: Mandatory Valid values: 0 ~ 16384 (seconds) Default value: 0 sec (due to profile generated) |
| < ds-down-shift > | Defines downstream down-shift margin time interval due to dynamic rate of seamless adaptation action. Type: Mandatory Valid values: 0 ~ 16384 (seconds) Default value: 0 sec (due to profile generated) |
| < ds-up-shift > | Defines downstream up-shift margin time interval due to dynamic rate of seamless adaptation action. Type: Mandatory Valid values: 0 ~ 16384 (seconds) Default value: 0 sec (due to profile generated) |
| < max-us-latency> | Defines upstream interleaved path latency. Type: Mandatory Valid values: 1 ~ 255 (milliseconds) Default value: 0 msec (due to profile generated) |
| < max-ds-latency> | Defines downstream interleaved path latency. Type: Mandatory Valid values: 1 ~ 255 (milliseconds) Default value: 0 msec (due to profile generated) |
| <min-us-inp> | Defines upstream minimum protection symbol time, Type: Mandatory Valid values: {0 ¦ 1/2 ¦ 1 ¦ 2 ¦ 4 ¦ 8 ¦ 16} Default value: 0 (due to profile generated) |
| <min-ds-inp> | Defines downstream minimum protection symbol time. Type: Mandatory Valid values: {0 ¦ 1/2 ¦ 1 ¦ 2 ¦ 4 ¦ 8 ¦ 16} Default value: 0 (due to profile generated) |
| <us-psd> | Defines upstream power spectrum density level. Type: Mandatory Valid values: -40.0 ~ 4.0 (dB/Hz) Default value: 0 dB/Hz (due to profile generated) |
| <ds-psd> | Defines downstream power spectrum density level. Type: Mandatory Valid values: -40.0 ~ 4.0 (dB/Hz) Default value: 1.0 dB/Hz (due to profile generated) |
| <target-snr-mgn> | Defines target SNR margin for upstream or downstream signal. Type: Mandatory Valid values: 0.0 ~ 31.0 (dBm) Default value: 6.0 dBm (due to profile generated) |
| <min-snr-mgn> | Defines minimum SNR margin for upstream or downstream signal. Type: Mandatory Valid values: 0.0 ~ 31.0 (dBm) Default value: 0 dBm (due to profile generated) |
| <max-snr-mgn> | Defines maximum SNR margin for upstream or downstream signal. Type: Mandatory Valid values: 0.0 ~ 31.0 (dBm) Default value: 31.0 dBm (due to profile generated) |
| <down-shift-snr> | Defines down-shift SNR margin for upstream or downstream signal. Type: Mandatory Valid values: 0.0 ~ 31.0 (dBm) Default value: 0 dBm (due to profile generated) |
| <up-shift-snr> | Defines up-shift SNR margin for upstream or downstream signal. Type: Mandatory Valid values: 0.0 ~ 31.0 (dBm) Default value: 0 dBm (due to profile generated) |
| <mode> | Defines power management mode Type: Mandatory Valid values: {automatic ¦ manual} Default value: manual |
| < l2-min-rate> | Defines minimum and low-bound for L2 state in 32 kbps steps. Type: Mandatory Valid values: 32 ~ 29984 kbps Default value: 64 kbps |
| < l2-max-rate> | Defines maximum data rate for L2 state in 32 kbps steps. Type: Mandatory Valid values: 32 ~ 29984 kbps Default value: 29984 kbps |
| <l2-low-rate-time> | Defines continuous time (sec.) of rate lower than L2 low-bound Type: Mandatory Valid values: 0 ~ 65535 seconds Default value: 300 seconds |
| <l0-time> | Defines minimum time (sec.) in L0 state to check L2 criteria Type: Mandatory Valid values: 0 ~ 65535 seconds Default value: 900 seconds |
| <denied ¦ accepted> | Defines the IP-DSLAM to allow L3 command from CPE site or not. Type: Mandatory Valid values: {denied ¦ accepted} Default value: accepted |

This example shows how to generate a new ADSL port connection profile with desired parameters, use “enable” command to activate the given profile.

- Add a new ADSL port connection profile with desired values

CLI(config profile adsl-conf)# add bank

OK

CLI(config profile adsl-conf)# set adaptive-rate bank 512 2048 1024 8192

OK

CLI(config profile adsl-conf)# set line-mode bank interleave 10 10 1 1

OK

CLI(config profile adsl-conf)# enable bank

OK

CLI(config profile adsl-conf)# show bank

profile [bank]

status       : enabled

line mode    : interleave

rate mode    : adaptive

up-stream   down-stream

-----------  -----------

fast rate (min/max)               :    512/2048    1024/8192 kbps

interleave rate (min/max)         :    512/2048    1024/8192 kbps

interleave max delay              :          10           10 ms

interleave min INP symbol time    :           1            1

target SNR margin                 :         6.0          6.0 dB

min./max. SNR margin              :    0.0/31.0     0.0/31.0 dB

down/up shift SNR margin          :    3.0/20.0     3.0/20.0 dB

down/up shift time                :   1000/1000    1000/1000 sec

PSD                               :         0.0          0.0 dBm/Hz

power management setting:

L2-mode   L2-min-rate  L2-max-rate   CPE L3

---------  -----------  -----------  --------

manual      32 kbps   29984 kbps  accepted

Attaching the ADSL port connection profile to the proper ADSL line port can be task at “config port” sub-group directory, refer to the “Chapter 5 Configuring the ADSL Line Port”.

## Managing the SHDSL Port Connection Profile

A profile corresponds to a particular set of parameters, and can be referenced to by separate SHDSL line port.

Enter to the “config shdsl profile conf” sub-group directory to manage the SHDSL port connection profile.

CLI# config shdsl profile conf

CLI(config shdsl profile conf)#

Table 4-30	SDSL Port Connection Profile Configuration

| Use this command to generate a new SHDSL port connection profile. |  |
| --- | --- |
| CLI(config shdsl profile conf)# add <name> |  |
| Use this command to remove the specific SHDSL port connection profile. |  |
| CLI(config shdsl profile conf)# del <name> |  |
| Use this command to activate the specific SHDSL port connection profile. |  |
| CLI(config shdsl profile conf)# enable <name> |  |
| Use this command to deactivate the specific SHDSL port connection profile. |  |
| CLI(config shdsl profile conf)# disable <name> |  |
| Use this command to modify the profile rate mode to adaptive with desired parameters. |  |
| CLI(config shdsl profile conf)# set <name> [minrate <minrate> maxrate <maxrate> psd <psd> transmission <transmission> downcurrentsnr <downcurrentsnr> downworstsnr <downworstsnr> upcurrentsnr <upcurrentsnr> upworstsnr <upworstsnr> usedsnr <usedsnr> lineprobe <lineprobe>] |  |
| Use this command to monitor the SHDSL port connection profile information. |  |
| CLI(config shdsl profile conf)# show [<name>] |  |
| Parameters | Task |
| <name> | This specifies the SHDSL port connection profile name Type: Mandatory Valid values: String of up to 32 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |
| <minrate> | Defines minimum transmit rate, this parameter is available for adaptive and dynamic rate mode. Type: Mandatory Valid values: 72 ~ 2312 (multiple of 64 kbps) Default value: 72 kbps (due to profile generated) |
| <maxrate> | Defines maximum transmit rate, this parameter is available for adaptive and dynamic rate mode. Type: Mandatory Valid values: 72 ~ 2312 (multiple of 64 kbps) Default value: 72 kbps (due to profile generated) |
| <psd> | Defines power spectrum density level. Type: Mandatory Valid values: 1 = symmetric, 2 = asymmetric Default value: 1 |
| <transmission> | Defines the transmission mode, Annex A, Annex B, or both. Type: Mandatory Valid values: 1:Annex.A or 2:Annex.B or 3:Both. Default value: 3 |
| < downcurrentsnr > | Defines downstream current target SNR margin. Type: Mandatory Valid values: -10 ~ 21 (dBm) Default value: 6 dBm (due to profile generated) |
| < downworstsnr > | Defines downstream worst target SNR margin Type: Mandatory Valid values: -10 ~ 21 (dBm) Default value: 6 dBm (due to profile generated) |
| <upcurrentsnr> | Defines upstream current target SNR margin. Type: Mandatory Valid values: -10 ~ 21 (dBm) Default value: 6 dBm (due to profile generated) |
| <upworstsnr> | Defines upstream worst target SNR margin Type: Mandatory Valid values: -10 ~ 21 (dBm) Default value: 6 dBm (due to profile generated) |
| <usedsnr> | Defines used SNR bit-map, 0:down-current, 1:down-worst, 2:up-current, 3:up-worst Type: Mandatory Valid values: 0, 1, 2, 3 |
| <lineprobe> | Enable or disable the line probe support. 1 = enable, 2 = disable. Type: Mandatory Valid values: 1, 2 |

Fast and Interleaved Mode

Transmission Rate Mode

Power Spectrum Density

SNR Margin

## Managing the IP Traffic Profile

The IP traffic profile is design to specify the traffic attributes of the PVC on the ADSL line. The operator can create the IP Traffic Profile according to the Service Level Agreement (SLA) and apply it to the corresponding VC-to-VLAN on demand.

By configures IP Traffic Profile, the following traffic attributes of a PVC is specified.

- The maximum allowed upstream/downstream net-data rate.

The system drops upstream/downstream packets whenever it exceeds the corresponding specified rate

- The downstream priority of the PVC

The system forwards the downstream packets in a differentiated manner. That is, the system only forwards the traffic on PVC of lower priority whenever either one of the following conditions happens.

  - There is no traffic on PVC of higher priority to be forwarded.

  - The volume of traffic on PVC of higher priority exceeds the specified downstream net-data rate in a unit time.

- The filtering of the downstream broadcast traffic

Enter to the “config profile ip-traffic” sub-group directory to manage the IP traffic profile.

CLI# config profile ip-traffic

CLI(config profile ip-traffic)#

Table 4-31	IP Traffic Profile Configuration

| Use this command to generate a new IP traffic profile. |  |
| --- | --- |
| CLI(config profile ip-traffic)# add <name> |  |
| Use this command to remove a new IP traffic profile. |  |
| CLI(config profile ip-traffic)# del <name> |  |
| Use this command to configure the rate limit of specific IP traffic profile. |  |
| CLI(config profile ip-traffic)# set <name> <us-rate-limit> <ds-rate> <ds-priority> <bcast-filter> |  |
| Use this command to display the IP traffic profile information. |  |
| CLI(config profile ip-traffic)# show |  |
| Parameters | Task |
| <name> | This specifies the IP traffic profile name Type: Mandatory Valid values: String of up to 32 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |
| <us-rate-limit> | This specifies the upstream rate limit for subscriber IP traffic. Type: Mandatory Valid values: nolimit ¦ 32k ¦ 64k ¦ 128k ¦ 256k ¦ 384k ¦ 512k ¦ 768k |
| <ds-rate> | This specifies the downstream rate limit for subscriber IP traffic. Type: Mandatory Valid values: 0 ~ 29984 kbps (multiple of 32 kbps) |
| <ds-priority> | This defines the downstream priority, the lower the priority, the higher the chance to get drop due to traffic congestion. Type: Mandatory Valid values: low ¦ medium ¦ high ¦ highest |
| <bcast-filter> | This defines the downstream broadcast filter of ip-traffic profile. Available on the VLAN ID in which PVC used this ip-traffic. Type: Mandatory Valid values: drop ¦ forward |

## Managing the Traffic Policing Profile

According to SLA (Service Level Aggrements) between subscribers and ISP, the edge network equipment marks subscriber’s traffic with different service level. The traffic policing profile serves to keep the rule to re-mark the subscriber’s upstream traffic with designated DSCP values. The traffic policing profile is design to police on the unicast connection for user’s upstream traffic; once it is created you can apply it to distinct xDSL line interfaces (see Chapter 5 Configuring the ADSL Line Port).

The CIR(Commit Information Rate) define the upstream rate police to be forwarded. The Action defines action (either DSCP replacement or no action) applied to the traffic over CIR.

Enter to the “config profile metering” sub-group directory to manage the traffic policing profile.

CLI# config profile metering

CLI(config profile metering)#

Table 4-32	Traffic Policing Profile Configuration

| Use this command to generate a new traffic policing profile. |  |
| --- | --- |
| CLI(config profile metering)# add <profile-name> |  |
| Use this command to remove the specific traffic policing profile. |  |
| CLI(config profile metering)# del <profile-name> |  |
| Use this command to modify the traffic policing profile and it desired parameters. |  |
| CLI(config profile metering)# set <profile-name> <cir> <action> |  |
| Use this command to monitor the traffic policing profile information. |  |
| CLI(config profile metering)# show [<profile-name>] |  |
| Parameters | Task |
| <profile-name> | This specifies the traffic policing profile name Type: Mandatory Valid values: String of up to 32 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |
| <cir> | Defines the committed information rate of traffic policing profile. Type: Mandatory Valid values: 1 ~ 1000 (mbps) |
| <action> | This identifies which value will DSCP be replace, drop packets or do nothing when user’s upstream traffic exceeds CIR. Type: Mandatory Valid values: no-action ¦ drop ¦ BE ¦ AF11 ¦ AF12 ¦ AF13 ¦ AF21 ¦ AF22 ¦ AF23 ¦ AF31 ¦ AF32 ¦ AF33 ¦ AF41 ¦ AF42 ¦ AF43 ¦ EF |

|  | The Service Type Control shall be enabled when Traffic Policing Profile is assign to xDSL subscribers (refer to Chapter 5 Configuring the ADSL Line Port). |
| --- | --- |

|  | Please refer to Figure 8-6 for more details of Differentiated Service Code Point. |
| --- | --- |

## Managing the Multicast Service Profile

The multicast service profile embraces a number of multicast channel profiles. The multicast channel profile controls transmission rate and priority of multicast stream. Multicast channel uses class D addressing, which is a special form of the IP address designed for multicasting.

### Multicast Channel Profile Setting

The multicast channel profile sets value of multicast group IP and the associated downstream bandwidth resource, it is a menu list of the TV channel (multicast group) provided by the Content Service Provider (CSP) or Application Service Provider (ASP).

Enter to the “config profile mcast” sub-group directory to manage the multicast channel profile.

CLI# config profile mcast

CLI(config profile mcast)#

Table 4-33	Multicast Channel Profile Configuration

| Use this command to generate a new multicast group profile. |  |
| --- | --- |
| CLI(config profile mcast)# add <profile-name> |  |
| Use this command to remove the specific multicast group profile. |  |
| CLI(config profile mcast)# del <profile-name> |  |
| Use this command to activate the specific multicast group profile. |  |
| CLI(config profile mcast)# enable <profile-name> |  |
| Use this command to deactivate the specific multicast group profile. |  |
| CLI(config profile mcast)# disable <profile-name> |  |
| Use this command to modify the profile multicast group member and it desired parameters. |  |
| CLI(config profile mcast)# set <profile-name> <group-ip> <rate> {low ¦ medium ¦ high ¦ highest} |  |
| Use this command to monitor the multicast group profile information. |  |
| CLI(config profile mcast)# show [<profile-name>] |  |
| Parameters | Task |
| <profile-name> | This specifies the multicast channel profile name Type: Mandatory Valid values: String of up to 32 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |
| <group-ip> | Defines class D IP addressing for multicast channel. Type: Mandatory Valid values: 224.0.1.0 ~ 239.255.255.255 Default value: 0.0.0.0 (due to profile generated) |
| <rate> | Defines the downstream transmission limit rate of multicast channel. Type: Mandatory Valid values: 32~29984 +32 kbps Default value: 32 kbps (due to profile generated) |
| {low ¦ medium ¦ high ¦ highest} | Defines the forwarding priority Type: Mandatory Valid values: low, medium, high, highest Default value: low (due to profile generated) |

### Multicast Service Profile Setting

The multicast service profile is a set of TV channel profiles. Once the TV channel profiles are created, you can generate the multicast service profile to bind suitable TV channel profiles. Each multicast service profile is viewed as a service package for the subscriber to book. The operator then applies the booked multicast service profile to the distinct VC-to-VLAN associated with the subscriber. (see Chapter 7).

Whenever the subscriber clicks his remote controller to watch a TV channel transmitted via the ADSL line, the set-top-box sends the corresponding IGMP report packet. The NE will forward IGMP packet if its multicast IP hits the associated multicast service profile. Otherwise, the NE drops the IGMP packet. As a result, the subscriber is restricted to watch the TV programs that he booked.

Attaching the multicast profile to the proper ADSL line port can be task at “config mcau” sub-group directory, refer to the “Chapter 7”.

Enter to the “config profile mservice” sub-group directory to manage the multicast service profile.

CLI# config profile mservice

CLI(config profile mservice)#

Table 4-34	Multicast Service Profile Configuration

| Use this command to generate a new multicast service profile. |  |
| --- | --- |
| CLI(config profile mservice)# add <service-name> |  |
| Use this command to remove the specific multicast service profile. |  |
| CLI(config profile mservice)# del <service-name> |  |
| Use this command to add the multicast channel profile into specific multicast service profile. |  |
| CLI(config profile mservice)# subscribe <service-name> <profile-list> |  |
| Use this command to remove the multicast channel profile from specific multicast service profile. |  |
| CLI(config profile mservice)# cancel <service-name> <profile-list> |  |
| Use this command to monitor the multicast service profile information. |  |
| CLI(config profile mservice)# show |  |
| Parameters | Task |
| <service-name> | This specifies the multicast service profile name Type: Mandatory Valid values: String of up to 32 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |
| <profile-list> | This specifies the multicast group profile name. Type: Mandatory Valid values: String of up to 32 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). Format: xxx or xxx¦xxx¦…¦xxx (xxx indicate as multicast group profile) |

This example shows how to generate a new multicast channel profile with desired parameters, use “enable” command to activate the given profile.

- Create a new multicast channel profile with desired values

CLI(config profile mcast)# add HBO

OK

CLI(config profile mcast)# add ESPN

OK

CLI(config profile mcast)# add CNN

OK

CLI(config profile mcast)# set HBO 224.1.1.10 29984 high

OK

CLI(config profile mcast)# set ESPN 224.1.1.11 29984 medium

OK

CLI(config profile mcast)# set CNN 224.1.1.12 29984 highest

OK

CLI(config profile mcast)# show

profile [HBO]

grouip-ip     rate(kbps)  priority   status

---------------  ----------  --------  --------

224.1.1.10       29984      high  disabled

profile [ESPN]

grouip-ip     rate(kbps)  priority   status

---------------  ----------  --------  --------

224.1.1.11       29984    medium  disabled

profile [CNN]

grouip-ip     rate(kbps)  priority   status

---------------  ----------  --------  --------

224.1.1.12       29984   highest  disabled

The below example shown how to subscribe (wrap) the created multicast channel profile into a complete service program.

- Subscribe a sets of multicast channel into service profile

CLI(config profile mservice)# add program-1

OK

CLI(config profile mservice)# subscribe program-1 HBO|ESPN

OK

CLI(config profile mservice)# add program-2

OK

CLI(config profile mservice)# subscribe program-2 HBO|ESPN|CNN

OK

CLI(config profile mservice)# show

Profile  [program-1]

Mcast Profile: "HBO", "ESPN",

Profile  [program-2]

Mcast Profile: "HBO", "ESPN", "CNN",

# Managing the Subscriber Interface

This chapter describes how to apply the relative profile to Subscriber interface.

This chapter contains the following sections:

- Configuring the ADSL Line Port

- Monitoring the ADSL Connection Status

- Configuring the SHDSL Line Port

- Monitoring the SHDSL Connection Status

- Subscriber Interface Administrating

## Configuring the ADSL Line Port

The ADSL port connection profile is independent and irrelative with other ADSL port interface. Configuration takes effect on specific port only.

Enter to the “config port” sub-group directory to configure the relative profile on the ADSL line port.

CLI# config port

CLI(config port)#

Table 5-35	ADSL Port Interface Configuration

| Use this command to apply the PM alarm profile to specific ADSL line port. |  |
| --- | --- |
| CLI(config port)# set adsl-alarm-profile <port-range> <profile-name> |  |
| Use this command to apply port connection profile to specific ADSL line port. |  |
| CLI(config port)# set adsl-conf-profile <port-range> <profile-name> |  |
| Use this command to force the ADSL2/ADSL2+ power management status. (manual mode only) |  |
| CLI(config port)# set adsl-pwr-mgt <port-range> <pwr-state> |  |
| Use this command to apply the traffic policing profile to specific ADSL line port. |  |
| CLI(config port)# set metering <port-range> <profile-name> |  |
| Use this command to apply a remote ID to specific xDSL line port. |  |
| CLI(config port)# set remote-id <port-range> <idstring> |  |
| Use this command to remove the remote ID from specific subscriber port. |  |
| CLI(config port)# clean remote-id <port-range> |  |
| Use this command to remove traffic policing profile from specific subscriber port. |  |
| CLI(config port)# clean metering <port-range> |  |
| Use this command to remove the PM alarm profile from specific subscriber port. |  |
| CLI(config port)# clean alarm-profile <port-range> |  |
| Use this command to apply the PM alarm profile to specific SHDSL line port. |  |
| CLI(config port)# set shdsl-alarm-profile <port-range> <profile-name> |  |
| Use this command to apply port connection profile to specific SHDSL line port. |  |
| CLI(config port)# set shdsl-conf-profile <port-range> <profile-name> |  |
| Use this command to viewing the ADSL line port operation status. |  |
| CLI(config port)# show <port-range> |  |
| Use this command to apply the remote CPE wireless to the specified xDSL line port. |  |
| CLI(config port)# set cpe-wireless <port-range> <wireless-status> |  |
| Parameters | Task |
| <port-range> | Identify the port range of the system wish to apply the relevance profile of line port. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <name> | Defines the profile name; port connection profile, performance alarm profile or traffic policing profile. Type: Mandatory Valid values: The name of “port connection profile”, “performance alarm profile” or “traffic policing profile” |
| <pwr-state> | Defines the ADSL2/ADSL2+ power management operating status, switch between L0, L2 and L3 will only available if ADSL power management is in “Manual” mode. Type: Mandatory Valid values: L0, L2,L3 |
| <idstring> | Identify the remote ID information which is used for the DHCP option 82 tap and PPPoE tag. |
| <wireless-status> | Defines the wireless AP status of the remote CPE. Type: Mandatory Valid values: disabled ¦ enabled |

|  | The Service Type Control shall be enabled when Traffic Policing Profile is assign to xDSL subscribers (refer to Chapter 8 Card Module Operations). |
| --- | --- |

- Apply the profile to the specify of ADSL line port

CLI(config port)# set adsl-conf-profile 1.1.1 bank

OK

CLI(config port)# set adsl-alarm-profile 1.1.1 high-threshold

OK

CLI(config port)# set remote-id 1.1.1 1234

OK

CLI(config port)# set metering 1.1.1 policing-1

OK

CLI(config port)# show 1.1.1

port: 1.1.1

admin status        : enabled

oper status         : down

cfg. profile        : "bank"

alarm profile       : "high-threshold"

traffic policing    : "policing-1"

circuit ID          : "IP_DSLAM-100.168.3.97-00:00:00:00:00:00 atm 1/1:0.0"

remote ID           : "1234"

power state         : L0

circuit ID          : "IP_DSLAM-100.168.1.31-00:00:00:00:00:00 atm 1/37:0.0"

remote ID           : "1234"

power state         : L0

line standard       : G.992.5 Annex A

[physical status]

item          US      DS

----------------  ------  ------

attainable rate    1447   11223  kbps

attenuation     0.0     0.0  dB

SNR margin    34.6    33.6  dB

output power    -8.9   -14.5  dBm

[channel status]

item          US      DS

----------------  ------  ------

Tx rate     255     508  kbps

interleave delay       0       0  ms

CRC block length       7      15  ms

INP symbol time    0.00    0.00  DMT symbol

## Monitoring the ADSL Connection Status

Enter to the “status” group directory to monitoring the ADSL line port connection status.

CLI# status

CLI(status)#

Table 5-36	ADSL Connection Status Monitor

| Use this command to viewing the ADSL line port connection status. |  |
| --- | --- |
| CLI(status)# port show <port-range> [phy ¦ channel] |  |
| Parameters | Task |
| <port-range> | Identify the port range of the system wish to view the status of line port. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| [phy ¦ channel] | Defines the status of physical status or channel status, show both if not specify. Type: Optional Valid values: phy, channel |

- Display the ADSL Connection Status

CLI(status)# port show 1.1.1

port: 1.1

oper status         : up

power state         : L0

line standard       : G.992.5 Annex A

[physical status]

item          US      DS

----------------  ------  ------

attainable rate    1321   30515  kbps

attenuation     0.0     0.0  dB

SNR margin     6.7    33.1  dB

output power    12.1     8.9  dBm

[channel status]

item          US      DS

----------------  ------  ------

Tx rate    1320    8189  kbps

interleave delay      10       9  ms

CRC block length     204      47  ms

INP symbol time    1.17    6.39  DMT symbol

## Configuring the SHDSL Line Port

Enter to the “config shdsl” sub-group directory to configure the relative profile on the SHDSL line port.

CLI# config shdsl

CLI(config shdsl)#

Table 5-37	SHDSL Port Interface Configuration

| Use this command to apply the PM alarm profile to specific SDSL line port. |  |
| --- | --- |
| CLI(config shdsl)# port set alarmprofile <port-range> <name> |  |
| Use this command to apply port connection profile to specific SDSL line port. |  |
| CLI(config shdsl)# port set profile <port-range> <name> |  |
| Use this command to viewing the SDSL line port operation status. |  |
| CLI(config shdsl)# port show <port-range> |  |
| Parameters | Task |
| <port-range> | Identify the port range of the system wish to apply the relevance profile of line port. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <name> | Defines the profile name; port connection profile or performance alarm profile. Type: Mandatory Valid values: The name of “port connection profile” or “performance alarm profile” |

## Monitoring the SHDSL Connection Status

Enter to the “status shdsl” group directory to monitoring the SHDSL line port connection status.

CLI# status shdsl

CLI(config shdsl)#

Table 5-38	SHDSL Connection Status Monitor

| Use this command to viewing the ADSL line port connection status. |  |
| --- | --- |
| CLI(status shdsl)# show <port-range> |  |
| Parameters | Task |
| <port-range> | Identify the port range of the system wish to view the status of line port. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |

## Subscriber Interface Administrating

Enter to the “config port” sub-group directory to administrating the ADSL line port.

Enter to the “config shdsl” sub-group directory to administrating the SHDSL line port.

CLI# config port

CLI(config port)#

CLI# config shdsl

CLI(config shdsl)#

Table 5-39	Subscriber Service Administration

| Use this command to activate the subscriber service of ADSL line port. |  |
| --- | --- |
| CLI(config port)# enable <port-range> |  |
| Use this command to deactivate the subscriber service of ADSL line port. |  |
| CLI(config port)# disable <port-range> |  |
| Use this command to activate the subscriber service of SHDSL line port. |  |
| CLI(config shdsl)# port enable <port-range> |  |
| Use this command to deactivate the subscriber service of SHDSL line port. |  |
| CLI(config shdsl)# port disable <port-range> |  |
| Parameters | Task |
| <port-range> | Identify the port range of the system wish to enable or disable the connection of ADSL line port. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |

- Administrating the connection of ADSL line port

CLI(config port)# enable 1.1.2

OK

CLI(config port)# disable 1.1.2

OK

CLI(SHDSL)# port enable 1.4.1

OK

CLI(SHDSL)# port disable 1.4.1

OK

This page is leave in blank for note or memo use

# Managing the Network Interface

There are two GE network interfaces, GE1 and GE2, for DAS4 series IP-DSLAM. GE1 is stated as the uplink GE port. All traffic from the xDSL line interface is forwarded to this interface by default. (i.e., the so-called port isolation) GE2 is stated as the subtended GE port, and it connects to other equipment and forward traffics to GE1 if none of LACP or RSTP is enabled

This chapter contains the following sections:

- Configuring the Subtending

- Configuring the RSTP

- Configuring the LACP

- Network Interface Administrating

## Configuring the Subtending

The subtending feature provides a mechanism to link a number of NE (IP-DSLAM) into one network trunk port. You can link up the DAS4192, DAS4672 IP-DSLAM chassis through their GE interface.

In a subtended configuration, each chassis is connected by its GE interface to the NC card of the chassis above it, in the subtending hierarchy, or, if it is at the top of the hierarchy, to the network trunk.

By default, the NC card of DAS4192 sets the GE1 to be Trunk port, and GE2 to be subtend port if subtending function is enable, the parent chassis considers subtended IP-DSLAM management and data traffics to be a transparent network.

Subtending Topology

There is one subtending topology that you can create with the NC card, a daisy-chain, where only one chassis is connected to the parent chassis at a time. Figure 6-2 shows a daisy chain subtending topology.

Figure 6-2	Daisy-Chain Topology for DAS4192

Enter to the “config” group directory to manage the subtending function.

CLI# config

CLI(config)#

Table 6-40	Subtending Configuration

| Use this command to enable, disable, or show the subtend status of system. |  |
| --- | --- |
| CLI(config)# subtend <option> |  |
| Parameters | Task |
| <option> | Configure the subtend function of system. Valid values: enable, disable, show |

|  | RSTP and LACP can not work with subtending at the same time. |
| --- | --- |

## Configuring the RSTP

The RSTP protocol smartly prevents the loop connection in your uplink networks. It improves the Spanning Tree Protocol (STP) by reducing the fail-over time whenever there is network topology change. The configuration of RSTP is divided to 2 parts. One is the system-wise configuration, which is described in the subsection “Bridge”. The other one is the port-specific configuration, which is described in the subsection “Port GE1/Port GE2”.

Port Roles and the RSTP Topology

The RSTP provides rapid convergence of the spanning tree by assigning port roles and by determining the active topology. The RSTP builds upon the IEEE 802.1d STP to select the switch with the highest switch priority (lowest numerical priority value) as the root bridge. DAS4192 IP-DSLAM with RSTP/STP assigns one of their Network interface roles to individual ports as below:

- Root port – Provides the best path (lowest cost) when the switch forwards packets to the root switch.

- Designated port – Connects to the designated switch, which incurs the lowest path cost when forwarding packets from that LAN to the root switch. The port through which the designated switch is attached to the LAN is called the designated port.

- Alternate port – An alternate port is a port blocked by receiving more BPDUs form another bridge.

- Backup port – A backup port is a port blocked by receiving more useful BPDUs from the same bridge it is on.

Figure 6-3	Spanning Tree Active Topology

Enter to the “rstp” sub-group directory to manage the RSTP function.

CLI# config rstp

CLI(config rstp)#

Table 6-41	RSTP Switch Configuration

| Use this command to enable the RSTP function. |  |  |
| --- | --- | --- |
| CLI(config rstp)# enable |  |  |
| Use this command to disable the RSTP function. |  |  |
| CLI(config rstp)# disable |  |  |
| Use this command to specify the version, RSTP or STP compatible. |  |  |
| CLI(config rstp)# set forceversion <protocol> |  |  |
| Use this command to configure the forwarding-delay for all RSTP instance. |  |  |
| CLI(config rstp)# set forwarddelay <delay-sec> |  |  |
| Use this command to configure the interval between the generations of configuration messages by the root switch by changing the hello time. |  |  |
| CLI(config rstp)# set hellowtime <hello-sec> |  |  |
| Use this command to configure the maximum-aging time for all RSTP instance. |  |  |
| CLI(config rstp)# set maxage <aging-sec> |  |  |
| Use this command to Configure the switch priority and make it more likely that the switch will be chosen as the root switch. |  |  |
| CLI(config rstp)# set priority <priority-value> |  |  |
| Use this command to configure the Tx hold count for all RSTP instance. |  |  |
| CLI(config rstp)# set txholdcount <count> |  |  |
| Use this command to viewing the RSTP bridge information. |  |  |
| CLI(config rstp)# show bridge |  |  |
| Parameters | Task |  |
| <protocol> | This specifies the Network interface to be acting in RSTP mode or STP-Compatible mode. Valid values: rstp, stp Default: rstp |  |
| <delay-sec> | The forward delay is the number of seconds a port waits before changing from its spanning-tree learning and listening states to the forwarding state. Default: 15 Valid values: 4 ~ 30 (Second) |  |
| <hello-sec> | The hello time is the interval between the generations of configuration messages by the root switch. These messages mean that the switch is alive. Default: 2 Valid values: 1 ~ 10 (Second) |  |
| <aging-sec> |  | The maximum-aging time is the number of seconds a switch waits without receiving spanning-tree configuration messages before attempting a reconfiguration. Default: 20 Valid values: 4 ~ 60 (Second) |
| <priority-value> |  | Configure the switch priority for an RSTP instance, the range is 0x0000 to 0xF000 in increments of 0x1000. The lower the number, the more likely the switch will be chosen as the root switch. Default: 0x8000 Valid values: 0x0000 ~ 0xF000. All other values are rejected. |
| <count> | This specifies the value used by the port Transmit state machine to limit the maximum transmission rate. Default: 3 Valid values: 0 ~ 10 |  |

Table 6-42 lists the RSTP port configuration parameters, those parameters allows you to define the port cost, port character, and administration status.

Table 6-42	RSTP Port Configuration

| Use this command to configure the path cost of port interface. |  |
| --- | --- |
| CLI(config rstp)# set uge cost <uge-range> <cost-value> |  |
| Use this command to disable the STP function of UGE port. |  |
| CLI(config rstp)# set uge disable <uge-range> |  |
| Use this command to configure the edge port instance. |  |
| CLI(config rstp)# set uge edge <uge-range> {false ¦ true} |  |
| Use this command to enable the STP function of UGE port. |  |
| CLI(config rstp)# set uge enable <uge-range> |  |
| Use this command to migrate the operation of RSTP and STP swap ability. |  |
| CLI(config rstp)# set uge migration <uge-range> {false ¦ true} |  |
| Use this command to configure the point-to-pint instance. |  |
| CLI(config rstp)# set uge p2p <uge-range> {true ¦ false ¦ auto} |  |
| Use this command to configure the port interface priority. |  |
| CLI(config rstp)# set uge priority <uge-range> <port-priority> |  |
| Use this command to viewing the GE Network interface RSTP information. |  |
| CLI(config rstp)# show uge |  |
| Parameters | Task |
| <uge-value> | This specifies the Network interface number (UGE port). Valid values: 1 (UGE port 1), 2 (UGE port 2) |
| <cost-value> | The path cost default values is derived from the media speed of an interface. If a loop occurs, the RSTP uses cost when selecting an interface to put in the forwarding state. You can assign lower cost values to interfaces that you want selected first and higher cost values that you want selected last. Default: 20000 Valid values: 1 ~ 200000000 |
| set uge edge < uge-range > {false ¦ true} | If you configure a port as an edge port on an RSTP, the edge port immediately transitions to the forwarding state. An edge port is the same as a Port Fast-enable port, and you should enable it only on ports that connect to a single end station. Default: false Valid values: false, true |
| set uge migration < uge-range > {false ¦ true} | This specifies the operation version with STP-Compatible to be able to run under RSTP if all region switches is run in RSTP mode. Default: false Valid values: false, true |
| set uge p2p < uge-range > {true ¦ false ¦ auto} | If you connect a port to another port through a point-to-point link and the local port becomes a designated port, it negotiates a rapid transition with the other port by using the proposal-agreement handshake to ensure a loop-free topology. Default: auto Valid values: true, false, auto |
| <port-priority> | If a loop occurs, the RSTP uses the port priority when selecting an interface to put into the forwarding state. You can assign higher priority values (lower numerical values) to interfaces that you want selected first and lower priority values (higher numerical values) that you want selected last. Default: 128 Valid values: 0, 16, 32, 48, 64, 80, 96, 112, 128, 144, 160, 176, 192, 208, 224, and 240. All other values are rejected. |

## Configuring the LACP

The LACP is defined in IEEE 802.3ad and enables DAS4192 IP-DSLAM to manage 2 ports GE Network interface that conform to the 802.3ad protocol. By using the LACP, the IP-DSLAM learns the identity of partners capable of supporting LACP and the capabilities of each port. It then groups similarly configured ports into a single logical link (aggregate port).

LACP Modes

The GE-Channel LACP mode for Network interface configuration contains both the active and passive LACP modes enable ports to negotiate with partner ports to a GE-Channel based on criteria such as port speed and, for Layer 2 GE-Channel, trunking state and VLAN numbers.

Ports can form a GE-Channel when they are in different LACP modes as long as the modes are compatible.

- A port in the active mode can form a GE-Channel with another port that is in the active or passive mode.

- A port in the passive mode cannot form a GE-Channel with another port that is also in the passive mode because neither port stats LACP negotiation.

Figure 6-4	Typical GE-Channel Configuration

LACP System Priority

You can configure the system priority for the GE-Channel that are enable for LACP, by changing this value from the default, you can affect how the software selects active and standby links.

LACP Port Priority

By default, all port uses the same port priority. If the local system has a lower value for the system priority and the system ID then the remote system, you can affect which of the standby links become active first by changing the port priority of LACP GE-Channel ports to a lower value than the default. The standby ports that have lower port numbers become active in the channel first.

Enter to the “la” sub-group directory to manage the LACP function.

CLI# config la

CLI(config la)#

Table 6-43	LACP Configuration

| Use this command to enable the static link aggregation or LACP. |  |
| --- | --- |
| CLI(config la)# enable <option> |  |
| Use this command to disable the static link aggregation or LACP. |  |
| CLI(config la)# disable |  |
| Use this command to configure the LACP group is active or passive. |  |
| CLI(config la)# set group-activity <group-id> <activity> |  |
| Use this command to define the UGE port which the LACP group is. |  |
| CLI(config la)# set group-member <uge-range> <group-id> |  |
| Use this command to configure the timeout parameter of the LACP group. |  |
| CLI(config la)# set group-timeout <group-id> <timeout> |  |
| Use this command to configure the UGE’s priority of LACP. |  |
| CLI(config la)# set port-priority <uge-range> <priority> |  |
| Use this command to configure the system’s priority of LACP. |  |
| CLI(config la)# set sys-priority <priority> |  |
| Use this command to viewing the LACP information. |  |
| CLI(config la)# show |  |
| Parameters | Task |
| <option> | Configure the aggregation mode to LACP or force to static link aggregation. Valid values: lacp ¦ static |
| <group-id> | This indicates the LACP group ID. Valid values: 0 ¦ 1 |
| <uge-range> | This indicates the UGE port. Valid values: 1 ¦ 2 |
| <timeout> | Configure the LACP timeout. Timeout = long means that BPDU send every 30 seconds. Timeout = short means that BPDU send every 1 second. Valid values: long ¦ short |
| <priority> | This indicates the UGE or system’s priority of LACP. Valid values: 0 ~ 65535 or 0x0000 ~ 0xFFFF |

## Network Interface Administrating

There are two GE network interfaces, GE1 and GE2, for DAS4 series IP-DSLAM. GE1 is stated as the uplink GE port. All traffic from the xDSL line interface is forwarded to this interface by default. (i.e., the so-called port isolation) GE2 is stated as the subtended GE port, and it connects to other equipment and forward traffics to GE1 if none of LACP or RSTP is enabled.

On the other hand, the NE supports the IEEE 802.1Q VLAN forwarding function. The operator can set the xDSL subscriber ports as well as the GE ports to only forward either tagged traffic or untagged traffic. This section depicts the commands to set the IEEE 802.1Q VLAN forwarding function on GE ports. As to the setting on the xDSL subscriber ports, please refer to Section “Defining the Line Card Operation Mode” for the configuration of xDSL subscriber port to either only forward either tagged traffic or untagged traffic on a per-LC basis.

Table 6-44 depicts the NE behavior with the follwoing configurations.

- NC with various Tagged mode parameters.

- ADSL LC with various Tagged mode and VTP parameters.

It is noted that the run-time status of LC may be different to its corresponding configuration. In this case, the behavior of the NE is per the run-time status of NE instead of their configuration. To describe the NE behavior, the following notations are adopted in Table 6-44.

- QS represents the service VLAN-tag and its VLAN-ID value is provided by the NE.

- QS (CPE) represents the service VLAN-tag and the notation (CPE) indicates that its VLAN-ID value is provided by the CPE (or the subscriber’s PC behind the CPE).

- Q (CPE) represents the 802.1Q VLAN-tag.

- QC (CPE) represents the customer VLAN-tag and the notation (CPE) indicates that its VLAN-ID value is provided by the CPE (or the subscriber’s PC behind the CPE).

|  | Please refer to Section “Verifying Current Software and Hardware Versions” for the run-time status of the tagged mode on NC and LC. |
| --- | --- |

|  | The ADSL LC needs to be reset to perform the expected system behavior as depicted in Table 6-44 whenever its run-time status changes. |
| --- | --- |

|  | The NC needs to be reset to perform the expected system behavior as depicted in Table 6-44 whenever its configured tagged mode changes. |
| --- | --- |

|  | Whenever the GE2 is set as subtended port and the NC is set as “tagged-only” mode, in order to make the NE forward the VLAN-specific traffic between GE1 and GE2, the operator needs to manually set GE1 and GE2 as the member ports of VLANs in interest. Please refer Table 6-45 for the “subtend-vid” related CLI commands. |
| --- | --- |

Table 6-44	The NE behavior when configuring NC and ADSL LC with various Tagged modes and VTP parameters.

| NC Setting | ADSL LC setting |  | ADSL LC Run-Time Status |  | Expected NE behavior |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | VLAN-tagging Status of Egress Traffic |  | Acceptable Ingress Traffic |  |  |
| Tagged mode | Tagged mode | VTP | Tagged mode | VTP | On the GE port | On the ADSL line | On the GE port |  | On the ADSL line |
| Tagged | Tagged | Enabled | Tagged | Enabled | QS (CPE) | Q (CPE) | Tagged | Tagged |  |
|  |  | Disabled | Tagged | Disabled | QS+QC (CPE) | Q (CPE) | Tagged | Tagged |  |
|  | Untagged | Enabled | Untagged | Disabled | QS | Untagged | Tagged | Untagged |  |
|  |  | Disabled | Untagged | Disabled | QS | Untagged | Tagged | Untagged |  |
| Untagged | Tagged | Enabled | Untagged | Disabled | Untagged | Untagged | Untagged | Untagged |  |
|  |  | Disabled | Untagged | Disabled | Untagged | Untagged | Untagged | Untagged |  |
|  | Untagged | Enabled | Untagged | Disabled | Untagged | Untagged | Untagged | Untagged |  |
|  |  | Disabled | Untagged | Disabled | Untagged | Untagged | Untagged | Untagged |  |

|  | It is noted that the NE will drop the tagged Ethernet frames of VLAN-ID not configured by the VC-to-VLAN setting (see Section “VC-to-VLAN Connection Management”) in the following case. NC tagged mode = Tagged LC tagged mode Run-Time Status = Tagged LC VTP Run-Time Status = Enabled |
| --- | --- |

|  | The tagged mode (run-time) indicates the operational status of tagged mode. Tagged-only: LC (or NC) only forwards the tagged Ethernet frame and drops the untagged Ethernet frame. Untagged-only: LC (or NC) only forwards the untagged Ethernet frame and drops the tagged Ethernet frame. It is noted that the value of configured Tagged mode and its Run-Time Status may be different. Please refer to Table 6-44 for the NE behavior when configuring NC and ADSL LC with various Tagged mode and VTP parameters. |
| --- | --- |

Enter to the “config nc” sub-group directory to manege the GE network interface.

CLI# config nc

CLI(config nc)#

Table 6-45	Network Services Administration

| Use this command to add both of the GE ports as member-ports of vlan <vid>. (See the note at the end of this section to learn the usage of this command) |  |
| --- | --- |
| CLI(config nc)#add subtend-vid <vid> |  |
| Use this command to configure both of the GE ports to be not member-ports of any VLAN. |  |
| CLI(config nc)#clear subtend-vid |  |
| Use this command to configure both of the GE ports to be not member-ports of the VLAN specified by <vid>. |  |
| CLI(config nc)#del subtend-vid <vid> |  |
| Use this command to modify the planning NC card type. |  |
| CLI(config nc)# set planned-type <nc-id> {none ¦ cpu} |  |
| Use this command to modify the negotiation mode of GE port. |  |
| CLI(config nc)# set autoneg <uge-id> {off ¦ on} |  |
| Use this command to configure the both of the GE ports to operate either in the “tagged-only” or “untagged-only” mode. |  |
| CLI(config nc)# set tagged-mode { tagged-only ¦ untagged-only} |  |
| Use this command to set GE2 as a subtendded port (by “enable”) or an uplink port. (by “disable”) |  |
| CLI(config nc)# set subtend { disabled ¦ enabled} |  |
| Use this command to activate the network service of specific UGE port. |  |
| CLI(config nc)# enable <uge-id> |  |
| Use this command to deactivate the network service of specific UGE port. |  |
| CLI(config nc)# disable <uge-id> |  |
| Use this command to display the UGE interface status. |  |
| CLI(config nc)# show |  |
| Parameters | Task |
| <nc-id> | Identify the slot range of the NC card Type: Mandatory Valid values: 1 ~ 2 (value = 2 is only on DAS4672) |
| {none ¦ cpu} | Identify the NC type. |
| <vid> | Identify the vlan id of the VLAN which the GE ports belong to. |
| <uge-id> | This specifies the Network interface number (UGE port). Valid values: 1 (UGE port 1), 2 (UGE port 2) |
| {off ¦ on} | Identify the auto negotiation mode of specified UGE port. Type: Mandatory Valid values: off ¦ on |
| { disabled ¦ enabled} | Specify the GE2 as either a subtendded port or an uplink port. Enable: GE2 works as a subtend port of the NC Disable: GE2 works as an uplink port of the NC. Type: Mandatory Valid values: disabled ¦ enabled |
| { tagged-only ¦ untagged-only} | This specifies both of the GE ports to operate either in the “tagged-only” or “untagged-only” mode. Type: Mandatory Valid values: tagged-only ¦ untagged-only |

# Managing the Connection Services

This chapter describes how to manage the system connection services.

This chapter contains the following sections:

- VC-to-VLAN Connection Management

- Configuring the Multicast Services

- Monitoring the VC-to-VLAN Connection State

- Configuring the System Services

- Managing the Subscriber Access Services

- Configuring the Access Control List

- Configuring the VLAN MAC Limitation

- Monitoring the VLAN Group

- Monitoring the IGMP Proxy Information

- Monitoring the Subscriber MAC

- Filtering the NetBIOS and NetBEUI

## VC-to-VLAN Connection Management

The VC-to-VLAN setting can easily define the multiple to one or one to one mapping; you can group different PVCs to a single VLAN ID as well as single PVC to one VLAN mapping. Figure 7-5 illustrates the basic principle for VLAN assignment in the DAS4 Series IP-DSLAM. As shown in Figure 7-5, the NE forwards five data flows, A~E, which may be either owned by the same subscriber or by different subscribers. It is noted that these data flows are conveyed in five individual ATM PVCs, and they are grouped into 3 individual VLANs.

|  | The NE supports up to 8 PVCs per xDSL port . The NE supports up to 4094 VLANs per system . |
| --- | --- |

Figure 7-5	VC-to-VLAN Mapping Illustrate

According to IETF RFC2684, an IP packet is encapsulated in either bridged mode or routed mode. The VC-to-VLAN settings are similar but not the same in these two encapsulation modes. This section depicts their configuration procedures together.

|  | The VC-to-VLAN configuration procedures are the same to both the ADSL port and SHDSL port. |
| --- | --- |

### Configuring a VC-to-VLAN Connection for the VC of RFC2684 Bridged Mode

Enter to the “config ucast” sub-group directory to configure the bridged services of unicast connections.

CLI# config ucast

CLI(config ucast)#

Table 7-46	Bridged Services Configuration

| Use this command to create a new VC-to-VLAN connection on specific of xDSL line port. |  |  |
| --- | --- | --- |
| CLI(config ucast)# add vcvlan <port-range> <vpi> <vci> |  |  |
| Use this command to remove the VC-to-VLAN connection on specific of xDSL line port. |  |  |
| CLI(config ucast)# del vcvlan <port-range> <vpi> <vci> |  |  |
| Use this command to activate the VC-to-VLAN service on specific of xDSL line port. |  |  |
| CLI(config ucast)# enable vcvlan <port-range> <vpi> <vci> |  |  |
| Use this command to deactivate the VC-to-VLAN service on specific of xDSL line port. |  |  |
| CLI(config ucast)# disable vcvlan <port-range> <vpi> <vci> |  |  |
| Use this command to change the bridged VC-to-VLAN parameters on specific of xDSL line port. |  |  |
| CLI(config ucast)# set vcvlan <port-range> <vpi> <vci> <802_1p> <iptraffic-profile> bridged <vlan-id> <mac-limit> |  |  |
| Parameters | Task |  |
| <port-range> | Identify the port range of the system wish to configure in bridged services. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |  |
| <vpi> |  | Defines the VPI (Virtual Path Identifier) value. Type: Mandatory Valid values: 0 ~ 255 |
| <vci> |  | Defines the VCI (Virtual Channel Identifier) value. Type: Mandatory Valid values: 1 ~ 65535 (1 ~ 31 are reserved) |
| <802_1p> |  | Defines the tagging of VLAN 802.1p priority of egress switch fabric on specific of VC-to-VLAN connection. Type: Mandatory Default value: 0 Valid values: 0 ~ 7 (low ~ high) |
| <iptraffic-profile> |  | Defines the created IP traffic profile name. Type: Mandatory Valid values: The name of “ip traffic profile” |
| <vlan-id> |  | Defines the VLAN ID to be assign on specific of VC-to-VLAN connection. Type: Mandatory Default value: 1 Valid values: 1 ~ 4093 |
| <mac-limit> |  | Defines the limit of MAC address learning from specific bridged service per xDSL line port. Each xDSL line port allow maximum of 8 MAC address learning in total of VC-to-VLAN usage. Type: Mandatory Default value: 1 Valid values: 1 ~ 8 |

### Configuring a VC-to-VLAN Connection for the VC of RFC2684 Routed Mode

|  | According to IETF RFC2684, an IP packet is encapsulated in either bridged mode or routed mode. The VC-to-VLAN settings are similar but not the same in these two encapsulation modes. |
| --- | --- |

Enter to the “config ucast” sub-group directory to configure the routed services of unicast connection. (Next hop up to 4 sets)

CLI# config ucast

CLI(config ucast)#

Table 7-47	Routed Services Configuration

| Use this command to create a new VC-to-VLAN connection on specific of xDSL line port. |  |
| --- | --- |
| CLI(config ucast)# add vcvlan <port-range> <vpi> <vci> |  |
| Use this command to create a new ISP (Internet Service Provider) connection. |  |
| CLI(config ucast)# add nexthop <ispname> <ip-addr> <vid> |  |
| Use this command to remove the VC-to-VLAN connection on specific of xDSL line port. |  |
| CLI(config ucast)# del vcvlan <port-range> <vpi> <vci> |  |
| Use this command to remove the ISP connection. |  |
| CLI(config ucast)# del nexthop <ispname> |  |
| Use this command to activate the VC-to-VLAN service on specific of xDSL line port. |  |
| CLI(config ucast)# enable vcvlan <port-range> <vpi> <vci> |  |
| Use this command to deactivate the VC-to-VLAN service on specific of xDSL line port. |  |
| CLI(config ucast)# disable vcvlan <port-range> <vpi> <vci> |  |
| Use this command to change the routed VC-to-VLAN parameters on specific of xDSL line port. |  |
| CLI(config ucast)# set vcvlan <port-range> <vpi> <vci> <802_1p> <iptraffic-profile> routed <ispname> |  |
| Parameters | Task |
| <port-range> | Identify the port range of the system wish to configure in routed services. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <vpi> | Defines the VPI (Virtual Path Identifier) value. Type: Mandatory Valid values: 0 ~ 255 |
| <vci> | Defines the VCI (Virtual Channel Identifier) value. Type: Mandatory Valid values: 1 ~ 65535 (1 ~ 31 are reserved when VPI equal 0) |
| <ispname> | Defines the ISP name for routed service. Type: Mandatory Valid values: String of up to 32 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |
| <ip-addr> | Defines IP address of the ISP server. Type: Mandatory Valid values: 0.0.0.0 ~ 255.255.255.255 |
| <vid> | Defines the VLAN ID to be assign on specific of VC-to-VLAN connection. Type: Mandatory Default value: 1 Valid values: 1 ~ 4093 |
| <802_1p> | Defines the tagging of VLAN 802.1p priority of egress switch fabric on specific of VC-to-VLAN connection. Type: Mandatory Default value: 0 Valid values: 0 ~ 7 (low ~ high) |
| <iptraffic-profile> | Defines the created IP traffic profile name. Type: Mandatory Valid values: The name of “ip traffic profile” |

## Configuring the Multicast Services

Whenever the subscriber clicks his remote controller to watch a TV channel transmitted via the ADSL line, the set-top-box sends the corresponding IGMP report packet. The NE will forward IGMP packet if its multicast IP hits the associated multicast service profile. Otherwise, the NE drops the IGMP packet. As a result, the subscriber is restricted to watch the TV programs that he booked.

Enter to the “config mcau” sub-group directory to configure the multicast connection.

CLI# config mcau

CLI(config mcau)#

Table 7-48	Multicast Services Configuration

| Use this command to remove the multicast service on specific of xDSL line port. |  |
| --- | --- |
| CLI(config mcau)# del <port-range> |  |
| Use this command to activate the multicast service on specific of xDSL line port. |  |
| CLI(config mcau)# enable <port-range> |  |
| Use this command to deactivate the multicast service on specific of xDSL line port. |  |
| CLI(config mcau)# disable <port-range> |  |
| Use this command to change the multicast service with desired parameters on specific of xDSL line port. |  |
| CLI(config mcau)# set <port-range> <vpi> <vci> <vlan-id> <channel-limit> <mservice-name> |  |
| Use this command to show the multicast service with desired parameters on specific of xDSL line port. |  |
| CLI(config mcau)# show <port-range> |  |
| Parameters | Task |
| <port-range> | Identify the port range of the system wish to configure in multicast services. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <vpi> | Defines the VPI (Virtual Path Identifier) value. Type: Mandatory Default value: 8 Valid values: 0 ~ 255 |
| <vci> | Defines the VCI (Virtual Channel Identifier) value. Type: Mandatory Default value: 35 Valid values: 1 ~ 65535 (1 ~ 31 are reserved) |
| <channel-limit> | Defines the limit of concurrent multicast channel transmission on specific of VC-to-VLAN connection. Type: Mandatory Default value: 1 Valid values: 1 ~ 5 |
| <vlan-id> | Defines the VLAN ID to be assign on specific of VC-to-VLAN multicast connection Type: Mandatory Default value: 1 Valid values: 1 ~ 4093 |
| <mservice-name> | This specifies the multicast service profile name Type: Mandatory Valid values: String of up to 32 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |

- Display the multicast connection status

channel

port-id   status    VPI/VCI   VLAN  limit          service-profile

-------  --------  ---------  ----  -----  --------------------------------

1. 1. 1   enabled    8/   35   300      3                         program-2

## Monitoring the VC-to-VLAN Connection Status

Enter to the “config ucast” sub-group directory to monitoring the unicast connection status.

CLI# config ucast

CLI(config ucast)#

Table 7-49	Unicast Connection Status Monitor

| Use this command to viewing the VC-to-VLAN connection of specific xDSL line port. |  |
| --- | --- |
| CLI(config ucast)# show vcvlan <port-range> |  |
| Use this command to viewing the status of ISP server use for routed services. |  |
| CLI(config ucast)# show nexthop |  |
| Use this command to viewing the launched service type of specific xDSL line port. |  |
| CLI(config ucast)# show servicetype <port-range> |  |
| Parameters | Task |
| <port-range> | Identify the port range of the system wish to view the VC-to-VLAN connection. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |

- Display the unicast connection status

CLI(config ucast)# show vcvlan 1.1.1-1.1.2

Port  1. 1

# VPI  VCI  IP-traffic prof  VLAN 1p MAC RFC2684     next-hop      admin   oper

- --- ----- ---------------- ---- -- --- ------- ---------------- -------- ----

1   0    32             test  100  0   1 bridged                   enabled   up

Port  1. 2

# VPI  VCI  IP-traffic prof  VLAN 1p MAC RFC2684     next-hop      admin   oper

- --- ----- ---------------- ---- -- --- ------- ---------------- -------- ----

1   0    32             test  100  0   1  routed            xinet  enabled   up

CLI(UNICAST)# show nexthop

next-hop name             next-hop IP          MAC        VLAN  status

-------------------------------- --------------- ----------------- ---- --------

xinet   100.168.10.10 00:90:96:7c:31:0a  100 inactive

## Configuring the System Services

The service connection contains parameter to define on each service types.

### MAC Aging for Bridged Services

The MAC aging time sets the lifetime for the learned MAC address. A specific MAC address will be dropped when aging out until it gets learning again. Disable this function will keep the learned MAC addresses permanent in the learning table.

Enter to the “config bridge” sub-group directory to configure the system bridging and monitor its status.

CLI# config bridge

CLI(config bridge)#

Table 7-50	Bridged Services Setting

| Use this command to configure the bridging service aging time. |  |
| --- | --- |
| CLI(config bridge)# set aging-time <sec> |  |
| Use this command to viewing the bridging aging time status. |  |
| CLI(config bridge)# show |  |
| Parameters | Task |
| <sec> | Defines the bridging aging time in second. Type: Mandatory Valid values: 10 ~ 1000 (sec.) Default value: 300 (sec.) |

- Display the bridging status

CLI(config bridge)# show

MAC aging time: 5 min 0 sec (300 sec)

### DHCP Broadcast Control

Users can set the DHCP broadcast packet rate limit and set the action applied to the out-of-profile traffic.

Enter to the “config dhcp” sub-group directory to configure the DHCP broadcast control.

CLI# config dhcp

CLI(config dhcp)#

Table 7-51	DHCP Broadcast Control

| Use this command to define the action when DHCP packet over rate limit. |  |
| --- | --- |
| CLI(config dhcp)# set bc <rate-limit> <action> |  |
| Use this command to disable the DHCP broadcast control |  |
| CLI(config dhcp)# disable bc |  |
| Use this command to enable the DHCP broadcast control |  |
| CLI(config dhcp)# enable bc |  |
| Use this command to display the DHCP broadcast control information |  |
| CLI(config dhcp)# show |  |
| Parameters | Task |
| <action> | Defines the DHCP broadcast packets overflow action. Type: Mandatory Valid values: none, drop, alarm, both Default value: none |
| <rate-limit> | Defines the DHCP broadcast packets per second Type: Mandatory Valid values: 1 ~ 100000 (pkts/sec.) Default value: 100 (pkts/sec.) |

- Configure the DHCP broadcast control

CLI(config dhcp)# enable bc

OK

CLI(config dhcp)# set bc 1000 both

OK

CLI(config dhcp)# show

option82                      : disabled

broadcast control             : enabled

broadcast rate limit          : 1000 pkts/sec

broadcast action              : both (drop & alarm)

stateful level                : none - show nothing

relay                         : disabled

relay server                  : no server exists

|  | When the action is set to be either “Send alarm” and “Drop packet and send alarm”, the NE will launch SNMP traps to the SNMP trap managers as specified in the section Chap 3. |
| --- | --- |

### DHCP Relay Setting

The DHCP relay intercepts the DHCP request packets from subscriber interface and forwards them to the specified DHCP server. In the opposite direction, the DHCP relay transfers the DHCP reply packets from DHCP server to the specified xDSL subscriber.

Enter to the “config dhcp” sub-group directory to configure the DHCP relay.

CLI# config dhcp

CLI(config dhcp)#

Table 7-52	DHCP Relay Setting

| Use this command to define the DHCP relay server and its correspondent VLAN ID. |  |
| --- | --- |
| CLI(config dhcp)# add relay-server <server-ip> |  |
| Use this command to remove the DHCP relay server IP |  |
| CLI(config dhcp)# del relay-server <server-ip> |  |
| Use this command to enable the DHCP relay functionality. |  |
| CLI(config dhcp)# enable relay |  |
| Use this command to disable the DHCP relay functionality. |  |
| CLI(config dhcp)# disable relay |  |
| Use this command to configure the stateful mode of DHCP packets. |  |
| CLI(config dhcp)# set stateful <level> |  |
| Use this command to viewing the DHCP relay status. |  |
| CLI(config dhcp)# show |  |
| Parameters | Task |
| <server-ip> | This specifies the IP address of DHCP server. Type: Mandatory Valid values: Any valid class A/B/C address Default value: None |
| <level > | Define the print out mode when system receives DHCP packets. Type: Mandatory Valid values: none ¦ flow ¦ pf ¦ all None – show nothing Flow – show flow state only Pf – show packet content and flow state All – all content with hexadecimal data |

### DHCP Relay Option 82 Setting

Enter to the “config dhcp” sub-group directory to configure the DHCP relay option 82.

CLI# config dhcp

CLI(config dhcp)#

Table 7-53	DHCP Relay Option 82 Setting

Use this command to enable the DHCP relay option 82 functionality.

CLI(config dhcp)# enable op82

Use this command to disable the DHCP relay option 82 functionality.

CLI(config dhcp)# disable op82

|  | The setting of DHCP option 82 contents is performed by configuring the xDSL Port Agent ID |
| --- | --- |

### IGMP Snooping/Proxy Setting

Follow the commands to configure the IGMP snooping and IGMP proxy related parameters.

Enter to the “config igmp” sub-group directory to configure the IGMP snooping or proxy.

CLI# config igmp

CLI(config igmp)#

Table 7-54	IGMP Snooping/Proxy Setting

| Use this command to activate the IGMP snooping function for multicast services. |  |
| --- | --- |
| CLI(config igmp)# enable snooping |  |
| Use this command to activate the IGMP proxy function for multicast services. |  |
| CLI(config igmp)# enable proxy |  |
| Use this command to deactivate both the IGMP snooping and proxy function for multicast services. |  |
| CLI(config igmp)# disable |  |
| Use this command to configure the aging time of IGMP Snooping. |  |
| CLI(config igmp)# set snooping aging-time <time> |  |
| Use this command to viewing the IGMP status. |  |
| CLI(config igmp)# show |  |
| Use this command to enable the IGMP proxy processes the leave message from subscriber link. |  |
| CLI(config igmp)# set proxy immediated-leave {disabled ¦ enabled} |  |
| Use this command to configure the IGMP proxy response time against the subscriber link. |  |
| CLI(config igmp)# set proxy response-time <sec> |  |
| Use this command to configure the IGMP proxy retry counter. |  |
| CLI(config igmp)# set proxy retries <count> |  |
| Use this command to configure the stateful mode of IGMP packets. |  |
| CLI(config igmp)# set stateful <level> |  |
| Parameters | Task |
| <time> | Defines the IGMP snooping aging time in second. Type: Mandatory Valid values: 30 ~ 3600 (sec.) Default value: 300 (sec.) |
| <sec> | Defines the time period waiting for subscriber response the IGMP message. Type: Mandatory Valid values: 1 ~ 30 (sec.) Default value: 30 (sec.) |
| <count> | Defines the retry counting for STB response the IGMP message, if the system did not receive IGMP message from subscriber edge, system will treat as ‘leave’ hence will stop the multicast stream to the particular link. Type: Mandatory Valid values: 1 ~ 5 Default value: 3 (count.) |
| <level> | Define the print out mode when system receives IGMP packets. Type: Mandatory Valid values: none ¦ flow ¦ msg None – show nothing Flow – show flow state only Msg – show packet flag and error message |

- Display the IGMP snooping/proxy status

CLI(config igmp)# show

IGMP proxy

status               : enabled

immediated leave     : enabled

retries              : 5 (times)

response interval    : 25 (seconds)

IGMP snooping

status               : disabled

aging time           : 300 (seconds)

Stateful

level                : "none: show nothing"

### Configuring the PPPoE Suboption

PPPoE sub-option has similar mechanism as DHCP option 82. The NE can insert Circuit ID and Remote ID in all upstream PPPoE discovery stage packets, i.e. the PADI, PADR and upstream PADT packets.

|  | The setting of PPPoE sub-option contents is performed by configuring the xDSL Port Agent ID |
| --- | --- |

Enter to the “config pppoe” sub-group directory to configure the PPPoE suboption.

CLI# config pppoe

CLI(config pppoe)#

Table 7-55	PPPoE Suboption Setting

| Use this command to enable the PPPoE suboption function. |  |
| --- | --- |
| CLI(config pppoe)# enable suboption |  |
| Use this command to disable the PPPoE suboption function. |  |
| CLI(config pppoe)# disable suboption |  |
| Use this command to configure the stateful mode of PPPoE packets. |  |
| CLI(config pppoe)# set stateful <level> |  |
| Use this command to display the PPPoE suboption and stateful information. |  |
| CLI(config pppoe)# show |  |
| Parameters | Task |
| <level> | Define the print out mode when system receives PPPoE packets. Type: Mandatory Valid values: none ¦ flow ¦ msg None – show nothing Flow – show flow state only Msg – show flow message |

- Configure the PPPoE suboption

suboption         : disabled

stateful level    : none

## Managing the Subscriber Access Services

The system supports the so-called “service type control” function to restrict the type of traffic to be forwarded on the PVC of individual subscriber. In RFC2684 routed mode, the following service type is supported.

- Static IP

In RFC2684 bridged mode, the following three service types are supported.

- PPPoE

- DHCP

- Static IP

Enter to the “config ucast” sub-group directory to manage the access service control.

CLI# config ucast

CLI(config ucast)#

Table 7-56	Access Services Configuration

| Use this command to define the access service of particular PVC. |  |
| --- | --- |
| CLI(config ucast)# set servicetype <port-range> <vpi> <vci> <mode> |  |
| Use this command to define the authentic IP in static IP access mode. |  |
| CLI(config ucast)# set servicetype staticip <port-range> <vpi> <vci> <staticipbase> <iplimit> |  |
| Use this command to viewing the access service status in specific Subscriber port interface. |  |
| CLI(config ucast)# show servicetype <port-range> |  |
| Parameters | Task |
| <port-range> | Identify the port range of the system line card Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <vpi> | Defines the VPI (Virtual Path Identifier) value. Type: Mandatory Valid values: 0 ~ 255 |
| <vci> | Defines the VCI (Virtual Channel Identifier) value. Type: Mandatory Valid values: 1 ~ 65535 (1 ~ 31 are reserved) |
| <staticipbase> | This specifies the static IP address. Type: Mandatory Valid values: Any valid class A/B/C address Default value: None |
| <iplimit> | This specifies the maximum IP counter in PPPoE/DHCP, and continuous IP count for Static IP. Type: Mandatory Valid values: 1 ~ 8 |
| <mode> | This specifies the authentic of access service mode in particular PVC. Type: Mandatory Valid values: pppoe, dhcp, staticip |

- Configure the static IP access service

CLI(config ucast)# set servicetype 1.1.1 0 35 staticip

Port 1. 1. 1:OK

CLI(config ucast)# set servicetypestaticip  1.1.1 0 35 168.95.1.1 1

OK

CLI(config ucast)# show servicetype 1.1.1

Port  1. 1

PVC     sevice-type  static-IP base   IP counter

---------  -----------  ---------------  ----------

0/   32    static IP       168.95.1.1           1

|  | Enabling the Service Type Control makes the NE to provide the IP/MAC anti spoofing function. In the case that the subscriber acquires his IP address dynamically via PPPoE or DHCP, the NE will block the subscriber’s traffic before a valid IP address assignment. Once the subscriber possesses a valid dynamic or static IP, the NE will just forward the packet of valid source IP/MAC addresses. In other words, the NE drops the subscriber’s traffic of invalid source IP/MAC addresses. |
| --- | --- |

## Configuring the Access Control List

The access control list (ACLs) can block a single host’s access from Subscriber Interface to an IP-DSLAM, it provide filtering to a variety of MAC resources.

ACLs are a series of sequentially processed permit or deny statements that can be used to filter source MAC addresses from Subscriber Interface (PVC base), use permit or deny solo statements at each subscriber PVC connection, permit identify the specifics MAC address to be access while deny all others, same algorithm for deny statement, interlace statements are not support, if so, only the last control statement will be read.

Enter to the “config” sub-group directory to manage the ACL statement.

CLI# config acl

CLI(config acl)#

Table 7-57	Access Control List Configuration

| Use this command to generate a new ACLs rule. |  |
| --- | --- |
| CLI(config acl)# add <port-range> <vpi> <vci> <mac> {permit ¦ deny} |  |
| Use this command to remove a ACLs rule. |  |
| CLI(config acl)# del <port-range> <vpi> <vci> <mac> |  |
| Use this command to viewing the ACLs statement in specific Subscriber port interface. |  |
| CLI(config acl)# show <port-range> |  |
| Parameters | Task |
| <port-range> | Identify the port range of the system line card Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <vpi> | Defines the VPI (Virtual Path Identifier) value. Type: Mandatory Valid values: 0 ~ 255 |
| <vci> | Defines the VCI (Virtual Channel Identifier) value. Type: Mandatory Valid values: 1 ~ 65535 (1 ~ 31 are reserved) |
| <mac> | This specifies the MAC address wish to act from subscriber network. Type: Mandatory Valid values: xx:xx:xx:xx:xx:xx (x valid from 0 ~ 9, a ~ f) |
| {permit ¦ deny} | Defines the ACL action of specific MAC address in the PVC connection. Type: Mandatory Valid values: permit, deny |

## Configuring the VLAN MAC Limitation

To limit the number of source MAC address learned in a specific VLAN, the users can enable the MAC limiting function and configure the upper limit of allowed MAC for a specific VLAN.

Enter to the “vlan-mac-limit” sub-group directory to manage the VLAN MAC limitation.

CLI# config vlan-mac-limit

CLI(config vlan-mac-limit)#

Table 7-58	VLAN MAC Limiting Configuration

| Use this command to enable or disable the MAC limiting of specific VLAN ID. |  |
| --- | --- |
| CLI(config vlan-mac-limit)# set <vid> {on ¦ off} |  |
| Use this command to define the MAC number of specific VLAN ID. |  |
| CLI(config vlan-mac-limit)# set <vid> <maclimit> |  |
| Use this command to display the status of VLAN MAC limiting. |  |
| CLI(config vlan-mac-limit)# show [<vid>] |  |
| Parameters | Task |
| <vid> | This specifies the VLAN ID of system. Type: Mandatory Valid values: 1 ~ 4094 |
| {on ¦ off} | This enable/disable the VLAN MAC limiting function of specific VLAN ID. Type: Mandatory |
| <maclimit> | This defines the MAC number of specific VLAN ID to be accepted Type: Mandatory Valid values: 5 ~ 50000 Default values: 43 |

## Monitoring the VLAN Group

The monitoring of subscriber VLAN group function allows you to summarize the VLAN group (ID) with associated xDSL line port in DAS4192 IP-DSLAM.

Enter to the “status vlan” sub-group directory to display VLAN ID with associated xDSL line port.

CLI# status vlan

CLI(status vlan)#

Table 7-59	Viewing Subscriber VLAN Group

| Use this command to viewing the VLAN group (ID) with associated xDSL line port. |  |
| --- | --- |
| CLI(status vlan)# show vlan-id <vid> |  |
| Use this command to viewing the status of uge with associated xDSL line port. |  |
| CLI(status vlan)# show uge <uge-id> |  |
| Parameters | Task |
| <vid> | This specifies the VLAN ID of correspond xDSL line port. Type: Mandatory Valid values: 1~ 4094 |
| <uge-id> | This specifies the uge id of correspond xDSL line port. Type: Mandatory Valid values: 1~2 |

- Display the subscriber VLAN group

CLI(status vlan)# show vlan-id 100

VLAN [100] egress ports on each LC

LC 1: 37

LC 2: none

LC 3: none

LC 4: 24,26,37,48

## Monitoring the IGMP Snoopy/Proxy Information

Enter to the “status igmpproxy” sub-group directory to display the IGMP snoop and proxy information with associated xDSL line port.

CLI# status igmp

CLI(status igmp)#

Table 7-60	Viewing IGMP Proxy Information

| Use this command to viewing the IGMP group (IP) with associated xDSL line port. |  |
| --- | --- |
| CLI(status igmp)# group show [<group-ip>] |  |
| Use this command to show IGMP member information on this port. |  |
| CLI(status igmp)# member show <port-id> |  |
| Parameters | Task |
| <group-ip> | Defines class D IP addressing for multicast channel Type: Mandatory Valid values: 224.0.1.0 ~ 239.255.255.255 |
| <port-id> | Identify the port ID of the system line card Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |

## Monitoring the Subscriber MAC

The FDB (filtering Database) of DAS4192 system stored the MAC addresses learning from the associate ATM VC at bridged mode.

Enter to the “status fdb” sub-group directory to view learning MAC addresses from the associate ATM VC.

CLI# status fdb

CLI(status fdb)#

Table 7-61	VC MAC Learning Table

| Use this command to displays current list of specified xDSL line port learning MAC addresses. |  |
| --- | --- |
| CLI(status fdb)# show port <port-id> |  |
| Use this command to displays current list of specified xDSL line port learning MAC addresses. |  |
| CLI(status fdb)# show spoofed |  |
| Use this command to displays current list of specified xDSL line port learning MAC addresses. |  |
| CLI(status fdb)# show mac <mac-addr> |  |
| Parameters | Task |
| <port-id> | Identify the port id of the system wish to display current list of learning MAC addresses from their remote network. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <mac-addr> | Indicate the target MAC address. Type: Mandatory Valid values: Valid MAC addresses form. (for example: 00:1F:AA:19:78:03) |

- Display the learning MAC addresses from the xDSL line port

CLI(status fdb)# show port 1.1.2

Port 1.1.2:

ID  VPI  VCI     MAC Address    Status

--- --- ----- ----------------- ------

0   8    35 00:00:e2:6b:bf:f0     LU

## Filtering the NetBIOS and NetBEUI

The NE allows the operator to configure to forward or drop the name server protocol (NetBIOS and NetBEUI) traffics received on the subscriber interfaces and network interfaces.

Enter to the “config filter” sub-group directory to define the NetBIOS and NetBEUI filtering function.

CLI# config filter

CLI(config filter)#

Table 7-62	NetBIOS and NetBEUI Filter

| Use this command to define the action NetBIOS and NetBEUI filtering. |  |
| --- | --- |
| CLI(config filter)# netbios <netbios-action> |  |
| Use this command to displays current setting of NetBIOS and NetBRUI filtering. |  |
| CLI(config filter)# show |  |
| Parameters | Task |
| <netbios-action> | Identify the NetBIOS and NetBEUI filtering action. Type: Mandatory Valid values: drop, forward |

## Configuring the MAC Spoofed

The DAS4192 support a function to prevent duplicated MAC address from xDSL user, it maybe opportunist or hacker.

When there are two or more duplicated MAC addresses from user side learned by DAS4192 at the same time, the default action of DAS4192 is allow the first MAC address and block all the others.

In order to prevent the illegal user’s MAC address has be allowed with default action when it learned by DAS4192 firstly, DAS4192 support “MAC-Spoofed” function that allow operator to configure the rule for duplicated MAC address manually.

Enter to the “config spoofed” sub-group directory to define the MAC Spoofed function.

CLI# config mac-spoofed

CLI(config mac-spoofed)#

Table 7-63	MAC Spoofed Configuring

| Use this command to deny all the ports have duplicated MAC, operator assumes they are both illegal. |  |
| --- | --- |
| CLI(config mac-spoofed)# deny all <mac-addr> |  |
| Use this command to permit the legal MAC address when operator confirmed its right. |  |
| CLI(config mac-spoofed)# permit <mac-addr> <port-id> |  |
| Parameters | Task |
| <mac-addr> | Indicate the target MAC address. Type: Mandatory Valid values: Valid MAC addresses form. (for example: 00:1F:AA:19:78:03) |
| <port-id> | Identify the specified xDSL subscriber. Type: Mandatory Valid values: <shelf.slot.port> |

|  | The roles of access control function, Deny and Permit, are repulsive, i.e. a “deny” role will be replaced while a new role “permit” is be configured. |
| --- | --- |

This page is leave in blank for note or memo use

# Managing the System Operations

This chapter describes the system functions of DAS4192 IP-DSLAM.

This chapter contains the following sections:

- Card Module Operations

- System Administrating

- Alarm Definition and Relay Setting

- Configuring the CoS Traffic Mapping

## Card Module Operations

You can define the card module (network card or line card) operation state in DAS4192 system.

### Defining the Line Card Operation Mode

You are allowed to plan the expecting card type address in specific slot, there will have an alarm arise if the planned card type and the actual plug-in card type are mismatch.

The DAS4192 support the following functions on a per LC basis.

- Planning the card type of a LC slot

To ease the operator to plan the usage of each LC slot in advance, the NE support to configure the planned type of a LC slot. There will be an alarm arise if the planned card type and the actual plug-in card type are different.

- RFC 2684 encapsulation method for ADSL line card, either LLC or VCMUX.

- “Service Type Control” for ADSL line card.

Operator can define the service which allow user to pass, they are “DHCP”, “PPPoE” and “Static IP”.

- VLAN tag pass-through function for ADSL line card

Whenever the VLAN tag pass-through (VTP) is configured as enabled, the LC provides transparent transportation of the VLAN traffic from subscriber interface to network interface without any VLAN tag attachment. The LC will not attach any VLAN tag to the upstream subscriber traffic. In the mean time, the LC will also not replace the existing VLAN tag of the upstream subscriber traffic.

On the other hand, in the case that the VTP function is configured as disabled, the LC will attach a VLAN tag to all the traffic from subscriber interface to network interface.

- IEEE 802.1Q VLAN forwarding function for ADSL line card

The operator can set the xDSL subscriber ports as well as the GE ports to only forward either tagged traffic or untagged traffic. This section depicts the commands to set the IEEE 802.1Q VLAN forwarding function on the xDSL subscriber ports. As to the setting on the GE ports, please refer to Section “Configuring the Subtending” for the configuration of GE ports to either only forward either tagged traffic or untagged traffic.

|  | Please refer to Section “Verifying Current Software and Hardware Versions” for the run-time status of the tagged mode on NC and LC. |
| --- | --- |

|  | It is noted that the run-time status of Tagged mode and VTP on LC may be different to their corresponding configuration. In this case, the behavior of the NE is per the run-time status of NE instead of their configuration. Please refer to Table 6-44 for the expected NE behavior. |
| --- | --- |

|  | The ADSL LC needs to be reset to perform the expected system behavior as depicted inTable 6-44 whenever its run-time status changes. |
| --- | --- |

|  | It is noted that the NE will drop the tagged Ethernet frames of VLAN-ID not configured by the VC-to-VLAN setting (see Table 6-45) in the following case. NC tagged mode = Tagged LC tagged mode Run-Time Status = Tagged LC VTP Run-Time Status = Enabled |
| --- | --- |

|  | The tagged mode (run-time) indicates the operational status of tagged mode. Tagged-only: LC (or NC) only forwards the tagged Ethernet frame and drops the untagged Ethernet frame. Untagged-only: LC (or NC) only forwards the untagged Ethernet frame and drops the tagged Ethernet frame. It is noted that the value of configured Tagged mode and its Run-Time Status may be different. Please refer to Table 6-45 for the NE behavior when configuring NC and ADSL LC with various Tagged mode and VTP parameters. |
| --- | --- |

Enter to the “config lc” sub-group directory to plan the line card slot.

CLI# config lc

CLI(config lc)#

Table 8-64	Plan the Line Card Slot

| Use this command to plan the line card type address in specific slot. |  |
| --- | --- |
| CLI(config lc)# set planned-type <lc-range> <card-type> |  |
| Use this command to define the RFC 2684 encapsulation method for specific line card. |  |
| CLI(config lc)# set rfc2684-encap <lc-range> <encap-type> |  |
| Use this command to define the Service Type Control function for specific line card. |  |
| CLI(config lc)# set service-type <lc-range> <option> |  |
| Use this command to modify the VLAN tag pass-through (VTP) that configured as enables or not. (per LC setting). |  |
| CLI(config lc)# vlan-tag-pass <lc-id> { enabled ¦ disabled} |  |
| Use this command to define the tagged mode in specific slot. |  |
| CLI(config lc)# set tagged-mode <lc-id> <mode> |  |
| Use this command to monitor the line card plug-in and planned status. |  |
| CLI(config lc)# show |  |
| Parameters | Task |
| <lc-range> | Specify the slot range of the system Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <lc-id> | Specify the specific slot identifier of NE. Type: Mandatory Default values: 1 |
| <card-type> | Specify the planning line card type Type: Mandatory Valid values: none, adsl, shdsl |
| <encap-type> | Specify the RFC 2684 encapsulation method. Type: Mandatory Valid values: llc, vc-mux |
| <option> | Specify the VLAN tag pass-through status or Service Type Control, enable or disable. Type: Mandatory Valid values: on ¦ off |
| {enabled ¦ disabled } | Specify the the VLAN Tag Pass Through (VTP) of specified UGE port. Type: Mandatory Valid values: disabled ¦ enabled |
| <mode> | Specify the tagged mode is configured as either tagged or untagged mode. Type: Mandatory Valid values: tagged-only ¦ untagged-only |

- Display the line card type status

CLI(config lc)# show

planned  current  rfc2684  vlan-tag  service    configured

LC   type     type     encap     pass      type     tagged-mode

--  -------  -------  -------  --------  --------  -------------

1     ADSL     ADSL      LLC  disabled  disabled  untagged-only

2     ADSL      n/p      LLC  disabled  disabled  untagged-only

3     ADSL     ADSL      LLC  disabled  disabled  untagged-only

4     ADSL      n/p      LLC   enabled  disabled    tagged-only

## System Administrating

The system administrating provides command for you to logout the Telnet session or reboots the system device.

### Reset the Line Card and Port

Reset the line card and port using the “reset” command at the prompt for CLI#.

Table 8-65	Line Card and Port Reset Command

| Use this command to reset the specify line card. |  |
| --- | --- |
| CLI# reset lc <lc-id> |  |
| Use this command to reset the specify NC card. |  |
| CLI# reset nc <nc-id> |  |
| Use this command to reset the specify xDSL port interface. |  |
| CLI# reset port <port-id> |  |
| Use this command to reset (reboot) the system device. |  |
| CLI# reset system |  |
| Parameters | Task |
| <lc-id> | Identify the slot id of the system Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <nc-id> | Identify the slot id of the network card Type: Mandatory Valid values: 1 ¦ 2 |
| <port-id> | Identify the port id of the system Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |

- Reset the line card and xDSL port

CLI# reset lc 2

OK

CLI# reset port 1.2.1

OK

|  | The pop-up information for reset line card command shows only on Console port access. |
| --- | --- |

### Reboot the System

The reboot command activates the software restart of system device. The configuration change will be lost if you did not committed (store) it.

Reboot the system using the “reboot” command at the prompt for CLI#.

Table 8-66	System Reboot Command

Use this command to reboot the system device.

CLI# reboot

### Session Logout

Use this command to terminate the Telnet session or quit the console session from CLI Ex mode.

To logout the sessions using the “logout” command at the prompt for CLI#.

Table 8-67	Session Logout Command

Use this command to logout the session (Telnet or Console).

CLI# logout

If you are using Telnet access for the CLI Ex mode, the command “logout” will terminate the current Telnet session, and the CLI Ex will return to the login prompt if using Console access.

## Alarm Definition and Relay Setting

The alarm definition profile allows you to define the rule of alarm element in system. Through this profile, you are able to change the severity of individual alarm element and decide to report it or not. Alarm element is specified in the class of module or port. Different types of module may present different alarm element. Different types of port may also present different alarm element.

The relay input management allows you to define the alarm relay input. Please see “System Installation Guide” for the definition. Once the normal status of input signal is different from the current status, the NE will launch an “abnormal status” alarm of the specified relay input to LCT and AMS server.

### Configuring the Alarm Definition

Enter to the “config alarm definition” sub-group directory to manage the alarm definition.

CLI# config alarm definition

CLI(config alarm definition)#

Table 8-68	Alarm Definition Configuration

| Use this command to change the default alarm severities |  |
| --- | --- |
| CLI(config alarm definition)# set <vendorType> <alarmType> {none ¦ critical ¦ major ¦ minor ¦ info} {true ¦ false} <suppressby> |  |
| Use this command to viewing the status of system alarm severities. |  |
| CLI(config alarm definition)# show |  |
| Parameters | Task |
| <vendorType> | Defines the vendor type of alarm. Type: Mandatory Valid values: noEntity, cpuModule, adslModule, powerModule, fanModule, adslPort, alarmRelayModule, gePort, alarmRelayInPort |
| <alarmType> | Defines the alarm type in numerical form. Type: Mandatory Valid values: 1 ~ 31 |
| {none ¦ critical ¦ major ¦ minor ¦ info} | Defines the severity level of alarm type. Type: Mandatory Valid values: none, critical, major, minor, info |
| {true ¦ false} | Defines the filtering status of specific alarm type. Type: Mandatory Valid values: true, false |
| <suppressby> | Defines the prevent alarms from being reported on another alarm, when an alarm or condition exists but you do not want it to appear instead of another. Type: Mandatory Valid values: Hexadecimal number |

- Display the system alarm definition

CLI(config alarm definition)# show

Alarm definition

vendor-type  type          name           severity  filtered  supress-by

-----------  ----  ---------------------  --------  --------  ----------

noEntity     0                  EMPTY      none     false         0x0

cpuModule     0                MISSING     major     false         0x0

cpuModule     1                   TEMP     major     false         0x0

cpuModule     2                    VOL     major     false         0x0

cpuModule     3               MISMATCH     major     false         0x0

cpuModule     6            TCA_DHCP_BC   warning     false         0x0

cpuModule    30            HW_INFO_INV     major     false         0x0

adslModule     0                MISSING     major     false         0x0

adslModule     1                   TEMP     major     false         0x0

adslModule     2                    VOL     major     false         0x0

adslModule     3               MISMATCH     major     false         0x0

adslModule     4           NOT_OPERABLE     major     false         0x0

adslModule    30            HW_INFO_INV     major     false         0x0

shdslModule     0                MISSING     major     false         0x0

shdslModule     1                   TEMP     major     false         0x0

shdslModule     2                    VOL     major     false         0x0

shdslModule     3               MISMATCH     major     false         0x0

shdslModule     4           NOT_OPERABLE     major     false         0x0

shdslModule    30            HW_INFO_INV     major     false         0x0

powerModule     0                MISSING     major     false         0x0

powerModule     4           NOT_OPERABLE     major     false         0x0

powerModule     5               PWR_FAIL     major     false         0x0

fanModule     0                MISSING     major     false         0x0

fanModule     1                   FAN1     major     false         0x0

fanModule     2                   FAN2     major     false         0x0

fanModule     9                    VOL     major     false         0x0

|  | The alarm suppression (suppressed by) allows you to mask specific alarms when there are sequences occurred at the same time. For example, let the LOF (Loss of Frame) be configured to be suppressed by the LOS (Loss of Signal), the LOF will not be display on the screen but only LOS whenever the corresponding ADSL loop is cut. |
| --- | --- |

### Configuring the System Relay-In Alarm

The DAS4192 support housekeep alarm relays for input signals.

Enter to the “config alarm relayin” sub-group directory to activate and monitor the alarm relay-in.

CLI# config alarm relayin

CLI(config alarm relayin)#

Table 8-69	System Relay-In Alarm Configuration

| Use this command to configure the system relay-in alarm function. |  |
| --- | --- |
| CLI(config alarm relayin)# set <relayin-index> <name> {open ¦ close ¦ disable} |  |
| Use this command to viewing the relay-in alarm status. |  |
| CLI(config alarm relayin)# show |  |
| Parameters | Task |
| <relayin-index> | Identify the port number of relay-in alarm. Type: Mandatory Valid values: 1 ~ 4 |
| <name> | This specifies the name of given relay-in alarm port. Type: Mandatory Valid values: String of up to 10 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |
| {open ¦ close ¦ disable} | Identify one of the parameter of expects normal status of the relay-in alarm port. Type: Mandatory Valid values: open, close, disable |

- Display the system relay-in alarm port status

CLI(config alarm relayin)# show

Relay-in table

relayin                                       normal   current

index                  name                  status   status

-------  -----------------------------------  -------  -------

1                    << Not Defined >>  disable  disable

2                    << Not Defined >>  disable  disable

3                    << Not Defined >>  disable  disable

4                    << Not Defined >>  disable  disable

## Configuring the CoS Traffic Mapping

User priority: The IEEE 802.1p user priority is a label carried with the frame that communicates the requested priority to the next hop (bridge, router or end systems). Typically, the user priority is not modified in the intermediate hop. Thus, the user priority has end-to-end significance across bridged LANs.

Queue (traffic class): A bridge can be configured so that multiple queues are used to hold frames waiting to be transmitted on a given outbound port, in which case the traffic class is used to determine the relative priority of the queues. Whenever the bridge’s physical port is configured as strict priority (SP), all waiting frames at a higher traffic class are transmitted before any waiting frames of a lower traffic class. As with access priority, traffic class is assigned by the bridge on the basis of incoming user priority.

|  | Currently, the NE supports 8 traffic classes (queues) on its GE ports with the strict priority (SP) scheduling policy only. |
| --- | --- |

Enter to the “config cos-queue” sub-group directory to configure the CoS traffic mapping.

CLI# config cos-queue

CLI(config cos-queue)#

Table 8-70	CoS Traffic Mapping

| Use this command to configure the CoS queue mapping between 802.1p priority and system queue index. |  |
| --- | --- |
| CLI(cos-queue)# mapping <802_1p> <queue-index> |  |
| Use this command to viewing the CoS mapping information. |  |
| CLI(cos-queue)# show |  |
| Parameters | Task |
| <802_1p> | This indicates the 802.1p priority for VLAN traffic. Type: Mandatory Valid values: 0 ~ 7 |
| <queue-index> | The system switch queue index, the higher the number, the higher the forwarding priority. Type: Mandatory Valid values: 1 ~ 8 |

## Configuring the Differentiated Service

DiffServ: RFC 2474/2475 defines the DiffServ field, which replaces the Type of Service (ToS) field in the IPv4 header. It facilitates the network devices behind IP-DSLAM to fulfill the end-to-end QoS.

Enter to the “config diffserv” sub-group directory to configure the DiffServ function.

CLI# config diffserv

CLI(config diffserv)#

Table 8-71	Configuring the DiffServ

| Use this command to enable diffserv function. |  |
| --- | --- |
| CLI(config diffserv)# enable |  |
| Use this command to disable diffserv function. |  |
| CLI(config diffserv)# disable |  |
| Use this command to configure the DiffServ action mapping between 802.1p priority and DSCP value. |  |
| CLI(config diffserv)# mapping <802_1p> <dscp> |  |
| Use this command to viewing the diffserv information. |  |
| CLI(config diffserv)# show |  |
| Parameters | Task |
| <802_1p> | This indicates the 802.1p priority for VLAN traffic. Type: Mandatory Valid values: 0 ~ 7 |
| <dscp> | Defines the DSCP value mapping to 802.1p priority. Type: Mandatory Valid values: BE ¦ AF11 ¦ AF12 ¦ AF13 ¦ AF21 ¦ AF22 ¦ AF23 ¦ AF31 ¦ AF32 ¦ AF33 ¦ AF41 ¦ AF42 ¦ AF43 ¦ EF |

Figure 8-6	DiffServ Field

Differentiated Service Code Point (DSCP): The six bits in front of DiffServ field called DSCP. The network device classifies packets and marks them with DSCP value, according to these values, other network devices in the core that support DiffServ can make decision for packets behavior and provide the Quality of Service properly.

A network device classify the priorities of traffic with 6 different levels, they are Express Forwarding (EF), Assured Forwarding Class 4 (AF4), Assured Forwarding Class 3 (AF3), Assured Forwarding Class 2 (AF2), Assured Forwarding Class 1 (AF1) and Best Effort (BE). These classifications are filled into the first 3 bits of DSCP. See Table 8-72 to understanding the precedence level.

Then, the network device differentiates three drop probabilities in AF4~AF1 respectively into last 3 bits of DSCP, they are Low Drop Precedence, Medium Drop Precedence and High Drop Precedence.

Table 8-72	Precedence Level

| Precedence Level | Description |
| --- | --- |
| 7 | For link layer and routing protocol keep alive. |
| 6 | For using for IP routing protocols. |
| 5 | Express Forwarding (EF) |
| 4 | Assured Forwarding Class 4 (AF4) |
| 3 | Assured Forwarding Class 3 (AF3) |
| 2 | Assured Forwarding Class 2 (AF2) |
| 1 | Assured Forwarding Class 1 (AF1) |
| 0 | Best Effort (BF) |

Expedited Forwarding: The code point of EF is 101110, the packets marked with EF is to be transmitted with highest priority, lowest drop.

Assured Forwarding: There are 4 classes of AF, and 3 drop precedence with each class. It guarantees a certain amount of bandwidth to a AF class if available. According to a given network’s policy, the packets can be selected a behavior based on required throughput, delay, jitter, loss or according to priority of access to network services. Table 8-73 indicates the relationship of the 4 AF class.

Table 8-73	DSCP Class Relationship

|  | Class |  |  |  |
| --- | --- | --- | --- | --- |
| Drop | AF1 | AF2 | AF3 | AF4 |
| Low Drop Prec | 001010 (AF11) | 010010 (AF21) | 011010 (AF31) | 100010 (AF41) |
| Medium Drop Prec | 001100 (AF12) | 010100 (AF22) | 011100 (AF32) | 100100 (AF42) |
| High Drop Prec | 001110 (AF13) | 010110 (AF23) | 011110 (AF33) | 100110 (AF43) |

# Diagnostic and Performance Monitoring

This chapter describes the filtering rule in different network layer.

This chapter contains the following sections:

- Performance Monitoring on System and Network Interface

- Performance Monitoring on Subscriber Interface

- Monitoring System Alarms

- OAM and Loop Diagnostic Test on Subscriber Interface

- Network Ping Test

- Monitoring the System Environment

- Monitoring the System Performance

## Performance Monitoring on System and Network Interface

Follow the commands to obtain data for evaluating the GE interface performance.

Enter to the “status perf” sub-group directory to display PM on the Network interface.

CLI# status perf

CLI(status perf)#

Table 9-74	Performance Monitoring on System and Network Interface

Use this command to viewing the PM on the Network interface.

CLI(PERF)# show nc

- Display the performance monitor on network interface

CLI(STA-PERF)# show nc

interface       unicast    broadcast   multicast    discard      error

----------------  ----------  ----------  ----------  ----------  ----------

UGE-01  inPkts           0           0           0           0           0

outPkts           0           3           0           0           0

UGE-02  inPkts           0           0           0           0           0

outPkts           0           0           0           0           0

LC-01   inPkts        6218          84           0           0           0

outPkts        6281           1           0           0           0

LC-02   inPkts        9448          88           0           0           0

outPkts        9522           1           0           0           0

LC-03   inPkts        5912          80           0           0           0

outPkts        5976           1           0           0           0

LC-04   inPkts          79          83           0           0           0

outPkts          78          78           0           0           0

interface       pause/RX    pause/TX

----------------  ----------  ----------

UGE-01  pkts           0           0

UGE-02  pkts           0           0

## Performance Monitoring on ADSL Subscriber Interface

Enter to the “status perf” sub-group directory to display PM on the ADSL Subscriber interface.

CLI# status perf

CLI(status perf)#

Table 9-75	Performance Monitoring on ADSL Subscriber Interface

| Use this command to viewing the PM on the specified ADSL line port. |  |
| --- | --- |
| CLI(status perf)# show port <port-id> [near ¦ far] |  |
| Use this command to viewing the PM on NC. |  |
| CLI(status perf)# show nc |  |
| Parameters | Task |
| <port-id> | Identify the port id of the system wish to display the PM with associated time period. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| [near ¦ far] | Identify the PM display on Near-End or Far-End, show both if not specify. Type: Optional Valid values: near, far |

- Display the performance monitor on subscriber interface

CLI(status perf)# show port 1.1.2

[UserCells/1.1.2]

Curr15Min     Prev15Min      Curr1Day      Prev1Day

--------      --------      --------      --------

rxCells        0             0             0             3

txCells        0             0             0             0

[Perf/NE/1.1.2]

Curr15Min     Prev15Min      Curr1Day      Prev1Day

--------      --------      --------      --------

UAS        0             0             0          8627

LOFs        0             0             0            30

LOSs        0             0             0             0

LPRs        0             0             0             0

INITs        0             0             0             6

FullINITs        0             0             0             6

ES        0             0             0             0

SES        0             0             0             0

CV        0             0             0             0

[Perf/FE/1.1.2]

Curr15min     Prev15Min      Curr1Day      Prev1Day

--------      --------      --------      --------

UAS        0             0             0          8577

LOFs        0             0             0             8

LOSs        0             0             0             7

LPRs        0             0             0          8329

ES        0             0             0           144

SES        0             0             0            45

CV        0             0             0          1618

## Performance Monitoring on SHDSL Subscriber Interface

Follow the commands to obtain data for evaluating the current xDSL line performance.

Enter to the “status shdsl” sub-group directory to display PM on the SHDSL Subscriber interface.

CLI# status shdsl

CLI(status shdsl)#

Table 9-76	Performance Monitoring on SHDSL Subscriber Interface

| Use this command to viewing the PM on specific SHDSL line port. |  |
| --- | --- |
| CLI(status shdsl)# perf <current ¦ history> |  |
| Use this command to show current 15-min, current 1-day, previous 15-min, and previous 1-day performance data of an SHDSL ports. |  |
| CLI(status shdsl)# show <port-range> |  |
| Parameters | Task |
| <current ¦ history> | Identify the current or history PM of SHDSL port interface. Type: Mandatory |
| <port-range> | Identify the port id of the system wish to perform the loop diagnostic, the define line port must operate in run-time status. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |

## Monitoring System Alarms

This section explains how to monitor alarms with CLI Ex, which includes viewing current and historical alarm data.

The CLI Ex detects and reports system alarms generated by the DAS4192 and the adjacent network. You can use CLI Ex to monitor alarms at a card, port, or network level and view alarm with severities.

Enter to the “status alarm” sub-group directory to monitor system alarms.

CLI# status alarm

CLI(status alarm)#

Table 9-77	Viewing the System Alarm

| Use this command to determine the current alarm to be report on the CLI Ex in real-time. |  |
| --- | --- |
| CLI(status alarm)# reportconsole {on ¦ off} |  |
| Use this command to viewing the current alarm data. |  |
| CLI(status alarm)# show current |  |
| Use this command to viewing the historical alarm data. |  |
| CLI(status alarm)# show alarmhistory |  |
| Use this command to viewing the status of current alarm where to be report on the CLI Ex mode. |  |
| CLI(status alarm)# show reportconsole |  |
| Use this command to viewing the status of current alarm where to be report on the CLI Ex mode. |  |
| CLI(status alarm)# show detail <unit> |  |
| Parameters | Task |
| {on ¦ off} | This defines the current alarm to be report on the CLI Ex in real-time. Type: Mandatory Valid value: on, off |
| {unit} | This indicates the unit on IP-DSLAM. Type: Mandatory Valid value: All the alarm unit on IP-DSLAM |

- Diagnostic the alarm of system

CLI(status alarm)# show current

unit        online type       planned type  alarm   last change     severity

-----------  ----------------  ----------------  -  -----------------  --------

shelf             shelf             shelf  -  10-10-00 08:01:40      none

LC1        adslModule        adslModule  -  10-10-00 08:02:10      none

LC1/port01          adslPort          adslPort  -  10-10-00 08:02:30      none

LC1/port02          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port03          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port04          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port05          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port06          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port07          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port08          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port09          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port10          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port11          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port12          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port13          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port14          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port15          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port16          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port17          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port18          adslPort          adslPort  v  10-10-00 08:02:09      info

LC1/port19          adslPort          adslPort  v  10-10-00 08:02:09      info

CLI(status alarm)# show alarmhistory

History Table

physical                            alarm

index     index  planned type  on-line type  type      occur time

-----  -------  -------------  -----------  ----------  ------------------

12562    10236       adslPort     adslPort          20   10-10-00 09:18:44

12563    10236       adslPort     adslPort          21   10-10-00 09:18:44

12564    10236       adslPort     adslPort          22   10-10-00 09:18:44

12565    10236       adslPort     adslPort          23   10-10-00 09:18:44

12566    10236       adslPort     adslPort          24   10-10-00 09:18:44

12567    10236       adslPort     adslPort          25   10-10-00 09:18:44

12568    10236       adslPort     adslPort          26   10-10-00 09:18:44

12569    10236       adslPort     adslPort          29   10-10-00 09:18:44

12570    10236       adslPort     adslPort          30   10-10-00 09:18:44

12571    10236       adslPort     adslPort          31   10-10-00 09:18:44

12572    10237       adslPort     adslPort           1   10-10-00 09:18:44

12573    10237       adslPort     adslPort           2   10-10-00 09:18:44

12574    10237       adslPort     adslPort           3   10-10-00 09:18:44

12575    10237       adslPort     adslPort           4   10-10-00 09:18:44

12576    10237       adslPort     adslPort           5   10-10-00 09:18:44

12577    10237       adslPort     adslPort           6   10-10-00 09:18:44

12578    10237       adslPort     adslPort           7   10-10-00 09:18:44

12579    10237       adslPort     adslPort           8   10-10-00 09:18:44

12580    10237       adslPort     adslPort           9   10-10-00 09:18:44

12581    10237       adslPort     adslPort          10   10-10-00 09:18:44

CLI(status alarm)# show detail LC1/port20

Detail alarm list is:

alarm name        severity                description

----------------------  --------  ----------------------------------------

DISABLED      info                      The port is disabled

## OAM and Loop Diagnostic Test on Subscriber Interface

In order to diagnose and fix problem, DAS4192 IP-DSLAM Subscriber interface provides for diagnosis of ATM OAM F5 at data connection layer and ADSL loop diagnostic at physical layer, respectively.

### OAM F5 VC Diagnostic

To configure the PVC to send Operation, Administration, and Maintenance (OAM) F5 loopback cells to verify connectivity on the VC (Virtual Circuit) situated in the ATM layer of Subscriber interface. The remote end (CPE) must respond by echoing back such cells

Enter to the “diag” group directory with “oam” command to perform the OAM F5 VC diagnostic.

CLI# diag

CLI(diag )#

Table 9-78	OAM F5 VC Diagnosis Test

| Use this command to testing the OAM F5 on both End-to-End and Segment-to-Segment. |  |
| --- | --- |
| CLI(diag)# oam set F5 <port-id> <vpi> <vci> both |  |
| Use this command to testing the OAM F5 on End-to-End only. |  |
| CLI(diag)# oam set F5 <port-id> <vpi> <vci> end-to-end |  |
| Use this command to testing the OAM F5 on Segment-to-Segment only. |  |
| CLI(diag)# oam set F5 <port-id> <vpi> <vci> seg-to-seg |  |
| Parameters | Task |
| <port-id> | Identify the port id of the system wish to perform the OAM F5, the define VC must existed at defines line port. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <vpi> | Defines the VPI (Virtual Path Identifier) value. Type: Mandatory Valid values: 0 ~ 255 |
| <vci> | Defines the VCI (Virtual Channel Identifier) value. Type: Mandatory Valid values: 1 ~ 65535 (1 ~ 31 are reserved) |

This example shows how to perform the OAM F5 diagnostic, if the OAM cell from the selected VCL replies the echo, the CLI Ex will shows “alive”, check both physical layer and ATM layer setting if shows “OAM timeout” otherwise.

- Diagnostic the OAM F5 in ATM layer of Subscriber interface

CLI(diag oam )# set F5 1.1.2 0 35 both

Port 1.1.2 pvc 0/35: alive.

OK

CLI(diag oam)# set F5 1.1.1 0 35 both

Port 1.1.1 pvc 0/35: OAM timeout.

OK

### ADSL Loop Diagnostic

The ADSL loop diagnostic determines the cause of problems on ADSL line port; this function performs real-time performance monitoring capabilities that provide measurements on line quality and noise conditions at both ends of the line.

This function available on ADSL2 and ADSL2+ connection only, the ADSL CPE who did not complied with ITU-T standard G.992.3, G.992.4, and G.992.5 may not be able to perform the loop diagnostics.

Enter to the “diag” group directory with “delt” command to perform the ADSL loop diagnostic.

CLI# diag

CLI(diag)#

Table 9-79 	ADSL Loop Diagnostic Test

| Use this command to start the run-time loop diagnostic to the specific ADSL line port and it corresponding data connection profile. |  |
| --- | --- |
| CLI(diag delt)# start <port-id> <profile-name> |  |
| Use this command to manually terminating the loop diagnostic if such function has been started. |  |
| CLI(diag delt)#stop |  |
| Use this command to viewing the result due to loop diagnostic completed. |  |
| CLI(diag delt)#show |  |
| Parameters | Task |
| <port-id> | Identify the port id of the system wish to perform the loop diagnostic, the define line port must operate in run-time status. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <profile-name> | This specifies the ADSL port connection profile of the specific ADSL line port. Type: Mandatory Valid values: String of up to 32 characters ('0'~'9', 'A'~'Z', 'a'~'z', '-', '_', '.', '@'). |

- Diagnostic the run-time ADSL line port loop performance

CLI(diag delt)# start 1.2.1 test

Ok

CLI(diag delt)# show

Loop diag result:

Port  1. 1

Used Profile:  "test"

ATU-C  ATU-R

Attainable Rate(Kbps)   26528.0 1284.0

Loop Attenuation(dB)    0.4   0.0

Signal Attenuation(dB)  0.0   0.0

SnrMargin(dB)           6.0   0.0

TxPower(dBm)            15.3  12.3

H(f) logarithmic representation( Hlog(f) )

DS              Unit: dB

[  1]  -78.0  -39.0  -42.0  -42.0  -45.0  -44.0  -45.0  -48.0

[  9]  -48.0  -48.0  -47.0  -48.0  -48.0  -48.0  -47.0  -48.0

[ 17]  -51.0  -47.0  -48.0  -52.0  -53.0  -41.0  -41.0  -41.0

[ 25]  -40.0  -36.0  -34.0  -31.0  -28.0  -26.0  -24.0  -22.0

[ 33]  -19.0  -17.0  -15.0  -13.0  -12.0  -10.0   -8.0   -7.0

[ 41]   -6.0   -5.0   -5.0   -4.0   -4.0   -3.0   -3.0   -3.0

[ 49]   -3.0   -3.0   -3.0   -2.0   -2.0   -2.0   -2.0   -2.0

[ 57]   -2.0   -2.0   -2.0   -1.0   -1.0   -1.0   -1.0   -1.0

[ 65]   -1.0   -1.0   -1.0    0.0    0.0    0.0    0.0    0.0

[ 73]    0.0    0.0    0.0    0.0    0.0    0.0    0.0    1.0

[ 81]    1.0    1.0    1.0    1.0    1.0    1.0    1.0    1.0

[ 89]    1.0    1.0    1.0    1.0    1.0    1.0    1.0    1.0

### ADSL Link Monitoring

The ADSL link monitoring function provides the records of ADSL loop characteristics measured during the last training. It is noted that the measured results are only available in the show-time.

Enter to the “diag” group directory with “portmon” command to perform the ADSL link monitoring.

CLI# diag

CLI(diag)#

Table 9-80 	ADSL Link Monitoring

| Use this command to start running the link monitoring to the specific ADSL line port. |  |
| --- | --- |
| CLI(diag portmon)# portmon start <port-id> |  |
| Use this command to manually terminating the ADSL link monitoring if such function has been started. |  |
| CLI(diag portmon)# portmon stop |  |
| Use this command to viewing the result due to ADSL link monitoring completed. |  |
| CLI(diag portmon)# portmon show |  |
| Parameters | Task |
| <port-id> | Identify the port id of the system wish to perform the link monitoring, the define line port must operate in running status. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |

### SELT Link Monitoring

The SELT loop function diagnosis function is to estimate the distance of the DSL connection from the NE to the subscriber’s location without connecting a subscriber device.

Enter to the “diag” group directory with “selt” command to perform the SELT link monitoring.

CLI# diag selt

CLI(diag selt)#

Table 9-81 	SELT Link Monitoring

| Use this command to start running the SELT link monitoring to the specific ADSL line port. |  |
| --- | --- |
| CLI(diag selt)# start <port-id> |  |
| Use this command to viewing the result due to SELT link monitoring completed. |  |
| CLI(diag selt)# show |  |
| Parameters | Task |
| <port-id> | Identify the port id of the system wish to perform the link monitoring, the define line port must operate in running status. Valid values: See “Chapter 2 Port Interface Indication” |

- Diagnostic the run-time ADSL line port loop performance

CLI(diag selt)# start 1.1.2

OK: But the result displays by diag selt show.

CLI(diag selt)# show

Port single end loop test result: Port  1.37

Cable Type: 24 AWG

Loop Length: 14219 (ft.)

|  | Please refer to ITU-T 992.3 for the details of SELT. |
| --- | --- |

## Network Ping Test

The “ping” command is a very common method for troubleshooting the accessibility of devices. It uses a series of ICMP (Internet Control Message Protocol) Echo messages to determine a target host is active or inactive.

To diagnose the remote hosts using the “ping” command at the prompt for CLI#. (From UGE or MGE)

Table 9-82	Network Ping Test

| Use this command to send the ICMP Echo message to target host. |  |
| --- | --- |
| CLI# ping <hostname> |  |
| Parameters | Task |
| <hostname> | Defines IP address or hostname of the target host to reply ICMP Echo message. Type: Mandatory Valid values: 0.0.0.0 ~ 255.255.255.255 ¦ string |

- Using Ping command to test the remote host status

CLI# ping 10.1.29.236

10.1.29.236 PING Statistics: 5 packets transmitted, 5 packets received

## Monitoring the System Environment

In the hardware monitoring list dialog, you can monitor the temperature and voltage status of any specific card module.

Enter to the “status” group directory with proper command to perform the system environment monitoring.

CLI# status

CLI(status)#

Table 9-83	System Environment Monitoring

| Use this command to display the system ventilation fan speed information. |  |
| --- | --- |
| CLI(status)# fanspeed show |  |
| Use this command to display the temperature of specific line card. |  |
| CLI(status)# temp show lc <slot-id> |  |
| Use this command to display the temperature of network card. |  |
| CLI(status)# temp show nc |  |
| Use this command to display the voltage of fan module. |  |
| CLI(status)# voltage show fan |  |
| Use this command to display the voltage of specific line card. |  |
| CLI(status)# voltage show lc <slot-id> |  |
| Use this command to display the voltage of network card. |  |
| CLI(status)# voltage show nc |  |
| Parameters | Task |
| <slot-id> | Identify the slot range of the line card Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |

## Monitoring the System Performance

Enter to the “status” group directory with proper command to perform the system environment monitoring.

CLI# status

CLI(status)#

Table 9-84	System Performance Monitoring

| Use this command to show adsl line historical performance data (15 minutes per interval). |  |
| --- | --- |
| CLI(status)# 15min-perf-history show <port-id> <start-interval> |  |
| Use this command to show those VLANs on specific line cards that currently are allowed to forward broadcast packets. |  |
| CLI(status)# broadcast dsfilter show [<slot-range>] |  |
| Use this command to display LACP status. |  |
| CLI(status)# lacp show |  |
| Use this command to display line card status. |  |
| CLI(status)# lcstatus show |  |
| Use this command to display RSTP status. |  |
| CLI(status)# rstp show [bridge ¦ uge] |  |
| Use this command to display the system up time. |  |
| CLI(status)# time show |  |
| Parameters | Task |
| <port-id> | Identify the port id of the system wish to perform the link monitoring, the define line port must operate in running status. Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| <start-interval> | This specifies the adsl line historical performance data (15 minutes per interval). Type: Mandatory Valid values: 1 ~ 96 |
| <slot-range> | Identify the slot range of the line card Type: Mandatory Valid values: See “Chapter 2 Port Interface Indication” |
| [bridge ¦ uge] | This specifies the Rapid Spanning Tree Protocol status Type: Mandatory Valid value: bridge, uge |

- show adsl line historical performance data (15 minutes per interval)

CLI(STATUS)# 15min-perf-history show 1.2.1 1

Port: 2.1  interval: 1/96

rxCells:         0   txCells:         0

Near End   Far End

--------- ---------

UAS         0         0

LOFs         0         0

LOSs         0         0

LPRs         0         0

ES         0         0

SES         0         0

CV         0         0

INITs         0      ----

FailINITs         0      ----

Port: 2.1  interval: 2/96

rxCells:         0   txCells:         0

Near End   Far End

--------- ---------

UAS         0         0

LOFs         0         0

LOSs         0         0

LPRs         0         0

ES         0         0

SES         0         0

CV         0         0

INITs         0      ----

FailINITs         0      ----

Port: 2.1  interval: 3/96

rxCells:         0   txCells:         0

Near End   Far End

--------- ---------

UAS         0         0

LOFs         0         0

LOSs         0         0

LPRs         0         0

ES         0         0

SES         0         0

CV         0         0

INITs         0      ----

FailINITs         0      ----

Port: 2.1  interval: 4/96

rxCells:         0   txCells:         0

Near End   Far End

--------- ---------

UAS         0         0

LOFs         0         0

LOSs         0         0

LPRs         0         0

ES         0         0

SES         0         0

CV         0         0

INITs         0      ----

FailINITs         0      ----

            - Abbreviations and Acronyms

The abbreviations and acronyms used in this document.

Table A-1	Abbreviations and Acronyms Table

| Abbreviations | Full Name |
| --- | --- |
| AAL | ATM Adaptation Layer |
| ADSL | Asymmetric Digital Subscriber line |
| AIS | Alarm Indication Signal |
| ATM | Asynchronous Transfer Mode |
| ATU-C | ADSL Transceiver Unit at the central office end |
| ATU-R | ADSL Transceiver Unit at the remote end |
| CBR | Constant Bit Rate |
| CV | Coding Violation |
| DSCP | Differentiated Service Code Point |
| DSLAM | Digital Subscriber line Access Multiplexer |
| ES | Error Seconds |
| EOA | Ethernet over ATM |
| GBIC | Gigabit Interface Converter |
| GE | Gigabit Ethernet |
| IP | Internet Protocol |
| LAN | Local Area Network |
| LOF | Loss of Frame |
| LOS | Loss of Signal |
| LPR | Loss of Power |
| OAM | Operation, Administration, and Maintenance |
| PCR | Peak Cell Rate |
| PSD | Power Spectral Density |
| PVC | Permanent Virtual Channel |
| rtVBR | Real time Variable Bit Rate |
| SCR | Sustainable Cell Rate |
| SFP | Small Form Pluggable |
| SNR | Signal-to Noise Ratio |
| SNMP | Simple Network Management Protocol |
| UAS | Unavailable Seconds |
| UBR | Unspecified Bit Rate |
| VC | Virtual Channel |
| VCI | Virtual Channel Identify |
| VCL | Virtual Channel Link |
| VDSL | Very high-speed Digital Subscriber line |
| VLAN | Virtual Local Area Network |
| VP | Virtual Path |
| VPI | Virtual Path Identifier |
| VTU-O | VDSL Transmission Unit at the Optical network interface |
| VTU-R | VDSL Transmission Unit at the remote end |
| WAN | Wide Area Network |
| xDSL | ADSL/VDSL |