### DP-692

#### Internet Server

#### Quick Installation Guide

First Edition (February, 2000)

6DP692QML.01

Printed In Taiwan

RECYCLABLE

### Introduction

This Quick Installation Guide gives step-by-step instructions for setting up the D-Link DP-692 Internet servers. The model you have purchased may appear slightly different from those shown in the illustrations. Examples in this manual are based on DP-692 Internet servers. For more detailed information about the router, its components, making network connections and technical specifications, please refer to the User’s Guide included with your router.

## Unpacking

Open the shipping cartons of your Internet server and carefully unpacks the contents.  The carton should contain the following items:

- One Internet server device

- One AC power adapter, suitable for your areas electrical power connections

- One 3.5” diskette with IS Admin software

- IS Admin User’s Guide

- One 3.5” diskette with IS Manager Server software

- IS Manager Server User’s Guide

- One 3.5” diskette with IS Mail Server software

- IS Mail Server User’s Guide

- Wall mount hardware

- One User’s Guide

- The Quick Installation Guide

Inspect the device and all accompanying items.  If any item is damaged or missing, report the problem immediately to your dealer.

## Wall Installation

The DP-692 comes complete with a wall mount kit. This kit includes two screws and two plastic anchors. For a proper placement on the wall, follow these steps:

- Select a site that is free of obstructions from other equipment or devices. Consider the following points for site selection:

- The Internet server should be placed high enough where LED indicators can be observed and cable and power connections can be made.

- Moreover, decide whether you would like it to point with the front panel facing either up or down.

- Drill two holes into the wall with the same distance as the screw support holes located on the bottom of the Internet server. Do not drill these holes too deep.

- Insert the nylon anchors into the holes in the wall and imbed them with gentle taps of a hammer.

- Screw in the screws provided with the wall mount kit into the plastic anchors.  Do not insert the screws with excessive torque. The screw’s head and with a small portion of the screw’s body should be sticking out.

- Gently, place the Internet server onto the wall with the front panel facing up or down by sliding the screws into the provided slots on the underside of the unit.

- Installing the Internet Server on a Wall

## Port Descriptions

- DP-692 Rear Panel

### RJ-45 – LAN

The Internet server has ten, NWay (auto-detect, auto-configure), RJ-45 LAN ports.  These MDI-X UTP ports are 10Mbps and 100Mbps capable and designed for use in an Ethernet or Fast Ethernet LAN network. By pressing the button next to the tenth port so that it is in the OUT position, the port 10X will become an MDI-II Uplink port for connecting the internet server to another hub.

### BNC – LAN

The DP-692 provides a BNC port for cascading with other hubs without sacrificing any RJ-45 ports. Use this port to connect to your thin coax backbone.

DP-692 Side Panel

### Serial – WAN

Serial ports are used for WAN connections either to a modem or an ISDN terminal adapter. Both serial ports are standard male 9-pin RS-232 connectors. The WAN ports can be set to operate between 4800bps and 460800bps.

## Diagnostic LED Descriptions

The Internet server has an LED array for indicating current port and transmission status and is shown below.

- DP-692 Front Panel LEDs

Power

- ON:  “power good”.

- OFF: “power bad.”  Check to see if the AC power adapter is properly connected, or if the correct AC power adapter is being used.

Collision

- Blinking: Packet collision is occurring. Packet collisions are not an abnormal situation. Collisions occur when two or more computers transmit packets on the network simultaneously, and a contention takes place on the network line. The computers should then back off, then retry transmission.  This trial-and-error process is repeated until no collision takes place. Note: Excessive collisions may result when multiple hubs are cascaded through a thin coaxial segment and many stations are connected on the network.

- Off:  No packet collision.

Link/Rx (for each UTP port)

- ON: Data link between (1) Mini Hub’s UTP port and (2) node’s or cascaded hub’s UTP port is successful.

- OFF: (1) No data link or (2) cable disconnected.  Check for bad cable or loose connectors. For cascaded hubs, check to see if UTP cable contains crossed-over wires.  Also check for a “power good” condition at both ends of the connection. If you suspect that the hub port is damaged, contact your authorized dealer for service.

- Blinking: Packet reception is occurring.

Rx (for BNC port).

- OFF: No packet reception.

- Blinking: Packet reception is occurring.

Partition (for each UTP port).

- ON:  The UTP port is being partitioned off due to excessive packet collisions.  Note that the UTP cable between a hub port and a non-repeater node must contain straight-through wires (no crossover).

- OFF:  Segment has no problem.

Partition (for BNC port).

- ON:  The BNC port is being partitioned off due to (1) no cable is connected, (2) faulty cable or connectors, (3) excessive packet collisions, (4) a disconnected point somewhere along the entire thin coaxial cable length, or (5) unterminated segment.  Check all connectors along the cable length.  If segment is not terminated, terminate both ends with 50-ohm terminators.

- OFF:  (1) Cable is connected and (2) segment has no problem.

COM1 / COM2

- ON: Each COM port LED provides an indication of the operating status of the corresponding WAN port.  When a WAN port is transmitting data, the related COM port indicator will light.

- OFF: No data is being transmitted.

## Connecting to the Local Network

This section describes how to connect the DP-692 Internet server to your Ethernet/Fast Ethernet network.

### Cables

- The EIA/TIA 568 Wiring Standard imposes a 100 meter limit on horizontal runs of twisted-pair cables; in this case, from the internet server to any other device.

- 10 Mbps Ethernet connections must use Category 3 or better twisted-pair cabling fitted with RJ-45 connectors.

- 100 Mbps Fast Ethernet connections must use shielded twisted pair (STP) or Category 5 or better-unshielded twisted-pair (UTP) cables fitted with Category 5 RJ-45 connectors.

- Cat 5 UTP cables use the same RJ-45 connector used with 10BASE-T, wired in the same configuration.

### Connecting to a Twisted-Pair LAN

As mentioned before, the Internet server comes equipped with ten NWay Ethernet/Fast Ethernet RJ-45 LAN ports. Together, these twisted-pair ports function as a normal Ethernet/Fast Ethernet hub. To connect the internet server to an existing 10BASE-T or 100BASE-TX LAN, simply plug one end of a straight-through cable into the 10X LAN port on the internet server, and the other end into a normal (non-Uplink) port on a hub or switch (as shown below). Both devices can be receiving power when making this connection. Please note that when making this connection the button next to the 10X port should be in the Uplink (OUT) position.

- LAN Uplink Connection

Please refer to the following table when making twisted-pair connections:

| DEVICE | PORT USED | DEVICE BEING CONNECTED | PORT TYPE | CABLE TO USE |
| --- | --- | --- | --- | --- |
|  |  | Hub or | Normal | Crossover (X) |
|  | Normal | Switch | Uplink | Straight-Through (¦¦) |
| Internet Server |  | Server (or PC) |  | Straight-Through (¦¦) |
|  |  | Hub or | Normal | Straight-Through (¦¦) |
|  | Uplink | Switch | Uplink | Crossover (X) |
|  |  | Server (or PC) |  | Crossover (X) |

### Connecting  Through Thin Coaxial Cable

The BNC port on the Mini Hub model DP-692 can be used to cascade hubs together.  Actually, you can cascade the DP-692 with any other IEEE 802.3 compliant 10BASE-T Ethernet hubs equipped with BNC ports.

You may attach up to thirty nodes to a thin coaxial segment.  In this sense, it is possible to cascade a maximum of thirty hubs together through the thin coaxial cable.

Be sure to leave a minimum of 0.5 meter (2 feet) of cable between two BNC ports.

### Connecting to a Thin Coaxial Segment

The BNC port on Mini Hub model DP-692 is used to connect to a thin coaxial segment.  Connect a T-connector to the BNC connector, then connect both ends of the T-connector to the thin coaxial cable.  If the thin coaxial segment terminates at the hub, attach a 50-ohm terminator to one end of the T-connector as shown below.

### Connecting Computers

The ten UTP ports, called a mini-hub, can be used as a normal hub to build a LAN. To do this, simply connect a normal straight-through Category 3,4 or 5 UTP or STP cable to a port and to a Network Interface Card (NIC) installed in a PC as shown below. Please note that the example shown below uses a BNC connection to a network backbone. You may also use the Uplink port (port 10X) connection described above but you must ensure that no signal loops are formed.

## Connecting to the Internet

The Internet server must be connected to the Internet (this is also known as a Wide Area Network or ‘WAN’ connection) using a COM serial port.  A COM port can either be connected to the serial port of a modem (or fax/modem), or to an ISDN terminal adapter.

This Internet server model has two COM ports and can therefore have two WAN connections.  Any combination of different types of WAN connections will work.  The faster the WAN connections, the faster your Internet access will be for all users.

Connect the COM port of the Internet server to a modem or ISDN terminal adapter using a standard serial cable.

- Modem Connection

## Connecting Power

Power is supplied to the Internet server through an AC power adapter.

- Connecting Power

Since the Internet server does not include a power switch, plugging its power adapter into a power outlet will immediately power it on.

After the Internet server is connected to the LAN and powered on, you can use the included IS Admin software to assign an IP address to the device. Once the Internet server has a valid IP address, you may set it up for normal operations (i.e. connecting to the Internet) from any network station through Telnet or a web browser.

## Installation

This section takes you through a step-by-step minimum installation and setup procedure for the Internet server. Please refer to the main text of this manual for detailed information about the setup and operation of this device.

### Getting Started

Step 1: Unpack the device. Make certain no components have been lost or damaged.

Step 2: Choose an installation site on a flat, level surface or wall. Make sure it is placed near the modem or ISDN/TA you plan to use for Internet connections and/or near the network line you plan to use for a LAN connection. Note that the Internet server can be hung on a wall using the wall mounting equipment included with the product.

### Making Connections

Step 3: Connect the Internet server to your LAN using a Category 3, 4 or 5, twisted-pair cable and the device’s single RJ-45 LAN port. This connection should be made to an Ethernet or Fast Ethernet switch or hub.  (The RJ-45 port looks like a phone jack.)

Step 4: Connect the Internet server to a modem or ISDN/TA, using one or both of the device’s serial ports (COM1 and COM2) located on the side panel.  (Note that your modem or ISDN/TA should already be connected and setup according to the instructions included with it.)

Step 5: Plug the power adapter into the device and into an electrical outlet.

### Configuration

Step 6: Before you can use your Internet server, IP addresses on your LAN’s PCs must be set so that they are compatible with the Internet server’s settings.  The Internet server comes with the default local IP address: 192.168.100.1 and the default subnet mask setting: 255.255.255.0. Assuming that you leave these settings unchanged and assign fixed IP addresses to machines on your LAN that will access the Internet through the Internet server, you must give those machines IP addresses in the range of 192.168.100.xxx, where xxx is a number from 2 to 255. (If you want to use a different IP address range, see the user’s guide “Setting IP Addresses”.) You can, alternatively, set the machines on your LAN to obtain their IP addresses automatically using DHCP to get IP addresses from the Internet server. Whether the machines use fixed IP addresses or DHCP, they must all use the same subnet mask setting as the Internet server, and the Internet server’s local (LAN) IP address must be set as each machine’s default gateway.

Step 7: The internet server can be configured and operated via Telnet or a web browser once PC IP addresses have been properly set.  (Note that some device settings can be manipulated using the IS Admin program included with the device.)  Start your Telnet or browser software and enter the IP address of the Internet server (either the default IP listed above or the new address you assigned using IS Admin). This should bring up the Internet server start menu.  See the next series of steps for information about settings that must be set for the device to work properly.

### Mandatory Settings for Internet Access

Step 8: ISP Account -> Phone Number, when you signed-up for an account with your ISP (Internet service provider), you should have been given an access phone number that your modem will dial.  Look under the “WAN Port Settings” menu for this item and enter the phone number provided by your ISP.

Step 9: ISP Account -> User ID, your ISP should also have assigned a User ID (aka, a username) that you will use for logging-in.  Also under “WAN Port Settings,” enter this user ID exactly as it was provided to you.

Step 10: ISP Account -> Password, finally, to complete the ISP login process, the Internet server must provide the password associated with the user ID assigned by your ISP.  Enter it.

You have now completed the basic steps necessary to install, configure, and begin using the Internet server.  Note that, with respect to steps 8–10, it may be necessary for you to use a “Login Script” instead.  If you enter the information required in those three settings correctly and still has trouble logging-in.

#### Additional Information

If you are encountering problems setting up your network, please refer to the User’s Guide that came with the hub. It contains many more rules, charts, explanations and examples to help you get your network up and running.

Additional help is available online at http://www.dlink.com, or through our offices listed at the back of the User’s Guide.