DFM-560I+

Voice/Fax/Data PCI

Modem

##### User’s Guide

### Rer. 01 (December, 2000)

RECYCLABLE

Printed in Taiwan

#### LIMITED WARRANTY

D-Link provides this limited warranty for its product only to the person or entity who originally purchased the product from D-Link or its authorized reseller or distributor.

Limited Hardware Warranty: D-Link warrants that the hardware portion of the D-Link products described below (“Hardware”) will be free from material defects in workmanship and materials from the date of original retail purchase of the Hardware, for the period set forth below applicable to the product type (“Warranty Period”) if the Hardware is used and serviced in accordance with applicable documentation; provided that a completed Registration Card is returned to an Authorized D-Link Service Office within ninety (90) days after the date of original retail purchase of the Hardware.  If a completed Registration Card is not received by an authorized D-Link Service Office within such ninety (90) period, then the Warranty Period shall be ninety (90) days from the date of purchase.

| Product Type | Warranty Period |
| --- | --- |
| Product (excluding power supplies and fans) | One (1) Year |
| Power Supplies and Fans | One (1) Year |
| Spare parts and spare kits | Ninety (90) days |

D-Link’s sole obligation shall be to repair or replace the defective Hardware at no charge to the original owner.  Such repair or replacement will be rendered by D-Link at an Authorized D-Link Service Office.  The replacement Hardware need not be new or of an identical make, model or part; D-Link may in its discretion may replace the defective Hardware (or any part thereof) with any reconditioned product that D-Link reasonably determines is substantially equivalent (or superior) in all material respects to the defective Hardware.  The Warranty Period shall extend for an additional ninety (90) days after any repaired or replaced Hardware is delivered. If a material defect is incapable of correction, or if D-Link determines in its sole discretion that it is not practical to repair or replace the defective Hardware, the price paid by the original purchaser for the defective Hardware will be refunded by D-Link upon return to D-Link of the defective Hardware.  All Hardware (or part thereof) that is replaced by D-Link, or for which the purchase price is refunded, shall become the property of D-Link upon replacement or refund.

Limited Software Warranty: D-Link warrants that the software portion of the product (“Software”) will substantially conform to D-Link’s then current functional specifications for the Software, as set forth in the applicable documentation, from the date of original delivery of the Software for a period of ninety (90) days (“Warranty Period”), if the Software is properly installed on approved hardware and operated as contemplated in its documentation.  D-Link further warrants that, during the Warranty Period, the magnetic media on which D-Link delivers the Software will be free of physical defects.  D-Link’s sole obligation shall be to replace the non-conforming Software (or defective media) with software that substantially conforms to D-Link’s functional specifications for the Software.  Except as otherwise agreed by D-Link in writing, the replacement Software is provided only to the original licensee, and is subject to the terms and conditions of the license granted by D-Link for the Software.  The Warranty Period shall extend for an additional ninety (90) days after any replacement Software is delivered.  If a material non-conformance is incapable of correction, or if D-Link determines in its sole discretion that it is not practical to replace the non-conforming Software, the price paid by the original licensee for the non-conforming Software will be refunded by D-Link; provided that the non-conforming Software (and all copies thereof) is first returned to D-Link.  The license granted respecting any Software for which a refund is given automatically terminates.

What You Must Do For Warranty Service:

Registration Card.  The Registration Card provided at the back of this manual must be completed and returned to an Authorized D-Link Service Office for each D-Link product within ninety (90) days after the product is purchased and/or licensed.  The addresses/telephone/fax list of the nearest Authorized D-Link Service Office is provided in the back of this manual.  FAILURE TO PROPERLY COMPLETE AND TIMELY RETURN THE REGISTRATION CARD MAY AFFECT THE WARRANTY FOR THIS PRODUCT.

Submitting A Claim.  Any claim under this limited warranty must be submitted in writing before the end of the Warranty Period to an Authorized D-Link Service Office.  The claim must include a written description of the Hardware defect or Software nonconformance in sufficient detail to allow D-Link to confirm the same.  The original product owner must obtain a Return Material Authorization (RMA) number from the Authorized D-Link Service Office and, if requested, provide written proof of purchase of the product (such as a copy of the dated purchase invoice for the product) before the warranty service is provided.  After an RMA number is issued, the defective product must be packaged securely in the original or other suitable shipping package to ensure that it will not be damaged in transit, and the RMA number must be prominently marked on the outside of the package.  The packaged product shall be insured and shipped to D-Link, 53 Discovery Drive, Irvine CA 92618, with all shipping costs prepaid.  D-Link may reject or return any product that is not packaged and shipped in strict compliance with the foregoing requirements, or for which an RMA number is not visible from the outside of the package.  The product owner agrees to pay D-Link’s reasonable handling and return shipping charges for any product that is not packaged and shipped in accordance with the foregoing requirements, or that is determined by D-Link not to be defective or non-conforming.

What Is Not Covered:

This limited warranty provided by D-Link does not cover:

Products that have been subjected to abuse, accident, alteration, modification, tampering, negligence, misuse, faulty installation, lack of reasonable care, repair or service in any way that is not contemplated in the documentation for the product, or if the model or serial number has been altered, tampered with, defaced or removed;

Initial installation, installation and removal of the product for repair, and shipping costs;

Operational adjustments covered in the operating manual for the product, and normal maintenance;

Damage that occurs in shipment, due to act of God, failures due to power surge, and cosmetic damage; and

Any hardware, software, firmware or other products or services provided by anyone other than D-Link.

Disclaimer of Other Warranties: EXCEPT FOR THE LIMITED WARRANTY SPECIFIED HEREIN, THE PRODUCT IS PROVIDED “AS-IS” WITHOUT ANY WARRANTY OF ANY KIND INCLUDING, WITHOUT LIMITATION, ANY WARRANTY OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NON-INFRINGEMENT.  IF ANY IMPLIED WARRANTY CANNOT BE DISCLAIMED IN ANY TERRITORY WHERE A PRODUCT IS SOLD, THE DURATION OF SUCH IMPLIED WARRANTY SHALL BE LIMITED TO NINETY (90) DAYS.  EXCEPT AS EXPRESSLY COVERED UNDER THE LIMITED WARRANTY PROVIDED HEREIN, THE ENTIRE RISK AS TO THE QUALITY, SELECTION AND PERFORMANCE OF THE PRODUCT IS WITH THE PURCHASER OF THE PRODUCT.

Limitation of Liability: TO THE MAXIMUM EXTENT PERMITTED BY LAW, D-LINK IS NOT LIABLE UNDER ANY CONTRACT, NEGLIGENCE, STRICT LIABILITY OR OTHER LEGAL OR EQUITABLE THEORY FOR ANY LOSS OF USE OF THE PRODUCT, INCONVENIENCE OR DAMAGES OF ANY CHARACTER, WHETHER DIRECT, SPECIAL, INCIDENTAL OR CONSEQUENTIAL (INCLUDING, BUT NOT LIMITED TO, DAMAGES FOR LOSS OF GOODWILL, WORK STOPPAGE, COMPUTER FAILURE OR MALFUNCTION, LOSS OF INFORMATION OR DATA CONTAINED IN, STORED ON, OR INTEGRATED WITH ANY PRODUCT RETURNED TO D-LINK FOR WARRANTY SERVICE) RESULTING FROM THE USE OF THE PRODUCT, RELATING TO WARRANTY SERVICE, OR ARISING OUT OF ANY BREACH OF THIS LIMITED WARRANTY, EVEN IF D-LINK HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.  THE SOLE REMEDY FOR A BREACH OF THE FOREGOING LIMITED WARRANTY IS REPAIR, REPLACEMENT OR REFUND OF THE DEFECTIVE OR NON-CONFORMING PRODUCT.

GOVERNING LAW: This Limited Warranty shall be governed by the laws of the state of California.

Some states do not allow exclusion or limitation of incidental or consequential damages, or limitations on how long an implied warranty lasts, so the foregoing limitations and exclusions may not apply.  This limited warranty provides specific legal rights and the product owner may also have other rights which vary from state to state.

Wichtige Sicherheitshinweise

- Bitte lesen Sie sich diese Hinweise sorgfältig durch.

- Heben Sie diese Anleitung für den spätern Gebrauch auf.

- Vor jedem Reinigen ist das Gerät vom Stromnetz zu trennen.  Vervenden Sie keine Flüssig- oder Aerosolreiniger.  Am besten dient ein angefeuchtetes Tuch zur Reinigung.

- Um eine Beschädigung des Gerätes zu vermeiden sollten Sie nur Zubehörteile verwenden, die vom Hersteller zugelassen sind.

- Das Gerät is vor Feuchtigkeit zu schützen.

- Bei der Aufstellung des Gerätes ist auf sichern Stand zu achten.  Ein Kippen oder Fallen könnte Verletzungen hervorrufen. Verwenden Sie nur sichere Standorte und beachten Sie die Aufstellhinweise des Herstellers.

- Die Belüftungsöffnungen dienen zur Luftzirkulation die das Gerät vor Überhitzung schützt.  Sorgen Sie dafür, daß diese Öffnungen nicht abgedeckt werden.

- Beachten Sie beim Anschluß an das Stromnetz die Anschlußwerte.

- Die Netzanschlußsteckdose muß aus Gründen der elektrischen Sicherheit einen Schutzleiterkontakt haben.

- Verlegen Sie die Netzanschlußleitung so, daß niemand darüber fallen kann.  Es sollete auch nichts auf der Leitung abgestellt werden.

- Alle Hinweise und Warnungen die sich am Geräten befinden sind zu beachten.

- Wird das Gerät über einen längeren Zeitraum nicht benutzt, sollten Sie es vom Stromnetz trennen.   Somit wird im Falle einer Überspannung eine Beschädigung vermieden.

- Durch die Lüftungsöffnungen dürfen niemals Gegenstände oder Flüssigkeiten in das Gerät gelangen.  Dies könnte einen Brand bzw. Elektrischen Schlag auslösen.

- Öffnen Sie niemals das Gerät.  Das Gerät darf aus Gründen der elektrischen Sicherheit nur von authorisiertem Servicepersonal geöffnet werden.

- Wenn folgende Situationen auftreten ist das Gerät vom Stromnetz zu trennen und von einer qualifizierten Servicestelle zu überprüfen:

  - Netzkabel oder Netzstecker sint beschädigt.

  - Flüssigkeit ist in das Gerät eingedrungen.

  - Das Gerät war Feuchtigkeit ausgesetzt.

  - Wenn das Gerät nicht der Bedienungsanleitung ensprechend funktioniert oder Sie mit Hilfe dieser Anleitung keine Verbesserung erzielen.

  - Das Gerät ist gefallen und/oder das Gehäuse ist beschädigt.

  - Wenn das Gerät deutliche Anzeichen eines Defektes aufweist.

- Bei Reparaturen dürfen nur Orginalersatzteile bzw. den Orginalteilen entsprechende Teile verwendet werden.  Der Einsatz von ungeeigneten Ersatzteilen kann eine weitere Beschädigung hervorrufen.

- Wenden Sie sich mit allen Fragen die Service und Repartur betreffen an Ihren Servicepartner.  Somit stellen Sie die Betriebssicherheit des Gerätes sicher.

- Zum Netzanschluß dieses Gerätes ist eine geprüfte Leitung zu verwenden, Für einen Nennstrom bis 6A und einem Gerätegewicht gr ßer 3kg ist eine Leitung nicht leichter als H05VV-F, 3G, 0.75mm2  einzusetzen.

Trademarks

Copyright 1999 D-Link Corporation.  Contents subject to change without prior notice.  D-Link is a registered trademark of D-Link Corporation/D-Link Systems, Inc.  All other trademarks belong to their respective proprietors.

Copyright Statement

No part of this publication may be reproduced in any form or by any means or used to make any derivative such as translation, transformation, or adaptation without permission from D-Link Corporation/D-Link Systems Inc., as stipulated by the United States Copyright Act of 1976.

Content

Chapter 1 	Introduction	1	Features	1

Systems Requirements	2

Box Includes	2

Chapter 2	Commands	3

Typing Commands	3

AT Commands	3

S-Register Definitions	5

Introduction

The DFM-560I+ Voice/Fax/Data PCI Modem connects your computer to Internet, all kinds of BBS, and other fax modems. This User's Guide describes the features and AT Commands.

Features

This modem supports the following communication standards. ITU-T is known as CCITT.

Data

- ITU-TV.90

- K56flex

- ITU-T V.34, V.32bis, V.32, V.22bis, V.22

- Bell 103 & 212A

- V.42bis & MNP 5 (Data compression)

- V.42 & MNP2-4 (Error correction)

- V.80 (H.324 video conferencing interface)

Fax

- V.17 (14400bps FAX)

- V.29 (9600bps FAX)

- V.27ter (4800bps FAX)

Voice

- Voice/Audio mode

- Full-Duplex speakerphone

- System Requirements

System Requirements

- WindowsTM  95, WindowsTM 98,

- WindowsTM 2000, WindowsTM NT 4.0

- 16 MB RAM

Box Includes

- The box contains the following items:

- One 56K Internal Modem

- One Phone Line Cable

- One CD with Drivers

- One User's Manual

- One Installation Guide

If any of these items are missing or damaged, please contact your dealer or sales representative for assistance.

Commands

Most people use the communication software programs to tell modems what to do. Therefore, you may not use the commands in this chapter. However, if you prefer to communicate with your modem directly, you can type the commands described below.

Here describes how to work in the terminal mode.

Typing Commands

- Use the BACKSPACE key to delete typing errors.

- Every command (except A/ and +++) must begin with the AT or at prefix and be entered by pressing the <Enter> key. For example, to execute the V command, you would type ATV and press the <Enter> key.

- When you see an n, replace the n with one of the letter or numeric

- options list§ed for that command. For example, for the En command, you might type ATE1.

AT Commands

A	Go off-hook and attempt to answer  a call.

&Cn				Control Carrier Detect (CD) Signal

&C0				CD override

&C1     Normal CD operations

Dn       							Dial

0-9 				DTMF digits 0 to 9

P 	Tone dial

W 	Wait for second dial tone

^ 	Toggles calling tone enable/disable

&Dn 	DTR Option

&D0 	Ignore an on-to-off transition of  DTR

&D1 	Switch to on-line command mode without disconnection

&D2  	Normal DTR operations.

&F 	Load Factory Configuration(Profile)

Hn 	Disconnect (Hang-up)

H0 	Hang up (go on-hook)

H1	 Go off-hook

In	Identification

I0 	Report product code

I2 	Report "OK" or "ERROR"

I3 	Report driver revision

I4 	Report OEM defined identifier string

Mn 	Speaker Control

M0	Speaker is always off

M1 	Speaker ON until CONNECT

M2 	Speaker is always on

M3 	Speaker off during dialing and receiving carrier and turn

speaker on during answering.

On 				Return to On-Line Data Mode

O0 	Go on-line

O1 	Go on-line and retrain

S-Register Definitions

Sr=n 	Set Register r to n

Sr?	Display contents of S-Register

S0 	Number of Rings to Auto-Answer

The number of rings the modem waits for before it auto

answers.

Range    	0-255 (rings)

Default  	0 (Country dependent)

S6 	Wait Time before Blind Dialing

The time to pause after off-hook before blind dialing.

Range 	2-255

Default 	2 (Seconds,Country dependent)

S7 	Waiting Time before Carrier Detect

The time to wait for a carrier from the remote

modem before hanging up.

Range 					1-255

Default 	50 (seconds,Country dependent)

S8 	Pause Time For Dial Delay

The time to pause for the pause dial modifier, "Comma".

Range 		0-255

Default 	2 (seconds)

S9 	Carrier Detect Response Time

The time a signal is detected an qualified as a carrier.

This timing lets your modem ignore spurious signals

that are the same frequency as the carrier. Higher S9

values reduce the chance of a carrier being detected.

Range 	1-255

Default 	6 (0.6 seconds)

S10 	Lost Carrier To Hang Up Delay

The time the modem waits before hanging up for carrier loss.

Range 			1-255

Default 	14 (1.4 seconds)

S11 	DTMF Tone Duration

The time for DTMF tone dialing and the time between the tone

spacing.

Range 	50-255

Default	 95 (ms, Country dependent )

Offices

AUSTRALIA	D-LINK AUSTRALASIA

Unit 16, 390 Eastern Valley Way, Roseville, NSW 2069, Australia

TEL: 61-2-9417-7100  FAX: 61-2-9417-1077

TOLL FREE: 1800-177-100 (Australia), 0800-900900 (New Zealand)

URL: www.dlink.com.au  E-MAIL: support@dlink.com.au, info@dlink.com.au

CANADA	D-LINK CANADA

2180 Winston Park Drive, Oakville, Ontario L6H 5W1 Canada

TEL: 1-905-829-5033  FAX: 1-905-829-5095  BBS: 1-965-279-8732  FREE CALL: 1-800-354-6522

URL: www.dlink.ca   E-MAIL: techsup@dlink.ca

CHILE	D-LINK SOUTH AMERICA

Isidora Goyenechea #2934 of.702, Las Condes, Santiago, Chile

TEL:	56-2-232-3185  FAX: 56-2-2320923  URL: www.dlink.cl

CHINA	D-LINK CHINA

2F., Sigma Building, 49 Zhichun Road, Haidian District, 100080 Beijing, China

TEL: 86-10-88097777  FAX: 86-10-88096789

URL: www.dlink.com.cn

DENMARK	D-LINK DENMARK

Naverland 2, DK-2600 Glostrup, Copenhagen, Denmark

TEL:45-43-969040  FAX:45-43-424347  URL: www.dlink.dk

E-MAIL: info@dlink.dk

EGYPT	D-LINK MIDDLE EAST

7 Assem Ebn Sabet Street, Heliopolis Cairo, Egypt

TEL: 202-2456176  FAX: 202-2456192  URL: www.dlink-me.com

E-MAIL: support@dlink-me.com

FRANCE	D-LINK FRANCE

Le Florilege #2, Allee de la Fresnerie

78330 Fontenay Le Fleury France

TEL: 33-1-30238688  FAX: 33-1-3023-8689  URL: www.dlink-france.fr

E-MAIL: info@dlink-france.fr

GERMANY	D-LINK Central Europe/D-Link Deutschland GmbH

Schwalbacher Stra_e74

65760 Eschborn, Germany

TEL: 49-6196-77990  FAX: 49-6196-7799300

URL: www.dlink.de  BBS: 49-(0)6192-971199 (Analog) 49-(0)6192-971198 (ISDN)

INFO LINE: 00800-7250-0000 (toll free)  HELP LINE: 00800-7250-4000 (toll free)

REPAIR LINE: 00800-7250-8000

INDIA	D-LINK INDIA

Plot No.5, Kurla-Bandra Complex Road,

Off Cst Road, Santacruz (E), Bombay - 400 098 India

TEL: 91-22-652-6696  FAX: 91-22-652-8914  URL: www.dlink-india.com

E-MAIL: service@dlink.india.com

ITALY	D-LINK MEDITERANEO SRL

Via Nino Bonnet No. 6/b, 20154 Milano, Italy

TEL: 39-02-2900-0676  FAX: 39-02-2900-1723    URL: www.dlink.it

E-MAIL: info@dlink.it

JAPAN	D-LINK JAPAN

10F, 8-8-15 Nishi-Gotanda, Shinagawa-ku, Tokyo 141 Japan

TEL: 81-3-5434-9678  FAX: 81-3-5434-9868  URL: www.d-link.co.jp

RUSSIA	D-LINK RUSSIA

Michurinski Prospekt 49, 117607 Moscow, Russia

TEL: 7-095-737-3389, 7-095-737-3492  FAX: 7-095-737-3390

SINGAPORE	D-LINK INTERNATIONAL

1 International Business Park, #03-12 The Synergy, Singapore 609917

TEL: 65-774-6233  FAX: 65-774-6322

URL: www.dlink-intl.com  E-MAIL: info@dlink.com.sg

S. AFRICA	D-LINK SOUTH AFRICA

Unit 2, Parkside 86 Oak Avenue

Highveld Technopark Centurion, Gauteng, Republic of South Africa

TEL: 27(0)126652165  FAX: 27(0)126652186

SWEDEN	D-LINK SWEDEN

P.O. Box 15036, S-167 15 Bromma Sweden

TEL: 46-(0)8564-61900  FAX: 46-(0)8564-61901  E-MAIL: info@dlink.se

URL: www.dlink.se

TAIWAN	D-LINK TAIWAN

2F, No. 119 Pao-Chung Road, Hsin-Tien, Taipei, Taiwan,

TEL: 886-2-2910-2626  FAX: 886-2-2910-1515  URL: www.dlinktw.com.tw

U.K.	D-LINK EUROPE

4th Floor, Merit House, Edgware Road, Colindale, London,  NW9 5AB, U.K.

TEL: 44-20-8731-5555  FAX: 44-20-8731-5511

URL: www.dlink.co.uk  E-MAIL: info@dlink.co.uk

U.S.A.	D-LINK U.S.A.

53 Discovery Drive, Irvine, CA 92618  USA

TEL: 1-949-788-0805  FAX: 1-949-753-7033  INFO LINE: 1-800-326-1688

BBS: 1-949-455-1779, 1-949-455-9616

URL: www.dlink.com  E-MAIL: tech@dlink.com, support@dlink.com

Registration Card

Print, type or use block letters.

Your name: Mr./Ms	_____________________________________________________________________________

Organization: ________________________________________________	Dept. ____________________________

Your title at organization:	________________________________________________________________________

Telephone:	_______________________________________ Fax:________________________________________

Organization's full address:	______________________________________________________________________

____________________________________________________________________________________________

Country:	_____________________________________________________________________________________

Date of purchase (Month/Day/Year):	_______________________________________________________________

| Product Model | Product Serial No. | * Product installed in type of computer (e.g., Compaq 486) | * Product installed in computer serial No. |
| --- | --- | --- | --- |

(* Applies to adapters only)

Product was purchased from:

Reseller's name:	______________________________________________________________________________

Telephone:	_______________________________________ Fax:________________________________________

Reseller's full address:	_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

Answers to the following questions help us to support your product:

1.	Where and how will the product primarily be used?

Home Office Travel Company Business Home Business Personal Use

2. How many employees work at installation site?

1 employee 2-9 10-49 50-99 100-499 500-999 1000 or more

3. What network protocol(s) does your organization use ?

XNS/IPX TCP/IP DECnet Others_____________________________

4. What network operating system(s) does your organization use ?

D-Link LANsmart Novell NetWare NetWare Lite SCO Unix/Xenix PC NFS 3Com 3+Open

Banyan Vines DECnet Pathwork Windows NT Windows NTAS Windows '95

Others__________________________________________

5. What network management program does your organization use ?

D-View HP OpenView/Windows HP OpenView/Unix SunNet Manager Novell NMS

NetView 6000 Others________________________________________

6. What network medium/media does your organization use ?

Fiber-optics Thick coax Ethernet Thin coax Ethernet 10BASE-T UTP/STP

100BASE-TX 100BASE-T4 100VGAnyLAN Others_________________

7. What applications are used on your network?

Desktop publishing Spreadsheet Word processing CAD/CAM

Database management Accounting Others_____________________

8. What category best describes your company?

Aerospace Engineering Education Finance Hospital Legal Insurance/Real Estate Manufacturing

Retail/Chainstore/Wholesale Government Transportation/Utilities/Communication VAR

System house/company Other________________________________

9. Would you recommend your D-Link product to a friend?

Yes No Don't know yet

10.Your comments on this product?

__________________________________________________________________________________________

__________________________________________________________________________________________