Installing SM56 Software on Windows NT 4.0

Important -- The SM56 software for Windows NT is Data/Fax only, there is no speakerphone or TAM supported.

Windows NT 4.0 does not fully support plug and play (PnP). Therefore, after installing the modem board in the computer and booting, the operating system will not automatically recognize that new hardware is present. You must manually install the modem as follows:

Step 1 -- Install a COM port for the modem

You must install a COM port for the SM56 modem as follows:

1. Open the Windows NT Control Panel.

2. Double click the Ports icon. The Ports window opens.

3. Select Add.

4. Select a COM port number (in the range 1-4 preferably) that is not already installed, and allocate to it the standard resources as follows:

COM Port		IO Address		IRQ

1			3F8			4

2			2F8			3

3			3E8			4

4			2E8			3

5. When your selections are complete click OK.

6. You will be prompted to restart the computer, do not do so yet. Select Don't Restart Now.

7. Close the Ports window.

Step 2 -- Install the SM56 modem software

1. Open the Windows NT Control Panel.

2. Double click the Modems icon....the Install New Modem dialog appears.

3. Click the "Don't detect my modem; I will select it from a list" check box. Then click Next.

4. Click the Have Disk button.

5. Browse to the Windows NT SM56 modem install software. Click misgmodm.inf to highlight it. Then click Open, then click OK.

6. Select the Motorola SM56 Modem (note that the product name listed will vary depending on which modem you have -- Data/Fax/, Data/Fax/TAM, etc.) from the list. Click Next.

7. Click the port number selected in Step 1 above to highlight it as the COM port for the modem. Then click Next. The wizard will now copy all the necessary files and install the SM56 modem on your PC.

8. When complete, restart the PC to activate and start the COM port and SM56 modem devices. You are now ready to use the modem.

SM56 Software Upgrade Procedure (Windows NT 4.0)

1. Make a backup of your current modem software version -- copy all the files in \windows\Motorola\MotsetNT folder.

2. Un-install your current software -- run the SM56 Modem Uninstall from Add/Remove Programs in the WindowsControl Panel.

3. Delete the Motorola SM56 modem INF files from \Program Files\Inf\Other

4. Restart the computer and perform a complete SM56 modem install (including COM port) again using the new drivers.

Recovering From an Unsuccessful Upgrade (Windows NT 4.0)

2. Un-install your current software -- run the SM56 Modem Uninstall from Add/Remove Programs in the Windows Control Panel.

3. Delete the Motorola SM56 modem INF files from \Program Files\Inf\Other.

4. Restart the computer and perform a complete SM56 modem install (including COM port) again, this time using your previous software version backup files (see Step 1 of NT 4.0 upgrade procedure).

Troubleshooting

If there is a problem making or receiving a call or transmitting data, and your communications application does not explain the problem, check the following list of symptoms and tips.

- The modem installs at COM 5 (or higher)

Some communications applications do not communicate with a COM port higher than COM4. If the SM56 installs on COM5 or higher, force the modem to a lower COM port as follows.

1.   Open the SM56 Control Panel application.

2.   Select the Advanced tab.

3.   Click on the radio button for the desired COM port. Note that unavailable COM ports are grayed out and not selectable.

- Windows 95/98 DOS box applications do not work with the modem

By default DOS box support is disabled after installing the SM56 modem. You must enable it via the Advanced tab in the SM56 Control Panel application (Start->Settings->Control Panel, then double-click the SM56 Modem icon). Make sure the DOS box check box is checked. If not, click it to check it, exit the SM56 Control Panel application and restart the computer. You must restart the computer to complete DOS box enabling.

If the Enable DOS box check box is already checked (i.e. enabled), and DOS box application still do not work, try:

1.   Open the Control Panel.

2.   Double click the System icon.

3.    Expand the Modem branch.

4.   Double click the Motorola SM56 modem.

5.   Select the Modem tab.  Make a note of the COM port number.

6.   Select the Resources tab. Make a note of the IRQ number listed.

7.   For DOS support to operate correctly, the SM56 modem cannot occupy the standard IRQ for the COM port number.

8.   Uncheck the Use Automatic Resources check box.

Double click on the Interrupt Request label and change the IRQ to a different number that is not in conflict with another device. (If there is no free IRQ:  free one, or change other device IRQ levels to free a non-standard IRQ).

- There is no dial tone

1.   Ensure that the telephone cable is securely connected at both ends.

2.   Unplug the telephone line cable from the computer, and connect it directly to a telephone from the wall outlet. Check for a dial tone. If there is none, the problem is in the telephone line or system. Call the service provider.

- The modem cannot complete a connection to another modem

1.   Ensure that your modem is dialing the correct number. Ensure that you've specified the correct area code, if one is required.

2.  Determine whether the remote modem is correctly configured to communicate with yours.

- The modem does not answer incoming calls

1.   Ensure that the automatic answer parameter is set to one of the enabled options, using the ATS0 command (ATS0=1 to answer after one ring, ATS0=2 to answer after two rings, and so on).

2.   Ensure that no other devices, such as fax or answering machines, are answering calls before the modem does.

- The modem disconnects while transmission is in progress

1.   Ensure that the telephone cable is securely connected at both ends.

2.   Ensure that call-waiting is disabled. In most areas, the command *70 or  #70 disables call-waiting. Check with your telephone company for the correct key sequence.  (With call-waiting, the incoming call's click sound may be disrupting your call.)

- Data is not transmitted or received for unusually long periods of time

1.   Re-dial the call.  (The telephone line connection may be poor.)

2.   Try another ISP number, the server could be busy.

3.   Call your ISP and find out if they are having server problems or any suggestions why data transmission is lethargic.

- The computer runs slower than usual

1.   Close any open applications that you are not using.

- You cannot enter tone selections successfully when calling tone-driven applications

1.   When dialing a remote system that requires you to enter selections using the telephone keys, such as a voice-mail       depot or bank-account information provider, you can lengthen the duration of the tones your modem sends, so that the            remote system can detect them better.  To adjust DTMF tone length, use the AT+VTDn command, where n specifies       the tone duration.

- The modem does not respond to AT commands

1.   Ensure that your communications software is configured to use the same COM port as the modem's COM port.

2.   Reset modem parameters to default options by entering AT&F; then re-enter custom options.

3.  	SM56 builds after Build 50 require setting the S46 register to 23 before AT&T1 will perform the Local Analog Loopback (LAL) hardware test.

- The modem responds to commands, but they do not appear on the screen

Ensure that the local display echo option is enabled by entering ATE1.

- You've installed a new peripheral device; now the modem does not work

1.   In the Windows desktop tray, select Start.

2.   Select Help. The Windows Help Topics window appears.

3.   Select the Contents tab.

4.   Select If You Have a Hardware Conflict.

5.   A series of troubleshooting actions appears.  Follow the appropriate sequence.

- You hear feedback (noise) from the PC sound system (Data/Fax/Speakerphone modems)

1.   Position the speakers at least three feet (1 M) away from the microphone.

2.   Ensure that the speakers are facing away from the microphone.

3.   Turn down the speaker volume.

4.   Speak into the microphone at a distance of at least 12 inches (30 cm) from your mouth. Minimize background noise.

5.   If there is still feedback, turn off the microphone boost, under the volume control panel.

- The modem connects; then meaningless characters appear

1.   Open the Control Panel. Double click the Modem icon.

2.   Select the Motorola SM56 modem

3.   Click on Properties.

4.   Select Connection.

5.   Click on Advanced.

6.   Check the Use Error Control-Required to Connect box.

- The modem cannot connect; the Error Control option is selected

The modem may be connecting at a rate higher than appropriate for the line conditions.

1.   Use the AT%B command to limit the SM56 maximum connection rate. (For a list of AT commands, refer to the on-line User's Guide.)

2.   Lower the rate, using AT commands, until the problem is corrected. You can add AT commands to do this; refer to the next section .

- How to Add AT Commands

1.   Open the Control Panel.

2.   Double-click the Modem icon.

3.   Select the Motorola SM56 modem.

4.   Click on Properties.

5.   Select Connection.

6.   Click on Advanced.

7.   In the Extra Settings box, add commands as needed. You do not need to enter the AT prefix, but it will not hurt if you do.