- SoftModem

Product Release Notes
(Linux V4.06.06.03)

- CONEXANT PROPRIETARY INFORMATION

DISSEMINATION OR USE OF THIS INFORMATION IS NOT PERMITTED WITHOUT THE WRITTEN PERMISSION OF CONEXANT SYSTEMS, INC.

December, 2001

- Table of Contents

1. iNTRODUCTION	3

2.  APPLICABLE DOCUMENTS	3

3.  SUPPORTED DEVICES	3

4.  VERSION NUMBERS	3

5.    CONFIGURATION INFORMATION	4

5.1  Device IDs	4

5.2  INF files	4

6.   DRIVER INSTALLATION	5

7.  CURRENT FEATURES AND FIXED ISSUES	7

7.1	Key Features of Current Release	7

7.2	Open Issues	7

# 1. iNTRODUCTION

The document describes the SoftModem Linux V4.06.06.03 release (A Beta release). The release supports Linux 2.2 and 2.4 kernels. The source kit is divided into two parts, pre-compiled kernel independent modules, and source code for building kernel-specific modules on the target machine. The installation of the drivers is covered in the installation section of this document.

The driver features V.90 Data modem only. All data AT commands are implemented. Fax and Voice are not are supported at this time. V.92 is not featured in this release.

# 2.  APPLICABLE DOCUMENTS

HSF/SmartHSF, HSFi/SmartHSFi, and SmartMC Modem Data Sheets

HSF/SmartHSF, HSFi/SmartHSFi, and SmartMC Modem Designer’s Guides

HSF/SmartHSF, HSFi/SmartHSFi, and SmartMC Reference Designs

HSF/HCF AT Command Set Reference Manual

KPPP Configuration Application Note

# 3.  SUPPORTED DEVICES

| Reference Board Number | Latest Revision Number |
| --- | --- |
| HSF Discrete DAA RD00-D480-031 | Rev. 2 |
| SmartHSF RD00-D840-581 |  |
| Churchill RD00-D180-031 |  |
| SmartHSFi |  |
| Athens |  |
| InterSmartMC |  |

# 4.  VERSION NUMBERS

| Current release | V.4.06.06.03 | 12/13/01 |
| --- | --- | --- |

# 5.    CONFIGURATION INFORMATION

### 5.1  Modem types and installation information files

The release supports a wide variety of deployed modem boards with Conexant modem chips. For Linux modem driver installation the correct modem type and INF file should be found. Vendor ID and Device ID are used to identify modem type.

Historically modem drivers were written for Windows® operating system where INF files are used for installation. For Linux modem drivers installation scripts are using files that have the same format as INF files for Windows® operating system.

The following Conexant chips are supported:

| Modem platform | Vendor ID | Device ID | INF file |
| --- | --- | --- | --- |
| HSF | 14F1 | 2013, 2014, 2015, 2016 | linux_hsf.inf |
| SmartHSF | 14F1 | 2043, 2044, 2045, 2046 | linux_smarthsf.inf |
| HSFi/SmartHSFi | 14F1 | 2F00 | linux_hsfi.inf |
| SmartMC on Intel AC-Link chip | 8086 | 2416 | linux_intel_smartmc.inf |
| Athens | 14F1 | 1631 | linux_athens.inf |

Each INF file contains support for the following countries:

| Australia | Greece | Malaysia | South Africa |
| --- | --- | --- | --- |
| Austria | Hong Kong | Mexico | Spain |
| Belgium | Hungary | Netherlands | Sweden |
| Bulgaria | Iceland | New Zealand | Switzerland |
| Canada | India | Norway | Taiwan |
| China | Indonesia | Philippines | Thailand |
| Croatia | Ireland | Poland | Turkey |
| Czech Republic | Israel | Portugal | UK |
| Denmark | Italy | Russia | United Arab Emirates |
| Finland | Japan | Singapore | USA |
| France | Korea | Slovakia |  |
| Germany | Luxembourg | Slovenia |  |

# 6.   DRIVER INSTALLATION

Prepare to install the drivers using the following sequence:

- Log as root on the target Linux machine.

- Find out the Vendor ID and Device ID of the PCI modem used and remember them (use the lspci command).

- Copy the modem driver package file to the directory where you wish to extract it.

- Extract the modem driver files from tar file

tar -xpvzf <tar-file-name>

This creates under the current directory the directory having the same name as the <tar-file-name> file, but without the .tar.gz suffix and extracts all files into it.

Example:

tar -xpvzf hsflinmodem-4.06.06.03.tar.gz

This creates the hsflinmodem-4.06.06.03 directory and extract modem driver files into it.

- Change directory to the newly created directory

Example:

cd  hsflinmodem-4.06.06.03

- Update the Vendor ID and Device ID in the .inf file corresponding to the modem board used:

Check the list of the Microsoft Windows style PnP strings under the [Generic] section and if no string corresponds to your PCI modem Vendor ID and Device ID, add the appropriate string to the [Generic] section.

Example: If the Vendor ID is 1234 and the Device ID is 5678, add the line in red to the [Generic] section. The SUBSYS part is redundant for Linux modem drivers and can be omitted:

[Generic]

%HSFModem%  = ModemX, PCI\VEN_14F1&DEV_2013&SUBSYS_201314F1

%HSFModem%  = ModemX, PCI\VEN_14F1&DEV_2014&SUBSYS_201414F1

%HSFModem%  = ModemX, PCI\VEN_14F1&DEV_2015&SUBSYS_201514F1

%HSFModem%  = ModemX, PCI\VEN_14F1&DEV_2016&SUBSYS_201614F1

%HSFModem%  = ModemX, PCI\VEN_1234&DEV_5678

The INF files are under the directory “inf”.

After the INF file has been configured, follow these steps to build/load the modem:

- CD to the directory in step 4 above, for example cd hsflinmodem-4.06.06.03

- Run: make install.

- Run: hsfconfig (If you have doubts on what to answer on one of the questions, press <enter> for the default answer).

- During the installation you’ll be asked to select your country, to change it after the installation is done, you can use at+gci .

- The installation should be done only once (no need to install it again every time the machine is restarted).

The modem may now be tested with Minicom: minicom –s

Note: minicom application isn’t a part of the standard linux installation.

Please see app note: KPPP Configurate Application Note for instructions on how to set up dial up networking in KDE.

Read the Install file in order to use the right installation script.

# 7.  CURRENT FEATURES AND FIXED ISSUES

### 7.1	Key Features of Current Release

| 1. | V.90 Data |
| --- | --- |

### 7.2	Open Issues

| 1. | V.92, Voice and Fax are not supported. |
| --- | --- |

Information provided by CONEXANT SYSTEMS, INC. is believed to be accurate and reliable. However, no responsibility is assumed by CONEXANT for its use, nor any infringement of patents or other rights of third parties which may result from its use. No license is granted by implication or otherwise under any patent rights of CONEXANT other than for circuitry embodied in CONEXANT products. CONEXANT reserves the right to change circuitry at any time without notice. This document is subject to change without notice.

K56flex is a trademark of CONEXANT SYSTEMS, INC. and Lucent Technologies.

Microsoft and Windows are registered trademarks of Microsoft Corporation.

CONEXANT and “What’s Next in Communications Technologies” are registered trademarks of CONEXANT SYSTEMS, INC.

1999-2002, CONEXANT SYSTEMS, INC.

Printed in U.S.A.

All Rights Reserved

- Worldwide Headquarters

4311 Jamboree Road,

P.O. Box C
Newport Beach, CA 92658-8902

Phone:	(949) 483-4600
Fax:	(949) 483-6375

For more information:
Call  1-800-854-8099

International information:
Call  1-949-483-6996

URL Address:
http://www.conexant.com

E-Mail Address:
literature@conexant.com

- REGIONAL SALES OFFICES

- US Northwest/Pacific Northwest

US Northwest Office
3600 Pruneridge Avenue
Suite 100
Santa Clara, CA  95051
Phone:	(408) 249-9696
Fax:	(408) 249-7113

- US Los Angeles

1000 Business Center Circle
Suite 215
Thousand Oaks, CA  91320
Phone:	(805) 376-0559
Fax:	(805) 376-8180

- US Southwest

4350 Von Karman Avenue
Suite 400
Newport Beach, CA  92660
Phone:	(949) 483-9119
Fax:	(949) 483-9090

- US North Central

Two Pierce Place
Chancellory Park
Suite 810
Itasca, IL  60143
Phone:	(630) 773-3454
Fax:	(630) 773-3907

- US South Central

17304 Preston Road
Suite 970
Dallas, TX 75252

Phone:	(972) 733-0723

Fax:	(972) 407-0639

- US Northeast

239 Littleton Road
Suite 4A
Westford, MA  01886
Phone:	(508) 692-7660
Fax:	(508) 692-8185

- US Southeast

3500 Parkway Lane, Suite 415
Norcross, GA  30092
Phone:	(770) 246-8283
Fax:	(770) 246-0018

- US Florida/South America

One Prestige Place
2600 McCormick Drive
Suite 350
Clearwater, FL  33759
Phone:	(813) 799-8406
Fax:	(813) 799-8306

- US Mid-Atlantic

Princeton Pike Corporate Center
993 Lenox Drive, Suite 200
Lawrenceville, NJ  08648
Phone:	(609) 219-7462
Fax:	(609) 895-2666

- European Headquarters

Les Taissounieres B1
1680 Route des Dolines

BP 283
06905 Sophia Antipolis Cedex
France
Phone:	(33) 4 93 00 33 35
Fax:	(33) 4 93 00 33 03

- Europe Central

Paul-Gerhardt-Allee 50 a
81245 München
Germany
Phone:	(49-89) 829-1320
Fax:	(49-89) 834-2734

- Europe Mediterranean

Via G. Di Vittorio, 1
20017 Mazzo Di Rho (MI)
Italy
Phone:	(39 2) 93179911
Fax	(39 2) 93179913

- Europe North

Berkshire Court
Western Road
Bracknell
Berkshire RG12 1RE
England
Phone:	44 (0) 1344 486444
Fax:	44 (0) 1344 486555

- Europe North (Satellite)

11 Galgaley Haplada Street
P.O. Box 12660
Herzlia 46733
Israel
Phone: 	(972) 9 9524004
Fax:	(972) 9 9602308

- Europe South

Tour GAN
Cedex 13
92082 Paris La Défense 2
France
Phone:	(33) 1 49 06 39 80 
Fax:	(33) 1 49 06 39 90

- APAC Headquarters

1 Kim Seng Promenade
#09-01 East Tower
Great World City

Singapore 237994
Phone:	(65) 737-7355
Fax:	(65) 737-9077

- Australia

Suite 603, 51 Rawson Street
Epping, NSW 2121
Australia
Phone:	(61-2) 9869 4088
Fax:	(61-2) 9869 4077

- China

Shanghai Representative Office
LT Square Building, Suite 3002
500 Chengdu North Road
Shanghai 200003 P.R.C.
Phone:	86-21-6361-2515
Fax:	86-21-6361-2516

- Hong Kong

13th Floor, Suites 8-10,
Harbour Centre
25 Harbour Road
Wanchai,
Hong Kong
Phone:	(852) 2 827-0181
Fax:	(852) 2 827-6488

- India

Regional Office - South Asia
Capital Trust House
47 Community Centre
Friends Colony
New Delhi - 110 065
India
Phone:	(91-11) 692-4780
Fax:	(91-11) 692-4712

- Korea

Room No. 1508
Korea Textile Centre Building
944-31, Daechi-3dong
Kangnam P.O. Box 2037
Kangnam-ku
Seoul
Korea
Phone:	(82-2) 565-2880
Fax:	(82-2) 565-1440

- Taiwan Headquarters

Room 2808 International Trade Bldg.
333, Keelung Road, Section I
Taipei,
Taiwan
10548 ROC
Phone:	(886-2) 2720-0282
Fax:	(886-2) 2757-6760

- Japan Headquarters

Shimomoto Bldg.
1-46-3 Hatsudai, Shibuya-ku
Tokyo, 151
Japan
Phone:	(81-3) 5371 1520
Fax:	(81-3) 5371 1501

- SOUD981210