NetQom

56K USB Modem

User’s Guide

Rev. A2 (September, 2001)

### Table of Contents

### USB MODEM	1

### Table of Contents	2

### Chapter 1	3

### Introduction	3

### Features	3

### System Requirements	4

### Package checklist	4

### Chapter 2	5

### Installation	5

### Hardware Installation	5

### Installation Procedures for Windows 98 / ME / 2000	7

### Chapter 3 	9

### Commands	9

### Typing Commands	9

### AT Commands	9

### S-register Definitions	11

### Introduction

The Voice/Fax/Data USB Modem connects your computer to Internet, all kinds of BBS, and other fax modems.

This manual describes the features, installation procedures and AT commands.

# Feature

This modem supports the following communication standards. ITU-T is known as CCITT

# System Requirements

- USB Port

- Windows 98 above

- 16 MB RAM

# Package Checklist

The package contains the following items:

- One USB modem

- One piece of phone cable

- One CD

- The USB modem user’s manual

- USB Cable

If any of these items are missing or damaged, please

contact your dealer or sales representative for assistance.

### Installations

Here describes how to install fax modem.

# Hardware Installation

1. Plug the device terminal of the USB cable into the

connector on the back of the modem.

2.Plug the other end of this cable into the USB port

on the back of your computer .

3.Plug one end of the phone cable into the modem’s

LINE jack. Plug the other end into the phone outlet.

4.To use telephone and the modem on the same line,

plug one end of the optional phone cable into the

PHONE jack on the modem; plug the other end

into the phone. Lift the telephone handset and listen

for a DIAL TONE to check the connection.

# Installation Procedures for Windows 98 / ME / 2000

1.When hardware installation complete, Windows will

auto detect your USB modem and show you the message.

Click Next

2. Select the Search for the best driver for your device

(Recommended) option, then click Next

3. Place the CD containing driver for Windows into your

CD ROM drive. Specify the location of the modem driver,

e.g. e:\driver\win98, e:\driver\winme, e:\driver\win2000
Click Next

4.Windows is now ready to install the the best driver

for Conexant USB HCF V.90 Data/ Fax/ Voice Moedm,

click Next

5. To finish the installation, click Finish

Commands

Most people use the communication software programs to tell modems what to do. Therefore, you may not use the commands in this chapter. However, if you prefer to communicate with your modem directly,

you can type the commands described below.

# Typing Commands

- Use the BACKSPACE key to delete typing errors.

- Every command (except A / and +++) must begin with

The AT or at prefix and be entered by pressing the

<Enter> key. For example, to execute the V command,

you would type ATV and press the <Enter> key.

- When you see an n, replace the n with one of the

letter or numeric options listed for that command. For

example, for the En command, you might type ATE1

- All defaults are based on the &F HardwareFlow Control

template load in NVRAM when the modem is shipped.

# AT Commands

A /  Re-executes the last issued command. Used

mainly to redial.

A  Go off-hook and attempt to answer a call.

ATS?  Read Selected S-Register.

This command reads and displays the selected S-Register.

An S-Register can be selected by using the ATSn command.

## Dn  D i a l

0-9  DTMF digits 0 to 9

P  Pulse (rotary) dial

T  Tone dial

W  Wait for second dial tone (X3 or higher) ; linked to S6 register

^   Toggles calling tone enable/disable

## En  Command Echo

E0  Disable command echo

E1  Enable command echo

Hn  Disconnect (Hang-up)

H0  Hang up (goes on-hook)

H1  Go off-hook

In   Identification

I0   Report product code

I1   Report “OK”

I2   Report “OK” or “ERROR”

I3   Report firmware revision

I4   Report OEM defined identifier string

I6   Report modem data pump model and internal code revision

Mn  Speaker Control

M0  Speaker is always off

M1  Speaker ON until CONNECT

M2  Speaker is always on

M3  Speaker off during dialing and receiving carrier and turn speaker on duringanswering.

On  Return to On-Line Data Mode

O0  Go on-line

O1  Go on-line and retrain

P   Set Pulse Dial (for phone line that does not support touch-tone dialing)

## S r=n Set Register r to n

Sn?  Display contents of S-Register n

Vn   Result Code Form

V0   Numeric codes

V1   Verbal codes

Zn   Soft Reset and Restore Profile

Z0   Restore stored profile 0 after warm reset

Z1   Restore stored profile 1 after warm reset

&C n Control Carrier Detect (CD) Signal

&C0  CD override

&C1  Normal CD operations

&Dn  DTR Option

&D0  Ignore an on-to-off transition of DTR.

&D1  Switch to on-line command mode without disconnection

&D2  Normal DTR operations.

&D3  Modem re-initialized. &Y determines which profile is loaded

## Vn    Result Code Form

V0   Numeric codes

V1   Verbal codes

Zn   Soft Reset and Restore Profile

Z0   Restore stored profile 0 after warm reset

Z1   Restore stored profile 1 after warm reset

&Cn  Control Carrier Detect (CD) Signal

&C0  CD override

&C1  Normal CD operations

&Dn  DTR Option

&D0  Ignore an on-to-off transition of DTR.

&D1  Switch to on-line command mode without disconnection

&D2  Normal DTR operations.

&D3  Modem re-initialized. &Y determines which profile is loaded

&Fn  Load Factory Configuration(Profile)

&F0  Restore factory configuration 0

&F1  Restore factory configuration 1

&Wn  Store Current Configuration

&W0 Store the current configuration as profile 0

&W1 Store the current configuration as profile 1

+MS Select Modulation

This command selects the modulation, optionally enables or disables automode, and optionally specifies the lowest and highest connection rates using one to four subparameters.

The format is:

AT+MS= <carrier>,<automode>,  <min_tx_rate>,<max_tx_rate>, <min_rx_rate>,<max_rx_rate>

For example

AT+MS=K56, 1, 75, 33600, 75, 56000

Where

K56: select K56FLEX

1: automode enabled

75: min_rate data speed 300 bps

56000: max_rate data speed 56000 bps

If you want to select V.34 28800 bps only, please insure AT+MS=V34, 0, 75, 28800, 75, 28800.

# S-Register Definitions

S0   Number of Rings to Auto-Answer

The number of rings the modem waits for before it auto answers.

Range 0-255 (rings)

Default 0

S6   Wait Time before Blind Dialing

The time to pause after off-hook before blind dialing.

Range 2-255 (U.S.)

Default 2 (Country dependent)

S7   Waiting Time before Carrier Detect

The time to wait for a carrier from the remote modem before hanging up.

Range 1-255

Default 50 (seconds)

## S8   Pause Time For Dial Delay

The time to pause for the pause dial modifier, “Comma”.

Range 0-255

Default 2 (seconds)

S9   Carrier Detect Response Time

The time a signal is detected an qualified as a carrier. This timing lets your modem ignore

spurious signals that are the same frequency as the carrier. Higher S9 values reduce the chance

of a carrier being detected.

Range 1-255

Default 6 (0.6 second)

## S10   Lost Carrier To Hang Up Delay

The time the modem waits before hanging up for carrier loss.

Range 1-255

Default 14 (1.4 seconds)

## S11   DTMF Tone Duration

The time for DTMF tone dialing and the time between the tone spacing.

Range 50-255

Default95 (0.95 second, country dependent)