# D-Link Corporation

MCNS DOCSIS Cable Modem

# Product External Specifications

Product Model Name:

DCM-100

Version: 0.80

07/24/2000

By: Sally Yeh

Product Planning Group

This document contains confidential proprietary information and is the property of D-Link Corporation. The contents of this document may not be disclosed to unauthorized persons without the written consent of D-Link Corporation.

### Product Description

The DCM-100 is a 2-way DOCSIS-compliant cable modem, providing customers with high-speed access to the Internet through the cable TV (CATV) network. It works on any cable system that complies with MCNS DOCSIS specifications and provides a high-bandwidth connection to the Internet that downloads web pages almost instantly, transfers files at incredible speeds and delivers real-time video and music with unsurpassed quality.

### Product Features

#### Full Functional MCNS/DOCSIS 1.0 compliant with:

- Automatic Signal Power Recognition/Adjustment

- Building MCNS MIB

- DES Encryption and Decryption

- Frequency Agility

- QPSK/16QAM Burst Modulator

- 64/256 QAM Receiver

- CPE Interface: Ethernet 10BASE-T

- Support Plug and Play

#### SNMP Remote Management

#### Static IP or DHCP IP Address Request, enable efficient management of IP allocation

### Specifications:

3.1 Downstream – Receiver

- Demodulation			64 QAM/ 256 QAM

- Data Speed			30.342Mbps (64 QAM)

42.88Mbps (256QAM)

- Frequency Range		91 MHz – 860 MHz

- Receive Level 		-15dBmV to +15dBmV

3.2 Upstream – Transmitter

- Data Speed 			0.3 to 5.1Mbps (QPSK)

0.6 to 10.2Mbps (16 QAM)

- Error Correction		Reed-Solomon

- Frequency Range		5MHz to 42MHz

- MAC Protocol			MCNS/DOCSIS 1.0 compliant

- Modulation			QPSK/ 16 QAM

- Transmission Level 	+8 to +55dBmV (16 QAM)

+8 to +58dBmV (QPSK)

3.3 Interface

- CPE Interface		10BASE-T Ethernet (RJ45)

- RF Interface		F-Type Female 75 ohm

### EMI certification

- FCC class B

- BSMI class B

### Safety Certifications:

CSA/ UL