Cable Modem

## MODEL NO.: DCM-1000  Rev.A1

PART NO.: DCM-1000..A1

This DOCSIS Cable Modem will provide you with high-speed access to the Internet through your cable TV (CATV) network. It works on any cable system that complies with MCNS DOCSIS specifications. Open your wide access gate into the Internet and you can enjoy the quality video, music, and mass information in just a click. Always-on connection makes your IP phone/fax or VPN (Virtual Private Network) connections more flexible and more convenient.

Product Features

The Cable Modem connection is many times faster than other Internet connections. Cable Modem transfer data in bursts and the maximum downstream data rate is 40Mbps. However, in the field, you can mostly you can have 4 ~ 10Mbps steady downstream data flow. Take a 10MB file download time as an example.

| MODEM SPEED/ TYPE |  | TRANSFER TIME |  |
| --- | --- | --- | --- |
| 9.6Kbps | Telephone Modem | 2.3 | Hours |
| 14.4Kbps | Telephone Modem | 1.5 | Hours |
| 28.8Kbps | Telephone Modem | 46 | Minutes |
| 56Kbps | Telephone Modem | 24 | Minutes |
| 128Kbps | ISDN Modem | 10 | Minutes |
| 1.54Mbps | T1 Connection | 52 | Seconds |
| 4Mbps | Cable Modem | 20 | Seconds |
| 10Mbps | Cable Modem | 8 | Seconds |

Transfer Rate for a 10-Megabyte File

### Specifications

The EC100 DOCSIS Cable Modem is MCNS DOCSIS compliant.

### Downstream

Demodulation:	64QAM / 256QAM

Physical Speed:	30 Mbps (64QAM) / 40 Mbps (256QAM)

Error Correction:	Reed Solomon + Trellis (Enhances Annex B)

Frequency range:	88 MHz to 860 MHz (edge-to-edge) in 62.5 kHz steps

Bandwidth:	6 MHz

Input signal level:	-15dBmV to+15dBmV

Input impedance:	75 ohms

Return loss:	> 6 dB from 88 MHz to 860 MHz

### Upstream

Demodulation:	QPSK / 16QAM

Physical Speed:	320,640,1280,2560,5120Kbps(QPSK)

640,1280,2560,5120,10240Kbps(16QAM)

Error Correction:	Reed Solomon

Frequency range:	5 to 42 MHz (edge-to-edge), in 1Hz steps

Bandwidth:	200, 400, 800, 1600, 3200 kHz

Output signal level:	8 dBmV to 58 dBmV (QPSK)

8 dBmV to 55 dBmV (16QAM)

### SNMP Management

MIB Group:	MIB II, MCNS MIB

### Interface

DTE:	IEEE802.3/10Base-T

Cable:	F type female 75ohm

### Power

Modem power input	12VDC

Power consumption:	8 W (Max)

#### Mechanical

Size:	1.96 in. (width) x 8.07 in. (depth) x 5.96 in. (height)

Weight:	2.1 lb

#### Environmental

Operating temperature:	0℃ ~ 40℃ degrees Fahrenheit

Humidity:	10 % ~ 90 % (non-condensing)

Storage temperature:	-20℃ ~ 60℃

Safety:	UL1310, UL1950

Emission:	FCC part 15, class B