Release Notes

| Project Name: | DCS-2100+ |  |
| --- | --- | --- |
| Version: | A2 |  |
| Pages: | 109 |  |
| Author: | Alex |  |
| Document number: |  |  |

TABLE OF CONTENT

1.	Introduction	2

1.1.	Document History	2

1.2.	References	2

2.	Hardware	3

3.	Firmware	4

4.	Installer Program	6

5.	Upgrade Wizard	7

6.	Surveillance Software	8

7. Device Label	11

8. CD-ROM	12

# 1.	Introduction

This document describes the differences among each version.

## Document History

| Version | Date | Log | Editor |
| --- | --- | --- | --- |
| A1 | 2003/5/13 | 1st time release | Alex |
| A1 | 2003/7/15 | IP Sentry renaming | Alex |
| A1 | 2003/8/4 | IP Surveillance renaming | Alex |
| A1 | 2003/11/19 | F/W&S/W | Alex |
| A2 | 2004/1/19 | F/W&H/W | Alex |
| A2 | 2004/5/21 | F/W&S/W | Alex |

## 1.2.	References

# 2.	Hardware

Version: 	A1

Date: 	May 13, 2003

- Initial release

Version: 	A2

Date: 	January 19, 2004

- Fixed Bugs

None

- Improved Features

- The WLAN module is changed to Maxim solution.

- Known Limitations

None

# 3.	Firmware

Version: 	IP3133-DLNK-0100

Date: 	May 13, 2003

- Initial release

Version: 	IP3133-DLNK-0101h

Date: 	September 18, 2003

- Fixed Bugs

None

- Improved Features

- Non-synchronization of audio and video in bad network environment.

- Bug fixed for ftp which accepts connection while username and password are both NULL.

- Get snapshots from secondary email address when primary and secondary email address are valid.

- Fixed the bug of getting the wrong image size by ftp.

- Sleep 10 ticks after start telnet task to avoid reboot fail.

- Modified the contrast parameter of CMOS.

- Modified the reboot page.

- Modified the wording of web UI.

- Revise the help content.

- Improve the wireless LAN driver

- Reboot system if the FTP port is changed

- Disable the "New" button in motion detection page when there are already three windows.

- Known Limitations

None

Version: 	IP3133-DLNK-0102h(1.02)

Date: 	December 25, 2003

- Fixed Bugs

- Fail to connect with DWL-7000AP+.

- Improved Features

- UPnP is added.

- Make UPnP can work while changing IP address and port.

- Discovery mechanism is added to support new IP Installer.

- DDNS is added.

- Change new D-Link logo.

- Review the help page.

- Add new range of MAC address 00-0D-88-xx-xx-xx.

- Modified firmware version format in device information page from 0102g to 1.02.

- Update verisign ID of plug-in.

- Modify the format of host name in Network Neighborhood from DCS-2100+-“MAC” to DCS-2100+-“last segment of IP”.

- Modify the wording in network settings from "Reset network at next boot" to "Reset the IP address at next boot".

- Known Limitations

None

Version: 	IP3133-DLNK-0103b(1.03)

Date: 	January 19, 2004

- Fixed Bugs

None

- Improved Features

- Change the WLAN driver for new module.

- Added SMTP authentication module.

- Added auto TX rate in network settings page.

- Known Limitations

None

Version: 	IP3133-DLNK-0104f (1.04)

Date: 	May 21, 2004

Checksum: 00183B14

- Fixed Bugs

- Fix the compatibility with Microsoft router	.

- Improved Features

- Time format is applied to the local setting of Windows.

- Add SMTP authentication.

- Adjust the interval of sequential snapshots to 5 digits for longer interval.

- Exchange the positions of video and audio ports in the network configuration page.

- Revise Help pages.

- Known Limitations

None

# 4.	Installer Program

Version: 	2.3.0.14

Date: 	May 13, 2003

- Initial release

Version: 	3.0.0.5

Date: 	November 19, 2003

- Fixed Bugs

None

- Improved Features

- Add search button to support discovery mechanism.

- Change the caption from "DCS2000/DCS2100+...." to "Internet Camera".

- Add new MAC range from 000d887ce333 to 000d887e69d2.

- Change the picture of cameras and add a logo.

- Known Limitations

None

# 5.	Upgrade Wizard

Version: 	1.1.0.8

Date: 	May 13, 2003

- Initial release

Version: 	2.0.0.2

Date: 	November 19, 2003

- Fixed Bugs

None

- Improved Features

- Support DCS-5300/DCS-5300W.

- Known Limitations

None

# 6.	Surveillance Software

Version: 	1.8

Date: 	May 13, 2003

- Initial release

- Known Limitation

DI/DO doesn't work when using the normal user count to login. To fit this spec, the scenario of ST3401 needs to be changed. (1) IP3132 shall provide all the user settings information (2) The telnet server in IP3132 shall allow more users to login simultaneously. Currently, we only allow the “admin” to control the DI/DO.

Version: 	1.9.0.4

Date: 	July 15, 2003

- Fixed Bugs

- The location names of multiple cameras will be disordered in Internet environment.

- Moving the location in the list may hang Monitor program.

- Change to “Invalid password. Access denied.”

- Change to “Connection to 67.112.37.149 has timed out or the model of camera is invalid.”

- Change the hint to “Quit” for consistency of Monitor and Playback.

- Change to “This application is already running.”

- Change to “There is no data between xxxx and xxxx.”

- Change to “You must restart the program for the changes to take effect.”

- Change to “Select an AVI compression method.”

- Improved Features

- Remove the left arrow in the playback shuttle panel.

- Save the changes made to AVI codec when the program closes.

- Playback program automatically loads video from the directory that is currently specified. If video is not present in the current directory, a message pops up indicating the lack of video in the current directory with a request asking a proper directory.

- Known Limitation

1. DI/DO doesn't work when using the normal user count to login. To fit this spec, the scenario of ST3401 needs to be changed. (1) IP3132 shall provide all the user settings information (2) The telnet server in IP3132 shall allow more users to login simultaneously. Currently, we only allow the “admin” to control the DI/DO.

Version: 	1.9.0.5

Date: 	August 4, 2003

- Fixed Bugs

None

- Improved Features

- Rename the program name of “IP Surveillance” to “IP surveillance” and change the text in the background image.

- Known Limitation

1. The folder name will be considered as the same between “IP Surveillance” and “IP surveillance”, so it won’t be changed.

Version: 	2.0(2.0.0.3)

Date: 	November 19, 2003

- Fixed Bugs

- Fail to connect if the admin’s password includes some special characters like "@".

- Recording for a long time may result in the recording data error (vv error).

- Fix spelling error in “configuration”.

- Improved Features

- Add camera control panel to support DCS-5300 and DCS-5300W.

- Known Limitation

None

Version: 	2.10 (2.10.0.19)

Date: 	May 21, 2004

- Fixed Bugs

- Cameras behind NAT may not be connected.

- Sometimes the channel will always be in connecting state.

- Improved Features

- Add audio live monitoring, recording and playback capability.

- Add a background program to start monitoring and recording automatically after Windows reboot.

- Simplify the schedule settings.

- Support multiple selecting channels.

- Add history list to call back previous added cameras.

- Add legend for understanding the histogram map.

- Known Limitation

None

# 7. Device Label

Version: 	A1

Date: 	May 13, 2003

- F/W

0100

- H/W

A1

Version: 	A1

Date: 	September 18, 2003

- F/W

0101h

- H/W

A1

Version: 	A1

Date: 	November 19, 2003

- F/W

1.02

- H/W

A1

Version: 	A2

Date: 	January 19, 2004

- F/W

1.03

- H/W

A1

# 8. CD-ROM

Version: 	1.01

Date: 	May 13, 2003

- Driver/IP Installer

2.3.0.14

- Surveillance Software

1.8

- Upgrade Wizard

1.1.0.8

Version: 	2.01

Date: 	July 15, 2003

- IP Installer/ipinstaller_23014

2.3.0.14

- IP Surveillance 1.9

1.9.0.4

- Upgrade Wizard/upgradewizard_1108

1.1.0.8

Version: 	2.03

Date: 	August 4, 2003

- IP Installer/ipinstaller_23014

2.3.0.14

- IP surveillance_1905

1.9.0.5

- Upgrade Wizard/upgradewizard_1108

1.1.0.8

Version:

Date: 	November 19, 2003

- IP Installer/ipinstaller_

- IP surveillance_

- Upgrade Wizard/upgradewizard_

END OF DOCUMENT