DCS-2121

Date: May 27, 2008

| Product Description: | Megapixel Wireless Internet Camera |
| --- | --- |
| Revision: | A1 |
| Availability: | 2008/5/29 |
| Picture: |  |
| Why Sell This Product: | DCS-2121 is the entry level network cameras with mega-pixel resolution for indoor surveillance applications and detail remote monitoring. Equipped with a 1.3 Mega-pixel (1280x1024) CMOS sensor delivering superior image details. |
| Key Features: | - Mega-pixel (1280x1024) CMOS sensor - 0.5 lux CMOS sensor can capture video in low-light environments - 3GPP mobile surveillance - 2-way audio - SD card slot for front-end storage - DI / DO interface - The Motion JPEG and MPEG-4 streams simultaneous allow systems to be optimized for both image quality and bandwidth-efficiency. - Support 3 profiles simultaneously - WPS support |

| Technical Details: | Hardware & System Key Components / Performance. Audio codec; ADPCM, AMR. Ethernet interface controller; 10/100BaseT. Sensor; 1.3 Mega pixel 1/4” CMOS Sensor. SDRAM; 64 MBytes. Flash Memory; 8 Mbytes. Lens; 5.01mm, F2.8. Port Functions. LAN; 10/100BaseT ports x1 Compliant to following standards: IEEE 802.3 compliance IEEE 802.3u compliance Support Full-Duplex operations MDI/MDIX auto-negotiation 802.3x Flow Control support for Full-Duplex mode Supported IEEE 802.11g standard. I/O Connector; 4 pin-contact terminal block 1x input(2pins), 1x output(2pins). Audio Out; 600mV. Reset Button; Reset to factory default (maintenance only). Power Consumption; Max 6W. Power Adaptor; Input: 100-240VAC, 50/60Hz Output: 5VDC, 2.5A. Storage; SD card slot. WPS Button; WPS setting. Software Features Feature Item; Feature; Detailed Description. Networking; Protocol; - IPV4, ARP, TCP, UDP, ICMP - DHCP Client - NTP Client (D-Link) - DNS Client - DDNS Client (D-Link) - SMTP Client - FTP Client - HTTP Server - Samba Client - PPPoE - RTP - RTSP - RTCP - 3GPP - LLTD - SSL - SIP - UPnP-x - UPnP AV. Ethernet; 10/100M BaseT Fast Ethernet auto negotiation. Wireless; WPS, WMM. Wireless; Security; WEP, WPA-PSK, WPA2. Video; Algorithm support; MPEG4/MJPEG dual format compression simultaneously JPEG for still image (up to 1280 x 1024). Display type; Support ActiveX, Java. Multi-profile; Support 3 profiles above simultaneously. Features; Adjustable image size and quality Time stamp and text overlay Three configurable motion detection windows Flip & mirror. Resolution; Up to 10 frames at 1280x1024 (MJPEG) Up to 10 frames at 1024x768 Up to 30 frames at 640x480 Up to 30 frames at 320x240 Up to 30 frames at 160x120. Digital zoom; Up to 10X. Min. illumination; 0.5 lux@F2.8. 3A control; AGC, AWB, AES. BLC; Yes. Electronic shutter; 1/60(1/50)~1/15,000 sec. Microphone; Directional; Omni-directional. Frequency; 50 ~ 16000Hz. S/N ratio; 6mm, – 40 db + 3db. LED; Enable/Disable. Security; Login authentication; Default Admin ID/PWD Admin/space. Firmware; Upgrade failure protection; Backup boot loader code. - OS Support - Device Windows 2000 / Windows XP/ Windows Vista/ Linux/ MAC OS X10.3 or above / Pocket PC/ 3GPP Mobile Phone - Utility Windows 2000 / Windows XP/ Windows Vista |
| --- | --- |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  IC No.  Others |  |  |
| Safety |  TUV No.  UL No.  CSA No.  CB No.  Others |  |  |
| Packaging Style |  Clamshell  White Box  Gift Box  Bulk  Brown Box  Others |  |  |
| Packaging Contents: | - Internet Camera x 1 - Power Adapter x 1 - Stand/Mounting Brackets x 1 - Master CD x 1 - CAT-5 Ethernet Cable x 1 - Quick Install Guide x 1 - Antenna x 1 |  |  |
| UPC Code: | 790069; 313998; ICS2121EEU...A1G. 790069; 314506; ICS2121BEU...A1G. 790069; 314773; ICS2121NEU…A1G. 790069; 314759; ICS2121ANA…A1G. 790069; 314766; ICS2121ENA…A1G. 790069; 314858; NCS2121CEU...A1G. 790069; 314797; TCS2121ANA…A1G. 790069; 314780; RCS2121EEU…A1G. |
| Units Per Carton: | For USA | For EU | For WW |
|  | N/A | N/A | 10 |
| Cartons Per Pallet: |  |  |  |
|  | N/A | N/A | 33 |
| Dimensions and Weight: | Box:  237x168x36mm (Type A)  225x150x75mm (Type B)  269x162x65mm (Type C)  274x208x68mm (Type D)  274x208x118mm (Type D1)  274x208x138mm (Type D2)  288x245x66mm (Type E) , Others , 0.799 kg Carton: 1. For USA mm, kg 2. For WW 373*294*463 mm, 8.51 kg Pallet: (by Air) mm, kg (for USA) mm, kg (for EU) 1100*1200*1524 mm, 299.33 kg (for WW) (by Sea) mm, kg (for USA) mm, kg (for EU) 1100*1200*1987 mm, 392.94 kg (for WW) INTRASTAT weight: 0.4591 kg (Box) Packing material weight (Box):0.4807 - Paper: 0.2617 kg - Plastic: 0.075 kg - Mixed: 0.0032 kg |  |  |

| Approved | Checked | PM |
| --- | --- | --- |