DCS-3220

Date: September 13, 2004

| Product name: | 2-Way Audio Internet Camera |
| --- | --- |
| Product Group & Type: |  |
| Revision: | A1 |
| Available: |  |
| Picture: |  |
| Why Sell This Product: | - Target users: business, factories, shop owners and wealthy residents needing surveillance systems. - The DCS-3220 is positioned as a mid-range model in your Internet camera line, supplementing the current DCS-2000 series. It is priced higher, but has more advanced functions like sharp video/image quality and digital zoom. - The DCS-3220 and DCS-2000 can be concurrently sold to different users with different technical requirements and budgets. Selling Points - All the functions that the DCS-2000 series offers, plus: 2-way audio acting like an intercom with video, CCD sensor/sharper image and 4x digital zoom. - Strong software with Motion Detection Recording, smart playback, event-trigger e-mail notification, etc. (same as the DCS-2000). - Can be installed along with the DCS-2000 series cameras in the same surveillance site. Users may try to experiment with a single DCS-3220 added to their existing security system. Once satisfied with the additional merits of the new camera, they may want to install more units. |
| Key Features: | - Stand-alone Internet camera with CPU & RAM, no need to attach to PC - Local access through Ethernet, remote access through Internet - 4x digital zoom for close-up viewing * - Listen and talk with 2-way audio ** - Surveillance software to control up to 16 cameras - Motion detection triggered recording or scheduled recording - Smart playback - High compression ratio recording - Connects to external devices for alarm-based event recording when trigger is activated - Implemented for intruder detection with still image capture & email notification - Powerful software for image capture & video recording - Easy deployment with UPnP & DNS support, web-based configuration * 4x digital zoom enlarges an image by magnifying the pixels in a selected portion of the image by 4 times. ** 2-way audio requires a powered speaker to be connected to the camera (not included). |
| Technical Details: | Camera Sensor - Built-in 1/4-inch CCD sensor - Resolution: 380 TV lines - Automatic Gain control (AGC): 24dB - Automatic Exposure (AE) - Automatic White Balance - Electronic shutter: 1/60 or 1/50 to 1/10000 sec. - Minimum Illumination: 1.0 ux @ f2.0 Lens - CS mount type - Fixed focal, (6mm, f1.8) with fine tuning LED indicator 1 bi-color LED indicator for status indication (POST, connection, heartbeat) Reset button Push button to reset to factory default settings Extended I/O - 1 relay alarm output (max. 24V DC 1A, 125V AC 0.5A - 1 Optic-isolated sensor input (max.12V DC 50mA) Memory - SDRAM: 16Mbytes - Flash ROM: 4Mbytes Video Resolution - Up to 30fps at 176 x 120 (NTSC) - Up to 30fps at 352 x 240 (NTSC) - Up to 10fps at 704 x 480 (NTSC) Modulation NTSC and PAL (different CCD sensors supporting NTSC or PAL in different camera models) Format - MPEG4 simple profile compression for streaming video - JPEG compression for still image - Compression ratio: 5 levels - Max. image frame rate: 30fps (NTSC), 25fps (PAL) - Adjustable image size, quality and bit-rate - Time stamp and text overlay Compression Options Medium, standard, good, detailed, excellent (user-selectable) Video Sizes QSIF/QCIF, SIF/CIF, 4SIF/4CIF (user-selectable) Information Display Image can be overlaid with text, timestamp and motion detection window Color Modes Color mode, monochrome mode (user-selectable) Position Flip and mirror can adjust image position Image Settings Brightness, contrast, hue, saturation (user-adjustable) Zoom-In Seamless digital zoom function with up to 4x magnification Audio Audio Input Built-in omni-directional microphone with S/N more than 60dB Audio Output Microphone socket for line-out unbalanced 0.05 to 1.0 Vpp Format - Proprietary, 8Kbps audio codec - 2-way audio, mono, supporting simplex, half-duplex, full-duplex modes Function - Can be muted at server side by administrator (camera microphone on/off) - Can be muted at client side (PC speaker on/off) - Adjustable audio volume (PC microphone on/off) Network Network Interface 802.3/3u 10/100Mbps Ethernet IP Installation IP address is installed at system reboot unless an option is set to fix it. Protocols and Applications - DDNS support with several famous DDNS servers - UPnP support - NTP client - SMTP client - FTP client - FTP server - FTP supports passive mode - HTTP server: max. 10 users to view at the same time - Adjustable server port Optional - IP filtering - SMTP Authentication Motion Detection Configuration - 3 motion-detection windows - 2 adjustable parameters: sensitivity and percentage Indicator When motion is detected, users can be informed from the image Extended I/O Digital Input - Status checking of high, low, rising, falling - DI triggered or motion triggered alert and notified via FTP or e-mail Digital Output On/Off Management & Viewing Remote Management Configuration can be accessed via Web browser Security - Administrator and user group protected - Password authentication Viewing System Requirements - Protocol: ActiveX - Operating System: Microsoft Windows XP, 2000, ME - Browser: Internet Explorer v.6 Physical & Environmental Power Input 12VDC, 1.5A Power Supply Through external AC switching power adapter Power Consumption 4.8 watts Dimensions 135 (L) x 89 (W) x 57 (H) mm (6.25 x 3.25 x 2.12 inches) (camera only) Weight 240 grams (8.5 oz.) (camera only) Operation Temperature 0 to 60 ℃ Storage Temperature -30 to 75 ℃ Humidity 5% to 95% non-condensing Emission (EMI) - FCC Class B - CE Class B Safety LVD (EN60950) Package Includes - DCS-3220 Internet Camera - 12V/1.5A external power adapter - Cat. 5 Ethernet cable - Quick Installation Guide - Master CD - Camera stand bracket |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |
| Safety |  TUV No.  UL No.  CSA No.  Others |
| Packaging Style |  Clamshell  White Box  Gift Box  Bulk  Others |
| Packaging Contents: | - DCS-3220 device - CAT5 Ethernet cable - CD-ROM - Stand/ Mounting Bracket - AC power adapter - QIG |
| UPC Code: | BCS3220A..A1; 790069269516. TCS3220A..A1; 790069270895. ECS3220E..A1; 790069270901. ICS3220A..A1; 790069270918. ICS3220E..A1; 790069270925. NCS3220CP.A1; 790069270932. ECS3220BP.A1; 790069270949. ECS3220EP.A1; 790069270956. ICS3220AP.A1; 790069270963. ICS3220EP.A1; 790069270970. ICS3220NP.A1; 790069270987. |
| Units Per Carton: | 10 |
| Cartons Per Pallet: | Air: 20 for WW, 21 for USA Sea: 24 for WW, 21 for USA |
| Dimensions and Weight: | Box: 286 x 247 x 84 mm, 1.2 kg Carton: 510 x 440 x 320 mm, 13 kg Pallet: (by Air) 1220 x 1020 x 1460 mm, 293 kg (for USA) 1200 x 1100 x 1700 mm, 280 kg (for WW) (by Sea) 1220 x 1020 x 1460 mm, 293 kg (for USA) 1200 x 1100 x 2140 mm, 332 kg (for WW) INTRASTAT weight: kg (Box) Packing material weight (Box): - Paper: 0.60 kg - Plastic: 0.20 kg - Mixed: 0.35 kg |