| Firmware Version: | V1.20 |
| --- | --- |
| Prom Code Version: |  |
| Published Date: | 2012/10/09 |

Content:

Upgrading Instructions:	2

New Features:	2

Problems Fixed:	2

Known Issues:	2

Related Documentation:	2

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| V1.10 | 2011/12/05 | DCS-6112V DCS-6113V | A1 |
| V1.20 | 2012/10/03 | DCS-6112V DCS-6113V | A1 |

# Upgrading Instructions:

Follow the steps below to upgrade the firmware:

1. Download the latest firmware file from the D-Link website. The file is in .rar file format.

2. Decompress the .rar file, there are three files included, two .pkg files and one .txt file.

3. Log-in camera web UI and enter setup/Maintenance/Firmware upgrade

4. Click Browse… and specify the firmware file “DCS-6112X_6113X_A1_FW_V1.20_preset.pkg”.

5. Click Upgrade. The camera starts to upgrade and will reboot automatically when the upgrade completes.

6. Execute upgrade process again and specify the firmware file “DCS-6112X_6113X_A1_FW_V1.20.pkg”

7. Click Upgrade. The camera starts to upgrade and will reboot automatically when the upgrade completes.

Note: Once user upgrade FW from v1.00 (or v1.10) to v1.20, it’s unnecessary to execute “DCS-6112X_6113X_A1_FW_V1.20_preset.pkg” for advanced FW in the feature, such as v1.30. The “xxx_preset.pkg” is applied for v1.00 and v1.10 only to re-manage internal memory utilization.

# New Features:

| Fireware Version | New Features |
| --- | --- |
| V1.10 | - Support both SSL and TLS - Support PPPoE CHAP |
| V1.20 | 1. Support ONVIF v1.02.4 2. Support NIPCA motion detection |

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.10 | - DUT couldn't send an E-mail via Gmail (Bug ID: DBG11080235) - Camera hang up issue when linked by D-ViewCam (Big ID: DBG11070124 & HQ20111003000012) |
| V1.20 | - DBG12070079: An error message would popup and the video settings couldn't be changed. - DBG11100155: After firmware upgrade the logo graph settings would disappear. - DBG11120366: When DUT HTTP port change to 8081, add DUT by auto search would show can't connect to IP camera. - DBG11120227//DBG11110318: The copy button in HTML Code Examples page didn't work. |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V1.10 | N/A |
| V1.20 | N/A |

# Related Documentation:

N/A