| Firmware Version: | V1.20 |
| --- | --- |
| Prom Code Version: |  |
| Published Date: | 2012/8/31 |

Content:

Upgrading Instructions:	2

New Features:	2

Problems Fixed:	2

Known Issues:	2

Related Documentation:	2

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| V1.01 | 2010/12/22 | DCS-7110 | A1 |
| V1.10 | 2012/8/01 | DCS-7110 | A2 |
| V1.20 | 2012/8/31 | DCS-7110 | A3 |

# Upgrading Instructions:

Just follow normal procedure to upgrade network camera FW.

# New Features:

| Fireware Version | New Features |
| --- | --- |
| V1.10 | Support PPPoE CHAP |
| V1.10 | Approval by ONVIF test tool v1.02.4 |
| V1.20 | Support NIPCA motion detection cgi |

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.10 | D-Track no. DBG11060237 & DRU20110614000005, camera doesn’t respond by the default IP address |
|  | Image noise issue in night mode |
| V1.20 | DBG10120098 : Add default icon in Image Settings (Profile 1) |
|  | DBG11110304 : After disable DUT HTTPS secure connection, DUT redirect to HTTP webpage with HTTPS port |
|  | DBG11110317 : After change logo graph, the title name would overlap the left logo graph in live view page. |
|  | DBG11110318 : The copy button in HTML Code Examples page didn't work. |
|  | DBG12010013: When change DUT HTTP/RTSP port to 8081/5544, browse DUT H.264 using QuickTime plugin HTML code example by FireFox would show connection failed. |
|  | DBG11100155 : After firmware upgrade the logo graph settings would disappear. |
|  | DBG11100146 : Recording notify message file didn't show the right folder path of network storage server. (Remove the path information) |
|  | DBG11110280 : The values of default and output frame size setting cannot be synchronized. |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |

# Related Documentation:

N/A