| Firmware Version: | V1.09_b2 |
| --- | --- |
| Prom Code Version: |  |
| Published Date: | 2013/12/26 |

Content:

Upgrading Instructions:	2

New Features:	2

Problems Fixed:	2

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| V1.09_b2 | 2013/12/26 | DCS-930L | A1/A2 |
| V1.08_b4 | 2013/2/20 | DCS-930L | A1/A2 |
| V1.07_b5 | 2012/11/19 | DCS-930L | A1 |
| V1.06_b5 | 2012/9/3 | DCS-930L | A1 |
| V1.05_b8 | 2012/6/18 | DCS-930L | A1 |

# Upgrading Instructions:

Follow the steps below to upgrade the firmware:

1. Download the latest firmware file from the D-Link website. The file is in .bin file format.

2. Log-in camera web UI and enter setup/Maintenance/Firmware upgrade

3. Click Browse… and specify the firmware file.

3. Click Upgrade. The camera starts to upgrade and will reboot automatically when the upgrade completes.

# New Features:

| Fireware Version | New Features |
| --- | --- |
| V1.09_b2 | - Add schedule feature for motion detection function. - NTP source port change to port 123 - Add user can configure interval setting for motion detection of mail/ftp. - User account name is limit as: 0-9, a-z, A-Z, “.”, “-“, “_”. - Update Mydlink agent to v2.0.17-b31 - Limit “Auto” FTP image frequency not over 5 frames/second. |
| V1.08_b4 | Firmware V1.08_b4 is only supported by wizard v1.03 or above. - Change maximum admin password length, from 8 characters to 30 characters. - Change PPPoE retry timing, retry connection on 30 seconds interval till connect. - New ActiveX to support long admin password. - Default time change to 2013-01-01 - Change IR LED Night to Day switch timing to 7 seconds. |
| V1.07_b5 | - Add mydlink agent version displayed on Web UI. - Update Java program to support new mjpeg streaming format - Change NTP request timing |
| V1.06_b5 | - Upgrade new mydlink agent to v2.0.16-b4 - Secure the configuration file with the new changed encryption method. - Remove two WebGUI Hidden Pages for RD develop using. - Remove the Telnet Daemon will be activated about 5 seconds when the Device is booting. - Remove the hidden FTP server. |
| V1.05_b8 | - Support Daylight Saving function - Support Korea language - Support Digest Authentication - Support language selection under live view page - Some Web UI English text wording change - Upgrade new mydlink agent to v2.0.15-b9 |

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.09_b2 | 1. Fixed ip task will be restarted issue on DHCP renew time. 2. Fixed E-mail/FTP cannot be started issue if motion detection is enabled first. 3. Fixed UpnP issue with DIR-826L router 4. The max length of Admin password change to 32 characters |
| V1.07_b5 | - Fixed ios6 live view hang issue – modify mjpeg streaming format - Fixed sometimes startup java view failed on Win 7 + IE9 - Fixed live view issue if http port is changed to 12345. - Fixed live view streaming cannot pass through router’s virtual server. - Fixed e-mail loss issue when live view the camera. |
| V1.06_b5 | - Fixed the live stream can’t display when using the https connection. - Fixed bad audio streaming on 3G internet access. |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |

# Related Documentation:

N/A