DCS-942L Firmware Release Note (WW Version)

-------------------------------------------------------------------------------------------------------------------------------------------------------------

Version: 1.10-1229_WW

Date: Feb. 29, 2012

Mydlink Agent: 2.0.14-B9

Important Notes: A factory reset is required after upgrading this firmware.

New Features/Changes:

1. Changed the NCSPlugin, which enables you to view live image streams in Non-IE browser on Microsoft Windows OS.

2. Changed the system default time to 2012-01-01.

3. Changed the system default IP to 192.168.0.20.

4. Changed the Video Type for SD Recording will both store one snapshot and video clip into the SD Card.

5. Added support Encode Method for H.264 and MPEG4 Streaming.

6. Added support H.264 streaming for Profile 4.

7. Improved error handle capability of SD hot-plug.

8. Improved noise of low-light streaming.

9. Improved live view compatibility for IE9.

10. NIPCA support H.264 at profile4.

11. Update mydlink agent to v2.0.14-B9.

12. Added the recording duration of SD recording can support 1/2/3/4/5/6 minutes.

Bug Fixes:

1. Extent manual time setting list.

2. Update ActiveX version to v6216.

3. Fine tune log and some log message to make sure log page display correctly.

4. Fixed video clip failures caused by the image frame size is too large.

Known Issue:

1. The bundle D-ViewCam can’t support the Encode Method setting for DCS-942L.

2. DCS-942L doesn’t provide the “FTP Test Button” function to allows you to test the FTP settings before running “Send Video Clips or Snapshot by FTP” to make sure they are valid for your server.

3. DCS-942L’s PIR is sometimes affected by wireless devices with high frequencies.

4. Send video clip or snapshot mail from yahoo or Gmail to Hinet would be failed.

-------------------------------------------------------------------------------------------------------------------------------------------------------------

Version: 1.02-1030_WW

Date: December 30, 2011

Mydlink Agent: 2.0.09

Note: This is Firmware Release for v1.02_WW.

New Features/Changes:

1. Enhance SD Card recording to make it not so picky to different class of SD card

2. Video only (without audio data) when Microphone is disabled for SD Recording and Video Clip

3. Resolve new SD sometime need to format twice

4. Support SMTP authentication when user key in ID/Password with no SSL-TLS.

-------------------------------------------------------------------------------------------------------------------------------------------------------------

Version: 1.01-0902_WW

Date: November 02, 2011

SVN Revision: 4166

Note: This is CN FW release.

New Features/Changes:

Correct issue of WPS cannot associate successfully after reboot.

-------------------------------------------------------------------------------------------------------------------------------------------------------------

Version: 1.01-0821_WW

Date: October 21, 2011

SVN Revision: 3949

Note: This is CN FW release.

New Features/Changes:

1. Refine CN and TC language for manual.

2. Update mydlink to v2.0.09-201110141349.

-------------------------------------------------------------------------------------------------------------------------------------------------------------

Version: 1.01-0818

Date: October 18, 2011

SVN Revision: 3818

Note: This is CN FW release.

New Features/Changes:

1. Add CN and TC language

-------------------------------------------------------------------------------------------------------------------------------------------------------------

Version: 1.00-0708

Date: Sept. 08, 2011

SVN Revision: 3356

Note: This is CN FW release.

New Features/Changes:

1. Fixed CN bugs from QA.

-------------------------------------------------------------------------------------------------------------------------------------------------------------

Version: 1.00-0706

Date: Sept. 06, 2011

SVN Revision: 3301

Note: This is CN FW release.

New Features/Changes:

1. Add CN language.

2. Update mydlink v2.0.08-201108261817

-------------------------------------------------------------------------------------------------------------------------------------------------------------

Version: 1.00-0522

Date: July 22, 2011

SVN Revision: 2734

Note: This is RC1 FW release.

New Features/Changes:

1. Resolve bugs on TSV tested by Alpha LAB

2. Resolve bugs on Redmine tested by Murphy’s QA team

-------------------------------------------------------------------------------------------------------------------------------------------------------------

Version: 1.00-0518

Date: July 18, 2011

SVN Revision: 2658

Note: This is Beta 2 FW release.

New Features/Changes:

Resolve bugs on TSV tested by Alpha LAB

Resolve bugs on Redmine tested by Murphy’s QA team

Update mydlink v2.0.06-201107151734

-------------------------------------------------------------------------------------------------------------------------------------------------------------

Version: 1.00-0430

Date: June 30, 2011

SVN Revision: 2461

Note: This is Beta FW release.

New Features/Changes:

1. Fix wording for D-Link Alpha Review

2. Resolved bugs on Redmine via Murphy’s QA Team.

-------------------------------------------------------------------------------------------------------------------------------------------------------------

Version: 1.00-0417

Date: June 17, 2011

Note: First alpha release

Please note:

1. Purpose of alpha release is for and only for…

- Web UI/functional review.

- Being given to NUUO for developing D-ViewCam (after Test LAB's validation).

2. The firmware is functional but it is also buggy.