| Firmware Version: | V1.12 |
| --- | --- |
| Prom Code Version: |  |
| Published Date: | 2012/7/23 |

Content:

Upgrading Instructions:	2

New Features:	2

Problems Fixed:	2

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| V1.12_1628 (EU) V1.12_1608 (WW) | 2012/7/23 | DCS-942L | A3 |
| V1.11_1426 | 2012/5/9 | DCS-942L | A3 |
| V1.00 | 2011/8 | DCS-942L | A1 |

# Upgrading Instructions:

Follow the steps below to upgrade the firmware:

1. Download the latest firmware file from the D-Link website. The file is in .bin file format.

2. Log-in camera web UI and enter setup/Maintenance/Firmware upgrade

3. Click Browse… and specify the firmware file.

3. Click Upgrade. The camera starts to upgrade and will reboot automatically when the upgrade completes.

# New Features:

| Fireware Version | New Features |
| --- | --- |
| V1.12_1628 (EU) V1.12_1608 (WW) | - Update mydlink agent to 2.0.15-B11. - Add SD card warning message. - Add mydlink agent version information. - Fine tune NTP failure when network is not accessible at boot time. - Support Direct View Web Pages. Using following URL to get live video - http://camera_ip/vaview.htm for Windows IE - http://camera_ip/vjview.htm for non-Windows IE |
| V1.11_1426 | 1. Enable playback in mydlink portal. 2. Changed the system default time to 2012-01-01. 3. Changed the system default IP to 192.168.0.20. 4. Changed the Video Type for SD card recording. It will store both one snapshot and video clip into the SD Card. 5. Support Encode Method for H.264 and MPEG4 Streaming. 6. Support H.264 streaming for Profile 4. 7. Improved error handling of SD hot-plug. 8. Reduces noise of low-light streaming. 9. Improved live view compatibility for IE9. 10. Added the recording period of SD recording can support 1/2/3/4/5/6 minutes. 11. Enables live streams in Non-IE browser on Microsoft Windows OS. 12. Add support PIR sensitivity function and set the default setting to low. |

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.11_1426 | 1. Extent manual time setting list. 2. Update ActiveX version to v6216. 3. Fine tune log and some log message to make sure log page display correctly. 4. Fixed video clip failures caused by the image frame size is too large. |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |

# Related Documentation:

N/A