| Firmware Version: | V1.26_5601 |
| --- | --- |
| Prom Code Version: |  |
| Published Date: | 2015/10/1 |

Content:

Upgrading Instructions:	2

New Features:	2

Problems Fixed:	6

Known Issues:	6

Related Documentation:	7

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| V1.26_5601 | 2015/10/1 | DCS-942L | A1~A4 |
| V1.25_4912 | 2015/3/12 | DCS-942L | A1~A4 |
| V1.24_4408 | 2014/10/8 | DCS-942L | A1~A4 |
| V1.23_3040 | 2014/1/23 | DCS-942L | A1~A4 |
| V1.22_2825 | 2013/7/24 | DCS-942L | A1~A4 |
| V1.21_2811 | 2013/6/21 | DCS-942L | A1~A4 |
| V1.20_2421 | 2013/3/29 | DCS-942L | A1~A4 |
| V1.12_1628 (EU) V1.12_1608 (WW) | 2012/7/23 | DCS-942L | A1~A3 |
| V1.11_1426 | 2012/5/9 | DCS-942L | A1~A3 |
| V1.00 | 2011/8 | DCS-942L | A1 |

# Upgrading Instructions:

Follow the steps below to upgrade the firmware:

1. Download the latest firmware file from the D-Link website. The file is in .bin file format.

2. Log-in camera web UI and enter setup/Maintenance/Firmware upgrade

3. Click Browse… and specify the firmware file.

3. Click Upgrade. The camera starts to upgrade and will reboot automatically when the upgrade completes.

# New Features:

| Firmware Version | New Features |
| --- | --- |
| V1.26_5601 | - Updated mydlink agent to v2.0.19-b06n - Updated Digital Signature |
| V1.25_4912 | - Upgrade mydlink agent to v2.0.19-b06 |
| V1.24_4408 | - Upgrade mydlink agent to v2.0.18-b11 - Support Windows 8.1/IE11 and MAC OS X10.9/Safari7 - The SD card can be recognized properly when formatting SD card by selecting default allocation unit size on Windows OS - To improve the image quality (NOTE: The image settings will be in effect after factory reset.) - Support new NIPCA CGI “/admin/videoclip.cgi” for Video Clip function |
| V1.23_3404 | - Upgrade mydlink agent to v2.0.17-b42 - Support schedule reboot - Support DDNS server address text input (currently only accept www.dlinkddns.com and www.DynDNS.org.) - Support Windows 8/ IE11 - Refine the NTP sync mechanism |
| V1.22_2825 | - Support 2-way audio for portal and App - Combine two firmware versions into one firmware. - Support Portuguese language. - Change the SD recording video clip file format default setting from avi to mp4. - Refine the available HTTP/RTSP/HTTPS port range and external HTTP/RTSP/HTTPS port range as 1~65535. - mydlink Agent: 2.0.17-b21. |
| V1.21_2811 | - Upgrade to UPnP v1.6.18. - mydlink Agent: 2.0.17-B15 |
| V1.20_2421 | Firmware V1.20_2421 is only supported by wizard v1.02.10 or above. - Support HTTPS - Support HTTPS Web Pages (not include Live Streaming) - Support HTTPS Port and HTTPS External Port Setting - Certiﬁcation management - WebUI for HTTPS connection (streaming keep on HTTP) - Conﬁgure/control under HTTPS - Support IPv6 - IPv6 certiﬁcation - IPv6 live view - Support ONVIF v2.2 - ONVIF Proﬁle S Certiﬁcation - Add Search Domain field for network setting - Add Set NTP Server from DHCP for NTP Sync - Frame rate support 6 fps - Support AAC audio - RTSP with AAC - NIPCA Audio CGIs, /audio/ACAS-AAC.cgi - Support .mp4 ﬁle format with AAC - SD recording - Video clip - Support Apple HTTP Live Streaming - Support IP Filtering - Support Sound Detection - Support Schedule on Day/Nigh Mode - Support no separated ﬁle on SD recording trigger by motion Enhancement: - Enhance NCSPlugin for live view - Windows - Mac OS X - Refine the DDNS timeout DDNS update interval: Case 1: If DDNS can be connected successfully. The reload interval will be a fixed time as DDNS timeout, and the valid range is between 24 and 65535. Case 2: If DDNS is failed to be connected. The retry interval will be an increasing time as 30 seconds, 60 seconds, 120 seconds, 240 seconds, 480 seconds, and then keeping retrying every 900 seconds until the DDNS can be connected. - Refine the interval for Video Clip/Snapshot feature The refined interval is as following. Function; Target; Trigger method; Available Range; Default Value. Snapshot; FTP; Always / Schedule; 10 ~ 86400; 300. Motion / Sound; 10 ~ 86400; 10. E-mail; Always / Schedule; 30 ~ 86400; 600. Motion / Sound; 30 ~ 86400; 30. Video Clip; FTP; Always / Schedule; 30 ~ 86400; 300. Motion / Sound; 10 ~ 86400; 10. E-mail; Always / Schedule; 60 ~ 86400; 600. Motion / Sound; 30 ~ 86400; 30. - Refine the display order for SD management - Refine the dwelling time of Day/Night mode switching. - Refine the available port setting as default port number and range from 1025 to 65535. |
| V1.12_1628 (EU) V1.12_1608 (WW) | - Update mydlink agent to 2.0.15-B11. - Add SD card warning message. - Add mydlink agent version information. - Fine tune NTP failure when network is not accessible at boot time. - Support Direct View Web Pages. Using following URL to get live video - http://camera_ip/vaview.htm for Windows IE - http://camera_ip/vjview.htm for non-Windows IE |
| V1.11_1426 | - Enable playback in mydlink portal. - Changed the system default time to 2012-01-01. - Changed the system default IP to 192.168.0.20. - Changed the Video Type for SD card recording. It will store both one snapshot and video clip into the SD Card. - Support Encode Method for H.264 and MPEG4 Streaming. - Support H.264 streaming for Profile 4. - Improved error handling of SD hot-plug. - Reduces noise of low-light streaming. - Improved live view compatibility for IE9. - Added the recording period of SD recording can support 1/2/3/4/5/6 minutes. - Enables live streams in Non-IE browser on Microsoft Windows OS. - Add support PIR sensitivity function and set the default setting to low. |

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.25_4912 | - Disable SSLv3 - Fixed the NIPCA XML response issue by adding <root> tag as well-format. |
| V1.24_4408 | - Fixed the issue that camera cannot reboot after schedule reboot is set. - Fixed the issue of FTP upload failure due to special characters as FTP password. |
| V1.23_3404 | - Fixed the ONVIF compatibility issues for “ONVIF Device Manager” - Fixed SMTP compatibility issue of Swiss e-mail provider “bluewin.ch” - Fixed NTP sync issue after power on at static IP status - Fixed Flicker issue in 50 HZ frequency light source environment |
| V1.22_2825 | - Refine Day/Night mode switching judgment. |
| V1.21_2811 | - RTSP Authentication is default enable. - Fix security vulnerability item CVE-2013-1602 from CoreSecurity. |
| V1.20_2219 | - Fix multiple login SD card playback error for mydlink |
| V1.11_1426 | - Extent manual time setting list. - Update ActiveX version to v6216. - Fine tune log and some log message to make sure log page display correctly. - Fixed video clip failures caused by the image frame size is too large. |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V1.24_4408 | - It’s recommended for users to download the latest D-ViewCam version from mydlink portal to avoid abnormal function operation when using the DCS-942L Ax camera with FW v1.24_4408. |

# Related Documentation:

N/A