| FW Version: | 2.00b06 |
| --- | --- |
| Published Date: | 2013/10/22 |

Content:

Revision History and System Requirement:	2

New Features:	2

Problems Fixed:	3

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| 2.00b06 | 2013/09/10 | DNR-326 | A2 |

MINIMUM SYSTEM REQUIREMENTS

OS Support

+ Windows XP SP2, Vista or 7 (32bit)

+ Web browser - Internet Explorer v7 to v9, Mac Safari, Chrome

+ Computer with:

-	Pentium 4 – 2.4 GHz

-	512MB RAM

# New Features:

| Firmware Version | New Features |
| --- | --- |
| 2.00b06 | 1. Support push notification with D-ViewCam mobile - Android v1.2 - iOS v1.3 2. Support cross browser include MS-IE, Mac Safari, Chrome with partial function Note: below functions are not supported in Safari and Chrome - LiveViewer: Preset/EMap/Full screen/Auto Scan/Manual Recording/Remote IO/Logout server -Playback: Smart search/Print/Backup/Image Enhancement/Logout Server 3. Support Win8 (Not including Metro Mode) 4. Support IE10 5. Support Onvif Profile S 6. Support new IP cameras DCS-932L/2136L/3010/3130/5010L/5020L/6513 |

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| 2.00b06 | 1. DBG11100238 A warning message would popup after live view for more than 60 hours 2. DBG11110188 Live view for a long term would casue the IE crashed 3. DBG12050017 Copyright of setup.exe would be incorrect 4. DBG12050172 DUT make DCS-6616 to focus near/far failed. 5. DBG12060230 [LiveView.exe]There Digital signature is wrong on LiveView.exe after installation. 6. DBG12060231 [Playback.exe]There Digital signature is wrong on Playback.exe after installation. 7. DBG12100369 After add camera to DUT, the camera 2nd stream setting is enable not default disable status. 8. DBG12100435 DCS-7413 30 FPS is available to change in DUT when resolution is 1920X1080. 9. DBG13030373 There is uncontinious voice on live video and playback file on DCS-931L and DCS-933L. 10. DBG13030413 DUT didn’t auto backup any file to the FTP server DNS-1200 at root path. 11. DBG13040006 Pop up warning message “Save dump file……” when reopen Muti-NVR Viewer. 12. Fixed email show “NVRmini” bug 13. Fixed the issue that error message “The format of URL login is invalid” would pop-up after user apply camera settings 14. Fixed the issue that Live view/playback cannot work after enable SSL in WEB GUI 15. Fixed the issue that NVR response failed for IP camera search 16. Fixed the bug that DCS-5010L/5020L audio support fail 17. Fixed below security issues 1) Authentication bypass vulnerability 2) Remote Unauthenticated Information Disclosure Vulnerability 3) Remote unauthenticated arbitrary firmware upload vulnerability 4) Remote unauthenticated Denial-of-Service Vulnerability 5) Remotely create a new USER 6) Remote admin password reset |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| 2.00b06 | 1. Open video live view with browser or NVR pro AP for a long time might popup C++ error Root cause: The root cause is memory fragmentation issue – DNR-326 FW manages memory is not enough efficient and it cannot get enough memory to process decode tasks of live view. So it might popup C++ error message in MS-IE browser or NVR Pro AP. When NVR popup C++ error message, it won’t affect NVR recording and other tasks. Improvement: Base on the NVR recording performance 720P@270FPS to implement connected cameras. Don’t let NVR over loading to cause this problem happen. Suggestion - Re-open MS-IE browser or NVR Pro AP and re-connect NVR to view live video again. - This issue will be fixed by re-design code base in the next FW version. We expect the new FW version will formal release in Q1, 2012. Others: - Can’t switch current view of DCS-6010L because camera firmware is not ready. - Can’t fix bug DBG12030320 with firmware v2.00.03 (DUT modify DCS-3710B1 bit rate quality would become CBR.) because root cause is camera firmware issue. - User selects Low/Minimum profile on NVR client for DCS-2121, the main profile would be modified accordingly because DCS-2121 uses sensor output for different profile setting - DCS-6010L switch current view “when display mode is 1P2R, user shall click down-left corner to switch to up-right view.” - DCS-6915: - Motion detection and IO can only trigger once when camera connects to NVR - Do not support G.726 audio codec - DCS-7413: When frame rate is 1fps, camera would disconnected with NVR easily - DCS-5605/5635: camera will reboot when switch to dual Stream - DCS-3112: Preset01 will be overwrited by Preset02, so it won’t find Preset01 when it executes Patrol - Chrome on MAC is not supported because of JAVA compatibility issue http://java.com/en/download/faq/chrome.xml - When user remotely watches video which resolution is higher than D1 on non-IE browsers, Live view would crash. - When user remotely watches video which resolution is higher than D1 on MAC, it will pop-up error message. User only needs to select stream profile low/minimum for avoiding this issue. |

# Related Documentation:

- User Manual

- Datasheet

# Note:

- Please refer to Camera Support List in PMD system.