D-Link Corporation

ver. A1.0

Date: 2009/04/02

Revision History

| Version | Revised Date | Person Name | Content Revised |
| --- | --- | --- | --- |
| 0.1 | 2009/02/26 | Miles Lee | First Edition |
| 0.2 | 2009/03/02 | Miles Lee | Update the size of DDR2 |
| 0.3 | 2009/03/04 | Miles Lee | Update Power LED behavior |
| 01.0 | 2009/04/02 | Miles Lee | - Update the CC1100 specification. - Update the LED behavior. - Update the packaging content. |

Table of Contents

1	Introduction	4

2	Application	4

3	Product Features	4

3.1	H/W Features	4

3.2	LED Indicators	5

3.3	Firmware Features	6

3.4	Support Media Format	7

3.4.1	Video File Formats	7

3.4.2	Video Decoder	8

3.4.3	Audio Decoder	9

3.4.4	Pictures	10

3.5	Device Portal Features	10

3.6	User Portal Features for DSM-350	11

3.7	TV UI Features	12

3.8	Remote Controller	13

3.8.1	Appearance	13

3.8.2	Key Definitions	14

3.9	OS Support	15

3.9.1	Device Portal (mydlink)	15

3.9.2	User Portal	15

4	Housing Design	15

5	Physical & Environment	16

5.1	Operation	16

5.2	Storage	16

6	Emission (EMI), Safety & Other Certifications:	16

7	Package Contents	16

### Introduction

The DSM-350 is a media adapter, which supports plenty of video formats and supports up to 720p HD video resolution. It provides HDMI out to allow connecting the TV via HDMI interface and provides an USB host in front panel to allow connecting to a USB device. With a 2.5-inch HDD inside, the DSM-350 can be a network storage device so the user doesn’t need extra storage to access the files.

Different from other vender’s media player, the DSM-350 can also help users to channelize their media contents from anywhere, anytime over the Internet. After doing so, users just need to turn on the device, and then they can watch the contents by pressing arrow keys on the remote controller. The behavior is like you turn on the TV to watch the program.

The DSM-350 also builds in a BT downloader allowing downloading the media contests from the Internet via P2P technology.

### Application

The application of DSM-350 is shown in below diagram.

### Product Features

#### H/W Features

| Key Components / Performance |  |
| --- | --- |
| CPU & Codec | C2 CC1100 |
| Flash Memory | 32MB |
| DDR2 SDRAM | 256MB |
| HDMI controller | ANALOGIX ANX9030 |
| Ethernet controller | RTL8103-EL |
| Audio DAC | WM8521 |
| Port Functions |  |
| LAN | 10/100BaseT ports x1 Compliant to following standards: IEEE 802.3 compliance IEEE 802.3u compliance Support Full-Duplex operations MDI/MDIX auto-negotiation 802.3x Flow Control support for Full-Duplex mode |
| IR Receiver | IP Remote sensor ; NEC protocol; |
| USB Host | USB type A X1; support up to 1A output |
| USB client | Mini - B receptacle X1; connect to PC |
| SATA connector | 2.5” HD connection; inside the housing. |
| HDMI (Video Out) | HDMI A type X 1; support video resolution up to 720p |
| S/PDIF | Digital audio signal output; fiber optical type. |
| Composite (Video Out) | RCA plug X1 (yellow) |
| Analogue Audio | Analogue audio output; RCA plugs X2 (red for right channel, white for left channel) |
| DC Jack | Connect to an external power adapter. |
| Power Consumption | Max 24W; support power saving mode |
| Power Adaptor | Input: 110-240VAC, 50/60Hz Output: 12VDC/2A |

#### LED Indicators

| - Location | - LED Indicative | - Color | - Status | - Description |
| --- | --- | --- | --- | --- |
| - Device | - HDD | - Blue | - On | - HDD is connected |
|  |  |  | - Off | - HDD is not connected |
|  |  | - Red | - Blinking | - HDD space is full or malfunction |
|  | - Power | - Blue | - On | - Power on the system |
|  |  | - Red | - On | - Power is connected; TV output is off |
|  |  |  | - Off | - Power off the system |
|  | - LAN | - Orange | - Blinking | - Transmitting or receiving data |
|  |  |  | - Off | - No transmitting or receiving data |
|  |  | - Green | - Off | - LAN is connected in 10Mbps |
|  |  |  | - On | - LAN is connected in 100Mbps |

#### Firmware Features

| Feature Item | Feature | Detailed Description |
| --- | --- | --- |
| Networking | Protocol | - IPV4, ARP, TCP, UDP, ICMP - DHCP Client - DNS Client - DDNS Client - FTP Client/server (phase II) - HTTP Server - Samba Client /Server - PPPoE - UPnP Port Forwarding |
|  | Ethernet | 10/100M BaseT Fast Ethernet auto negotiation |
| Video system | NTSC/PAL |  |
| Settings & Maintenance | PPPoE | For dial-up connection |
|  | Static IP | for static IP connection |
|  | DHCP | Factory default |
|  | Password change |  |
|  | wireless | optional |
|  | FW download | Click a button to check and download the latest FW |
| Device IP | Auto IP |  |
| Security | Login authentication | ID: admin /PWD: admin |
| Firmware | Upgrade failure protection | Backup boot loader code |
| File System | HDD connection | NTFS file system & Unicode file name |

#### Support Media Format

##### Video File Formats

##### Video Decoder

##### Audio Decoder

##### Pictures

#### Device Portal Features

| Item | Feature | Description |
| --- | --- | --- |
| Language | Multiple | Supports Simple Chinese, Traditional Chinese, French, German, Spanish, Italian, Russian, and English. |
| Torrent | URL download | Get Torrent from URL |
|  | PC download | Get Torrent form PC |
|  | Sharing Pool download | Get Torrent from sharing pool. |
|  | Select channel | Allow creating a new channel or selecting an old channel; One channel is pre-defined in factory default. |
|  | Download Status | - Display Torrent download status, such as priority, name, size, progress, status, up/down speed, left time, seed, and peers, etc. - Support the following buttons: - Stop - Start - Remove - Pause - Priority move up - Priority move down - Click to view the download event in details - Share to your friend (phase II) |
| Channel | Management | - Add CH - Del CH - Edit CH - Program priority move up - Program priority move down |
| Sharing (Phase II) | Torrent sharing | - Shared torrent list. - Shared device list. |
| File | Management | - Media file search - Media file from/ to directory selections - Display the directory the file is located - Move media files - Delete media files - Backup media files to NAS or USB stick (phase II) |
| Settings | Network & Function Settings | - Network Settings: - PPPoE - Static IP - DHCP - password change - FW upgrade - Format HDD - Function Settings: - Upstream BW limitation - Downstream BW limitation - Torrent download limitation - Reset above Items’ value to factory default - Display device information such as model name, IP, and MAC address, storage capacity |

#### User Portal Features for DSM-350

| Item | Feature | Description |
| --- | --- | --- |
| Language | Multiple | Current only support English |
| Device Settings | Setting the devices | Click Go To My Device button to login in the web page of the device. |
| Device Info | Display information of device | Display device information, such as device name, IP, and MAC address, etc. |

Note: other portions please refer to the specifications of mydlink portal.

#### TV UI Features

| Mode | Feature | Description |
| --- | --- | --- |
| TV | Video | - Delay time (minute :second : mini second) (move to TV) - Text position - Text color (black/white/green) (default: white) |
|  | Information | - Channel No. - Channel Name - Program name - Progress for both channel and program - Audio information - Subtitle - Playback status - Audio volume - Message - Smart preview - Network status when it is error |
| Settings | System Settings | - System reset (back to default settings) - HDD format - Language selection (Simple Chinese, Traditional Chinese, French, German, Spanish, Italian, Russian, or English.) (Default: depends on the SKU.) - FW check/update - Device information: name/mac/network information (IP/connection type/Internet status) - Storage space indication |
|  | Video Settings | - Aspect ratio: 4:3/16:9/auto (default: auto) - Video output: HDMI/Composite (default: composite) - Display resolution:480i/480p (default: 480p) - Video system: NTSC/PAL (default: depends on the SKU) |
|  | Playback Settings | - Audio track (left/Right/Both or Chinese/English…) (default: Both) - Smart preview: on/off (default: on) - Digital/Stereo: stereo/Digital (default: stereo) - Font size: small/medium/large (default: medium) - Subtitle: on/off (default: on) - Channel No. Display: on/on+title/off (default: on+title) |
| Friends | Friend List | - Add/Accept/Reject/Delete the list |
| Download | Torrent Download Status | - Status: Downloading - Status: Pause - Status: Stop - Status: Complete - Status: Queue (waiting) - Function: Share the torrent |
|  | Shared Torrent List | - Display shared torrent list with program name and file name - Add shared torrent to the download list - Delete shared torrent |
| EPG | Channel List | - Display all channel (channel number/name/time) - Display new channel (channel number/name/time) - Display new program - Program List - Display program list in a channel (sequence/snapshot/file name/program title/time/) |

#### Remote Controller

##### Appearance

##### Key Definitions

| Mode | Key | Description |
| --- | --- | --- |
| TV | MyChannel | Press to jump into TV channel |
|  | Mute | Press to turn on/off the audio of video |
|  | Information | Press to display/non display the program information |
|  | Smart Preview | Press for smart preview on/off. |
|  | Font Size | Press to select the font size. |
|  | Subtitle | Press to select the subtitle. |
|  | Audio | Press to select audio track, such as stereo, left mono, right mono |
|  | VOL + | Press to increase the volume of audio. |
|  | VOL - | Press to decrease the volume of audio. |
|  | CH/ Page + | Press to jump to next channel. |
|  | CH /Page - | Press to jump to previous channel. |
|  | Play/Pause | Press to play/pause the video. |
|  | Previous | Press to jump to the previous segment of the video. |
|  | Next | Press to jump to the next segment of the video. |
|  | Stop | Press to stop the video. |
|  | Options | Option function |
|  | Backward | Press to reverse the video. 1X/2X/4X cyclically. |
|  | Forward | Press to forward the video. 1X/2X/4X cyclically. |
|  | 10 Number Keys | 0,1,2,3,4,5,6,7,8,9 |
|  | Power | Press to turn device on/standby |
| Media | Menu | Display main menu |
|  | Search | Search media |
|  | Repeat | For music repeat |
|  | Shuffle | Random play for music |
|  | Videos | Go to video mode |
|  | Music | Go to Music mode |
|  | Photos | Go to Photo mode |
|  | Network Media | Go to Network Media mode |
| Download | Download On/Off | Press to Start/Stop all download events. |
| EPG | EPG | Press to jump into EPG mode |
|  | Left | Press to go left or change to previous program |
|  | Right | Press to go right or change to next program |
|  | Up | Press to go up or channel up |
|  | Down | Press to go down or channel down |
|  | OK | Press to execute “Enter” command or play media |
|  | Back | Press to return to previous page |

#### OS Support

##### Device Portal (mydlink)

Windows XP / Vista

##### User Portal

Windows XP / Vista

### Housing Design

### Physical & Environment

#### Operation

- Operation Temperature: 0-40°C

- Humidity: 20%-80% RH non-condensing

#### Storage

- Storage Temperature: -15-60°C

- Humidity: 0%-90% RH non-condensing

### Emission (EMI), Safety & Other Certifications:

- FCC

- CE

- C-Tick

- BSMI

- CCC (waiting for request)

- Compliant to RoHS & WEEE

### Package Contents

  - Media Player x 1

  - 12V2A External Power Adapter x 1

  - Remote controller including a separated battery set x 1

  - CAT5 Ethernet cable (1.5M) x 1

  - AV cable (1.5M) x 1

  - Quick Installation Guide + System & Remote control user manual x 1

  - CDROM including a wizard and user’s manual

  - Warranty card x 1

  - GNU GPL insert page x 1

  - HDD Kit