DSM-520

HD Digital Media Adapter

Product External Specification

Date: May 2008

# Software Specification

## Main Features

| Logo Certification |  |
| --- | --- |
| * Logo Certification | Windows Connect Now Logo （In case with USB port） |
|  | PlaysForeSure Audio/Video Network Deivce |
|  | HDMI 1.0 |
| DLNA |  |
| DLNA | Support DLNA 1.0 |
|  | Support Render Device function |
| Language |  |
| Languages | Russia、English、French、Italy、German、Spanish |
| Software Update |  |
| Software Update | Internet Update Server |
|  | USB memory |
| Support Format |  |
| WMDRM 10 | Content protected WMV/WMA |
| DVR-MS | Video record format by Microsoft |
| Others | Please refer to Section 1.2 below: |
| Content Protection |  |
| WMDRM 10 | Content protection for WMV/WMA file format |
| HDCP | Content protection for digital output through HDMI |

## Supported Format

### DMA Application

| Type |  | Extension | Stream |  | Limitation |  |
| --- | --- | --- | --- | --- | --- | --- |
| VIDEO |  |  |  |  |  |  |
|  | MPEG PS | Mpg mpeg mpe m2p | Video | MPEG-1 MPEG-2 | Profile Level | up to MP@HL |
|  |  |  |  |  | Resolution | Max. 1920×1080 |
|  |  |  |  |  | Bit rate VBR/CBR | Max 15Mbps |
|  |  |  | Audio | MPEG-1/2 Layer-II | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Channels | Max. 2ch |
|  |  |  |  |  | Bit rate VBR/CBR | Max 384kbps |
|  |  |  |  | LPCM | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Bits per sample | Max. 16bit |
|  |  |  |  |  | Channels | Max. 2ch |
|  |  |  |  | AC3 | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Channels | Max. 6ch |
|  |  |  |  |  | Bit rate | Max. 640kbps |
|  | MPEG-2 TS/TTS | - | Video | MPEG-2 | Profile Level | up to MP@HL |
|  |  |  |  |  | Resolution | Max. 1920×1080 |
|  |  |  |  |  | Bit rate VBR/CBR | Max 25Mbps |
|  |  |  | Audio | MPEG-1/2 Layer-II | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Channels | Max. 2ch |
|  |  |  |  |  | Bit rate VBR/CBR | Max 448kbps |
|  |  |  |  | AC3 | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Channels | Max. 6ch |
|  |  |  |  |  | Bit rate | Max. 640kbps |
|  |  |  |  | MPEG-2/4 AAC LC | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Channels | Max. 6ch |
|  |  |  |  |  | Bit rate | Max 6 ch |
|  | WMV *1,*2,*3 | Asf Wmv | Video | WMV9 | Profile Level | up to MP@HL |
|  |  |  |  |  | Resolution | Max. 1920×1080 |
|  |  |  |  |  | Bit rate VBR/CBR | Max 10Mbps |
|  |  |  | Audio | WMA8 | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Channels | Max. 2ch |
|  |  |  |  |  | Bit rate | Max. 320kbps |
|  |  |  |  | WMA9 WMA9 Professional *5 | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Channels | Max. 6ch |
|  |  |  |  |  | Bit rate VBR/CBR | Max. 768kbps |
|  | DVR-MS | dvr-ms | - | - | Support recored TV program in Microsoft Media Center edition |  |
|  | MPEG4 AVI 1.0/2.0 | Avi | Video | 3ivx | Resolution | Max. 1280×720 |
|  |  |  |  | XviD | Resolution | Max. 1280×720 |
|  |  |  | Audio | MPEG-1/2 Layer-III | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Channels | Max. 2ch |
|  |  |  |  |  | Bit rate VBR/CBR | Max. 320kbps |
|  |  |  |  | AC3 | Sampling Frequency | Max 48kHZ |
|  |  |  |  |  | Channel | Max 6 ch |
|  |  |  |  |  | Bit rate | Max 640kbps |
|  | MPEG4 | mp4 | Video | MPEG-4 ASP | Resolution | Max 1920X1080 |
|  |  |  |  |  | Bit rate VBR/CBR | Max 5Mbps |
|  |  |  | Audio | MPEG-4 AAC LC | Sampling Frequency | Max 48Khz |
|  |  |  |  |  | Channel | Max 6ch |
|  |  |  |  |  | Bit rate | Max 640kbps |
|  | IFO | - | - | - | IFO file specified in DLNA 1.0 errata |  |
| MUSIC |  |  |  |  |  |  |
|  | MP3 | mp3 | MPEG-1/2 Layer-III |  | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Channels | Max. 2ch |
|  |  |  |  |  | Bit rate VBR/CBR | Max. 320kbps |
|  |  |  |  |  | D3 Tag *5 | ID3v1.X ID3v2.0,2.1,2.2,2.3, 2.4 |
|  | LPCM | - | LPCM |  | Sample Frequency | Min. 8kHz Max. 64kHz |
|  |  |  |  |  | Bits per sample | Max. 16bit |
|  |  |  |  |  | Channels | Max. 2ch |
|  | WAV | Wav | PCM |  | Sample Frequency | Min. 8kHz Max. 64kHz |
|  |  |  |  |  | Bits per sample | Max. 16bit |
|  |  |  |  |  | Channels | Max. 2ch |
|  | WMA | asf wma | WMA8 |  | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Channels | Max. 2ch |
|  |  |  |  |  | Bit rate | Max. 320kbps |
|  |  |  | WMA9 WMA9 Professional *3 |  | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Channels | Max. 6ch |
|  |  |  |  |  | Bit rate ABR/CBR | Max. 768kbps |
|  | Ogg | Ogg | Vorbis |  | Sample Frequency | Max. 48kHz |
|  |  |  |  |  | Channels | Max. 2ch |
|  |  |  |  |  | Bit rate ABR/VBR | Max 496kbps |
|  | M4A | m4a | MPEG-2/4 AAC8AAC LC) |  | Sample Frequency | Max 48Khz |
|  |  |  |  |  | Channels | Max 5.1 ch |
|  |  |  |  |  | Bit rate CBR/VBR | Max 640kbps |
| PHOTO |  |  |  |  |  |  |
|  | JPEG | Jpg Jpeg |  |  | Resolution | Max. 67108864pixel (e.g. 8192×8192) *6,*7 |
|  | BMP | Bmp |  |  | Resolution | Non compression |
|  | PNG *8 | Png |  |  | Resolution | Max. 2097152pixel (e.g.2048×1024) |
|  | TIFF | tif tiff |  |  | Resolution | Max 2097152pixel (e.g. 2048x1024) |
|  |  |  |  |  | Compression | Non compression, ZIP, LZW, Packbits, Huffman, RLE, CCITT Fax3, CCITT Fax4 |
|  | GIF *9 | Gif |  |  | Resolution | Max 2097152pixel (e.g. 2048x1024) |

*1 Combination of WMV HD and WMA Pro may result in dropping frames.

*2	Depending on some WMV HD contents, noise may be generated in the audio signal when 
starting playback or during seek operation.

*3 Does not support “Complex” content

*4 WMA Lossless/Voice formats are not supported.

*5 Only support for English

*6 Support with of following coindition

-Color space is YCbCr

-non Progressive mode

-Any format of YUV420,YUV422,YUV444

*7 Other than *6 case, supported JPEG is max 983040 pixel(e.g.1280x768)

*8 Although transparent PNG file can be played, they will be displayed as non-transparent 
images.

*9 Animation GIF formats are not supported.

### Supported Audio Output Formats and Specifications

| Audio Output |  |  |
| --- | --- | --- |
| Audio Format | Analog | S/PDIF |
| AC3 | 2ch | LPCM 2ch as is |
| AAC LC | 2ch | LPCM 2ch |
| WMA | 2ch | LPCM 2ch |
| MPEG-1/2 Layer-II | 2ch | LPCM 2ch |
| MP3 | 2ch | LPCM 2ch |
| LPCM | 2ch | LPCM 2ch |

| Multichannel Audio Output |  |  |
| --- | --- | --- |
| Audio Format | Analog | S/PDIF |
| AC3 | 2ch downmix, 5.1ch | LPCM 2ch downmix, as is |
| AAC LC | 2ch downmix, 5.1ch | LPCM 2ch downmix |
| WMA | 2ch downmix, 5.1ch | LPCM 2ch downmix |
| LPCM | 2ch downmix | LPCM 2ch downmix |

| Audio L/R Switching |  |  |
| --- | --- | --- |
| VIDEO MUSIC PHOTO | L/R Switchable | LPCM |
|  |  | MPEG-1/2 Layer-II, Layer-Ⅲ |
|  |  | AAC LC 2ch |
|  |  | WMA 2ch |
|  | L/R Non-Switchable | AC3 |
|  |  | AAC LC Multichannel(4.0ch, 5.0ch, 5.1ch) |
|  |  | WMA9 Professional |

# H/W Specification

| Item |  | Detailed Description |
| --- | --- | --- |
| CPU + MPEG-1/2/4 Decoder |  | Sigma Design EM8620L |
| SDRAM for CPU |  | 64MB DDR SDRAM |
| Flash Memory |  | 8MB |
| USB 2.0 Host Controller |  | NEC Logic Upd720110 |
| Stereo Audio DAC |  | Wolfson WM8728 |
| Wireless 11b/g mPCI Module |  | Atheros 11g mPCI module |
| NIC |  | IC PLUS IP100A |
| IrDA |  | 38K NEC code |
| HDMI Transmitter |  | Silicon Image Sil164CT64 |
| Output | Video output | Composite, S-Video, HDMI |
|  | Audio output | SPDIF, Composite audio |
| LAN | Wired LAN | 802.3 100BASE-T |
|  | Wireless LAN | 801.11 b/g |
|  |  | Encription ： Non, WEP, TKIP, AES, WPA |
| USB port |  | USB 2.0 |
| TV output |  | TV specification ： NTSC, PAL |
|  |  | TV output type ： Analogue, HDMI |
|  |  | TV aspect ratio ： 4x3, 16x9 |
|  |  | TV display resolution ： Analogue /Composite ： HDMI 480i/480p/1080i/720p |
| LED |  | PowerLED |
|  |  | LAN /Wireless Access LEDs |

# Block diagram

8MB Flash

32MB SDRAM

16-bit

32-bit

64MB DDR SDRAM

EM8620L MPEG-1/2/4

PCI

I2S

WM8728 Stereo DAC

Audio-R

Audio-L

Composite Video

Pr

Pb

Y

S/PDIF Coax

Atheros 11a/b/g mPCI

WLAN

S/PDIF Fiber

S-Video

Single TXMR

LAN

TX/RX

NEC

USB 2.0 Host

USB

IC Plus

IP100A

HDMI

Silicon Image Sil9030

# Outlook

Front Panel:

Rear Panel:

Dimension: L = 425 mm /16.75 inches xW =286mm/ 11.25 inches xH =38 mm/ 1.5 inches

Weight: 2.5Kg / 5.5 lbs

| Feature | Detailed Description |
| --- | --- |
| Front Panel |  |
| Power Button | Power soft-button with built-in lighted Power LED |
| LED | Power •Off – Power Off •Solid Blue – Power On The Power LED will light solid blue when this device is powered on to indicate the ready state of the device. Indicator will be dark when this switch is powered off. LAN/Wireless Link/ACT LED •Off – No Link (LAN only) •Solid Blue – Link (LAN only) •Blinking Blue – Activity |
| USB | 1 USB 2.0 connectors |
| Remote Sensor | IR remote sensor |
| Rear Panel |  |
| Ethernet 802.3u | RJ-45 connector |
| Wireless 802.11b/g | Reverse SMA connector |
| RGB Composite connectors | a.Yellow – Video b.Red – Audio right c.White – Audio left |
| Component YPbPr connector (Supporting Progressive Scan/Interlaced Scan) | a.Blue – Pb b.Green – Y c.Red - Pr |
| S-Video Connector | 4 Pin Mini-Din Female |
| S/PDIF | 1*Optical Digital Connector 1*Coax Digital Connector |
| Reset Bottom | System reset to factory default |
| HDMI connector | For HD digital video support |
| Power cord | Attached power cord |

# Standard Compliance

- IEEE 802.3; IEEE 802.3u;

- IEEE 802.3x

- IEEE 802.11b or g

- CCIR601/CCIR656

# International Specific Test:

- EMI: FCC class B,   BSMI Class B, CE Class B, C-Tick Class B

- ESD: Air discharged 8 KV , Contact 4 KV.

- Hi-pot: 1500Vac

- Dry Heat testing

- Damp Heat cycle testing

- Cold testing

- Damp Heat cycle testing

- Safety Certification: CSA international

# Power Consumption :

- Power supply:

- 100 to 240 VAC w/ +/-10% margin

- 50/60 Hz w/ +2%, -4% margin

- EN61000-3-2 compliant

- Power consumption: 10 W

# Environment Specification:

- Operating temperature: 0 to 45 °C w/+5°C margin

- Operating humidity: 10% to 95% RH non-condensing

- Storage temperature: -25 to 60°C

- Storage humidity: 0% to 95% RH non-condensing