# D-ViewCam Software Release Note

Version: 3.0.12.02

Date: December 2, 2009

Notes for Using D-ViewCam 3.0 Application

- When accessing cameras, the system only supports the account “admin” in cameras.

- The system will not disable PTZ panel/ focus icon in UI when connecting cameras which don’t support those functions. (However, the functions are invalid.)

- Support IE-8. (Refer to Appendix A for more details.)

- Under the minimum hardware requirement, how to calculate the FPS for 32 channels with different video resolutions (Refer to Appendix B for more details.)

Device limitations:

  - DCS-5220

  - Cannot test UPnP in version 2.00.

  - DCS-3710

    - The settings of enable/ disable microphone/ speaker cannot be synchronized between the system and the camera web page. (version: 0.24.06)

    - In MPEG4 mode, the image is not acceptable in live view and playback.

 This issue can be solved in firmware version 0.24.06.

    - In MPEG4 mode, end users cannot modify option quality through Mainconsole.

 No CGI command.

  - DCS-6615

    - AutoPan is not integrated into Mainconsole.

    - PTZ speed config is not synced between Mainconsole and web page.

  - DCS-2120

    - Mirror and flip function is not integrated into Mainconsole.

    - B/W mode is not integrated into Mainconsole.

    - Variable bitrate is not integrated into Maincosnole.

| Camera Firmware tested |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model |  |  | 3420 | 5220 | 5220tw | 5300 | 5300G | 6620 | 6620G |  | 5610 | 2121 | 2102 |
| Firmware |  |  | 1.03 | 2.00 | 0200 | 1.06 | 2.03 | 1.06 | 1.06 |  | 1.01 | 1.04 | 1.04 |
| Model | 6410 | 6110 | 6111 | 1100 | 1130 | 3110 | 3415 | 7410 | 7510 | 2120 | 2120tw | 3710 |  |
| Firmware | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0201 | 0201tw | 0.24.06 |  |
| Model |  | 210-1 | 310-1 | 5230 | 6815 |  |  |  |  |  |  |  |  |
| Firmware |  | 1.5.9 | 1.5.9 | 1.01 | 1.01 |  |  |  |  |  |  |  |  |

Compatibility:

  - Windows

|  | Windows 2000 | Windows XP | Windows Vista | Server 2003 | Server 2008 | Windows 7 |
| --- | --- | --- | --- | --- | --- | --- |
| 3.0.12.02 | Supported | Supported | Supported | Supported | N/A | N/A |

  - Windows Internet Explorer Browser

|  | IE 6 | IE 7 | IE 8 |
| --- | --- | --- | --- |
| 3.0.12.02 Remote Application | Supported | Supported | Supported* |

* Refer to Appendix A for more details..

Appendix A – Windows Internet Explorer 8

In order to access the system with Windows Internet Explorer 8, we need to setup following settings to install ActiveX control.

Generally, there are two settings to make it work.

Note: Please make sure user has already installed vcredist_x86.exe

- Add the Server IP into Trusted Web Site.

  - Click Tools on IE toolbar, and then select Internet Options to start setting.

  - Switch to Security tab, select Trusted sites, and then click Sites to add server.

  - Enter IP address of the system and click on Add, then, the IP address will be listed on trust websites.

  - Back to Security tab of Internet Options window, click on Custom level… button to setup Security Settings.

Note: Please make sure that you have focus on Trusted sites section.

  - Select Low and click Reset… button to setup custom settings to Low. And then click OK to finish configuration.

- Add the Server IP Compatibility View Setting

- Click Tools on IE toolbar, and then select Compatibility View Settings to start setting.

- Enter IP address of the system and click Add, then, the IP address will be set as compatibility views.

- After setting, the compatibility button will show on browser when connecting to the system.

- If it still cannot work, try to use this step.

- Click Tools on IE toolbar, and then select Internet Options to start setting.

- Switch to Advanced tab, disable the Enable memory protection to help mitigate online attacks option of Security setting.

- Restart Internet Explorer after apply the new setting.

Appendix B – Minimum System Hardware Requirement

To ensure D-ViewCam system can maintain high video monitoring and recording performance, need to refer to the following table when establishing the system hardware.

Minimum Hardware Requirement

| S of All Cameras | 1200~600 | 600~480 | 480~240 | 240~120 | 120~ |
| --- | --- | --- | --- | --- | --- |
| CPU | Intel Core 2 Quad Q6600 | Intel Core 2 Duo E6400 | Intel Core 2 Duo E5200 | Intel Core 2 Duo E5200 | Intel Core 2 Duo E5200 |
| RAM | 2 GB | 1 GB | 1 GB | 512 MB | 512 MB |
| Motherboard | Intel G31 or above, MB vendor Asus, Gigabyte or MSI with Intel Chipset recommended |  |  |  |  |
| Display | ATi Radeon 9200 , nVIDIA GeForce FX-5200 , Intel 945G/965G or above (ATi recommended) |  |  |  |  |
| Ethernet | 100 baseT or above, Gigabit LAN recommended |  |  |  |  |
| Hard Disk | 80 GB or above |  |  |  |  |
| OS | MS Windows 2000 / XP pro sp2 / 2003 / Vista |  |  |  |  |

For example, if users apply Intel Core 2 Quad CPU with D-ViewCam software, according to the table, the total recording and live view ability of this 32 channels system will be 1200 FPS with CIF resolution. Refer to the following section for different video resolution combinations.

M: parameter of resolution of camera.

1 Mega-pixel : M=10

VGA or D1 resolution : M=5

CIF resolution : M=1

N: FPS of Camera

Users can calculate the S value to evaluate hardware requirement by the formula :

S = M * N, it means S value = M (depend on video resolution) * N (FPS of Camera)

Or users can calculate the N value to evaluate FPS for 32 channels by the formula :

N = S ÷ M, it means N (FPS of Camera) = S value ÷ M (depend on video resolution)

For example:

If users apply Intel Core 2 Quad CPU with D-ViewCam software and they want to calculate the FPS per channel, then it could be:

N (FPS per Cahnnel)= S ÷ M ÷ 32

1. FPS for CIF resolution with 32 channels = 1200 ÷ 32 ÷ 1 = 37.5 FPS/Ch

It means D-ViewCam software can record 30 FPS per channel, and it can also display 30 FPS for each channel with CIF resolution.

2. FPS for VGA or D1 resolution with 32 channels = 1200 ÷ 32 ÷ 5 = 7.5 FPS/Ch

It means D-ViewCam software can record at least 7.5 FPS per channel and display at least 7.5 FPS for each channel with VGA or D1 resolution.

3. FPS for Mega-pixel resolution with 32 channels = 1200 ÷ 32 ÷ 10 = 3.75 FPS/Ch

It means D-ViewCam software can record at least 3.75 FPS per channel and display at least 3.75 FPS for each channel with Mega-pixel resolution.