# DSN-1100-10 Firmware Release Note

Firmware: 2.7. 3.1

Hardware: A1

Date: April, 9, 2010

Enhancements:

Management Center (Java GUI) Changes

1. Faster and Smoother Display

The Java GUI has been improved to display information more quickly. It also eliminates any

flickering and other display anomalies that existed in previous versions.

2. Reorganized System Administration View

The System Administration View now displays individual tabs for all of the dialogs, rather

than using multiple “leaves” for the General Settings, System Policies, User Accounts and

Advanced Settings dialogs. In addition, the “Schedule” tab has been renamed to “Tasks”,

and the Settings tab has been renamed to “Control”.

3. Ability to Select Drives when Reconfiguring a Volume

A new dialog appears in the “Reconfigure Volume Wizard” that allows you to select

specific drives when reconfiguring a volume. This feature can also be used to make a

volume wider when new drives are added, while maintaining the same volume composition.

4. Display of System Cache Mode on the Summary Dialog

The current system cache mode of either “Write Thru” or “Write Back” now appears on

the System Summary dialog. The “Write Back” mode provides the highest system

throughput by utilizing the cache memory in the controller, and it is only available when a

fully charged battery is present.

5. Configurable Read-Ahead Cache Size

A new selection in the Volume View->Advanced Settings dialog allows you to set the

readahead cache size for optimizing performance with a large number of concurrent data

streams.

6. Optimize for Multi-Track Audio

A new selection in the Volume View->Advanced Settings dialog allows you to optimize

the read-ahead caching algorithm for multi-track audio playback on a volume-by-volume

basis.

7. Simplified Battery Policies

Reduced the number of selections on the System Administration->System Policy dialog

for the Battery Policy to include only “Ignore Battery Status” and “Stop Buffering I/O”.

8. Ability to Enable or Disable the iSCSI Network Ports

A new selection in the Network Settings->Details dialog allows you to enable or disable

each of the iSCSI network ports (support for standalone systems only)

9. Display of NAND Flash Memory Size

The size of the NAND Flash memory module now appears in the System Administration->

Controller dialog. That information is also included in the system startup information that

appears on the RS-232 Serial Diagnostic port output.

10. Synchronize Date and Time

A new button in the System Administration->Advanced Settings dialog allows you to set

the date and time of the storage array to be the same as the management host system.

Support for Volumes Larger than 2TB

When using a standard volume block size of 512 bytes, most 32-bit operating systems (such as Windows XP) can support a maximum volume size of 2TB. This version introduces an alternative volume block size of 4096 bytes that will support volumes up to 16TB. A new setting in the Volume Advanced Settings dialog allows you to specify a block size of 4096 bytes for specific volumes that require it. You can also use a new setting in the System Administration->Advanced Settings dialog to set a system-wide default block size of 4096 bytes, so that any new volumes that are created will use that new block size.

New Spare Drive and Rebuild Policies

Volume rebuilds will now look for a spare or available drive that is the same type drive and capacity as the drive that failed or was removed. If a matching drive cannot be found, the system will attempt to allocate any other similar type drive from the spare or available pool for the rebuild. If a similar drive type is not available, the system will use any spare or available drive, even if it is a different type. Finally, if there is no spare or available drive, the system will use any available capacity on any drive that is already in use. To reserve a drive as a spare, select the drive in the Physical Storage View, and click on the "Reserve as Spare" item. As always, reserving a drive as a spare will prevent it from being used when creating a new volume.

Support for SNMP Traps

In addition to generating an e-mail notification message for serious events, the system can now generate an SNMP (Simple Network Management Protocol) Trap that can be used with any third party network management application that supports SNMP version 2. The SNMP parameters (including SNMP Server IP address, SNMP Server Port Number and SNMP Community String) can be configured in the System Administration->Advanced Settings dialog. In addition, a new “Send Test Notifications” button has been added to the bottom of the Advanced Settings dialog to allow you to validate your e-mail and/or SNMP parameters.

Updated Linux Kernel

A newer version of the embedded Linux kernel has been implemented to improve the reliability and performance of the JFFS2 Flash file system in the controller.

Improved Diagnostic Capture Capabilities

The System Diagnostic Capture has been expanded to capture more detailed information about the system state, including all configuration and system logs, as well as the Serial Diagnostic Console messages. This new capability requires 256MB of NAND Flash memory on your controller. Most newer controllers are already equipped with 256MB of NAND Flash memory, but a few older controllers may only have 128MB of NAND Flash memory.

Problems Resolved:

The following additional issues have been fixed in this release:

• Recurring Fan Fault/Clear messages occur with some enclosure types

• Repeated link transitions (Link Down/Up) can result in a non-responsive data port

• System with SATA drives can bugcheck under heavy I/O load when the Java GUI is active

• Improved iSCSI session disconnect/reconnect handling

• No email notification on battery failure

• No indication on SMART data changes

• System restart occurred on iSCSI reinstate connection

• MC shows incorrect drive percentage used

• MC shows extents drive status "Blank" when reconfiguring volumes

• Changing the VLAN setting for a LAG does not show up on the Management Center

• No event log entries for a disk configured as “Spare”

============================================================

Firmware: 2.5.4.2

Hardware: A1

First release