# Pre-Product Announcement

Date: November 05,2001

To:  All Locations

# Subject: New Product Release for D-Link DP-100+;

Ethernet Print Server

D-Link is pleased to make this formal announcement for following new product. Please start to promote for this new product and advise if we can be helpful in any aspects.

PART NO.: DP100+A…A1, DP100+B…A1, DP100+E…A1, DP100+N…A1, DP100+U…A1

MODEL NO.: DP-100+/A, DP-100+/B, DP-100+/E, DP-100+/N, DP-100+/U

### About DP-100+

The DP-100+ is a compact print server which connects to Ethernet network anywhere you wish to locate printer servers. It manages the flow of print files from your workstations or file servers to its connected printers, delivering print job to high performance printers much faster than a file server or a PC acting as a print server can.

It provides two high-speed parallel print ports and one serial print port which supports 10Mbps Ethernet connection, allowing you to share the latest generation of bi-directional laser printers, color ink jets and others.

Functions and Features

- Equipped with 2 high-speed parallel ports and 1 serial port transmitting at up to 115.2Kbps

- Multiple protocol support ideal for shared printing in LAN environments

- Both parallel ports support HP PJL protocol for bi-directional network printer communication

- Supports SNMP and MIB2 (RFC 1213)

- Easy troubleshooting through five LED indicators

- Flash Memory For Easy Software Upgrade

- Web Configuration Supported

Technical Specifications

- Printer Connection

Standards: IEEE 1284 Bi-directional parallel interface, RS-232

Ports: Bi-directional 25-pin parallel port

Parallel Port Bi-directional Communication: Hewlett-Packard PJL (Printer Job Language) supported

- Network Connection

Network Standards: IEEE 802.3 10BASE2/10BASE-T Ethernet

Network Data Transfer Rate: 10Mbps

Network Connectors: BNC connector for 10 BASE2 shielded pair connection, RJ-45 connector for 10 BASE-T unshielded twisted pair connection (auto-switching)

- Network Protocols

Transport Protocols: TCP/IP, NetBEUI, AppleTalk/EtherTalk

TCP/IP Protocols Supported: BOOTP, SNMP, Telnet, TFTP, FTP, LPD, RARP, DHCP

- Management and Diagnostics

Standard: SNMP

MIBs: MIB-II (RFC 1213)

Diagnostic LED Indicators: Power/Tx, Link/Rx, LPT1, LPT2, COM

- Environmental and Physical

Power Supply: DC 5V 2.4A

Dimensions: 190mm x 116.8mm x 30.9mm

Weight: Approx. 360g

Operating Temperature: 0 to 50℃

Storage Temperature: -20 to 55℃

Humidity: 5% to 95% non-condensing

Emissions: FCC Class A, CE Class A, C-Tick Class A

Safety: UL (UL 1950), CSA(CSA 950), TUV/GS (EN 60950)

## Box Includes

- DP-100+ Ethernet/Fast Ethernet Print Server

- Power Adapter

- Stacking Bracket

- Quick Install Guide

- Print Server Software

# Ordering Information

DP-100+/A	(with USA power adapter and worldwide packaging)

DP-100+/B	(with U.K. power adapter and worldwide packaging)

DP-100+/E	(with Europe power adapter and worldwide packaging)

DP-100+/N	(with Australia power adapter and worldwide packaging)

DP-100+/U	(without power adapter in worldwide packaging)

### Release Schedule

December mid., 2001

Ordering Price

Please make reference to the Price Book from BBC

Lead Time:

About one month per order