# DP-300+ Firmware Release Note

Firmware: v3.10 Build 4

Hardware: C1

Date: April 7th, 2006

Problems Resolved:

- Fixed didn’t district user key in more than 8 characters on DP-300+ web page, since it’s not saved after press “apply”.

- Fixed configuration file can't backup the IP address and subnet mask.

- E-mail Notification-- EPSON Color 600 doesn't report 'paper out' status to print server, so print server no action. For most of printers, it will report 'paper out' status, so print server can send an e-mail notification.

Firmware: v3.10

Hardware: C1

Date: June 06, 2005

Enhancements:

- Support Password setting with maximum length up to 32 characters from Web GUI, and can be login from Web GUI with long password.
Note: Long password setting/login only support on Web GUI, PS Admin is not supported.

Firmware: v3.00

Hardware: C1

Date: Oct 19, 2004

Problems Resolved:

- Change hardware version from B1 to C1, Re-layout PCB and change CPU from RDC 8822 to RDC 2880.

- Change RAM from 512KB to 8MB, Flash from 512KB to 2MB for the new HTML image file and new features.

- Change to Middle Housing.

- Solve the lack of key component problem.

Enhancements:

- First release firmware v3.00 to fit the new hardware version.

- Web supports download firmware.

- Support UPnP, MAC Rendezvous, SMB/NBT, IPP

- Support Protocols Printing Information.

- Support Users Printing Information.

- Support Users Printing Access Control.

- Support Printer Status E-mail Notification.

- Support Configuration Backup/Restore

Notice:

- The firmware cannot be downloaded to the B1 version of Hardware.