# DP-G310 Firmware Release Note

Firmware: v1.40

Hardware: A1/A2/B1

Date: Jan 31, 2005

Problems Resolved:

When try to get IP address by “automatically assign”, disable UPnP and enable second HTTP port, this function was failed in FW v1.30, and has been fixed in this version.

Firmware: v1.30

Hardware: A1/ A2/B1

Date: Jan 28, 2005

Problems Resolved:

Fixed the ping problem in the environment (DI-624+). In the environment, Enable UPnP feature would cause problem, and this version fixes the bug.

Firmware: v1.26

Hardware: A1/ A2/B1

Date: Jan 07, 2005

Problems Resolved:

1. When user enable Adhoc mode for the wireless connection, the connection did not work stably, this version fixes the bug.

2. In some networking environment, E-mail Printing feature was failed. The version fixes the bug.

3. When disable E-mail notification, E-mail Notification Message will create the correction. The version fixes the bug.

Firmware: v1.25

Hardware: A1/ A2/B1

Date: Dec 24, 2004

Problems Resolved:

Fix these items, which were the same with DP-G321 HWB1 (Ralink wireless module)

- Wireless link Failed.

- UPnP icon would disappear

- WEP Key did not auto refresh when length change

- Disable WEP, Shared Key mode still can be selected

- IPP Printing

- SSID can type in more 32 chars

- Edit field need to limit input length and check value.

- E-mail notification Fixed.

- Default Gateway Restore

Firmware: v1.21

Hardware: A1/ A2

Date: Oct 18, 2004

Problems Resolved:

- Wireless site survey is modified as the same with DP-G321 HW B1.

- Fixed wireless link problem with TI solution AP.

- If LAN port was linked, and detect wireless link to an AP that is at the same LAN segment, then disable wireless, and wireless link status will be displayed as 'Disable'.

.

Enhancements:

- Support RALINK solution wireless module.