LPR for Windows 95

TCP/IP Printing

User’s Guide

First Edition

RECYCLABLE

P/N: 6DP100S.001

Printed in Taiwan, R.O.C.

Copyright Statement

Copyright ©1997 D-Link Corporation

No part of this publication may be reproduced in any form or by any means or used to make any derivative such as translation, transformation, or adaptation without permission from D-Link Corporation/D-Link Systems Inc., as stipulated by the United States Copyright Act of 1976.

Trademarks

D-Link is a registered trademark of D-Link Corporation/D-Link Systems, Inc.

All other trademarks belong to their respective owners.

Limited Warranty

The software and reference material are provided “as are,” without warranty as to their performance, merchantability or fitness for any particular purpose.  D-Link Corporation/D-Link Systems, Inc. reserve the right to revise this publication and to make changes to its contents at any time, without obligation to notify any person or entity of such revisions or changes.

# Установка TCP/IP печати для Windows 95

Сетевой принт-сервер D-Link может обеспечить сервис печати для сетей TCP/IP используя сетевой протокол печати lpr/lpd.  Большинство систем Unix также как и Windows NT поддерживают lpd.

Клиент LPR для печати TCP/IP фирмы D-Link предоставляет рабочим станциям Windows 95 возможность печати использовать протокол печати lpr/lpd.  Хотя существует возможность печатать через принт-сервер D-Link из Windows 95, но данная возможность важна для случаев, если администрирование сети происходит серез Unix-системы или печать с рабочей станции необходимо осуществлять толькопо протоколу IP.

## Установка LPR для клиента печати под TCP/IP

До того как рабочия станция с Windows 95 смогла печатать используя протокол lpr необходимо инсталлировать клиентстскую программу D-Link как приведено ниже:

- Убедитесь, что протокол TCP/IP инсталиирован и правильно сконфигурированы установки TCP/IP Windows 95. Если необходимо, то можно инсталлировать протокол из панели Network control.

- From the Start menu, choose the Settings submenu, then the Control Panel item within it.  Windows will display the Control Panel folder.

- Double-click on the Network icon in the Control Panel folder.

- Click on the Add... button on the Configuration tab.

- Windows will ask for the type of component to add.  Select Client and click Add... to continue.

- When Windows asks you to select a network client, click the Have Disk... button.

- Insert the LPR client diskette in your computer’s floppy drive (A: or B:), enter the proper pathname, and click OK.

- Select the LPR for TCP/IP Printing client and click OK.

- Windows will copy the necessary files, and return to the Network dialog.  Click OK to let Windows restart.

## Adding LPR Printer Ports

Once you have installed the client, and have enabled TCP/IP printing for the D-Link network print servers, then you need to add a printer port for each printer you will be printing to using the lpr/lpd protocol.  To add a printer port,

- From the Start menu, choose the Settings submenu, then the Control Panel item within it.  Windows will display the Control Panel folder.

- Double-click on the Network icon in the Control Panel folder.

- Choose the LPR for TCP/IP Printing client and click the Properties button.

- Click the Add Port... button to add a new lpd-accessible printer port.

- Enter the IP address of the D-Link print server and the name of the printer port that you will be printing to, then press OK.

- Click OK to leave the LPR for TCP/IP Printing Properties window.

## Printing from Windows 95

After you have added the necessary printer ports, you can add a printer device for printing to the port.  To enable printing to an lpr-accessible printer port from Windows 95,

- From the Start menu, choose the Settings submenu, then the Printers item within it.  Windows will display the Printers folder.

- Double-click on the Add Printer icon in the Printers folder.

- Windows will start the Add Printer Wizard.

- Click the Next > button to continue to the next screen.

- Choose the Network Printer selection and click the Next > button to continue.

- Enter the name of the port, as displayed in the LPR for TCP/IP Printing Port List.

- Alternatively, you can click Browse... and locate the printer port.  TCP/IP printing ports are located under the Entire Network icon, within the Print Servers - TCP/IP LPR icon.

- Press OK when you have located the proper printer port.

- Click Next > to continue to the next step.

- At this point Windows will ask you to choose the correct printer driver for the printer.  Choose you printer’s make and model from the list, or use the driver disk included with the printer.  When you have chosen the correct printer, click Next > to continue.

- Windows will then ask for a name for the new printer, and ask whether or not you want to set the printer as the Windows default printer.  Click Finish (or Next >) to continue.

- Windows may ask you whether or not you wish to print a test page to make sure that the printer will work correctly.  When the test page is done printing, you will be asked whether or not the test page printed.

## Setting TCP/IP Printing Settings

The lpr client’s Properties dialog allows you to change several of the client’s settings.  To change these settings,

- From the Start menu, choose the Settings submenu, then the Control Panel item within it.  Windows will display the Control Panel folder.

- Double-click on the Network icon in the Control Panel folder.

- Choose the LPR for TCP/IP Printing client and click the Properties button.

- Click on the Settings tab to access the lpr client settings.  Change the settings as necessary, and press OK to let the changes take effect.

The parameters are as follows:

- Communication Timeout  Determines how long, in seconds, the system will wait before determining that it has lost connection with lpr/lpd print servers.

- Retry Interval  Determines how long, in seconds, the system will wait before retrying a lost connection.

- Retry Times  Determines how many times the system will retry a lost connection with an lpr/lpd print server.  If Infinite is checked, it will retry indefinitely.

- Spool Path  Gives the directory path where the system will temporarily store lpr/lpd print server job files.